3GPP TSG RAN WG3 Meeting #124	R3-243016

Fukuoka, Japan, 20 - 24 May, 2024

3GPP TSG-WG SA2 Meeting #162	S2-2405535

Changsha, China, April 15th – 19th, 2024	(revision of S2-2405493, 4089)

Title:	LS OUT on RAN feedback during handover

Response to:	LS (R3-238038/S2-2403866) on RAN feedback during handover from RAN3

Release:	Release 18

Work Item:	TRS_URLLC

Source:	SA2

To:	RAN3

Cc:

Contact Person:

Name:	Haiyang, Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks RAN3 on LS on RAN feedback during handover and the hard work of TRS_URLLC-NR-Core WI.

RAN3 kindly inform SA2 that the TRS_URLLC-NR-Core WI is completed in RAN3.

RAN3 would also like to inform SA2 that RAN3 made the following decisions on the RAN feedback during handover:

- For proactive RAN feedback, the proactive parameters (including the Burst Arrival Time Window and Periodicity Range) are not signalled during handover, i.e. not provided by the source gNB to the target gNB during the Xn based handover, and are not provided by the AMF to the target gNB during the NG based handover.

- For reactive RAN feedback, the reactive parameter (i.e. Capability for BAT Adaptation) is signalled from the source gNB to the target gNB.

SA2 confirms that the proactive parameters (including the Burst Arrival Time Window and Periodicity Range) do not need to be signalled during handover considering:

- The BAT offset can be provided from NG-RAN to the SMF in the response to the QoS Flow establishment or modification request and QoS Flow modification can happen after a Handover. Thus, the Target RAN can still get the proactive parameters from the SMF after Handover, e.g., when there is requirement from AF.

-     SA2 has agreed that the SMF can update the locally stored TSCAI so that after a handover SMF provides the adjusted periodicity and BAT to the target NG-RAN node. This has no impact on the RAN.

SA2 also confirms transferring the reactive parameter (i.e., Capability for BAT Adaptation) from the source gNB to the target gNB works if it can be assumed that all RAN nodes that support Xn interface with each other support the different versions of TSCAI (Rel-16, Rel-17 or Rel-18 TSCAI) homogenously.

2. Actions:

To RAN3 group.

ACTION: 	SA2 asks RAN3 group to take the above into account and update their specifications if needed.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #163		27th – 31st May, 2024			Jeju, KR

TSG-SA2 Meeting #164		19th – 23rd August, 2024			Maastricht, NL