3GPP TSG-RAN3 Meeting # 125-bis 	R3-245043

# Hefei, China, 14th – 18th Oct. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | xxxx | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft correction to group paging conflicts for multicast reception in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  | Date: |  |  | 2024-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Previous RAN3 discussion for eMBS followed a principle that whether to enable the RRC_INACTIVE reception feature should be per gNB decision, i.e., one gNB should not tell another gNB how to do scheduling on which one, RRC_INACTIVE or RRC_CONNECTED based multicast reception. That is why RAN3 did not introduce the enhanced group paging on XnAP. For example, - assume there is at least one UE with multicast session RRC_INACTIVE reception enabled, and whose RNA covers cells belong to both gNB1 and gNB2. - the gNB1 decides to turn on this feature, upon session activation, while the neighbour gNB2 is not willing to. - only legacy group paging on XnAP will be issued, i.e., the scheduling decision from gNB1 won’t keep UE that reselected to cells under gNB2 in RRC_INACTIVE, wrongly. However, there is another case that raises our concern: - gNB1, upon session activation/data arrival, wants to follow Rel-17 scheduling and initiates a legacy (Rel-17) group paging. - while gNB2 wants to enable a Rel-18 based RRC_INACTIVE reception. - gNB2 will have to follow the legacy group paging from gNB1 via XnAP, which means UE in cells under gNB2 will be brought back to RRC_CONNECTED, even this is against gNB2’s intention. Of course gNB2 might follow its own will, when it receives Rel-17 XnAP group paging message and the session activation from 5GC, to still apply the enhanced group paging (i.e., Rel-18 version). There is no way to notify gNB1, though. gNB1, aims to wake UE up upon session activation, might try another round of group paging to gNB2. - It is questionable to whether gNB2 is smart enough to ignore the Xn group paging again. - Or gNB1 might try unicast paging, and if there is large number of UEs in RRC_INACTIVE that re-select to cells belong to gNB2, there might be trouble in gNB2. - One might say gNB1 might simply give up, e.g., releasing AS context and potentially resulting in data loss. A simple solution is to confine related UE’s RNA area to be within the scope of one gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add in stage 2 description to confine the RNA configuration: “The decision to enable data reception of MBS multicast session in RRC_INACTVE state is in a per cell granularity, by each gNB the cell belongs to. The gNB may decide to confine the RNA area of the UE that is released to RRC_INACTIVE for MBS multicast session reception to be within the scope of one gNB, to avoid the scheduling conflicts between different gNBs.” Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Scheduling conflicts between gNBs, e.g., one gNB might not be able to really enable the RRC_INACTIVE reception feature upon session activation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.10.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

// change starts.

16.10.5	Multicast Handling

16.10.5.1	Session Management

There are two delivery modes as specified in TS 23.247 [45]:

-	5GC Shared MBS traffic delivery;

-	5GC Individual MBS traffic delivery.

As specified in TS 23.247 [45], if the gNB supports MBS, the network shall use the 5GC Shared MBS traffic delivery in which case an MBS Session Resource context for a multicast session is setup in the gNB when the first UE joins the multicast session.

For 5GC Shared MBS traffic delivery mode, shared NG-U resources are used to provide MBS user data to the gNB. The gNB initiates the Multicast Distribution Setup procedure towards the 5GC, to allocate shared NG-U resources for a multicast session. In case multiple MBS session areas as specified in TS 23.247 [45] are associated with the same multicast session for location dependent MBS services, multiple NG-U shared resources are established for the same multicast session per MBS Area Session ID served by the gNB.

A shared NG-U resource applies one of the following transport options:

-	unicast transport;

-	multicast transport.

For 5GC Shared MBS traffic delivery an MBS Session Resource comprises one or several MRBs. If minimisation of data loss is applied for a given MRB, synchronisation of allocation of PDCP COUNT values is applied by either or a combination of the following methods:

-	derivation of the PDCP COUNT values by means of a DL MBS QFI Sequence Number provided on NG-U. Synchronisation in terms of MBS QoS flow to MRB mapping and PDCP SN size of the corresponding MRB among gNBs are achieved by means of network implementation.

-	deployment of a Shared NG-U Termination at NG-RAN, shared among gNBs, which comprises a common entity for assignment of PDCP COUNT values. Synchronisation in terms of MBS QoS flow to MRB mapping and PDCP SN size of the corresponding MRB among gNBs may be achieved by means of network implementation.

If PDCP COUNT values are derived from a DL MBS QFI Sequence Number provided on NG-U and only one QoS Flow is mapped to an MRB, the gNB shall set the PDCP COUNT value of PDCP PDU to the value of the DL MBS QFI Sequence Number provided with the received packet over NG-U. If PDCP COUNT values are derived from a DL MBS QFI Sequence Number provided on NG-U and multiple QoS Flows are mapped to an MRB, the gNB may derive the PDCP COUNT value of the PDCP PDU from the sum of the DL MBS QFI Sequence Numbers of the QoS Flows mapped to this MRB.

NOTE:	Synchronisation of PDCP COUNT values in case user data for MBS QoS flows mapped to the same MRB arrive over NG-U at different gNBs in different order or in case of loss of data over NG-U, and related handling of minimisation of data loss is left to implementation.

As specified in TS 23.247 [45], the gNB may receive from the 5GC MBS Assistance Information associated with a multicast MBS session for a UE, which assists the gNB in configuring the UE properly. The MBS Assistance Information indicates that the UE is expected to require dedicated resources very frequently. Based on this information, the gNB may decide the RRC state of the UE. The QoS requirements of the multicast session apply regardless of the RRC state within which the UE receives multicast session data.

The decision to enable data reception of MBS multicast session in RRC_INACTVE state is in a per cell granularity, by each gNB the cell belongs to. The gNB may decide to confine the RNA area of the UE that is released to RRC_INACTIVE for MBS multicast session reception to be within the scope of one gNB, to avoid the scheduling conflicts between different gNBs.

// change ends.