3GPP TSG-RAN WG3 Meeting #125bis		R3-245056

Hefei, China, 14th – 18th, October, 2024

Source:	CATT

Title:	(TP to BL CR for TS 38.413) Support of MBS broadcast service for NR NTN

Agenda Item:	14.2

Document for:	Discussion and Decision

Introduction

For MBS broadcast service topic, the NR NTN Phase 3 WID [1] content is as follows:

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

The last RAN3 meeting has agreed that the mapped cell ID can be used to define MBS broadcast service in NTN. And there are different views on GNSS description. In this contribution, we will provide corresponding observations and proposals.

Discussion

In the last RAN3 meeting, we’ve discussed and agreed that Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN. We understand that the mapped cell IDs and corresponding geographical areas are pre-configured/OAM configured, they’re stable. In some of the cases, using the mapped cell ID(s) may not be a proper way to describe the intended MBS service area, e.g. for some public security events, we also need to consider the other way, e.g. using GNSS description, it could accurately and precisely define the service area of MBS broadcast. To make E2E solution more efficient, it’s beneficial to align the format of MBS service area in NG interface with the MBS service area used in the NR-Uu, which is working in RAN2.

Observation 1: Mapped cells are pre-planned/pre-configured, it may not suitable to describe the intended service area, e.g. in case of some public security events.

In the previous RAN2 meetings, they discussed the definition of MBS broadcast service, and the conclusion is as follows:

The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

Observation 2: RAN2 has decided to define intended service area as a set of circles (reference location and radius) or a set of polygons.

As above, it is proposed to introduce end-to-end solution. For example, CN provides broadcast service contained by a (set of) polygon(s) to NG-RAN over NG and then NG-RAN provides it to UE over Uu.

Proposal 1: In the NG-AP, the MBS service area could be defined with the same format as NR-Uu, i.e. a (set of) reference Location and radius or by a (set of) polygon(s).

To support the GNSS description for MBS service area, corresponding TP for BLCR on TS 38.413 is provided in the section 6. RAN3 is requested to discuss and agree the TP.

Proposal 2: It is proposed to discuss and agree the TP for BLCR to TS 38.413 on support of GNSS based MBS service area description.

In Summary, MBS broadcast area to be used in NGAP for NR NTN can be defined by TAI list or NR CGI list (as legacy), Mapped cell ID list, or GNSS description. With that, it’s beneficial to send an LS to SA2, to indicate that mapped cell ID(s) and GNSS description could be used to describe the intended MBS service area in NGAP for NTN, and ask them to take it into account in their work. The draft LS is provided in appendix 5.

Proposal 3: It is proposed to send LS to SA2 on MBS broadcast service for NR NTN system and the draft LS is provided in appendix 5.

Conclusion

According to the analysis in section 2, the following observations and proposals are provided:

Observation 1: Mapped cells are pre-planned/pre-configured, it may not suitable to describe the intended service area, e.g. in case of some public security events.

Observation 2: RAN2 has decided to define intended service area as a set of circles (reference location and radius) or a set of polygons.

Proposal 1: In the NG-AP, the MBS service area could be defined with the same format as NR-Uu, i.e. a (set of) reference Location and radius or by a (set of) polygon(s).

Proposal 2: It is proposed to discuss and agree the TP for BLCR to TS 38.413 on support of GNSS based MBS service area description.

Proposal 3: It is proposed to send LS to SA2 on MBS broadcast service for NR NTN system and the draft LS is provided in appendix 5.

Reference

- RP-240775 Rel-19 Moderator WID NR NTN phase 3

- RAN3_125_agenda_20240823EOM

Draft LS to SA2

Title:	[Draft] LS on MBS broadcast service for NR NTN system

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	CATT (to be RAN3)

To:	SA2

Cc:	RAN2

Contact Person:

#### Name:	Jiancheng Sun

#### E-mail Address:	sunjiancheng@catt.cn

Attachments:	n/a

1. Overall Description:

RAN3 has discussed the MBS broadcast service for NR NTN system. RAN3 assumes that the MBS Service Area Information defined by a list of NR-CGI or a list of TAI could also be applied for NTN system.

Besides that, both mapped cell ID(s) and GNSS description could be used to describe the intended MBS service area for NTN in the NGAP, especially for the case the intended service area is small than a cell. How the GNSS description is encoded will be aligned with the NR-Uu design to be decided by RAN2, e.g. a (set of ) circle(s) or a set of a (set of) polygon(s).

2. Actions:

To SA2:

ACTION: RAN3 respectfully requests SA2 to take the above information into consideration in their future work.

3. Date of Next TSG-RAN WG3 Meetings:

RAN3#126	       Nov 18th – Nov 22nd	2024		Orlando, US

RAN3#127	       Feb. 17th – Feb. 21st  	2024		Athens, GR

TP to BLCR for TS 38.413, support of NTN MBS broadcasting

START OF CHANGES

9.3.1.209	MBS Service Area Information

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  | - | - |
| >NR CGI | M |  | 9.3.1.7 |  | - | - |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  | - | - |
| >TAI | M |  | 9.3.3.11 |  | - | - |
| GNSS Description | O |  | FFS |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is 8192. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is 1024. |

Editor’s Note: the detail definition to be aligned with RAN2.

NEXT CHANGE

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

NEXT CHANGE

id-XrDeviceWith2Rx,

id-MaximumDataBurstVolume,

id-MBS-NGUFailureIndication,

id-UserPlaneFailureIndication,

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-GNSS Description,

NEXT CHANGE

MBS-ServiceAreaInformation ::= SEQUENCE {

mBS-ServiceAreaCellList		MBS-ServiceAreaCellList																OPTIONAL,

mBS-ServiceAreaTAIList		MBS-ServiceAreaTAIList																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformation-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

ID id-GNSS Description		CRITICALITY ignore	TYPE (FFS)	PRESENCE optional，

...

}

NEXT CHANGE

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

NEXT CHANGE

id-MBS-NGUFailureIndication								ProtocolIE-ID ::= 434

id-UserPlaneFailureIndication							ProtocolIE-ID ::= 435

id-UserPlaneFailureIndicationReport						ProtocolIE-ID ::= 436

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-GNSS Description										ProtocolIE-ID ::= xxx

END OF CHANGES