3GPP TSG-RAN WG3 #125bis                                          R3-245059

Hefei, China, 14th – 18th, October, 2024

Source:	CATT

Title:	(TP to TR 38.769) Procedures for AIoT Command

Agenda Item:	16.3

Document for:	Decision

Introduction

The following agreements and FFS were captured in last RAN3 meeting:

RAN3 starts with inventory-only procedure. Discuss the content in Inventory/Command.

The AIoT CN to select suitable reader (AIoT enabled gNB or AIoT enabled UE reader) for both topology 1 and topology 2?

Assistance information from AIoT CN to AIoT RAN?

In this contribution, we will further analyze the procedures for AIoT Command, and base on the discussion, we will provide our observations and proposals accordingly.

Discussion

In last RAN3 meeting, procedures for A-IoT Inventory have been captured in TR38.769, here we continue discussing the candidate procedures for A-IoT inventory and command and command only cases based on the overall procedure in TR38.769 below:

Figure 6.3.1-1 Overall AS procedures between A-IoT device and reader

For inventory only and inventory and command procedure, following steps would be performed in AIoT radio interface.

Then, above AS procedure can support indoor inventory and indoor command use cases by the following manners:

-	For the detailed use case of “inventory-only”, it is supported by the procedure with step A and step B as baseline.

-	For the detailed use case of “inventory and command”, it is supported by the procedure with step A, step B, step C1 and step C2, as baseline.

According to above text in TS38.769, command procedure is triggered after inventory procedure at A-IoT radio interface. It may be AIoT CN or AIoT RAN to trigger command request after inventory procedure.

For AIoT CN triggered solution, the following candidate procedure for A-IoT inventory and command for Topology 1 is provided.

Figure 1: CN based A-IoT Inventory and Command in Topology 1

1. The AIoT CN sends an Inventory request message to the AIoT RAN node.

2. The AIoT RAN node sends an Inventory response message to the AIoT CN.

NOTE 1: In step 2, the AIoT RAN node may instead send an Inventory failure message to the AIoT CN Indicating that the inventory procedure could not be initiated towards the AIoT device(s).

3. The AIoT RAN node initiates inventory procedure towards the AIoT device(s) over the AIoT radio interface.

4. After receiving inventory result from the AIoT device(s), the AIoT RAN node may send Inventory report towards the AIoT CN including the received inventory result.

5. After receiving inventory result from AIoT RAN, the AIoT CN sends a Command request message to the AIoT RAN.

NOTE 2: AIoT CN may send device context setup request message which includes command request to AIoT RAN in step 5 in order to setup device context in AIoT RAN for the following command procedure.

6. The AIoT RAN node initiates Command procedure towards the AIoT device(s) over the AIoT radio interface.

7. After receiving Command result from the AIoT device(s), the AIoT RAN node may send Command response towards the AIoT CN including the received command result.

For AIoT RAN triggered solution, the following candidate procedure for A-IoT inventory and command for Topology 1 is provided.

Figure 2: RAN based A-IoT Inventory and Command in Topology 1

1. The AIoT CN sends a Command request message to the AIoT RAN node.

2. The AIoT RAN node initiates inventory procedure towards the AIoT device(s) over the AIoT radio interface.

3. After receiving inventory result from the AIoT device(s), the AIoT RAN node initiates Command procedure towards the AIoT device(s) over the AIoT radio interface.

4. After receiving Command result from the AIoT device(s), the AIoT RAN node may send Command response towards the AIoT CN including the received command result.

We slightly prefer AIoT CN triggered solution but it is finally pending to SA2.

For the procedure of A-IoT Inventory and Command for Topology 2, the following figure is provided for AIoT CN triggered solution, if RRC-based solution is used.

Figure 3: CN based A-IoT Inventory and Command in Topology 2 (if RRC-based solution is used)

1.  The AIoT CN sends an Inventory request message to the AIoT-enabled gNB.

NOTE 1: Before initiating inventory procedure, the AIoT CN may select suitable AIoT-enabled UE(s).

Editor’s Note: Whether the AIoT-enabled gNB can select AIoT-enabled UE(s) is FFS.

2. The AIoT-enabled gNB sends an Inventory response message to the AIoT CN.

NOTE 2：In step 2, the AIoT-enabled gNB may instead send an Inventory failure message to the AIoT CN indicating that the inventory procedure could not be initiated towards the AIoT device(s).

3. The AIoT-enabled gNB requests the AIoT-enabled UE(s) to trigger inventory procedure towards the AIoT device(s).

4. After receiving inventory result reported from the AIoT enabled UEs, the AIoT-enabled gNB may send Inventory reports towards the AIoT CN including the received inventory result.

5. After receiving inventory result from AIoT-enabled gNB, the AIoT CN sends a Command request message to the AIoT-enabled gNB.

6. The AIoT-enabled gNB requests the AIoT-enabled UE(s) to trigger Command procedure towards the AIoT device(s).

7. After receiving Command result from the AIoT device(s), the AIoT-enabled gNB may send Command response towards the AIoT CN including the received command result.

For the procedure of A-IoT Inventory and Command for Topology 2, the following figure is provided for AIoT CN triggered solution, if NAS/UP based based solution is used.

Figure 4: CN based A-IoT Inventory and Command in Topology 2 (if NAS/UP based solution is used)

1.  The AIoT CN sends an Inventory request message to the AIoT-enabled UE(s).

NOTE 1: Before initiating inventory procedure, the AIoT CN may select suitable AIoT-enabled UE(s).

2. The AIoT-enabled UE(s) sends an Inventory response message to the AIoT CN.

NOTE 2：In step 2, the AIoT-enabled UE(s) may instead send an Inventory failure message to the AIoT CN Indicating that the inventory procedure could not be initiated towards the AIoT device(s).

3. The AIoT-enabled UE(s) triggers inventory procedure towards the AIoT device(s).

4. After receiving inventory result from the AIoT device(s), the AIoT enabled UE(s) may send Inventory report towards the AIoT CN including the received inventory result.

5. After receiving inventory result from AIoT-enabled UE(s), the AIoT CN sends a Command request message to the AIoT-enabled UE(s).

6. The AIoT-enabled UE(s) to trigger Command procedure towards the AIoT device(s).

7. After receiving Command result from the AIoT device(s), the AIoT-enabled UE(s) may send Command response towards the AIoT CN including the received command result.

Proposal 1: It is proposed for RAN3 to discuss above procedure for A-IoT Inventory and Command case.

For command only case, the candidate solution in AIoT radio interface is as below.

-	For the detailed use case of “command-only”:

-	It can be also supported by the baseline procedure with Step A, Step B, Step C1 and Step C2.

-	In addition, another candidate to support this use case is following, whose feasibility still depends on the conclusion from [7] and [8]:

-	Step A’: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message including the command, indicating device(s) to process/respond the command.

-	Step C2: Possible D2R data transmission (e.g. the device ID or the corresponding response to command), via the A-IoT random access procedure or without using the A-IoT random access procedure.

After receiving Command request message from AIoT CN, AIoT RAN node may initiate command procedure directly via AIoT paging message or inventory and command procedure towards the AIoT device(s).

Figure 5: Message flow for A-IoT Command only in Topology 1

1: The AIoT CN sends a Command request message to the AIoT RAN node

2a: The AIoT RAN node initiates command procedure towards the AIoT device(s) over the AIoT radio interface. In step2, AIoT RAN triggers AIoT paging message including command indication (Step A) in AIoT radio interface and AIoT device sends response message to AIoT RAN (Step C2) after receiving Step A.

2b: The AIoT RAN node initiates inventory procedure towards the AIoT device(s) over the AIoT radio interface. After receiving inventory result from the AIoT device(s), the AIoT RAN node initiates Command procedure towards the AIoT device(s) over the AIoT radio interface.

3: AIoT RAN sends command response message to AIoT CN.

For Topology 2, the following figure is provided.

Figure 6: Message flow for A-IoT Command only in Topology 2 (if RRC-based solution is used)

1: The AIoT CN sends a Command request message to the AIoT-enabled gNB.

2a: The AIoT-enabled gNB requests the AIoT-enabled UE(s) to trigger command procedure towards the AIoT device(s) over the AIoT radio interface.

2b: The AIoT-enabled gNB requests the AIoT-enabled UE(s) to trigger inventory procedure towards the AIoT device(s) over the AIoT radio interface. After receiving inventory result from the AIoT-enabled UE(s), the AIoT-enabled gNB requests the AIoT-enabled UE(s) to trigger Command procedure towards the AIoT device(s) over the AIoT radio interface.

3: After receiving command result reported from the AIoT-enabled UEs, the AIoT-enabled gNB may send command response towards the AIoT CN including the received command result.

Figure 7: Message flow for A-IoT Command only in Topology 2 (if NAS/UP based solution is used)

1.  The AIoT CN sends a Command request message to the AIoT-enabled UE(s).

2a. The AIoT-enabled UE(s) triggers command procedure towards the AIoT device(s).

2b: The AIoT-enabled UE(s) triggers inventory procedure towards the AIoT device(s) over the AIoT radio interface. After receiving inventory result from the AIoT device(s), the AIoT-enabled UE(s) triggers Command procedure towards the AIoT device(s) over the AIoT radio interface.

3. After receiving command result from the AIoT device(s), the AIoT-enabled UE(s) may send Command response towards the AIoT CN including the received command result.

Proposal 2: It is proposed for RAN3 to discuss above procedure for A-IoT Command only case.

Conclusions

In this contribution, we mainly discussed the network signalling for topology 1. Based on the discussion above, the following proposals are provided:

Proposal 1: It is proposed for RAN3 to discuss above procedure for A-IoT Inventory and Command case.

Proposal 2: It is proposed for RAN3 to discuss above procedure for A-IoT Command only case.

Reference

[1] RP-240826 Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2] R3-244183 (TP for TR 38.769) A-IoT architecture aspects, CATT

TP for TR 38.769

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

- 6.5.2	Signaling and Procedures for Topology 1

- 6.5.2.1	Candidate procedures for A-IoT Inventory for Topology 1

Editor’s note 1: Future discussions on A-IoT Inventory will take place based on the following message flows, working on the content of the messages including ownership, associated functions, scope, etc.

Figure 6.5.2.1-1: Message flow for A-IoT Inventory in Topology 1

- 6.5.2.X	Candidate procedures for A-IoT Inventory and Command for Topology 1

Editor’s note 1: Future discussions on A-IoT Inventory and Command will take place based on the following message flows, working on the content of the messages including ownership, associated functions, scope, etc. Command procedure is triggered after inventory procedure at A-IoT radio interface. It may be AIoT CN or AIoT RAN to trigger command request after inventory procedure.

Figure 6.5.2.X-1: CN based A-IoT Inventory and Command in Topology 1

1. The AIoT CN sends an Inventory request message to the AIoT RAN node.

2. The AIoT RAN node sends an Inventory response message to the AIoT CN.

NOTE 1: In step 2, the AIoT RAN node may instead send an Inventory failure message to the AIoT CN Indicating that the inventory procedure could not be initiated towards the AIoT device(s).

3. The AIoT RAN node initiates inventory procedure towards the AIoT device(s) over the AIoT radio interface.

4. After receiving inventory result from the AIoT device(s), the AIoT RAN node may send Inventory report towards the AIoT CN including the received inventory result.

5. After receiving inventory result from AIoT RAN, the AIoT CN sends a Command request message to the AIoT RAN.

NOTE 2: AIoT CN may send device context setup request message which includes command request to AIoT RAN in step 5.

6. The AIoT RAN node initiates Command procedure towards the AIoT device(s) over the AIoT radio interface.

7. After receiving Command result from the AIoT device(s), the AIoT RAN node may send Command response towards the AIoT CN including the received command result.

Figure 6.5.2.X-2: RAN based A-IoT Inventory and Command in Topology 1

1. The AIoT CN sends a Command request message to the AIoT RAN node.

2. The AIoT RAN node initiates inventory procedure towards the AIoT device(s) over the AIoT radio interface.

3. After receiving inventory result from the AIoT device(s), the AIoT RAN node initiates Command procedure towards the AIoT device(s) over the AIoT radio interface.

4. After receiving Command result from the AIoT device(s), the AIoT RAN node may send Command response towards the AIoT CN including the received command result.

- 6.5.2.Y	Candidate procedures for A-IoT Command only for Topology 1

Editor’s note 1: Future discussions on A-IoT Command only will take place based on the following message flows, working on the content of the messages including ownership, associated functions, scope, etc.

Figure 6.5.2.Y-1: Message flow for A-IoT Command only in Topology 1

1: The AIoT CN sends a Command request message to the AIoT RAN node

2a: The AIoT RAN node initiates command procedure towards the AIoT device(s) over the AIoT radio interface.

2b: The AIoT RAN node initiates inventory procedure towards the AIoT device(s) over the AIoT radio interface. After receiving inventory result from the AIoT device(s), the AIoT RAN node initiates Command procedure towards the AIoT device(s) over the AIoT radio interface.

3: AIoT RAN sends command response message to AIoT CN.

- 6.5.3	Signaling and Procedures for Topology 2

- 6.5.3.1	Candidate procedures for A-IoT Inventory for Topology 2

Editor’s note 1: Future discussions on A-IoT Inventory will take place based on the following message flows, working on the content of the messages including ownership, associated functions, scope, etc.

Figure 6.5.3.1-1: Message flow for A-IoT Inventory in Topology 2 (if RRC-based solution is used)

Figure 6.5.3.1-2: Message flow for A-IoT Inventory in Topology 2 (if NAS/UP based solution is used)

Editor’s note 2: how and where to depict signalling suitable for triggering A-IoT RAN node functions for A-IoT radio resource management needs further discussions for direct communication between A-IoT CN and A-IoT-enabled UE.

- 6.5.3.X	Candidate procedures for A-IoT Inventory and Command for Topology 2

Editor’s note 1: Future discussions on A-IoT Inventory and Command will take place based on the following message flows, working on the content of the messages including ownership, associated functions, scope, etc.

In Topology 2, to support A-IoT Inventory and Command solution, AIoT CN or AIoT RAN could trigger command request after inventory procedure which is the same as topology 1. The procedures for CN based solution are provided below:

Figure 6.5.3.X-1: CN based A-IoT Inventory and Command in Topology 2 (if RRC-based solution is used)

1.  The AIoT CN sends an Inventory request message to the AIoT-enabled gNB.

NOTE 1: Before initiating inventory procedure, the AIoT CN may select suitable AIoT-enabled UE(s).

Editor’s Note: Whether the AIoT-enabled gNB can select AIoT-enabled UE(s) is FFS.

2. The AIoT-enabled gNB sends an Inventory response message to the AIoT CN.

NOTE 2：In step 2, the AIoT-enabled gNB may instead send an Inventory failure message to the AIoT CN indicating that the inventory procedure could not be initiated towards the AIoT device(s).

3. The AIoT-enabled gNB requests the AIoT-enabled UE(s) to trigger inventory procedure towards the AIoT device(s).

4. After receiving inventory result reported from the AIoT enabled UEs, the AIoT-enabled gNB may send Inventory reports towards the AIoT CN including the received inventory result.

5. After receiving inventory result from AIoT-enabled gNB, the AIoT CN sends a Command request message to the AIoT-enabled gNB.

6. The AIoT-enabled gNB requests the AIoT-enabled UE(s) to trigger Command procedure towards the AIoT device(s).

7. After receiving Command result from the AIoT device(s), the AIoT-enabled gNB may send Command response towards the AIoT CN including the received command result.

Figure 6.5.3.X-2: CN based A-IoT Inventory and Command in Topology 2 (if NAS/UP based solution is used)

1.  The AIoT CN sends an Inventory request message to the AIoT-enabled UE(s).

NOTE 1: Before initiating inventory procedure, the AIoT CN may select suitable AIoT-enabled UE(s).

2. The AIoT-enabled UE(s) sends an Inventory response message to the AIoT CN.

NOTE 2：In step 2, the AIoT-enabled UE(s) may instead send an Inventory failure message to the AIoT CN Indicating that the inventory procedure could not be initiated towards the AIoT device(s).

3. The AIoT-enabled UE(s) triggers inventory procedure towards the AIoT device(s).

4. After receiving inventory result from the AIoT device(s), the AIoT enabled UE(s) may send Inventory report towards the AIoT CN including the received inventory result.

5. After receiving inventory result from AIoT-enabled UE(s), the AIoT CN sends a Command request message to the AIoT-enabled UE(s).

6. The AIoT-enabled UE(s) to trigger Command procedure towards the AIoT device(s).

7. After receiving Command result from the AIoT device(s), the AIoT-enabled UE(s) may send Command response towards the AIoT CN including the received command result.

- 6.5.3.Y	Candidate procedures for A-IoT Command only for Topology 2

Editor’s note 1: Future discussions on A-IoT command only will take place based on the following message flows, working on the content of the messages including ownership, associated functions, scope, etc.

Figure 6.5.3.Y-1: A-IoT Command only in Topology 2 (if RRC-based solution is used)

1: The AIoT CN sends a Command request message to the AIoT-enabled gNB.

2a: The AIoT-enabled gNB requests the AIoT-enabled UE(s) to trigger command procedure towards the AIoT device(s) over the AIoT radio interface.

2b: The AIoT-enabled gNB requests the AIoT-enabled UE(s) to trigger inventory procedure towards the AIoT device(s) over the AIoT radio interface. After receiving Inventory result from the AIoT-enabled UE(s), the AIoT-enabled gNB requests the AIoT-enabled UE(s) to trigger Command procedure towards the AIoT device(s) over the AIoT radio interface.

3: After receiving command result reported from the AIoT-enabled UEs, the AIoT-enabled gNB may send command response towards the AIoT CN including the received command result.

Figure 6.5.3.Y-2: A-IoT Command only in Topology 2 (if NAS/UP based solution is used)

1.  The AIoT CN sends a Command request message to the AIoT-enabled UE(s).

2a. The AIoT-enabled UE(s) triggers command procedure towards the AIoT device(s).

2b: The AIoT-enabled UE(s) triggers inventory procedure towards the AIoT device(s) over the AIoT radio interface. After receiving inventory result from the AIoT device(s), the AIoT-enabled UE(s) triggers Command procedure towards the AIoT device(s) over the AIoT radio interface.

3. After receiving command result from the AIoT device(s), the AIoT-enabled UE(s) may send Command response towards the AIoT CN including the received command result.

----------------End of the Changes----------------