3GPP TSG-RAN WG3 Meeting #125-bis	R3-245074

Hefei , CN, 14 - 18 Oct, 2024

Title:	(TP to TR 38.769) A-IoT Device Locating

Agenda Item:	16.4

Source:	Huawei

Document for:	other

# 1	Introduction

In RAN3#125 meeting, regarding to A-IoT Device Locating, there are four Editor’s Notes needs to be solved:

Editor’s Note 1:	How to know the “reader” location is FFS. Whether to use more than one “readers” within one A-IoT RAN for location purposes is FFS.

Editor’s Note 2:	Signalling details on how to provide the location information of the A-IoT device to the A-IoT CN needs further study.

Editor’s Note 3:	Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4:	Analysing the gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

In this paper, we provide our further analyses on Ambient IoT device locating, try to solve these FFSs, and provide the corresponding TP to the TR38.769 in section 5.

# 2	Discussion

## 2.1	How to report the location information?

In last meeting, it was agreed that A-IoT RAN node ID can be considered as a location of A-IoT device in topology 1, and UE ID of the A-IoT-enabled UE can be considered as a location of A-IoT device in topology 2, but signalling details on how to provide the location information of the A-IoT device to the A-IoT CN needs further study.

Editor’s Note 2:	Signalling details on how to provide the location information of the A-IoT device to the A-IoT CN needs further study.

In Topology 1, during the setup of XX interface, the A-IoT CN will get the A-IoT RAN node ID of the A-IoT RAN, afterwards, during the A-IoT services related procedures after XX setup procedure, the A-IoT CN knows which A-IoT RAN it’s communicating with, the A-IoT RAN does not need to report the A-IoT RAN node ID to the A-IoT CN during these A-IoT services related procedures.

Proposal 1: In topology 1, the A-IoT RAN informs the A-IoT RAN node ID to the A-IoT CN during the setup of XX interface.

Considering that an A-IoT RAN node may support multiple TRPs, each TRP within the A-IoT RAN node can be a RAN reader, and thus an A-IoT RAN node can be regarded as multiple RAN Readers, then during the Inventory or the command after inventory procedure, the A-IoT RAN node can include TRP ID within the Inventory Report message towards the A-IoT CN.

Proposal 2: In topology 1, the A-IoT RAN may include TRP ID within the Inventory Report message towards the A-IoT CN.

In Topology 2 RRC based solution, considering that the A-IoT-enabled gNB will forward the information received from A-IoT-enabled UE to the A-IoT CN, the most straight forward way to report the UE ID of the A-IoT-enabled UE is to include it within the Inventory Report message.

In Topology 2 NAS/UP based solutions, there is no need for the A-IoT-enabled gNB to provide any location information to the A-IoT CN, as the A-IoT procedures terminated between the A-IoT-enabled UE and the A-IoT CN directly, CN knows which A-IoT-enabled UE the Ambient IoT device accessing from.

Proposal 3: In topology 2 RRC based solution, the A-IoT-enabled gNB includes the Ambient IoT device location information (UE ID of the A-IoT-enabled UE) within the Inventory Report message.

Proposal 4: In topology 2 NAS/UP based solutions, the A-IoT CN knows which A-IoT-enabled UE the device comes from by default.

## 2.2	How to know the “reader” location?

Another Editor’s note 1 in [1] includes two FFSs, the first one is how to know the “reader” location.

Editor’s Note 1:	How to know the “reader” location is FFS. Whether to use more than one “readers” within one A-IoT RAN for location purposes is FFS.

In Topology 1, considering that the A-IoT RAN does not move, there is no need to update the its location to the A-IoT CN frequently. Therefore, the A-IoT CN can get the A-IoT RAN’s location (may with per TRP information) based on e.g., OAM configuration, or A-IoT XX Setup procedure.

Proposal 5: In topology 1, the A-IoT CN can get the A-IoT RAN’s location (may with per TRP information) based on e.g., OAM configuration.

In Topology 2, as we mentioned above, the A-IoT CN is able to get the A-IoT-enabled UE’s location by existing mechanisms, there is no need to introduce new method to enable this in this study.

Proposal 6: In topology 2, the A-IoT CN is able to get the A-IoT-enabled UE’s location by existing mechanisms.

## 2.3	Finer granularity?

In Editor’s Note 1, there is also another FFS about whether to use more than one “readers” within one A-IoT RAN for location purposes is FFS:

Editor’s Note 1:	How to know the “reader” location is FFS. Whether to use more than one “readers” within one A-IoT RAN for location purposes is FFS.

based on the offline discussion in last meeting [3], companies agree with the moderator’s proposal 4 to not consider the utilization of more than one “reader” for location purposes in this release, especially such complex solution inlvolvs multiple WGs.

On the other hand, there are two newly added Editor’s notes related to finer granularity:

Editor’s Note 3:	Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4:	Analysing the gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

In the SID [2], the device positiong/locating related objectives are copied as below:

Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). - RAN3-led: - … - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network.

For Editor’s Note 3, in last meeting, it is the common understanding that there is no need to consider this in this release or RAN3 could not decide this in the Study. From our view there is no need to discuss this in the study, as we need to figure out the basic solutions first.

For Editor’s Note 4, it is clear that the requirement from the SID is to identify solutions with no or minimal specification impact, and it is noted that the existing user location report is in per cell per TAI granularity, in the discussion so far, RAN3 concluded to support reader id granularity, and the related solutions does not have device impact, therefore, we do not see the gap mentioned in the Editor’s note 4.

Proposal 7: “Use more than one “readers” within one A-IoT RAN for location purposes”, and “Introduce finer granularity for locating an Ambient device besides the reader ID granularity” are not considered in this release.

Proposal 8: Remove the Editor’s Notes 1~4.

# 3	Proposals

In this contribution, we discuss Ambient IoT device locating, and get the following proposals:

Proposal 1: In topology 1, the A-IoT RAN informs the A-IoT RAN node ID to the A-IoT CN during the setup of XX interface.

Proposal 2: In topology 1, the A-IoT RAN may include TRP ID within the Inventory Report message towards the A-IoT CN.

Proposal 3: In topology 2 RRC based solution, the A-IoT-enabled gNB includes the Ambient IoT device location information (UE ID of the A-IoT-enabled UE) within the Inventory Report message.

Proposal 4: In topology 2 NAS/UP based solutions, the A-IoT CN knows which A-IoT-enabled UE the device comes from by default.

Proposal 5: In topology 1, the A-IoT CN can get the A-IoT RAN’s location (may with per TRP information) based on e.g., OAM configuration.

Proposal 6: In topology 2, the A-IoT CN is able to get the A-IoT-enabled UE’s location by existing mechanisms.

Proposal 7: “Use more than one “readers” within one A-IoT RAN for location purposes”, and “Introduce finer granularity for locating an Ambient device besides the reader ID granularity” are not considered in this release.

Proposal 8: Remove the Editor’s Notes 1~4.

It is also proposed to capture the corresponding TP provided in section 5 into the TR 38.769.

# 4	Reference

[1] 3GPP TR 38.769 Study on solutions for ambient IoT (Internet of Things) (Release 19) V1.0.0 (2024-09)

[2] RP-240826 Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA

[3] R3-244705 Summary of Offline Discussion – CB: # AIoT3_Location, CMCC

# 5	Text Proposal for TR 38.769

------------Start of the Change-------------

## 6.9	Locating ambient IoT devices

### 6.9.x	General

A-IoT device location information may be used for the following purposes:

(a) improving the A-IoT operation itself, e.g. by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the A-IoT service.

Locating an A-IoT device at “reader ID granularity” is useful for both purposes.

For topology 1:

- A-IoT RAN node ID can be considered as a location of A-IoT device.

- The A-IoT RAN informs the A-IoT RAN node ID to the A-IoT CN during the setup of XX interface.

- The A-IoT RAN may report TRP ID to the A-IoT CN during the Inventory Report.

- The A-IoT CN gets the A-IoT RAN’s location (may with per TRP information) based on e.g., OAM configuration.

For topology 2:

- UE ID of the A-IoT-enabled UE can be considered as a location of A-IoT device.

- In RRC based solution, the A-IoT-enabled gNB includes the A-IoT device location information (UE ID of the A-IoT-enabled UE) within the Inventory Report message.

- In NAS/UP based solutions, the A-IoT CN knows which A-IoT-enabled UE the device comes from by default.

- The A-IoT CN gets the A-IoT-enabled UE’s location by existing mechanisms.

-----------End of the Change-------------