3GPP TSG-RAN WG3 Meeting #125bis	R3-245082

Hefei, China, 14th – 18th October 2024

Agenda item:	16.3

Source:	Xiaomi

Title:	(TP to BL CR for TR 38.769) AIoT interface impacts between RAN and CN

Document for:	Other

1	Introduction

In this contribution, we’d like to discuss the interface impacts according to the discussion in RAN3 125 meeting.

With regard the RAN and CN interface impacts, following topics will be discussed in this Agenda item:

For topology 1

- Information Inventory request

- Device info transfer and device context

- Command operation

For topology 2

- UE selection and AIoT resource allocation

2	Discussion

## Stage2 procedure for topology 1

The proposed procedure for Topology 1 includes the stage2 inventory and command procedures. In addition to these procedures, we suggest introducing a reader information exchange procedure between AIoT NF and AIoT RAN. This will enable the AIoT NF to effectively select readers for specific inventory tasks, taking into account factors such as inventory areas and reader capabilities.

In step 0, the AIoT NF requests AIoT RAN about the reader information, as one based station may have multiple readers deployed in different places and have different AIoT capabilities and resources, the AIoT RAN may response a list of readers and corresponding reader capabilities.

In step 1, the AIoT NF sends an AIoT inventory request to the AIoT RAN along with a specific task ID that uniquely identifies an AIoT inventory operation.

In step 2, as the inventory process may take a considerable amount of time, the AIoT RAN node can transmit the inventory response to AIoT NF in order to indicate that the inventory operation has been executed successfully.

In step 3, acting as the reader, the AIoT RAN initiates the inventory procedure with one or more devices. The details of the handshake are based on discussions in RAN1/2. During or after the handshake, the AIoT device(s) may transmit its UL AIoT device information to the AIoT RAN.

In step 4, along with task ID, the AIoT RAN node transfers one or more UL AIoT device information to the AIoT AMF. If it is an inventory + command operation, then a specific RAN device ID can be generated by the AIoT RAN node for each device and included in any messages associated with that particular device.

In step 5, if further command operation is needed for the specific device, the AIoT NF sends the AIoT command request to the AIoT RAN, along with the RAN specific device ID, AIoT command and DL AIoT device info (if any).

In step 6, the AIoT RAN node performs the AIoT command operation, if any, it may transmit the DL AIoT device to the AIoT device, and/or receive the UL AIoT device info from the AIoT device. Considering that the command operation is aimed for a specific device, the response message can be sent after the AIoT command operation over Uu.

In step7, the AIoT RAN node send the AIoT command response to the AIoT AMF, along with the RAN specific device ID, AIoT command and UL AIoT device info (if any).

Figure 1 stage 2 procedure for AIoT operation in T1

Observation 1, it’s useful to introduce a reader information procedure for effective reader selection.

Proposal 1, introduce a new class 1 message (e.g., Reader Information Exchange) between AIoT RAN and AIoT NF for AIoT reader selection.

## Reader information

The following information can be considered as reader information provided to the AIoT NF for reader selection.

| Reader information | Analysis |
| --- | --- |
| Reader ID | The reader ID can be RAN node ID and TRP ID, which can be further discussed in the topic of locating AIoT device. |
| Location information of the reader | The location information of a reader can be utilized for reader selection in AIoT NF, taking into account the geographical data associated with a specific AIoT service request. |
| AIoT resource information of the reader | The frequency, timing, and power information of the reader can provide valuable support for AIoT operations when selecting readers in AIoT NF based on QoS and device types for specific AIoT service |

Proposal 2, The following information can be provided as reader selection criteria from AIoT RAN to AIoT NF, taking into account the AIoT service request:

- Reader ID

- Location information of the reader

- AIoT resource information of the reader

## Inventory request

In RAN2#127 meeting, RAN2 achieved the following agreements for A-IOT functions.

Agreements

-	Send an LS to RAN1 regarding TBS size. Ask SA2 on the max and typical upper layer packet size.

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

-	RAN2 will study the need and use of a simple message “size” reporting to the reader

-	At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

-	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

The following information is considered useful to be visible to the AIoT RAN from CN from RAN2 perspective.

•	The service type of A-IoT (e.g. inventory, command)

•	Targeted for one or more than one devices

•	Approximate number of target devices (if available)

And RAN3 agreed that the AIoT Device Identification and Scope of inventory request can be included in inventory request.

The following are some analysis for information in the inventory request message:

| Information in inventory request message | Analysis |
| --- | --- |
| AIoT Device Identification (to find a single device, a group of devices, or all devices) | This identification can be MASK info, which will be further transfer to device, so that device can decide whether to response the inventory request. |
| Scope of inventory request (e.g., a certain area in which the inventory is to be triggered) | We think this area information can be used for gNB to decide whether a reader needs to perform this inventory request. This area information can be list of TRPs, a list of cells, etc. |
| service type of A-IoT (e.g. inventory, command) | Agreed by RAN2, could be valuable for a reader in initiating CFRA or CBRA to allocate dedicated/shared resources for subsequent D2R transmission. In addition, this information can also be useful for the AIoT RAN to decide whether to keep the device context for subsequent AIoT operations. |
| Targeted for one or more than one device | Agreed by RAN2, can be utilized by readers to determine the type of RA and schedule the resource. |
| Estimated Number of devices (Approximate number of target devices) | Agreed by RAN2. This may help reader to allocate radio resources, to perform re-triggering or to set Q value, but the above can also be done by reader’s implementation. E.g. the reader can know from the identification info whether a single or multiple device are expected, the reader can set Q and re-trigger inventory by knowing the Collision over AIoT radio. This info may help to reduce the processing complexity of the reader. |
| Inventory Session/Task ID | gNB may receive multiple inventory requests from AIoT NF(s), similar to positioning, there’s transaction ID to identify a specific procedure. |
| Periodicity | It is possible that the inventory process is periodic |
| The type of the ID included in the A-IOT paging message (e.g., group ID/no ID or independent device ID) | RAN2 agrees to study the following cases for AIoT paging message: • a message containing an ID of a single A-IoT device. • a message containing a group ID that maps to multiple A-IoT devices. • a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. • a message containing multiple IDs of A-IoT devices. the information of one or more than one device is used by the reader to determine the type of random access, e.g., CFRA or CBRA. However, this information alone is not sufficient if more than one device is targeted. This is because these three cases are all targeted at multiple devices, however if the group ID is contained or no ID is contained, reader can only trigger CBRA while if multiple independent IDs are contained, reader can either trigger CFRA or CBRA. |
| QoS requirement (e.g., E2E latency) | In TR 38.848, it indicates the latency requirement for A-IOT, which is E2E latency between the reader and the device. the reader should also be able to derive the latency request from the service request from CN so that the reader can execute the related AS procedure according to the latency requirement |

Proposal 3, the AIoT Device Identification in inventory request message can be MASK info that will finally be sent to device.

Proposal 4, Scope of inventory request in the inventory request message can be list of TRPs.

Proposal 5, the following information can be introduced in inventory request from AIoT NF to AIoT RAN

- Service type of A-IoT (e.g. inventory, command)

- Targeted for one or more than one device

- Estimated Number of devices (Approximate number of target devices)

- Inventory Session/Task ID

- Periodicity

- Type of the ID included in the A-IOT paging message (e.g., group ID/no ID or independent device ID)

- QoS requirement (e.g., E2E latency)

## Inventory report

As there may be more than one AIoT operations for a AIoT device, e.g., inventory and command operation, in this case, it’s beneficial to introduce a RAN specific Device ID for the device when the AIoT RAN receives the response from a reader. the RAN specific device ID (similar to NGAP UE ID) is used to identify a specific device within its own when receiving responses from reader

Proposal 6, AIoT RAN can generate a specific RAN level device ID for each inventoried device and include it in the associated inventory report.

## Command request

For command operation, the following information is useful for command request:

| Information in command request message | Analysis |
| --- | --- |
| RAN specific device ID | This ID can be used for the reader to know which device is targeted to, which means the command procedure can be device-associated. |
| command type (e.g., read/write/disable) | this information is useful since the response for different command is of different size, with this information, the reader is able to schedule the following D2R resource accordingly e.g., relatively larger resource for response of read command than write command. In addition, this is aligned with RFID where the command type is visible to reader. |

Proposal 7, the following information can be introduced in command request from AIoT NF to AIoT RAN:

- RAN specific device ID

- Command type (e.g., read/write/disable)

## UE reader selection and resource allocation in topology 2

For AIoT resource allocation for a UE reader, the AIoT-enabled gNB must verify if the UE is authorized as a UE reader to allocate AIoT radio resources. Similar to other authorizations (e.g., SL, SL positioning), the AMF sends the UE’s authorization information via UE context-related messages to the AIoT-enabled gNB, which stores this information as part of the UE context. This authorization information is transferred during UE handover and UE context retrieval procedures.

Proposal 8, RAN3 agrees that the AIoT authorization information for the UE reader should be sent from the serving AMF to the AIoT-enabled gNB in UE context-related messages and stored as part of the UE context in the AIoT-enabled gNB.

Regarding UE reader selection, most companies think the AIoT NF is responsible for UE reader selection, we can make this as working assumption, and then discuss the details for different signalling solutions, although we think there may be some potential issues if no serving gNB involvement.

For RRC-based solution, the AIoT NF can sends the AIoT operation info to the AIoT-enabled gNB of the selected UE reader via AMF, then the AIoT-enabled gNB sends the AIoT operation info to UE reader via RRC signalling, the AIoT-enabled gNB is aware of the AIoT operation for the UE reader, it can allocate AIoT radio resources directly.

Figure 2 UE selection and resource allocation based RRC-based solution in T2

For NAS -based solution, the AIoT NF can sends the AIoT operation info to UE via NAS, in addition, the serving AMF may include the AIoT resource request in the NAS transfer message, so that the AIoT-enabled gNB can allocated resources to the UE as shown in step 1, 2a and 3a. An alternative way is that after UE reader receives the AIoT info from the serving AMF, it sends the AIoT resource request to the AIoT-enabled gNB, and then the AIoT-enabled gNB allocate resources for it, as shown in step 2b and 3b.

Figure 3 UE selection and resource allocationfor NAS-based solution in T2

For UP-based solution, the only simple way is the UE sends the AIoT resource request to the gNB after it receives the AIoT info via PDU session, and then the gNB allocate resources for it, as shown in step 2b and 3b.

Proposal 9, RAN3 discuss the different impacts on UE reader selection for different signalling solutions.

In the Annex, we provided a TP for TR 38.769 to reflect the potential RAN-CN interface impacts for topology 1 and topology 2.

Proposal 10, RAN3 agrees the TP for TR 38.769 in the Annex.

3	Conclusion

In this contribution, we discussed the RAN-CN interface impacts to support AIoT in topology 1 and topology 2, and have the following observations and proposals.

## Reader information exchange

Observation 1, it’s useful to introduce a reader information procedure for effective reader selection.

Proposal 1, Introduce a new class 1 message (e.g., Reader Information Exchange) between AIoT RAN and AIoT NF for AIoT reader selection.

Proposal 2, The following information can be provided as reader selection criteria from AIoT RAN to AIoT NF, taking into account the AIoT service request:

- Reader ID

- Location information of the reader

- AIoT resource information of the reader

## Inventory request

Proposal 3, the AIoT Device Identification in inventory request message can be MASK info that will finally be sent to device.

Proposal 4, Scope of inventory request in the inventory request message can be list of TRPs.

Proposal 5, the following information can be introduced in inventory request from AIoT NF to AIoT RAN

- Service type of A-IoT (e.g. inventory, command)

- Targeted for one or more than one device

- Estimated Number of devices (Approximate number of target devices)

- Inventory Session/Task ID

- Periodicity

- Type of the ID included in the A-IOT.

## Inventory report

Proposal 6, AIoT RAN can generate a specific RAN level device ID for each inventoried device and include it in the associated inventory report.

## Command request

Proposal 7, the following information can be introduced in command request from AIoT NF to AIoT RAN:

- RAN specific device ID

- Command type (e.g., read/write/disable)

## UE reader selection and resource allocation in topology 2

Proposal 8, RAN3 agrees that the AIoT authorization information for the UE reader should be sent from the serving AMF to the AIoT-enabled gNB in UE context-related messages and stored as part of the UE context in the AIoT-enabled gNB.

Proposal 9, RAN3 discuss the different impacts on UE reader selection for different signalling solutions

Proposal 10, RAN3 agrees the TP for TR 38.769 in the Annex.

4	Reference

[1]	R3-244838, (TP for TR38.769) NG interface impact for Ambient-IoT

5	Annex (TP to BL CR for TR 38.769)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

6.3.1.x1	Reader Information Exchange

In Topology 1, the Reader Information Exchange procedure is to allow the AIoT NF to request the AIoT RAN to provide detailed information for reader(s) hosted by the AIoT RAN so that the AIoT NF can perform the reader selection, taking account the AIoT service request. The AIoT NF initiates the procedure by sending a Reader Information Request message and the AIoT RAN responds with a Reader Information Request message that contains the requested reader information.

6.3.1.x2	UE reader authorization

The AIoT-enabled gNB of UE receives the UE reader’s AIoT authorized information from the serving AMF of UE in initial UE context setup procedure, and it may also receive the updated authorized information from the AMF in UE context modification procedure, path switch procedure. The AIoT authorized information can be used by AIoT-enabled gNB for UE reader selection (FFS) and AIoT radio resource allocation.

6.3.1.x3	UE reader selection and AIoT resource allocation

In case of UE selection in AMF and RRC-based signalling is used, the AIoT NF notifies the AIoT-enabled gNB of UE reader the AIoT operation information for the UE reader, the AIoT-enabled gNB can allocate AIoT radio resources accordingly and configured to UE reader.

Figure 6.3.1.4-1: UE reader selection for RRC-based solution.

In case of UE selection in AMF and NAS/UP-based signalling is used, either AIoT NF or UE reader sends AIoT radio resource request to the AIoT-enabled gNB, the AIoT-enabled gNB can allocate AIoT radio resources accordingly and configured to UE reader.

Figure 6.3.1.4-2: UE Reader selection for NAS-based solution.

In case of UE selection in gNB, AIoT NF sends AIoT operation request to gNB, and gNB performs the UE selection and allocate AIoT radio resources accordingly and configured to UE reader.

Figure 6.3.1.4-3: UE Reader selection for UP-based solution.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>