3GPP TSG-RAN WG3 Meeting #125 bis	             R3-245108

Hefei, China, 14th – 18th Oct 2024

Agenda Item:	20.2

Source:	CMCC

Title:	Support AI/ML Assisted Positioning

Document for:	Discussion

Introduction

In the RAN #105 meeting, the following WID relevant to RAN3 work item of AI/ML for positioning has been reached [1]:

[WID] - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Discussions on AI/ML Assisted Positioning

AI/ML-assisted positioning utilizes AI/ML models to produce intermediate measurement statistics that play a critical role in determining location. These statistics can include metrics such as the probability of a signal being Line-of-Sight (LOS) or Non-Line-of-Sight (NLOS), the angle of arrival (AoA) or departure (AoD) of signals, and time-of-arrival data (ToA). These measurements are essential for refining positioning accuracy, especially in complex environments.

The AI/ML assisted positioning has the following cases that have been discussed and prioritized in other RAN WGs.

- (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

Case 3a (1st Priority)

The Case 3a is illustrated in Figure 1.

Figure 1: Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

Model Training

For Case 3a, the training samples are preferable to be generated from the PRUs. The NG-RAN node measures SRS, with the legacy configuration of SRS to be adopted for obtaining measurements for training.

The samples from the PRUs should be prioritized for label generation for model training and updating, which could be applied to all cases.

For gNB-side model, the measurement data can be generated by the gNB, while the labels can be generated by OAM or LMF. RAN1 116 Bis meeting has the following agreement [3]:

Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

Figure 2 shows the procedure of model training and updating in Case 3a.

Figure 2: Case 3a model training, transfer of measurements and labels from PRU/UE to UE-side training entity

For Case 3a, the labels and their related data (e.g., time stamp) can be transferred from the LMF to the NG-RAN node.

Model Inference

For Case 3a, the UE’s position is determined by the LMF based on the assisting information, which is the inference output from the model trained and reported by the NG-RAN Node, as shown in Figure 3. Same as the model training, measurements at the NG-RAN Node based on SRS are used as model input for the inference.

Figure 3: Case 3a model inference

For Case 3a, the inference output as assisting information needs to be sent from the NG-RAN node to the LMF, which are used by the LMF to calculate the UE’s position by legacy approaches.

Model Monitoring

For Case 3 model monitoring, RAN1 116 Bis meeting has the following agreement [3]:

Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation

RAN1 118 meeting further discussed on it and has the following agreement [4]:TR

Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

According to RAN 1, that both options are possible. RAN 3 should discuss which option to be adopted. The following we provide some discussion for both options:

Option A: NG-RAN node performs monitoring metric calculation for its own model

In Case 3a, both the NG-RAN Node and the LMF are involved in deriving the monitoring metric as outlined in TR 38.843 [2]. With the model being trained and maintained by the NG-RAN Node, the LMF does not have visibility into the model. Therefore, the potential monitoring method for Case 3a could be: the NG-RAN Node calculates the metric and makes the monitoring decision independently. In this case, the metric used by the NG-RAN Node could vary based on the implementation.

On the other hand, the LMF is the entity that gathers data from multiple NG-RAN nodes and calculates the UE’s position in the final step. Therefore, it may be advantageous for the LMF to provide assistance to the NG-RAN Node in calculating the monitoring metric or making the monitoring decision. What assistance information could be provided from the LMF to the NG-RAN node is for further discussion.

For Case 3a, the NG-RAN Node could be the entity that calculates the monitoring metric and making monitoring decision.

For Case 3a, the LMF can provide assistance information for the monitoring metric calculation and monitoring decision making to the NG-RAN Node if Option A is adopted.

Option B: LMF performs monitoring metric calculation for the model located at the NG-RAN node

In Case 3a, if the LMF performs monitoring metric calculation for the model located at the NG-RAN Node, given that the LMF only collects all the inference output from relevant NG-RAN Nodes and calculates the UE’s position, the LMF does not have the information of the original input of the AI model located at the NG-RAN Node. Therefore, the selection of monitoring metrics and the decision-making process at the LMF will require support from the NG-RAN Node. The specific assistance information that needs to be transferred from the NG-RAN Node to the LMF remains for further discussion.

For Case 3a, if Option B is adopted for model monitoring, the NG-RAN Node shall provide assistance information to the LMF for monitoring metric selection, calculation and monitoring decision making.

Support of AI-based positioning in F1

If the CU/DU split architecture is considered for Case 3a, RAN 3 should discuss and determine where the AI/ML model should be located—whether in the CU, in the DU, or be distributed across both entities. The decision should consider multiple factors, such as processing capacity, latency requirements, and the specific functions performed by each unit.

In order to support Case 3a in the CU/DU split architecture, RAN 3 should discuss and determine where the AI model is located and possible interactions between gNB-CU and gNG-DU.

Case 2a (2nd Priority)

The Case 2a is illustrated in Figure 4.

Figure 4: Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

Model Training, Model Inference, and Model Monitoring

If Case 2a is supported, the existing PRS configuration can be utilized to gather training measurements for Case 2a at the UE side. The model is trained at the UE’s side or an OTT server. The inference should be done in a similar manner as for Case 3a and the inference output is sent from the UE to the LMF for the final calculation of the UE’s position through the legacy approaches. The model monitoring should be done in a similar manner as for Case 3a as well, with the signalling being between the UE and the LMF.

However, given the increased air-interface load, we believe that Case 2a should de-prioritized in RAN 3.

RAN 3 should deprioritize the study on the Case 2a. If the Case 2a is considered, no RAN 3 impact is foreseen. RAN 3 will closely monitor RAN 1’s progress after RAN 1 resumes the discussion on Case 2a.

Conclusions

In this submission, we discussed the AI/ML assisted positioning cases and potential RAN 3 impact. We presented the following proposals:

- The samples from the PRUs should be prioritized for label generation for model training and updating, which could be applied to all cases.

- For Case 3a, the labels and their related data (e.g., time stamp) can be transferred from the LMF to the NG-RAN node.

- For Case 3a, the inference output as assisting information needs to be sent from the NG-RAN node to the LMF, which are used by the LMF to calculate the UE’s position by legacy approaches.

- For Case 3a, the NG-RAN Node could be the entity that calculates the monitoring metric and making monitoring decision.

- For Case 3a, the LMF can provide assistance information for the monitoring metric calculation and monitoring decision making to the NG-RAN Node if Option A is adopted.

- For Case 3a, if Option B is adopted for model monitoring, the NG-RAN Node shall provide assistance information to the LMF for monitoring metric selection, calculation and monitoring decision making.

- In order to support Case 3a in the CU/DU split architecture, RAN 3 should discuss and determine where the AI model is located and possible interactions between gNB-CU and gNG-DU.

- RAN 3 should deprioritize the study on the Case 2a. If the Case 2a is considered, no RAN 3 impact is foreseen. RAN 3 will closely monitor RAN 1’s progress after RAN 1 resumes the discussion on Case 2a.

References

RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

3GPP TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

Final Report of 3GPP TSG RAN WG1 #116b v1.0.0 (Changsha, China, April 15th – 19th, 2024).

Draft Report of 3GPP TSG RAN WG1 #118 v0.1.0 (Maastricht, The Netherlands, August 19th – 23rd, 2024).