3GPP TSG RAN3 125bis	R3-245133

# Hefei, China, 14 - 18 October, 2024

Agenda item:	19.2

Source: 	Samsung

Title: 	Discussion on information exchange among gNBs for CLI handling

Document for:	Discussion and decision

Introduction

RAN3 would start the discussion on sub-band full duplex as per the objectives of the WID [1] below:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request

Some latest progress of RAN1 are informed to RAN3 in the incoming LS [2] of this meeting, in which RAN1 provided their agreements and basic understanding on the potential information to be transferred between gNBs .

In this contribution, we provide some discussion on the above issues based on the LS from RAN1.

Discussion

2.1 Semi-static cell-specific SBFD time/frequency location configuration

Before RAN3 starts the discussion on SBFD, RAN1 has converged to the support of cell-specific configuration on time and frequency location of SBFD subbands within a TDD carrier. RAN2 have also discussed the semi-static cell specific SBFD time and frequency location and achieved several agreements on the basic principles and parameters when configuring SBFD. At RAN2#126 meeting in Netherland, the following agreement was captured:

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

To our understanding, the cell-specific SBFD time and frequency configuration in the SIB1 shall be configured along with the TDD DL/UL configuration, which can be sent from a gNB to other gNBs via the XnAP Setup Request and/or NG-RAN node configuration update message. with the intention to reduce the cross link interference between gNBs, it is useful to send the SBFD time and frequency configuration to the other gNBs for information, such that the neighbouring gNBs can avoid transmitting via the DL resources which could cause CLI.

For split architecture, similarly, the cell specific SBFD configuration can be sent from to DU along with the TDD DL/UL configuration via F1 Setup Request message and gNB-DU Configuration Update message.

Proposal 1: The semi-static cell specific SBFD time and frequency configuration can be exchanged between gNBs via Xn Setup Request message and NG-RAN node Configuration Update message.

Proposal 2: The semi-static cell specific SBFD time and frequency configuration can be sent from the gNB-DU to the gNB-CU via F1 Setup Request message and gNB-DU Configuration Update message.

when it comes to the specific parameters within the configuration, RAN1 has the following agreements or working assumptions:

time domain: For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot indexes - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot frequency domain: working assumption For cell-specific configuration of frequency locations of SBFD UL sub-band, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured

The RAN1 agreements on specific parameters of the SBFD time and frequency configuration would be taken into account when RAN2 specify the SBFD configurations in SIB1, it is suggested that RAN3 wait for the stable specification of RAN2 before RAN3 define the detailed IEs over network interfaces.

Proposal 3: For the detailed IEs of cell-specific time and frequency location configuration, RAN3 wait RAN2 normative work for corresponding specification in SIB1.

2.2 Measurement resource allocation

For the measurement resource allocation, according to the information from RAN1 and TS 38.331, the resource information such as SSB and/or periodic NZP CSI-RS are already defined in the CSI-ResourceConfig IE of 38.331, where the SSB and NZP CSI-RS resource for channel measurement are configured. For the sake of simplicity, RAN3 can refer to the CSI-ResourceConfig IE when defining the IE(s) for measurement resource allocation information exchange between gNBs. As far as we can see, there are two options RAN3 could adopt:

Option 1: To define an IE which is an OCTECT STRING simply referring to the CSI-ResourceConfig IE of 38.331

Option 2: To define the IEs explicitly in the network interfaces as in CSI-ResourceConfig

RAN3 could discuss between the two selections, and we slightly prefer option 1 because it is relatively simply and straight-forward.

With Regard to the procedure used to transfer the measurement resource allocation, similar to the SBFD configuration, the measurement resource allocation configuration can also be included in the XnAP Setup Request message and NG-RAN node configuration Update message. As for split architecture, as mentioned above, the F1 Setup Request and gNB-DU Configuration Update message can be used to send the measurement resource allocation configuration from the gNB-DU to the gNB-CU.

Proposal 4: Adopt Option 1, i.e., to simply refer to 38.331, when defining the measurement resource allocation configuration to be exchanged between gNBs and from the gNB-DU to the gNB-CU.

2.3 Strongest DL beam information

The downlink strongest DL beam information is provided in the CRI value and could be defined as an SSB index from the perspective of higher layer, as indicated by RAN1 in the LS.

Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB. Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.

Thus, the strongest SSB index could be transferred to the other gNBs.

For split architecture, since the CSI measurement are lower layer measurement results transferred via PUCCH and PUSCH, only DU is aware of this information, to support the exchange between gNBs, the gNB-DU shall send the strongest DL beam information to the gNB-CU, after which the gNB-CU can send this information to the other gNB-CU(s).

Proposal 5: The strongest SSB index could be transferred over XnAP and F1AP.

2.4 CLI-mitigation request

It is noted that RAN1 did not provide any specific information or opinion about the CLI-mitigation request in the incoming LS. To some extent, it should depend on our RAN3 discussion whether to introduce such kind of CLI-mitigation request.

In the first place, the motivation and benefit of exchanging the CLI-mitigation request between gNBs should be evaluated. with the information in the previous parts exchanged between gNBs, the receiving node can adjust its resource configuration to mitigate the CLI by network implementation. The necessity for the gNB to explicitly indicate to the aggressor node about the request for CLI-mitigation seems not obvious.

So, we would prefer to discuss in RAN3 about the motivation and benefits to send explicit CLI-mitigation requests between gNBs.

Proposal 6: Discuss in RAN3 whether an explicit CLI mitigation request is needed between gNBs.

With all the above discussion, TPs to the BLCRs of TS38.423 and TS38.473 is provided in [3].

Conclusion

In this paper, we provide further considerations for QoE on support of NR-DC. The following proposals are provided,

Proposal 1: The semi-static cell specific SBFD time and frequency configuration can be exchanged between gNBs via Xn Setup Request message and NG-RAN node Configuration Update message.

Proposal 2: The semi-static cell specific SBFD time and frequency configuration can be sent from the gNB-DU to the gNB-CU via F1 Setup Request message and gNB-DU Configuration Update message.

Proposal 3: For the detailed IEs of cell-specific time and frequency location configuration, RAN3 wait RAN2 normative work for corresponding specification in SIB1.

Proposal 4: Adopt Option 1, i.e., to simply refer to 38.331, when defining the measurement resource allocation configuration to be exchanged between gNBs and from the gNB-DU to the gNB-CU.

Proposal 5: The strongest SSB index could be transferred over XnAP and F1AP.

Proposal 6: Discuss in RAN3 whether an explicit CLI mitigation request is needed between gNBs.

References

[1] RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2] R3-245006 LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation

[3] R3-245134 TPs to BLCRs of TS38.423 and TS38.473 on CLI handling