3GPP TSG-RAN WG3 Meeting #125bis	R3-245135

Hefei, China, 14th – 18rd Oct, 2024

Title:	Discussion on AI/ML based Coverage and Capacity Optimization

Source:				China Telecom

Agenda item:		11.3

Document for:		Discussion and Decision

1. Introduction

In RAN#105 meeting, the new WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN was approved [1] with following objectives:

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743. The detailed objectives of the WI are listed as follows: - Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing and AI/ML based CCO. [RAN3] - Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]: - Mobility Optimization for NR-DC - Split architecture support for Rel-18 use cases - Continuous MDT collection targeting the same UE across RRC states Note: Coordination with RAN2, SA5 when needed if any.

In this document, we analysis the AI/ML based Coverage and Capacity Optimization (CCO) and provide our proposals.

2. Discussion

In the study phase, we discussed use case and solutions for AI/ML based Coverage and Capacity Optimization (CCO). The following content was captured in TR 38.743 [2]:

## 4.2 AI/ML based Coverage and Capacity Optimization ### 4.2.1 Use case description The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. An NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included. In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it. With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

### 4.2.2 Solutions and standard impacts #### 4.2.2.1 Locations for AI/ML Model Training and AI/ML Model Inference The following solutions can be considered for supporting AI/ML-based CCO: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB. - AI/ML Model Training and AI/ML Model Inference are both located in the gNB. In case of CU-DU split architecture, the following solutions are possible: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU. - AI/ML Model Training and Model Inference are both located in the gNB-CU. #### 4.2.2.2 Input data of AI/ML based CCO For a proactive prediction and resolution of a CCO issue, a gNB may need the following information as input data for AI/ML-based CCO: From local node: - Measured/Predicted radio resource status - Current CCO State From neighbouring gNBs: - Measured/Predicted radio resource status From the UE: - UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements - SON Reports (e.g., RLF, CEF, RA) #### 4.2.2.3 Output data of AI/ML based CCO AI/ML-based CCO model in a gNB can generate following information as output: - Predicted CCO issue (including predicted affected cells/beams) - Future CCO State NOTE 1: Future CCO State can also be derived by legacy means. Signalling future CCO state will not be described as prediction over Xn. NOTE 2: Future CCO state and predicted CCO issue will be provided together with timing information. NOTE 3: Future CCO state can be derived based on the predicted CCO issue and other information. #### 4.2.2.4 Feedback of AI/ML based CCO To optimize the performance of AI/ML-based CCO model, following feedback can be considered to be collected from gNBs: - Measured radio resource status - Legacy UE performance feedback for those UEs handed over from the source gNB SON Reports (e.g., RLF, CEF, RA) #### 4.2.2.5 Potential standard impacts Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18 based on the CCO framework described in TS 38.300 [2] and TS 38.401 [3]. Xn interface: Enhance existing procedure to collect information between gNBs: - Future CCO State together with an associated coverage modification cause - Timing Information F1 interface: - Predicted CCO Issue (including predicted affected cells/beams) from gNB-CU to gNB-DU. - Future CCO State from gNB-DU to gNB-CU - Timing Information NOTE 4: Whether gNB-CU generates the suggested future CCO state and sends it to gNB-DU as assistance information is left to normative discussion. NOTE 5: For split architecture, future CCO State is derived based on the predicted CCO issue signaled from gNB-CU and other information in the gNB-DU.

Based on TR 38.743 [2], we have following observations:

Observation 1: Xn interface impacts and F1 interface impacts need to be discussed for AI/ML based CCO in normative phase.

As for Xn interface impact, AI/ML support functions and AI/ML support procedures for Xn interface are defined in TS 38.420 [3]. The AI/ML support functions are used to support AI/ML for NG-RAN, including initiation of data collection reporting of collected data. The AI/ML support procedures are used to initiate data collection and report collected data to support, e.g., AI/ML for NG-RAN, including:

-	Data Collection Reporting Initiation.

-	Data Collection Reporting.

Observation 2: AI/ML support function and AI/ML support procedures for XnAP are defined in TS 38.420.

Based on Observation 1 and 2, we have following proposal:

Proposal 1: The existing AI/ML support function and AI/ML support procedures for XnAP should be reused for AI/ML based CCO.

Furthermore, according to TR 38.743 [2], to support the AI/ML based CCO, the following information need to be collected between gNBs:

-	Future CCO State together with an associated coverage modification cause

-	Timing Information

Therefore, we have following proposal:

Proposal 2: The existing AI/ML support procedures (i.e. Data Collection Reporting Initiation and Data Collection Reporting) should be enhanced to collect following information between gNBs:

-	Future CCO State together with an associated coverage modification cause

-	Timing Information

According to TS 38.423 [4], the Data collection Initiation procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN. The successful operation of Data Collection Reporting Initiation is as Figure 1. NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. If NG-RAN node2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

Figure 1: Data Collection Reporting Initiation, successful operation

The Data Collection Reporting procedure is initiated by an NG-RAN node to report information accepted by the NG-RAN node following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN. The successful operation of Data Collection Reporting procedure is as Figure 2. In Figure 2, NG-RAN node2 shall report the accepted information in DATA COLLECTION UPDATE message. The accepted information is the information that was successfully initiated during the preceding Data Collection Reporting Initiation procedure.

Figure 2: Data Collection Reporting, successful operation

Observation 3: The Data Collection Reporting Initiation procedure is implemented via DATA COLLECTION REQUEST message and DATA COLLECTION RESPONSE message.

Observation 4: The Data Collection Reporting procedure is implemented via DATA COLLECTION UPDATE message.

Based on the observation 3 and 4, we have following proposal:

Proposal 3: The DATA COLLECTION REQUEST message, DATA COLLECTION RESPONSE message and DATA COLLECTION UPDATE message should be enhanced for collecting following information between gNBs:

-	Future CCO State together with an associated coverage modification cause

-	Timing Information

As for F1 interface impact, to support the AI/ML based CCO, the following information need to be collected between gNB-CU and gNB-DU:

-	Predicted CCO Issue (including predicted affected cells/beams) from gNB-CU to gNB-DU.

-	Future CCO State from gNB-DU to gNB-CU.

-	Timing Information.

However, there is no AI/ML related functions and procedures for F1AP to support AI/ML based CCO. Therefore, we have the following proposals:

Proposal 4: Introduce new AI/ML support functions and AI/ML support procedures for F1AP to support AI/ML for NG-RAN in CU/DU split architecture.

Proposal 5: The introduced new AI/ML support procedures for F1AP should be used for collecting following information between gNB-CU and gNB-DU:

-	Predicted CCO Issue (including predicted affected cells/beams) from gNB-CU to gNB-DU.

-	Future CCO State from gNB-DU to gNB-CU.

-	Timing Information.

3. Conclusion

In this contribution, we further discuss AI/ML based Coverage and Capacity Optimization (CCO). We have following observations:

Observation 1: Xn interface impacts and F1 interface impacts need to be discussed for AI/ML based CCO in normative phase.

Observation 2: AI/ML support function and AI/ML support procedures for XnAP are defined in TS 38.420.

Observation 3: The Data Collection Reporting Initiation procedure is implemented via DATA COLLECTION REQUEST message and DATA COLLECTION RESPONSE message.

Observation 4: The Data Collection Reporting procedure is implemented via DATA COLLECTION UPDATE message.

Based on observations, we have following proposals:

Proposal 1: The existing AI/ML support function and AI/ML support procedures for XnAP should be reused for AI/ML based CCO.

Proposal 2: The existing AI/ML support procedures (i.e. Data Collection Reporting Initiation and Data Collection Reporting) should be enhanced to collect following information between gNBs:

-	Future CCO State together with an associated coverage modification cause

-	Timing Information

Proposal 3: The DATA COLLECTION REQUEST message, DATA COLLECTION RESPONSE message and DATA COLLECTION UPDATE message should be enhanced for collecting following information between gNBs:

-	Future CCO State together with an associated coverage modification cause

-	Timing Information

Proposal 4: Introduce new AI/ML support functions and AI/ML support procedures for F1AP to support AI/ML for NG-RAN in CU/DU split architecture.

Proposal 5: The introduced new AI/ML support procedures for F1AP should be used for collecting following information between gNB-CU and gNB-DU:

-	Predicted CCO Issue (including predicted affected cells/beams) from gNB-CU to gNB-DU.

-	Future CCO State from gNB-DU to gNB-CU.

-	Timing Information.

4. Reference

- RP-242385, New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, ZTE Corporation, NEC

- TR 38.743, Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (Release 19)

- TS 38.420, Xn general aspects and principles (Release 18)

- TS 38.423, Xn application protocol (XnAP) (Release 18)