3GPP TSG-RAN WG3 Meeting #125bis	   				         R3-245141

Hefei, China, 14th – 18th Oct 2024

Agenda Item:		16.2

Source:			China Telecom

Title:		(TP to TR 38.769) RAN architecture for Ambient-IoT

Document for:	 	Other

- Introduction

The last RAN3 meeting discussed the RAN architecture of AIoT, including the definition of the device entities that provide AIOT services and the system architecture diagram of topology 1&2. However, there are still some open issues regarding the protocol stack design for topology 2 due to time constraints. In this contribution, we will continue to discuss RAN architecture aspects from the perspective of RAN3.

- Discussion

For Topology 2, there are three signalling solutions:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

From the RAN3 perspective, for NAS or UP based solutions, the signalling might not be recognized by A-IoT-enabled gNB, implying that there may be no RAN3 impacts for supporting these two solutions.

The protocol stacks can be as follows:

Figure 1: Protocol stack for RRC based solution of Topology 2

Figure 2: Protocol stack for NAS based solution of Topology 2

Figure 3: Protocol stack for UP based solution of Topology 2

The above Protocol stack for Topology 2 should be captured in the TR 38.769. The TP is provided in the annex.

Proposal 1: Capture these three protocol stacks for Topology 2 into the TR.

In addition, SA2 agreed in TR 23700-13[2] to introduce a new core network function to support topology 1, whether the new core network function also applies to topology 2 is FFS. There is still no consensus on whether the interface between AIoT RAN and AIoT CN should reuse NG or define a new XX interface in SA2, which may depend on the collaborative relationship between the new AIoT NF and AMF. It is suggested that RAN3 further discuss whether to reuse NG interface for A-IoT signaling interaction, and coordinate with SA2. We slightly prefer to reuse existing NG interface to support both topology 1 and topology 2, in order to minimize standard impacts. Whether to define a new interface between AIoT NF (similar to NRPPa) needs to wait the progress of SA2.

Proposal 2: It is proposed to enhance the existing NG interface to support both topology 1 and topology 2.

- Conclusions

In this paper, we have the following proposal:

Proposal 1: Capture these three protocol stacks for Topology 2 into the TR.

Proposal 2: It is proposed to enhance the existing NG interface to support both topology 1 and topology 2.

- References

[1]	R3-243962, [TP for TR 38.769] CB:#AIoT1_Architecture (Ericsson, Huawei)

[2] 	3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things, V1.0.0 (2024-09).

- Text Proposal for TR 38.769

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>

### 6.4.2	Support of Topology 2

Figure 6.4.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB.

The following definitions apply:

A-IoT-enabled gNB:	A gNB supporting A-IoT RAN node function, which is able to communicate with the A-IoT enabled UE via NR Uu interface.

A-IoT-enabled UE:	A UE supporting Common reader function, which is able to communicate with the A-IoT device via the A-IoT radio interface.

Figure 6.4.2-1 Logical system architecture for topology 2

Editor’s Note 1:	Figure 6.3.2-1 doesn’t illustrate the protocol between A-IoT enabled UE and A-IoT CN, if needed, the figure needs to be revised in case such is defined by SA2.

Editor’s Note 2:	In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS.

Editor’s Note 3:	The A-IoT CN could include AMF and A-IoT related functions. This is up to SA2 decision.

Editor’s Note 4:	The A-IoT enabled gNB performs radio resource management for A-IoT related radio resources, details are pending on RAN1 and RAN2 mechanisms.

#### 6.4.2.1	Solutions for Topology 2

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

For NAS or UP based solutions, the signalling might not be recognized by the A-IoT-enabled gNB, implying that there may be no RAN3 impacts for supporting these solutions.

Figures 6.4.2.1-1 to 6.4.2.1-3 show the Protocol stack for Topology 2:

Figure 6.4.2.1-1: Protocol stack for RRC based solution of Topology 2

Figure 6.4.2.1-2: Protocol stack for NAS based solution of Topology 2

Figure 6.4.2.1-3: Protocol stack for UP based solution of Topology 2

Editor’s note 1: How to enable radio resource control, i.e. trigger A-IoT RAN node functions for above solutions is FFS.

Editor’s note 2: Depiction and further details of the options above are FFS

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>