3GPP TSG RAN WG3#125bis	R3-245151

Heifei, China, 14 – 18 Oct. 2024

Agenda item:	12.3

Source: 	China Telecom

Title: 	On NR Femto Architecture and functional split

Document for:	Discussion and decision

- Introduction

In RAN#105, the work on “Additional topological enhancements for NR” goes on the work item phase from the study item [1]. The Femto related objectives in the new work item were listed below:

The objectives of the 5G Femto work are as follows: - Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3]. - Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3]. NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective. NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

In this paper, we discuss issues related to NR Femto architecture and present our proposals.

- Discussion on NR Femto architecture

At Study Item phase, three options related to NG interface and two options related with Xn interface are captured in the section 5.2 in TR 38.799, listed below:

The architecture related to NG:

- Option 1: a NR Femto node connects to the 5GC directly as a gNB by means of the NG interface.

- Option 2: a NR Femto node connects to the 5GC via a NR Femto Gateway which serves as a concentrator for the C-Plane.

- Option 3: a NR Femto node connects to the 5GC via an SCTP concentrator acts as an IP proxy.

The architecture related to Xn:

- Option A: the NR Femto Node connects to the other NR Femto Node(s)/gNB(s) directly as a gNB by means of the Xn interface

- Option B: NR Femto node connects to the other NR Femto Node(s)/gNB(s) via the Xn GW.

In the recent approved WID, the Femto-related objects only include the normative work on optional NR Femto GW for NG interface and do not cover the Xn-related work. Therefore the Xn-related topics will not be discussed in the paper [1].

For the NG-related normative work, we will begin the discussion from two aspects:

- Architecture of Femto GW for NG interface

- Functional split of Femto GW

2.1 Architecture of Femto GW for NG interface

In TR 38.799, Figure 5.2.1.2-1 presents a logical architecture for the NR Femto which include a set of NG interfaces connecting the NR Femto node to the 5GC, as shown in Figure 1. We can follow the approach used in LTE and incorporate this figure into TS 38.300 as the NR Femto Logical Architecture.

Figure 1: NR Femto Architecture (i.e. Figure 5.2.1.2-1 in TR 38.799)

In addition to Figure 1, we also need a figure (referred to as Figure 2) to illustrate the overall NR architecture with a deployed NR Femto GW. Figure 2 show that a standard gNB can connect to an NR Femto via the Xn interface, an NR Femto can connect to an NR Femto GW via NG interface, and an NR Femto GW can connect to one or more AMF/UPFs through the NG interface. However, there is no connection between ng-eNB and NR Femto, and neither ng-eNB not gNB have any connection with NR Femto GW. Since not all of this information is reflected in Figure 1, it is necessary to include Figure 2 in the technical specification (i.e TS 38.300).

Figure 2: Overall NR Architecture with deployed NR Femto GW.

Proposal 1: The figure depicting the overall NR architecture with deployed NR Femto GW (i.e. Figure 2) should be included in TS 38.300.

2.2 Functional split of NR Femto GW

In the NR architecture with deployed NR Femto GW, while an NR Femto hosts the same functions as a gNB, it should also additional capability specifics in case of its connection to the NR Femto GW, such as the following:

- A NR Femto should discover a suitable serving NR Femto GW;

- A NR Femto shall connect to only one single NR Femto GW at one time;

- The TAC and PLMN ID used by the NR Femto shall also be supported by the NR Femto GW.

For NR Femto GW, referring to the functionality of the HeNB GW, the functionality of it should be as follows at least:

- A NR Femto GW can relay UE-associated NG application part messages between the AMF serving the UE and the NR Femto serving the UE;

- Terminating non-UE associated NR application part procedures towards both the NR Femto and the AMF;

- Optionally terminating NR-U interface with the NR Femto and with the UPF;

- Supporting TAC and PLMN ID used by the NR Femto.

- Ensuring that Xn interfaces shall not be established between the NR Femto GW and other nodes.

Proposal 2: The basic functions outlined above for NR Femto and NR Femto GW should be included in TS 38.300.

- Conclusion

Based on the above discussion regarding architecture and functional split for NR Femto and NR Femtos, we provide the following proposals:

Proposal 1: The figure depicting the overall NR architecture with deployed NR Femto GW (i.e. Figure 2) should be included in TS 38.300.

Proposal 2: The basic functions outlined above for NR Femto and NR Femto GW should be included in TS 38.300.

- Reference

[1] RP-242395, New WID on additional topological enhancements for NR, NTT DOCOMO, AT&T.

[2] RP-241925, TR 38.799 v2.1.0 NR; Study on additional topological enhancements for NR, NTT DOCOMO INC., AT&T.

[3] TS 38.300, NR; NR and NG-RAN Overall description; Stage-2 Rel-18, 3GPP.

[4] TS 38.413, NG-RAN; NG Application Protocol (NGAP) Rel-18, 3GPP.

[5] TS 36.300, E-UTRAN; Overall description; Stage 2 Rel-18, 3GPP.

[6] TS 36.413, E-UTRAN; S1 Application Protocol (S1AP) Rel-18, 3GPP.