3GPP TSG-RAN WG3 Meeting #125bis													R3-245175

Hefei, China, 14th - 18th October 2024

Agenda item:	12.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.401) Discussion on high level aspects for WAB

Document for:	Discussion and Decision

# 1	Introduction

RAN#105 agreed WI on topological enhancements for NR [1]. The objectives for WAB are following, [1]:

The objectives of the Wireless Access Backhaul (WAB) work are as follows: - Specifications for the support of WAB including [RAN3]: - Support of a WAB-node including a WAB-gNB and a WAB-MT. - Support of backhauling of the WAB-gNB’s NG, Xn and OAM traffic over the WAB-MT’s PDU session(s). - Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs. - Defining the behaviour of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes. - Network integration procedures for WAB nodes. - Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes. - Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB. - UE’s ULI that reflect the WAB node’s location. - The handling of: - PCI collision avoidance. - Reconfiguration of TAC and RANAC on WAB-gNBs. - Mechanisms to avoid multi-hop WAB topology. - Radio-resource coordination between access and backhaul links. - NG connection management. NOTE 1: For PCI collision avoidance and reconfiguration of TAC and RANAC on WAB-gNBs, follow the conclusion of mobile IAB. NOTE 2: NG connection management should take the NTN conclusion into account, avoiding parallel discussions. NOTE 3: No impact on the UE. NOTE 4: Coordination with other WGs (e.g. SA2, RAN2) when needed. NOTE 5: Backhaul link for WAB-MT can be TN or NTN. NOTE 6: Mobility procedures to be used for the UEs served by a WAB-gNB are legacy UE mobility procedures. Mobility of the WAB-MTs is based on legacy UE mobility procedures. NOTE 7: The interface between the WAB-MT and the co-located WAB-gNB is out-of-scope for the normative phase. NOTE 8: Split architecture of the WAB-gNB is out-of-scope for the normative phase. NOTE 9: RAN2 impact should be identified as early as possible, and should be minimal.

This contribution analyses the architecture, protocol stack and high-level aspects.

# 2	architecture and protocol stack

The conclusion in the SI, as captured in the TR38.799 can be a starting point. It is proposed to capture the architecture as described in Figure 4.2-1 of TR 38.799, the NG-C/U Protocol Stack as described in Figure 4.2-3 of TR 38.799, and Xn-C/U Protocol Stack as described in Figure 4.2-4 of TR 38.799 in Stage-2 specification (TS 38.401). (also copied as below)

Figure 4.2-1: The WAB architecture example for 5GS when the WAB-gNB traffic is transported via PDU session backhaul

Figure 4.2-3: Protocol stack examples of NG Control plane and User plane transport for a UE connected via WAB-node

Figure 4.2-4: Protocol stack examples of Xn Control plane and User plane transport

Proposal 1: Capture the architecture as described in Figure 4.2-1 of TR 38.799, the NG-C/U Protocol Stack as described in Figure 4.2-3 of TR 38.799, and Xn-C/U Protocol Stack as described in Figure 4.2-4 of TR 38.799 in Stage-2 specification (TS 38.401).

# 3	Integration procedure

TR 38.799 has captured the following integration procedure.

### 4.3.1	WAB-node integration procedure

Figure 4.3.1-1 WAB-node integration procedure

Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling.

Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases:

Sub-phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is configured by the OAM (e.g., with the information of AMF(s) to serve the UE) and service-authorized by the SeGW or by the OAM.

Sub-phase 2-2: NG connection setup. The WAB-gNB establishes NG connection(s) toward the AMF(s). This step may follow legacy procedures. After the NG is set up, the WAB-gNB can start serving UE(s).

Sub-phase 2-3: Xn connection setup. If needed, the WAB-gNB may establish Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s).

Proposal 2: Capture the integration procedure as described in Clause 4.3.1 of TR 38.799 in Stage-2 specification (TS 38.401).

# 4	Reconfiguration of TAC and RANAC on WAB-gNBs

This can be similar to mobile IAB. TR 38.799 has captured following:

The TAC/RANAC of WAB-gNB’s cell is configured by the OAM, and it can be reconfigured by the OAM during the mobility of WAB-node. The TAC/RANAC of the WAB-gNB’s cell may be the same as, or different than, the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcasted by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location.

Proposal 3: It is proposed to capture above text for Reconfiguration of TAC and RANAC in TS 38.401

# 5	NG interface management

NR NTN WI have concluded

Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management.

WAB is similar to the gNB in NR NTN Regenerative system.  It is proposed to adopt the same decision from NR NTN. Stage-2 TP is needed, e.g. to clarify the WAB-gNB can initiate the NG Removal procedure. Any Stage-3 impact can be discussed later after the BL CR for NR NTN is agreed.

Proposal 4: adopt the NR NTN decision for NG Removal.

The proposed Stage-2 TP can be found in Annex – TP for TS 38.401

# 6	Xn interface management

Previous meeting RAN3 agreed

Establishment of Xn Connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported and can follow legacy procedures.

However, there is no TP captured in the TR 38.799 for Xn management (e.g. Xn establishment) for the WAB-gNB.

Xn Establishment

Xn establishment between WAB-gNB and the BH-gNB serving WAB-MT

When the WAB-MT connects to a BH-gNB serving WAB-MT, the WAB-MT knows the TAI/CGI of the serving cell. After the integration procedure (i.e. WAB-MT connected with the gNB serving WAB-MT, WAB-gNB setup NG with the AMF, BH PDU session establishment), the WAB-gNB can initiate the Xn-C TNL address discovery procedure towards the BH-gNB serving WAB-MT. The WAB-gNB may then initiate the TNL/Xn Setup with the BH-gNB serving WAB-MT.

Proposal 5-1: WAB-gNB can reuse existing Xn-C TNL address discovery procedure to know the Xn-C TNL address of BH-gNB serving WAB-MT, then setup Xn with BH-gNB serving WAB-MT.

Xn establishment between WAB-gNB and neighboring gNB

The surrounding RAN node can be a stationary gNB, or a WAB-gNB. There is different view on whether the Xn can be setup between the two WAB-gNBs.

In Rel-18, the mobile IAB acts as a gNB-DU/cell. The Xn between the IAB-donor and its neighboring gNB may be already established before mobile IAB connects with IAB-donor. The arrival/departure of a mobile IAB does not affect the Xn between the IAB-donor and its neighboring gNB, other than possibly informing neighbors about newly deployed/released IAB cell(s). From the perspective of a neighboring gNB, the arrival/departure of a mobile IAB can be considered to add/remove cell(s) of the serving IAB-donor.

The WAB brings a new challenge to the inter-gNB neighbor relation. Unlike a new Rel-18 mobile IAB appearing as a new gNB-DU/cell to neighboring gNB, WAB appears as a new gNB to the neighboring gNB. The arrival/departure of mobile WAB causes dynamic setup/release of the Xn between the WAB-gNB and its neighboring gNB(s).

In legacy system, the gNB can initiate the ANR procedure whenever the UE reports an unknown PCI, i.e. a PCI that is not in the NCRT. This can be similar for WAB, e.g. WAB initiate the ANR procedure whenever the UE reports an unknown PCI. However, WAB faces another challenge, for example, when the UE reports a known PCI, the WAB cannot know whether the PCI is related to an existing cell/entry in its NCRT, or the PCI belongs to a new neighboring cell that is not in the NCRT. This is due to the mobility nature of WAB and PCI reuse for cells in different geographical areas.

Figure 1: PCI reuse by cells of gNB1 and gNB3

An example is shown in above figure.

- When the WAB is at location A, it detects the new cell with PCI#1 (i.e. gNB1’s cell), WAB initiate ANR procedure to add gNB1’s cell (PCI#1) in its NCRT.

- Later, when WAB move to location B which is covered by gNB3’s cell with same PCI#1, WAB cannot know whether the PCI#1 corresponds to an existing entry in NCRT that is related to gNB1’s cell, or corresponds to a new cell.

So, it is necessary for WAB-gNB to know the neighboring cell, and a neighboring gNB to know WAB cell as early as possible. In legacy system, the Xn setup is either triggered by OAM, or by ANR based on the UE’s measurement report. However, in the mobile WAB case, the WAB-gNB or neighboring gNB may not be able to know the right time that it needs to ask UE to perform a measurement in order to detect the cell using a specific PCI is changed.

Observation 5-1: Due to PCI reuse, it may be difficult for WAB-gNB (or gNB) to determine the right time to ask UE to perform a measurement in order to detect the new neighbouring cell.

One option is to exchange the neighboring cell information over Xn interface, in order for WAB-gNB to know the neighboring cell, and a neighboring gNB to know WAB cell as early as possible.

The WAB-gNB can reuse the existing mechanism to detect the neighboring cell of a gNB. Since the WAB has a gNB functionality, it can initiate the ANR procedure based on the measurement report from a connected UE, or from the measurement report of the co-located WAB-MT. In addition, the WAB-gNB can also use the information received from BH-gNB serving WAB-MT over Xn.  In most of the scenarios, the WAB’s neighboring gNB is also a neighboring gNB to the BH-gNB. Since the BH-gNB may already setup Xn with the neighboring gNB, BH-gNB can provide the Xn-C TNL address of neighboring gNB to WAB-gNB, thus WAB-gNB can directly initiate the Xn setup procedure with neighboring gNB. Alternatively, during the Xn Setup or NG-RAN node configuration update procedure, the BH-gNB serving WAB-MT can provide its neighboring cell information to the WAB-gNB. The WAB-gNB can use the neighboring cell information received from the BH-gNB to update its NCRT, or initiate Xn-C TNL address discovery procedure for further TNL/Xn Setup with the neighboring gNB, without waiting for the measurement report from UE (or WAB-MT).

Proposal 5-2: BH-gNB can provide the Xn-C TNL address of neighboring gNB to WAB-gNB, so WAB-gNB can directly initiate Xn Setup with neighbour gNB.

Proposal 5-3: WAB-gNB can also use the neighboring cell information received from the BH-gNB to update its NCRT or initiate the Xn-C TNL address discovery procedure towards the neighboring gNB for further TNL/Xn Setup with the neighboring gNB, without waiting for the measurement report from UE (or WAB-MT).

Furthermore, in case Xn is to be avoided between WAB-gNBs (as proposed by some companies in the previous meeting due to the nature of WAB being mobile, which would lead frequent Xn setup and Xn release), this could even be avoided during TNL discovery procedure, i.e., as early as possible. For example, source WAB-gNB can inform the target gNB that it is a WAB during TNL discovery, and target can refrain sending TNL information. That way, Xn establishment can be avoided at the earliest time possible and there would be no unnecessary additional signalling.

Proposal 5-4: If Xn is to be avoided among WAB-gNBs, TNL discovery procedure can be enhanced to avoid Xn establishment as early as possible among WAB-gNBs.

Xn Removal

When the WAB move away, the WAB may need to remove the Xn with previous surrounding gNBs. This can be supported by the existing Xn Removal procedure.

Observation 5-2: Existing Xn Removal procedure can be reused to remove the Xn between WAB-gNB and surround gNB.

Support for RRC INACTIVE

A UE connected to the WAB-gNB may transition to RRC INACTIVE, then move to the surrounding gNB’s coverage.  Similarly, a UE connected to the surrounding gNB may transition to RRC INACTIVE, then move to the WAB-gNB’s coverage. The RNA assigned to the UE may include the RAN area ID/cell ID of WAB-gNB and surrounding gNB.

Observation 5-3: A UE’s RNA can consist of RAN area ID(s)/cell ID(s) of WAB-gNB(s) and surrounding gNB(s).

Observation 5-4: An RRC_CONNECTED UE can transition to RRC_INACTIVE via WAB-gNB.

However, to support the Xn Retrieve UE Context procedure, the Xn Removal procedure should be carefully designed to avoid the scenario where UE context cannot be retrieved due to released Xn interface between the target gNB and the WAB-gNB. This is especially an issue considering WAB’s are moving devices, and Xn may be established and removed dynamically as the WAB moves. In our view, there are at least a couple of ways to tackle the issue:

- “Too early Xn Removal”, i.e. Xn is removed before Xn Retrieve UE Context Procedure is initiated.

- In case Xn is to be released between the WAB-gNB and another gNB, whose cell(s) are in the RNA of at least one of the UEs that WAB-gNB sent to RRC_INACTIVE, UE context of the corresponding UE(s) are provided to the gNB by the WAB-gNB.

Proposal 5-5: RAN3 discuss the enhancement to avoid the UE context retrieval problems when Xn is removed between a WAB-gNB and a surrounding gNB.

# 7	Traffic Mapping for Backhaul PDU session carrying N2/N3 traffic

The WAB-gNB sets up NG with UE’s AMF. The N2/N3 (or NG-C/U) traffic is conveyed over the BH PDU session of the WAB-MT. At least one PDU session needs to be setup, and to be used for the UE’s N2/N3 backhaul. Up to the operator’s configuration, the WAB-MT may setup one or multiple PDU sessions, e.g. for OAM, for N2/N3, or for Xn, etc.

In uplink direction, the WAB-gNB knows the QoS for each UL packet received from the UE. The WAB-gNB can inform the co-located WAB-MT. Based on the QoS information for the UL packet, the WAB-MT can map the related N3 packet to a specific QoS flow of the WAB-MT’s BH PDU session. The WAB-MT can also determine whether an already established BH PDU session is enough, or modify the existing BH PDU session, or request the establishment of a new BH PDU session.

For the uplink N2 traffic, the co-located WAB-MT can know whether it is a UE-associated signaling or non-UE-associated signaling, the WAB-MT can map it to the related QoS flow of the BH PDU session.

Proposal 6-1: In the uplink direction, WAB can perform the traffic mapping based on the QoS of the UL packet, and the QoS of the QoS flow of its BH PDU session.

In downlink direction, UE’s UPF uses the packet filter provided by the UE for packet detection, e.g. identify the related QoS Flow and add the QFI in the NG-U GTP-U header. The NG-U GTP-U packet is routed to the UPF of the WAB-MT. The WAB-MT’s UPF performs a further traffic mapping to a QoS flow of the BH PDU session. The WAB-MT’s UPF can only see the IP header, but not the inside GTP-U packet from the UE’s UPF. So, the traffic mapping in the WAB-MT’s UPF can only be based on the IP Header.  This is also applicable to the downlink N2 traffic.

To perform a correct traffic mapping in the WAB-MT’s UPF, the WAB-MT’s UPF needs to be configured with the mapping rule.

Proposal 6-2: For downlink N2/N3 traffic, the WAB-MT’s UPF performs the traffic mapping based on the IP header of the N2/N3 traffic.

The mapping rule can be provided by the WAB-MT, e.g. based on the existing UE requested PDU session modification procedure.

If the N2 traffic is on the same PDU session as the N3 traffic, the traffic classification for N2, is based on identification of the SCTP protocol. Else, if the N2 traffic has its own PDU session, the traffic is identified in DL by the IP addresses used for N2 over this connection (i.e. the IP address of the connection) so normal IP routing is sufficient.

Note that the following conclusion is made by the SA2 on the matter:

- If QoS optimization of the N3 connection is required, then the MWAB-gNB is configured (e.g. by OAM) with the mapping of the 5QIs to the DSCP field for TNL used by the UPFs/gNBs in the PLMN/SNPN serving the UE via the MWAB-gNB. This is then used to establish SDFs at the BH-PDU session used for N3 connection(s) based on existing procedures.

The traffic mapping for N3 over the appropriate BH PDU session QoS flow, is therefore to be based on the approach documented in Solution 3 of TR 23.700-06 i.e based on the differentiated services code point (DSCP) value to be used by the UE’s UPF for sending the DL NG-U packet. This value can be known per 5QI and be configured by OAM at the WAB gNB which then can request the suitable update of the N3 PDU session by PDU session modification including this classification rule.

Proposal 6-3: For downlink traffic mapping to the appropriate N3 BH PDU session QoS flow, the WAB-gNB needs to know the DSCP information for the further DL NG-U packet during the UE’s PDU Session Resource Setup/Modification procedure.

# 8	Conclusion

In this contribution, we discussed the architecture and other high-level aspects of WAB. Our proposals are:

Proposal 1: Capture the architecture as described in Figure 4.2-1 of TR 38.799, the NG-C/U Protocol Stack as described in Figure 4.2-3 of TR 38.799, and Xn-C/U Protocol Stack as described in Figure 4.2-4 of TR 38.799 in Stage-2 specification (TS 38.401).

Proposal 2: Capture the integration procedure as described in Clause 4.3.1 of TR 38.799 in Stage-2 specification (TS 38.401).

Proposal 3: It is proposed to capture above text for Reconfiguration of TAC and RANAC in TS 38.401

Proposal 4: adopt the NR NTN decision for NG Removal.

Proposal 5-1: WAB-gNB can reuse existing Xn-C TNL address discovery procedure to know the Xn-C TNL address of BH-gNB serving WAB-MT, then setup Xn with BH-gNB serving WAB-MT.

Proposal 5-2: BH-gNB can provide the Xn-C TNL address of neighboring gNB to WAB-gNB, so WAB-gNB can directly initiate Xn Setup with neighbour gNB.

Proposal 5-3: WAB-gNB can also use the neighboring cell information received from the BH-gNB to update its NCRT or initiate the Xn-C TNL address discovery procedure towards the neighboring gNB for further TNL/Xn Setup with the neighboring gNB, without waiting for the measurement report from UE (or WAB-MT).

Proposal 5-4: If Xn is to be avoided among WAB-gNBs, TNL discovery procedure can be enhanced to avoid Xn establishment as early as possible among WAB-gNBs.

Proposal 5-5: RAN3 discuss the enhancement to avoid the UE context retrieval problems when Xn is removed between a WAB-gNB and a surrounding gNB.

Proposal 6-1: In the uplink direction, WAB can perform the traffic mapping based on the QoS of the UL packet, and the QoS of the QoS flow of its BH PDU session.

Proposal 6-2: For downlink N2/N3 traffic, the WAB-MT’s UPF performs the traffic mapping based on the IP header of the N2/N3 traffic.

Proposal 6-3: For downlink traffic mapping to the appropriate N3 BH PDU session QoS flow, the WAB-gNB needs to know the DSCP information for the further DL NG-U packet during the UE’s PDU Session Resource Setup/Modification procedure.

The proposed Stage-2 TP can be found in Annex – TP for TS 38.401

# References

- RP-242395, New WID on additional topological enhancements for NR

- TR 38.799 v19.0.0

# Annex – TP for TS 38.401

### 6.1.x1	Overall Architecture of WAB

Figure 6.1.x1-1 shows the WAB architecture for 5GS.

Figure 6.1.x1-1: Overall Architecture of WAB

### 6.1.x2	Protocol stacks of WAB

Figure 6.1.x2-1 shows protocol stack of NG Control plane and User plane transport for a UE connected to the network via a WAB-node.

Figure 6.1.x2-1: Protocol stack examples of NG Control plane and User plane transport for a UE connected via WAB-node

Figure 6.1.x2-2 shows protocol stack of Xn Control plane and User plane transport for WAB-node.

Figure 6.1.x2-2: Protocol stack examples of Xn Control plane and User plane transport

# 11	Overall procedures in NG-RAN Architecure

## 11.1	Multiple TNLAs for Xn-C

In the following, the procedure for managing multiple TNLAs for Xn-C is described.

Figure 11.1-1: Managing multiple TNLAs for Xn-C.

1.	The NG-RAN node1 establishes the first TNLA with the NG-RAN node2 using a configured TNL address.

NOTE:	The NG-RAN node1 may use different source and/or destination IP end point(s) if the TNL establishment towards one IP end point fails. How the NG-RAN node1 gets the remote IP end point(s) and its own IP address are outside the scope of this specification.

2-3.	Once the TNLA has been established, the NG-RAN node1 initiates the Xn Setup procedure to exchange application level configuration data

4-6.	The NG-RAN node2 may add additional TNL Endpoint(s) to be used for Xn-C signalling between the NG-RAN node1 and the NG-RAN node2 pair using the NG-RAN node Configuration Update procedure. NG-RAN node Configuration Update procedure also allows the NG-RAN node2 to request the NG-RAN node1 to modify or release TNLA(s).

7-9.	The NG-RAN node1 may add additional TNL Endpoint(s) to be used for Xn-C signalling between the NG-RAN node1 and the NG-RAN node2 pair using the NG-RAN node Configuration Update procedure. NG-RAN node Configuration Update procedure also allows the NG-RAN node1 to request the NG-RAN node2 to modify or release TNLA(s).

The XnAP UE TNLA binding is a binding between a XnAP UE association and a specific TNL association for a given UE. After the XnAP UE TNLA binding is created, the NG-RAN node1 or the NG-RAN node2 can update the UE TNLA binding by sending the first available XnAP message for the UE to the peer NG-RAN node via a different TNLA. The peer NG-RAN node shall update the XnAP UE TNLA binding with the new TNLA.

## 11.x1	WAB-node integration procedure

Figure 11.x1-1 WAB-node integration procedure

Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling.

Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases:

Sub-phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is configured by the OAM (e.g., with the information of AMF(s) to serve the UE) and service-authorized by the SeGW or by the OAM.

Sub-phase 2-2: NG connection setup. The WAB-gNB establishes NG connection(s) toward the AMF(s). This step may follow legacy procedures. After the NG is set up, the WAB-gNB can start serving UE(s).

Sub-phase 2-3: Xn connection setup. If needed, the WAB-gNB may establish Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s).

### 11.x2	WAB-node OAM

The WAB-node receives commands, configuration data and software downloads (e.g. for equipment software upgrades) from its OAM system. The WAB-node can also send alarms and traffic counter information to its OAM system. The transport connection between the WAB-node and its OAM, using IP, is provided by the WAB-MT’s PDU session via 5G network.

Alarms in the wAB generate bursts of high-priority traffic, to be transported in real time. Traffic counters generate bursts of traffic, but their transport need not be real-time. Configuration messages from OAM to the WAB will also generate small bursts of traffic, possibly with lower priority than alarms but still delay-sensitive: when a configuration is committed on the OAM, the time interval between the commitment and the effect on the equipment shall be small. Alarm messages and commands should be transported on a high-priority bearer, while counters may be transported on a lower priority bearer.

OAM software download to the WAB may generate larger amounts of data, but both the required data rate and the priority of this kind of traffic are much lower than in the case of alarms, commands and counters.

For different types of OAM traffic, it is necessary to use different DRBs between the WAB-MT and the serving BH-gNB, with different QoS parameters. Aggregation of OAM traffic with other non-OAM traffic on the same DRB is not precluded.

The continuity of OAM connectivity needs to be ensured as the mobile IAB-node moves across the mobile network.

### 11.x3	TAC/RANAC (re-)configuration for WAB

The TAC/RANAC of mobile WAB’s cell is configured by the OAM, and it can be reconfigured by the OAM during the mobile WAB-node mobility. The TAC/RANAC of the mobile WAB’s cell may be same as or different than the TAC/RANAC of the co-located mobile WAB-MT’s serving cell. The TAC/RANAC broadcasted by the mobile WAB cell can be changed in order to reflect the mobile WAB-node’s physical location.

### 11.x4	Management of NG interface for WAB

Based on the OAM configuration, the WAB-gNB can setup NG interface with an AMF. When disconnecting from an AMF is required due to e.g. the WAB-node move out of the serving area of the AMF, the WAB-gNB may request the removal of the NG interface by triggering the NG Removal procedure toward the AMF.

### 11.x5	Management of Xn interface for WAB

The WAB-gNB may establish Xn interface with surrounding gNBs. The BH-gNB may provide the information of neighboring gNB (e.g. gNB ID, TNL address) to the WAB-gNB via Xn interface, which enables the WAB-gNB to establish Xn with the neighboring gNB(s).

The WAB-gNB may release the Xn interface with a neighboring gNB, e.g. when the WAB node move away from the neighboring gNB. The Xn removal should also consider the stored UE context for RRC INACTIVE UE(s), so the Xn interface removal will not cause issue to further XnAP Retrieve UE Context procedure.

### 11.x6	WAB-gNB mobility without change of UE’s AMF(s)

During WAB-node movement, radio configuration parameters of the WAB-gNB may be changed (e.g., cell ID, PCI and/or TAC) without the change of the UE’s AMF(s). This change of radio configuration parameters may require the UE handling by means of, e.g., intra-gNB handover and/or Mobility Registration Update as defined in TS 23.502 [7].