3GPP TSG-RAN WG3 #125bis                                     R3-245187

Hefei, China, 14th – 18th Oct, 2024

Source:	CATT

Title:	Discussion on Evolution of Duplex Operation

Agenda Item:	19.2

Document for:	Discussion and decision

- Introduction

This document discusses intra-NTN Mobility for SONMDT based on WID [1].

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

  - Information exchange among gNBs, including [RAN3]

    - Semi-static cell-specific SBFD time and frequency location configuration

    - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

    - Strongest DL beam information

    - CLI-mitigation request

- Discussion

  - Measurement resource configuration and Strongest DL beam information

GNB measures based on the NZP CSI-RS resource information provided by neighbour cell, and then send measurement result i.e., the strongest DL beam information back. According to the strongest DL beam information, neighbour gNBs know the interference.

A LS [1] received from RAN1 as below

From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported.

Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB.

Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.

Xn impacts

It can be found that there are two steps need to be supported in Xn

- Step 1: gNB1 sends NZP CSI-RS resources to gNB2

- Step 2: gNB2 provides strongest DL beam information in the context of NZP-CSI-RS/SSB to gNB1

For step 1, NZP CSI-RS resources defined in TS38.331 is used to configure Non-Zero-Power (NZP) CSI-RS transmitted in the cell where the IE is included, which the UE may be configured to measure on. It can be added in Served Cell Information NR.

Proposal 1: Add NZP CSI-RS resource in Served Cell Information NR in Xn.

For step 2, the strongest DL beam information is defined as CRI and SSB index.

The question is that if the reporting of the strongest DL beam is periodic or one-off or event trigger.

- In case of periodic response, the strongest DL beam information can reuse the RESOURCE STATUS REQUEST/RESPONSE/UPDATE procedure.

- In case of one-off response, the strongest DL beam information can be added in NG-RAN NODE CONFIGURATION UPDATE.

- In case of event trigger, gNB2 reports the strongest DL beam information when the interference over the threshold.

Proposal 2: RAN3 discuss how neighbour cells response the strongest DL beam information in Xn i.e., periodic response or one-off response or response base on event trigger.

F1 impacts

It can be found that there is one step need to be supported by RAN3 in F1

- Step 1: After gNB1-CU receiving strongest DL beam information in the context of NZP-CSI-RS/SSB from gNB2-CU, gNB1-CU sends strongest DL beam information to gNB1-DU.

The strongest DL beam information can be added in Neighbour Cell Information List in GNB-CU CONFIGURATION UPDATE message.

Proposal 3: Add strongest DL beam information in a Neighbour Cell Information List in GNB-CU CONFIGURATION UPDATE.

  - CLI-mitigation request

- Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling

- Notes: Following are assumed for information exchange among gNBs

  - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3

  - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact

The gNB1 sends the CLI-mitigation handling request, the gNB2 perform the CLI-mitigation measurement reporting to gNB1. The RESOURCE STATUS REQUEST/RESPONSE/UPDATE procedure can be considered to reuse or introduce a new signalling procedure to adapt more flexible request. In our understanding, reuse the existing one is enough to support per cell CLI-mitigation request.

Proposal 4: For CLI-mitigation, RAN3 discuss whether to reuse NG-RAN NODE CONFIGURATION UPDATE procedure or introduce a new procedure.

- Conclusion

Proposal 1: Add NZP CSI-RS resource in Served Cell Information NR in Xn.

Proposal 2: RAN3 discuss how neighbour cells response the strongest DL beam information in Xn i.e., periodic response or one-off response or response base on event trigger.

Proposal 3: Add NZP CSI-RS resource in a Neighbour Cell Information List in GNB-CU CONFIGURATION UPDATE.

Proposal 4: For CLI-mitigation, RAN3 discuss whether to reuse NG-RAN NODE CONFIGURATION UPDATE procedure or introduce a new procedure.

- Reference

[1] R1-2407533, LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation, RAN1.