3GPP TSG-RAN WG3 #125bis                                     R3-245189

Hefei, China, 14th – 18th Oct, 2024

Source:	CATT

Title:	Discussion on XR LS R3-245011 and R3-245017

Agenda Item:	8.1

Document for:	Discussion and decision

- Introduction

RAN3 receives two LS from RAN2 and SA2 about UL PSI based PDU discarding and multi-modality awareness separately. This paper discusses them from RAN3 perspective.

- Discussion

  - R3-245011 UL PSI based PDU discarding

RAN2 thank RAN3 for the questions on PSI-based SDU discard on UL split bearer in NR-DC. The following is the current UE behavior:

- UE activates PSI-based SDU discard on a UL split-bearer upon reception of a PSI-Based SDU Discard De-/Activation MAC CE activating the discard, regardless of whether the MAC CE is received from MN or SN;

- UE deactivates PSI-based SDU discard on a UL split-bearer upon reception of a PSI-Based SDU Discard De-/Activation MAC CE deactivating the discard, regardless of whether the MAC CE is received from MN or SN.

It is up to RAN3 to discuss whether any network side solution to improve PSI-based SDU discard is needed.

As described by RAN2, the UE will activate or deactivate PSI-based SDU discarding on a UL split bearer regardless of whether the MAC CE is received from the MN or the SN. It may cause UE receives two different status of PSI-based SDU discarding e.g., to activate from MN while to deactivate from SN.

Based on the current mechanism, the PSI-based SDU discard can work well, which is similar as PDCP duplication that UE executes the PSI-based SDU discarding based on the latest MAC CE.

Observation 1: Without any enhancement, PSI-based SDU discard can work well which is similar as PDCP duplication.

UL PSI based discard coordination for split bearer between MN and SN will not be considered for NR-DC. FFS whether MN/SN can inform SN/MN about the (de)activation of the UL PSI based discard for the split bearer.

An enhancement considered by RAN3 was to exchange the status of PSI-based SDU discarding between the MN and the SN which implicitly indicates the congestion status of each other. If the SN knows the MN is congested i.e., the MN actives PSI-based SDU discarding, the SN can handle UL split bearer traffic more proactively.

However, we do not consider it necessary—we believe that the UE is capable to balance the load on MCG and on SCG for UL split bearers, just like the (PDCP) hosting node capable to balance the load for DL split bearers.

For detail, if one cell group is congested—assume it to be the MCG, the MN will allocate less resource than the UE requested. As a result, the UE will automatically turns to the SCG, i.e. increase the volume of requested resource on the SCG. If the SCG is not congested the SN will allocated enough resource, and thus every UL packet can be delivered successfully and there is no need to discard anything. If the SCG is congested as well it will not allocated enough resource for the UE, so there will be necessity of discarding. After a while both the MN and the SN will find the UE keep requesting more resource than either one can provide, and thus activates the PSI-based discarding. Such mechanism does not require any direct coordination between the MN and the SN, since this work has been done at the UE.

Proposal 1: No RAN3 specification enhancement is needed for improving PSI-based SDU discard.

If proposal 1 is agreeable, there is no reply LS is needed to RAN2.

  - R3-245017 Multi-modality Awareness

SA2 specified policy control enhancements to support multi-modal services in Rel-18, with the use of Multi-modal Service ID (MMSID) in AF and PCF, please see TS 23.501 clause 5.37.2 for details.

Forwarding MMSID to RAN via control plane is in principle technically feasible, but needs confirmation from RAN3 on the NG-RAN protocols impacts.

Also, SA2 would like to first understand the usage RAN2 has envisioned with multi-modality awareness (i.e., the foreseen NG-RAN behaviour for "coordinated handling of the flows at NG-RAN") before SA2 can decide whether to provide the MMSID to RAN as part of Rel-19 alignment work. Such feedback from RAN2 needs to also be assessed by RAN3 and SA4 to confirm the applicability and usage of this information.

From technical point view, forwarding MMSID to RAN via control plane is feasible. The use case is UE mobility, providing MMSID to NG-RAN could help network to perform admission control. If target cell cannot accept all PDU set for multi-modality, the handover can be rejected by target.

Proposal 2: RAN3 confirm that forwarding MMSID to RAN via control plane is technically feasible. It can be beneficial to admission control in NG-RAN.

- Conclusion

Observation 1: Without any enhancement, PSI-based SDU discard can work well which is similar as PDCP duplication.

Proposal 1: No RAN3 specification enhancement is needed for improving PSI-based SDU discard.

Proposal 2: RAN3 confirm that forwarding MMSID to RAN via control plane is technically feasible. It can be beneficial to admission control in NG-RAN.

- Reference

[1] R3-245011, Reply to RAN3 LS on UL PSI based PDU discarding in NR-DC, RAN2.

[2] R3-245017, LS on multi-modality awareness at RAN, SA2.