3GPP TSG-RAN WG3 Meeting #125bis														R3-245193

Hefei, China, Oct 14th - Oct 18th, 2024

Agenda item:		11.2

Source:				CMCC

Title:	Discussion on AI-based network slice

Document for:		Discussion

Introduction

During RAN#105, new WI “Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” was approved [1].

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743. The detailed objectives of the WI are listed as follows: - Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing and AI/ML based CCO. [RAN3] - Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]: - Mobility Optimization for NR-DC - Split architecture support for Rel-18 use cases - Continuous MDT collection targeting the same UE across RRC states

In this paper, we provide our consideration of AI/ML based Network Slicing.

Discussion

During the study phase, RAN3 has agreed that the locations of AI/ML model training and inference are the same as R18, and the input/output/feedback information had been captured in TR 38.743. Compared with the R18 phase, RAN3 enhanced the prediction information obtained from local/neighboring nodes to slice level, and finer granularity of UE performance feedback was agreed to be introduced.

The SI phase mainly focuses on the solution discussion of slice use case, while for the WI phase, we believe that we need to first focus on the standard impacts of the solution and enhance the Xn/F1 interface to support the collection of the predicted/feedback information. Therefore, for the start point of R19 WI, we propose RAN3 first identifies the standard impacts of the solution.

Proposal 1: RAN3 focuses on the standard impacts of the solution and enhance the Xn interface first in R19 WI.

Then, the potential standard impacts in TR 38.743 mainly describes the enhancement of Xn interface for the slice use case. So, we propose the WI phase to focus on the non-split architecture for AI/ML-based network slicing use case first.

Proposal 2: Focus on the non-split architecture for AI/ML-based network slicing use case first.

Input/Output/feedback

In R18, RAN3 has introduced Data Collection Reporting Initiation and Data Collection Reporting procedures for AI/ML related data transmission, including predicted radio resource status, predicted UE trajectory, etc. Therefore, for the R19 slice use case, we propose to reuse the Data Collection Reporting Initiation and Data Collection Reporting procedures to support the new input/feedback information mentioned in TR 38.743, such as predicted radio resource status per slice and predicted slice available capacity.

Proposal 3: Reuse Data Collection Reporting Initiation and Data Collection Reporting procedures for Xn enhancement, and consider further enhancement to support R19 slice use case.

Input information:

Then, for the input information, slice use case introduces the predicted radio resource status per slice and predicted slice available capacity. As the Data Collection Reporting Initiation procedure already included the ‘Predicted Radio Resource Status’, and the ‘Predicted Radio Resource Status’ is referred to the ‘Radio Resource Status’ IE, which already including the radio resource status per slice, only the semantics description is needed to be updated to include the per slice level radio resource status.

Proposal 4: Update the semantics description of ‘Predicted Radio Resource Status’ IE in Data Collection Reporting procedure to support the slice use case.

For the predicted slice available capacity, we propose to introduce a new IE in Data Collection Reporting procedure, and introduce a new bit in ‘Report Characteristics’ IE in Data Collection Reporting Initiation procedure to support this information.

Proposal 5: Introduce a new IE in Data Collection Reporting procedure to include predicted slice available capacity information.

Proposal 6: Introduce a new bit in ‘Report Characteristics’ IE of Data Collection Reporting Initiation procedure to include predicted slice available capacity information.

When using the Data Collection Reporting Initiation procedure to obtain AI/ML related information, the local node should inform the neighbour nodes which slice’s predicted information is required. If the local node does not indicate the neighbour node which slice’s information it wants to obtain, the neighbour node may confuse about how to provide the requested information, or provide the requested information about all slices. Considering that there may be many slices supported by the neighbour node, if all the slices’ predicted information are collected and reported by the neighbour nodes, the total amount of collected data is large and some of the data is not required by the local node. Therefore, we propose to indicate in the Data Collection Reporting Initiation procedure which slice's predicted information is required by the local node.

Observation 1: The local node should inform the neighbour nodes which slice’s predicted information is required, which can prevent the neighbour nodes from confusing about which slices’ predicted information needs to be provided, or reporting information about all slices.

Proposal 7: Indicate in the Data Collection Reporting Initiation procedure which slices' predicted information are required by the local node.

Feedback information:

In R18, the feedback information is configured by Data Collection Request message and reported via Data Collection Update message, such as the UE performance, measured UE trajectory and energy cost. In R19 SI phase, the agreed feedback information of slice use case including measured radio resource status per slice, measured slice available capacity, legacy UE performance and finer granularity of UE performance feedback. From our perspective, the feedback information of R19 can reuse the R18 feedback information reporting mechanism, using Data Collection Request message to configure and reporting via Data Collection Update message.

Observation 2: For how to transfer the feedback information, reuse the R18 feedback information reporting mechanism, using Data Collection Request message to configure and reporting via Data Collection Update message.

Proposal 8: RAN3 agrees to transfer the measured feedback information via Data Collection Reporting procedure, e.g. measured radio resource status per slice and measured slice available capacity information.

Finer granularity of UE performance feedback

For the feedback information, the legacy UE performance feedback for those UEs handover from source gNB has agreed to be collected. But for slice use case, the UE level performance feedback is not enough. Also, during RAN3#125, RAN3 has agreed to introduce finer granularity UE performance feedback, but companies do not reach consensus on the granularity to support slice UE performance.

Four level UE performance feedback have been discussed in RAN3#125, including slice level, PDU session level, DRB level and QoS flow level. As mentioned in the discussion, QoS flow level is not measurable and DRB level has the remapping issue, slice level is preferred by majority companies. Except the slice level UE performance feedback, we think the PDU session level UE performance feedback (i.e. the service performance) can also be discussed, which can be used to evaluate whether the handover based on the AI/ML model inference make UE experience severe service performance degradation.

Observation 3: The PDU session level UE performance feedback can be used to evaluate the service performance after handover.

Proposal 9: RAN3 discusses the following granularity UE performance feedback:

- UE performance feedback per slice per UE

- UE performance feedback per PDU session

Based on the above discussion, we provide a TP for TR38.423 in R3-245194.

Proposal 10: Agree the TP in R3-245194 for 38.423.

Conclusion

In this contribution, we propose the following proposals:

Proposal 1: RAN3 focuses on the standard impacts of the solution and enhance the Xn interface first in R19 WI.

Proposal 2: Focus on the non-split architecture for AI/ML-based network slicing use case first.

Proposal 3: Reuse Data Collection Reporting Initiation and Data Collection Reporting procedures for Xn enhancement, and consider further enhancement to support R19 slice use case.

Proposal 4: Update the semantics description of ‘Predicted Radio Resource Status’ IE in Data Collection Reporting procedure to support the slice use case.

Proposal 5: Introduce a new IE in Data Collection Reporting procedure to include predicted slice available capacity information.

Proposal 6: Introduce a new bit in ‘Report Characteristics’ IE of Data Collection Reporting Initiation procedure to include predicted slice available capacity information.

Observation 1: The local node should inform the neighbour nodes which slice’s predicted information is required, which can prevent the neighbour nodes from confusing about which slices’ predicted information needs to be provided, or reporting information about all slices.

Proposal 7: Indicate in the Data Collection Reporting Initiation procedure which slices' predicted information are required by the local node.

Observation 2: For how to transfer the feedback information, reuse the R18 feedback information reporting mechanism, using Data Collection Request message to configure and reporting via Data Collection Update message.

Proposal 8: RAN3 agrees to transfer the measured feedback information via Data Collection Reporting procedure, e.g. measured radio resource status per slice and measured slice available capacity information.

Observation 3: The PDU session level UE performance feedback can be used to evaluate the service performance after handover.

Proposal 9: RAN3 discusses the following granularity UE performance feedback:

- UE performance feedback per slice per UE

- UE performance feedback per PDU session

Proposal 10: Agree the TP in R3-245194 for 38.423.

- References

[1] RP-242385, “New WID: Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, ZTE, NEC