3GPP TSG-RAN WG3 #125bis	              R3-245200

Hefei, China, 14th – 18th Oct 2024

Agenda Item:	18.2

Source:	CMCC

Title:	Discussion on Low-power wake-up signal and receiver for NR

Document for:	Discussion

- Introduction

Rel-19 NR LP-WUS/WUR WI was approved in [1], which states that in RRC IDLE/INACTIVE modes, it's observed that significant UE power saving gain (up to more than 90%) is obtained by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI). We will discuss on the following objectives in this contribution.

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

In RAN2#125bis, RAN2 agreed that the PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

- Discussion

2.1 CN assigned subgrouping

Functionally, in RRC IDLE/INACTIVE modes, LP-WUS is very similar to PEI. Both of them give an indication before the UE detects paging, and only the UEs in the corresponding group need to receive the subsequent paging message. According to TS 38.300, the procedure for CN controlled subgrouping in PEI is shown in the figure below:

Figure 9.2.5-1: Procedure for CN controlled subgrouping

Referring to the figure above, similarly, in the CN-assigned subgrouping of LP-WUS, some signalling should be supported between AMF and gNB(s) to inform gNB(s) about the related subgroup information. So, we propose to introduce UE LP-WUS Subgroup Information IE into PAGING message and Core Network Assistance Information IE for RRC INACTIVE IE over NGAP for waking up a UE by LP-WUS signal in RRC_IDLE/RRC_INACTIVE, where UE LP-WUS subgroup information IE includes at least UE LP-WUS Subgroup ID. Furthermore, the number of CN assigned subgroups is same among all cells within the registration area, the total number of CN-assigned subgroups can be configured by OAM. As for the number of subgroups in LP-WUS, we still need to wait for RAN1's ​​conclusion on payload and indication method (e.g. codepoint or bitmap).

Proposal 1: For NGAP, introduce UE LP-WUS Subgroup Information IE into PAGING message and Core Network Assistance Information IE for RRC INACTIVE IE over NGAP for waking up a UE by LP-WUS signal in RRC_IDLE/RRC_INACTIVE, where UE LP-WUS subgroup information IE includes at least UE LP-WUS Subgroup ID.

In order to support subgrouping efficient, similar to the solution in PEI, the "Assistance information to CN" can be carried by NAS signalling to CN, No RAN3 impact expect here.

Considering F1 paging, the UE LP-WUS Subgroup ID is also needed in F1 paging message for waking up a UE by LP-WUS signal in RRC_IDLE/RRC_INACTIVE. So, we propose to introduce UE Paging Subgroup Information IE into PAGING message over F1AP.

Proposal 2: For F1AP, introduce UE Paging Subgroup Information IE into PAGING message.

2.2 UE-ID based subgrouping

According to TS 38.300, the procedure for UE ID based subgrouping in PEI is shown in the figure below:

Figure 9.2.5-2: Procedure for UE ID based subgrouping

The same approach can still be used in LP-WUS subgrouping. The number of UE-ID based subgroups is controlled on a cell basis and can be different in different cells, OAM can configure such group number per cell, for CU/DU split gNB, the DU can configure such number for cells. Furthermore, UEs capability of supporting the UE-ID based subgrouping of LP-WUS can be reported to RAN by AS UE capability signalling. But if CN subgroup ID for LP-WUS is not provided, the gNB-DU shall know whether the UE supports UE-ID based subgrouping or not, then DU decides using UE-ID based subgrouping waking up by LP-WUS signal or PEI or legacy paging. So, it is proposed to add UE-ID based subgrouping support indication for LP-WUS in F1 paging message.

Proposal 3: For F1AP, introduce LP-WUS UE-ID based subgrouping support indication IE into PAGING message.

2.3 Co-exist of CN-assigned and UEID-based subgrouping

For LP-WUS, RAN2 agreed both UE ID-based and CN-assigned subgrouping can be supported simultaneously in a cell, but CN-assigned subgroups shall not be remapped to UE-ID based subgroup. For example, UE-ID based subgrouping can only utilize the rest of LP-WUS indication capacity (subgroupsNumPerPO-X subgroups assigned by CN). The SIB message can be used to indicate the total number of UE-ID based subgroups supported by the cell, and the gNB or DU can know the total number of CN-assigned subgroups via OAM, so no RAN3 impact here at this stage.

Proposal 4: For LP-WUS, no RAN3 impact on co-exist of CN-assigned and UE-ID based subgrouping.

2.4 LP-WUS related configuration over F1 interface

RAN2 already concluded that the LP-WUS configuration for RRC_IDLE/INACTIVE UEs should be carried in system information, as given as follows.

RAN2#126 Agreement The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - LP-SS configuration - LP-WUS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

In the CU/DU split case, RAN3 can discuss whether the LP-SS/LP-WUS configuration is generated by the DU or the CU. Since LP-SS configuration and LP-WUS configuration are more related to lower-layer resources, it seems more direct for the DU to generate the relevant configuration. But from another perspective, if LP-SS is not sent per cell, it may be more reasonable for the CU to generate the configuration. In addition, this situation may also involve the transmission of LP-SS configuration through the F1 interface and the Xn interface.

Proposal 5: RAN3 discuss the LP-SS and LP-WUS configuration is generated at the gNB-DU or the gNB-CU, which may be related to whether LP-SS is sent per cell.

RAN2#127 agreed that the network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE. Hence RAN3 can conclude that no further impacts when the idle/inactive UE enters or leave the LP-WUS monitoring state.

- Conclusions

In this contribution, we discussed supporting LP-WUS indicating paging monitoring in RRC_IDLE/INACTIVE mode, and following are the proposals.

Proposals:

Proposal 1: For NGAP, introduce UE LP-WUS Subgroup Information IE into PAGING message and Core Network Assistance Information IE for RRC INACTIVE IE over NGAP for waking up a UE by LP-WUS signal in RRC_IDLE/RRC_INACTIVE, where UE LP-WUS subgroup information IE includes at least UE LP-WUS Subgroup ID.

Proposal 2: For F1AP, introduce UE Paging Subgroup Information IE into PAGING message.

Proposal 3: For F1AP, introduce LP-WUS UE-ID based subgrouping support indication IE into PAGING message.

Proposal 4: For LP-WUS, no RAN3 impact on co-exist of CN-assigned and UE-ID based subgrouping.

Proposal 5: RAN3 discuss the LP-SS and LP-WUS configuration is generated at the gNB-DU or the gNB-CU, which may be related to whether LP-SS is sent per cell.

- References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)