3GPP TSG-RAN3 Meeting #125bis	R3-245246

# Hefei, China, 14 – 18 Oct 2024

Agenda Item:	13.2

Source:	CATT

Title:	(TP for LTM BLCR for TS38.300) Inter-CU LTM basic procedure

Document for:	Decision

- Introduction

This contribution contains a TP for LTM BLCR [1] for TS38.300.

- TP for LTM BLCR for TS38.300

##### 9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one candidate cell belonging to candidate gNB(s).

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response HO REQUEST ACKNOWLEDGE to the source gNB.

6.	The source gNB may request LTM configuration update to the candidate gNB(s) and send the collected information for subsequent LTM.

Editor’s Note: Details are FFS on step 6.

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE to response the source gNB.

Editor’s Note

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. Early TA acquisition to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) send the TA INFORMATION TRANSFER message to the source gNB.

Editor’s Note: Details are FFS on step 10/11.

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to execute inter-gNB LTM.

14.	The source gNB decides to send cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

Editor’s Note: Details are FFS on above note.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB.

20. The target gNB may send the UE CONTEXT RELEASE to inform the source gNB to release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

Editor’s Note: Details are FFS on step 20.

.

3. Reference

[1]	R3-244836 Inter-CU LTM basic procedure