3GPP TSG-RAN WG3 Meeting #125bis                                                             R3-245252

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Discussion on stage-2 aspects for WAB

Agenda Item:	12.2

Document for:	Discussion and Decision

1 Introduction

In RAN3#125 meeting, study of additional topological enhancements for NR was finished, and the conclusion for Wireless Access Backhaul (WAB) is provided in the TR 38.799 [1].

During RAN#105 meeting, the following objectives of WAB is captured in the WID [2] based on the recommended scope in the TR:

The objectives of the Wireless Access Backhaul (WAB) work are as follows: - Specifications for the support of WAB including [RAN3]: - Support of a WAB-node including a WAB-gNB and a WAB-MT. - Support of backhauling of the WAB-gNB’s NG, Xn and OAM traffic over the WAB-MT’s PDU session(s). - Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs. - Defining the behaviour of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes. - Network integration procedures for WAB nodes. - Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes. - Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB. - UE’s ULI that reflect the WAB node’s location. - The handling of: - PCI collision avoidance. - Reconfiguration of TAC and RANAC on WAB-gNBs. - Mechanisms to avoid multi-hop WAB topology. - Radio-resource coordination between access and backhaul links. - NG connection management. NOTE 1: For PCI collision avoidance and reconfiguration of TAC and RANAC on WAB-gNBs, follow the conclusion of mobile IAB. NOTE 2: NG connection management should take the NTN conclusion into account, avoiding parallel discussions. NOTE 3: No impact on the UE. NOTE 4: Coordination with other WGs (e.g. SA2, RAN2) when needed. NOTE 5: Backhaul link for WAB-MT can be TN or NTN. NOTE 6: Mobility procedures to be used for the UEs served by a WAB-gNB are legacy UE mobility procedures. Mobility of the WAB-MTs is based on legacy UE mobility procedures. NOTE 7: The interface between the WAB-MT and the co-located WAB-gNB is out-of-scope for the normative phase. NOTE 8: Split architecture of the WAB-gNB is out-of-scope for the normative phase. NOTE 9: RAN2 impact should be identified as early as possible, and should be minimal.

In this contribution, we will discuss the impact on stage-2 (TS38.300) for WAB.

2 Discussion

2.1 WAB Architecture

In the TR [1], the definition, architecture and protocol stack examples of WAB are generally OK. We propose that the architecture part in stage-2 can follow the definition, architecture and protocol stack examples of WAB in the TR. Refinement or supplement would be needed. For example, following figure should also be captured in stage-2 spec:

Proposal 1: The architecture part of stage-2 spec follows the definition, architecture and protocol stack examples in TR 38.799 with refinement or supplement if necessary.

2.2 Backhauling of WAB-gNB’s traffic

According to the WID [2], support of backhauling of the WAB-gNB’s NG, Xn and OAM traffic over the WAB-MT’s PDU session(s) needs to be specified. The interface between the WAB-MT and the co-located WAB-gNB is out-of-scope for the normative phase. On the other hand, the following conclusion is provided by SA2 [3]:

- If QoS optimization of the N3 connection is required, then the MWAB-gNB is configured (e.g. by OAM) with the mapping of the 5QIs to the DSCP field for TNL used by the UPFs/gNBs in the PLMN/SNPN serving the UE via the MWAB-gNB. This is then used to establish SDFs at the BH-PDU session used for N3 connection(s) based on existing procedures.

The QoS mapping of WAB-gNB’s traffic to the BH PDU session(s), the QoS mapping based on the mapping of 5QI to the DSCP field for TNL used by the UPF/gNB serving the UE, should be captured.

Proposal 2: Capture support of backhauling of WAB-gNB’s traffic over WAB-MT’s PDU session in stage-2 spec, including how to do the QoS mapping of WAB-gNB’s traffic to the BH PDU session.

2.3 WAB authorization

The behavior of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes should be specified. The TR 38.799 [1] has captured the WAB authorization part, where authorization of the WAB-MT is independent with the WAB-gNB service authorization. However, the TR only includes the behavior of WAB-node when the status of WAB-gNB authorization is changed from “authorized” to “not authorized”. The behaviors of WAB-node in case the authorization status is changed from “not authorized” to “authorized” and the authorization status of WAB-MT changes are not included. If the authorization status of WAB-gNB is changed from “authorized” to “not authorized”, the WAB-gNB can remove the NG/Xn connections after releasing/handing over all the UEs, but keep the SCTP connections alive, as the BH PDU session providing the IP connectivity is still available. If the authorization status of WAB-gNB is changed from “not authorized” to “authorized”, the WAB-gNB should re-establish NG connections (and Xn connections if needed).

The behaviors of WAB-node for the authorization status change of WAB-MT are similar with those in case authorization status of WAB-gNB is changed. The difference is that the WAB-node needs to release the BH PDU session(s) of the WAB-MT if the authorization status of WAB-MT is changed to “not authorized”, and thus the WAB-node should shut down the SCTP connections after removing the NG/Xn connections. Accordingly, the WAB-node should re-establish the BH PDU session(s) if the authorization status of WAB-MT is changed to “authorized”.

The stage-2 spec should capture the behavior of WAB-node for authorization status of WAB-gNB/WAB-MT changing.

Proposal 3-1: Capture the WAB-node behaviors in case authorization status of WAB-gNB/WAB-MT changes in stage-2 spec.

Proposal 3-2: Capture in stage-2 spec that the WAB-node needs to shut down the SCTP connections and release the BH PDU session(s) of the WAB-MT if the authorization status of WAB-MT is changed to “not authorized”, and re-establish the BH PDU session(s) if the authorization status of WAB-MT is changed to “authorized”.

2.4 Avoid multi-hop WAB topology

According to the WID [2], the mechanism to avoid multi-hop WAB topology should be specified. And one LS [4] was sent from SA2 for asking RAN to discuss several issues that included the issue to restrict WAB-MT accessing a WAB-gNB cell:

S2-2407345 Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

IDLE or INACTIVE mode

For case a) and b), there are two potential solutions:

- Solution 1: BH-AMF does not accept a WAB-MT accessing via a WAB-gNB during network integration

As BH-AMFs can recognize WAB-MT via subscription data, BH-AMF can decide not to authorize or refuses the registration request for the WAB-MT if it accesses via a WAB-node.

- Solution 2: WAB-MT does not select WAB-gNB cells

Alt.1: WAB-MT does not select the cell broadcasting a WAB-Cell indicator.

This alternative requires each WAB-gNB cell broadcast the WAB-Cell indicator. WAB-MT does not camp on a WAB cell, or deprioritize the reselection to a WAB cell. During cell selection/reselection, a WAB-MT can consider the cell broadcasting the WAB-Cell indicator as barred so that it does not camp on a WAB cell when it is operating as WAB-node. Alternatively, the WAB-MT can consider the frequency of the cell as lowest priority to deprioritize reselection to the cell.

Alt.2: WAB-MT only select the cell with “WAB-support” indication

This alternative requires all cells supporting the WAB to broadcast a “WAB-support” indication. And WAB-gNBs should not broadcast “WAB-support” indication. WAB-MT can only camp on the cell which broadcasts “WAB-support” indication. Based on this, the WAB-MT will not select the cells of WAB-gNB that multi-hop topology can be avoided.

- Solution 3: WAB-gNB refuses the access of WAB-MT

WAB-MT reports a WAB-node indication by RRC. The WAB-node indication can be explicitly included in the RRCSetupComplete message or implicitly derived via the UECapabilityInformation message. WAB-gNB can release the UE’s if the UE reports a WAB-node indication by the RRC signalling.

Per SA2’s latest conclusion [3], the WAB-gNB indicates to AMFs that it is a WAB-node when establishing NG connections to the AMFs. So, BH-AMF can recognize whether a WAB-MT is accessing via a WAB-gNB. That means Solution 1 is feasible in principle.

Compared with Solution 1, Solution 2 can prevent the multi-hop before the network integration procedure through WAB-MT’s cell (re)selection. However, the BH-RAN nodes should be upgraded to broadcast “WAB-support” indication for Alt.2. This requirement is applied to all NG-RAN nodes except the WAB-gNBs. On the contrary, Alt.1 only requires enhancement to the WAB-node. On solution 3, upgrade to existing NG-RAN nodes also the awareness of WAB by BH-RAN is required. So, only alt.1 of solution 2 is preferred. How to specify the WAB-Cell indicator and potential enhancements with respect to the WAB-cell indicator can be left to RAN2 discussion.

Observation 1: To avoid multi-hop WAB topology, BH-AMF does not accept WAB-MT’s access to a WAB-gNB during integration procedure. For performance optimization, an idle/inactive mode WAB-MT does not (re)select to WAB-gNB cells.

CONNECTED mode

For case a) and case b), two ways to prevent handover of connected mode WAB-MT to a WAB-node:

- Solution 1: BH-AMF does not accept a WAB-MT handing over to a WAB-gNB

BH-AMF de-authorizes the WAB-MT or refuses the handover of WAB-MT, if target RAN-node is a WAB-node.

NG based handover:

On reception of Handover Required message for WAB-MT from source node, if the target node is a WAB-node, the BH-AMF can refuse the handover requirement or turn the WAB-MT from “authorized” to “non-authorized” after handover. If WAB-MT is handed over to the WAB-node and turned to “non-authorized”, all BH PDU sessions should be release.

Xn based handover:

BH-AMF does not know the handover is pertaining to a WAB-MT until path switch procedure. Upon receiving the path switch request, BH-AMF can turn the WAB-MT from “authorized” to “non-authorized” or deregister the WAB-MT if the target node is a WAB-node.

- Solution 2: WAB-MT does not report measurement results of WAB-gNB cells

This solution requires WAB cells to broadcast a WAB-Cell indicator. WAB-MT does not report the measurement results of the cell(s) broadcasting the WAB-Cell indicator, and thus the source BH-RAN-node will not select the WAB cells as the target cells. How to make the WAB-MT skip reporting measurement of WAB cells is up to RAN2 decision. To reduce the WAB-MT complexity on reading the broadcast information of WAB cells, the gNB can informs the neighbouring WAB cell(s) to the WAB-MT, which is also up to RAN2.

- Solution 3: BH RAN does not carry out handover of WAB-MT to WAB-gNB

Alt.1: Source node does not initiate the handover

This alternative is used for Xn handover. It requires WAB-MT reporting a WAB-node indication via RRC and the WAB-gNB needs to exchange a WAB indication over Xn interface. The WAB-node indication can be explicitly included in the RRCSetupComplete message or implicitly conveyed in the UECapabilityInformation message.

Alt.2: WAB-gNB node does not accept the handover

For WAB-gNB to recognize a WAB-MT, it requires WAB-MT reporting a WAB-node indication via RRC and an indication of WAB-node included in the Handover Request message over Xn/NG interface. The WAB indicator would be an explicit indication in RRC as well as Xn/NG interfaces, or it can be implicitly derived in UE capability information.

Solution 1 can be supported by core network without effort of RAN, given WAB-gNBs can send the WAB-node indicator to the AMF. Compared with solution 1, solution 2 and 3 actively avoid the handover failure due to WAB-MT is handed over to WAB-gNB. Solution 2 only requires enhancement to the WAB-node. In solution 3, upgrade to the legacy NG-RAN nodes also the awareness of WAB by the legacy NG-RAN nodes is needed. Therefore, solution 2 is preferred in our view.

Observation 2: To avoid multi-hop WAB topology, BH-AMF does not accept the WAB-MT’s handover to WAB-gNBs. For performance optimization, a connected mode WAB-MT does not report measurement results of WAB cells.

Proposal 4-1: Multi-hop WAB topology can be avoided by BH-AMF not accepting WAB-MT’s access/ handover to a WAB-gNB. For performance optimization, an idle/inactive WAB-MT does not (re)select to WAB cells and connected WAB-MT does not report measurement results of WAB cells.

RAN3 should send the response LS to SA2 on the conclusion on multi-hop topology avoidance and send a LS to RAN2 on making WAB-MT does not select WAB cells and not report measurement results of WAB cells.

Proposal 4-2: RAN3 send response LS to SA2 and a request LS to RAN2 on multi-hop topology avoidance as Annex.

2.5 WAB-node integration procedure

In the TR [1], the WAB-node integration procedure includes two phases, WAB-MT setup and WAB-gNB setup. The WAB-gNB setup phase consists of three steps, WAB-gNB initialization, NG connection setup, Xn connection setup.

The statements in TR 38.799 for integration procedure are basically OK. However, per SA2’s latest conclusion [3], the WAB-gNB indicates to AMFs that it is a WAB-node when establishing NG connections to the AMFs. That should be a mandatory operation and therefore needs to be added to the NG connection setup phase of the integration procedure.

Proposal 5: Capture the WAB-node integration procedure in the TR to stage-2 spec, and WAB-node indicates it is a WAB-node during the NG connection setup phase of WAB-node integration procedure.

2.6 Handling the change of BH PDU session

According to WID [2], handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes, needs to be specified. During the WAB-node’s movement, the WAB-MT’s BH PDU session used for backhauling the WAB-gNB’s traffic may be re-established, for example, due to moving to a new region or moving between TN and NTN, that will cause the (outer) IP addressed used for transmitting the WAB-gNB’s traffic being changed. The existing mechanisms defined in TS 23.501/TS 23.502 can be used for re-establishment of BH PDU session(s).

WAB-gNB need not know the reason why the BH PDU session is re-established. However, the procedures for TNL address update for SCTP connection of the WAB-gNB’s would be needed. The WAB-gNB may have to establish new SCTP connections using the new TNL address for NG/Xn connection. Besides that, on NG interface, the WAB-gNB initiates RAN Configuration Update procedure/PDU Session Resource Modify Indication procedure to update the TNL addresses for NG-C and NG-U respectively. On Xn interface, the WAB-gNB can initiate NG-RAN Node Configuration Update procedure to update the TNL addresses for Xn-C and Xn-U.

Proposal 6: Capture in stage-2 spec that WAB-gNB may update the TNL addresses used for NG or Xn interface using the new IP address of the BH PDU session when the BH PDU session is re-established.

2.7 Support the UE’s AMF change

The following is concluded by SA2 [3]:

- The MWAB instantiates a new logical MWAB gNB when it is configured with a new TAC (and possibly Cell ID ) and with a different gNB ID. This may be used to support migrating / handing over of UEs served by the MWAB to the new logical gNB. Once all UEs have been migrated, the MWAB removes the old logical MWAB-gNB instance. This can happen e.g. when AMF, serving the UEs of the MWAB, change is required based on the operator policy. NOTE 4: This aspect will be coordinated with RAN WG3. Whether the gNB ID will be changed will be reviewed in normative phase based on RAN WG3 input taking into account potential AMF impacts. NOTE 5: The solution based on instantiating multiple virtual cells by the same MWAB-gNB needs to wait for RAN WG3 confirmation.

It can be observed that the solution captured in conclusion of SA2 for changing UE’s AMF is based on the scenario where the gNB ID is configured with a different gNB-ID when AMF with which the WAB-gNB connects changes from old AMF to a new AMF. But most probably the gNB-ID of WAB-gNB is not changed in that case.

Observation 3: The gNB-ID of WAB-gNB is probably not changed in case UE’s AMF is changed.

On the other hand, three options were generated for single gNB solution in TR 38.799 [1].

Option 1: Single WAB-gNB with a single cell using mobility registration update due to TAC change.

Option 2: Single WAB-gNB with two cells with different TACs, using NG-based HO.

Option 3: Single WAB-gNB single cell without TAC change.

Per SA2 conclusion [3], the TAC(s) and Cell ID(s) the WAB-gNB broadcasts should also change in that case, due to the new AMF(s) take charge of different TAs compared to the old AMF(s) and the new Cell ID(s) reflect the new geolocation of the WAB-node. So, regardless of whether two cells are powered up concurrently, TAC will change when AMF changes. Option3 should be excluded.

It is also concluded by SA2 [3] that the change of Cell ID (including gNB ID) triggers NG-based handover of connected UE(s) (the old AMF decides the new AMF during the NG HO preparation). On option 2, the procedure for changing the UE’s AMF is exactly the same as the two logical gNBs solution. The only difference is that the gNB-ID part of the cell ID of the new cell is the same as that of the old cell, and therefore it’s no need to instantiate a new logical gNB. Similar with the two logical gNB solution, the WAB-gNB can power up two cells corresponding to the old and new configurations respectively to support an interruption-less NG-based HO and shut down the old cell after all UEs’ handovers.

Option 1 has already been discussed by SA2 during the SI phase, however, it was not adopted as solution for UE’s AMF change (according to SA2’s conclusion on the WAB mobility issue). There is no need to continue Option 1 in normative phase.

Therefore, it can be concluded that Option 2 can be used to support changing UE’s AMF when WAB-gNB’s gNB-ID remaining unchanged.

Proposal 7: Capture the two logical gNB solution and Option 2 for single gNB solution in stage-2 spec.

2.8 NG connection management

During the WAB-node movement, if the AMF(s) that WAB-gNB should connect and the TAI(s) broadcasted by the WAB-gNB change, the WAB-gNB will not serve the old TAI(s) and the old AMF(s) any more. The WAB-gNB may stop using the NG connection with the old AMF(s) after all the UEs connected to the old AMF(s) are handed over to the new AMF(s). The old AMF(s) are still reachable because the BH PDU session providing the IP connection to NG connections with the old AMF(s) is available. The removal of NG connection during WAB-node’s movement has been agreed. The WAB-gNB can remove the NG connections and keep the underlying SCTP connection alive.

In the last meeting [1], some companies suggested that NG connections can be suspended. Since the WAB-node does not serve the old AMF(s) anymore, the old AMF(s) should not use the corresponding NG connections, e.g., sending paging messages to the WAB-node. Therefore, the WAB-node has to notify the old AMF(s) that the NG interface is not available through either new NGAP signalling or existing NGAP signalling (i.e., implicitly). Also, the WAB-node has to inform those AMF(s) to resume the NG connection when the WAB-node moves back. From our perspective, given the removal of NG connection is adopted, there is no need to introduce NG connection suspension.

Noted that NG connection removal is approved by NTN, while NG connection suspend is FFS for NTN. We need be aligned with NTN. Whether to introduce NG suspension procedure can wait for NTN's conclusion.

Proposal 8-1: Whether to introduce NG suspension procedure can wait for NTN's conclusion.

Proposal 8-2: Capture in stage-2 spec that NG connection can be removed during the WAB-node movement.

2.9 PCI collision avoidance and TAC reconfiguration

For PCI collision avoidance, as the scenario is quite similar with mobile IAB, the method of mobile IAB can be used as baseline for WAB. And it has only impact to stage-2 spec.

For reconfiguration of TAC and RANAC on WAB-gNBs, the description in TR 38.799 is OK. We propose that this description is captured in TS 38.300 directly.

Proposal 9-1: The mechanism for PCI collision avoidance in mobile IAB can be reused for WAB.

Proposal 9-2: Capture the mechanism for PCI collision avoidance and reconfiguration of TAC/RANAC on WAB-gNBs in stage-2 spec.

3 Conclusion

In the previous sections we made the following proposals:

Proposal 1: The architecture part of stage-2 spec follows the definition, architecture and protocol stack examples in TR 38.799 with refinement or supplement if necessary.

Proposal 2: Capture support of backhauling of WAB-gNB’s traffic over WAB-MT’s PDU session in stage-2 spec, including how to do the QoS mapping of WAB-gNB’s traffic to the BH PDU session.

Proposal 3-1: Capture the WAB-node behaviors in case authorization status of WAB-gNB/WAB-MT changes in stage-2 spec.

Proposal 3-2: Capture in stage-2 spec that the WAB-node needs to shut down the SCTP connections and release the BH PDU session(s) of the WAB-MT if the authorization status of WAB-MT is changed to “not authorized”, and re-establish the BH PDU session(s) if the authorization status of WAB-MT is changed to “authorized”.

Proposal 4-1: Multi-hop WAB topology can be avoided by BH-AMF not accepting WAB-MT’s access/ handover to a WAB-gNB. For performance optimization, an idle/inactive WAB-MT does not (re)select to WAB cells and connected WAB-MT does not report measurement results of WAB cells.

Proposal 4-2: RAN3 send response LS to SA2 and a request LS to RAN2 on multi-hop topology avoidance as Annex.

Proposal 5: Capture the WAB-node integration procedure in the TR to stage-2 spec, and WAB-node indicates it is a WAB-node during the NG connection setup phase of WAB-node integration procedure.

Proposal 6: Capture in stage-2 spec that WAB-gNB may update the TNL addresses used for NG or Xn interface using the new IP address of the BH PDU session when the BH PDU session is re-established.

Proposal 7: Capture the two logical gNB solution and Option 2 for single gNB solution in stage-2 spec.

Proposal 8-1: Whether to introduce NG suspension procedure can wait for NTN's conclusion.

Proposal 8-2: Capture in stage-2 spec that NG connection can be removed during the WAB-node movement.

Proposal 9-1: The mechanism for PCI collision avoidance in mobile IAB can be reused for WAB.

Proposal 9-2: Capture the mechanism for PCI collision avoidance and reconfiguration of TAC/RANAC on WAB-gNBs in stage-2 spec.

4 Reference

- R3-244849, 3GPP TR 38.799,  Study on additional topological enhancements for NR, v1.2.0 (2024-08)

- RP-242359, New WID on additional topological enhancements for NR, NTT DOCOMO, INC., AT&T, RAN#105

- 3GPP TR 23.700-06, Study on architecture enhancements for vehicle-mounted relays - Phase 2, v 19.0.0 (2024-06)

- S2-2407345, LS on questions regarding FS_VMR_Ph2, SA2#163

5 Annex A (Proposed response LS)

Title:	[DRAFT] Response LS on questions regarding FS_VMR_Ph2

Response to:	Reply LS on questions regarding FS_VMR_Ph2 (S2-2407345)

Release:	Rel-19

Work Item:	FS_NR_WAB_5GFemto

Source:	CATT (to be RAN3)

To:	SA2

Cc:	RAN2

Contact Person:

#### Name:	Yang Lu

Tel. Number:

E-mail Address:	luyang@catt.cn

Attachments:

1. Overall Description:

RAN3 thanks SA2 for providing issues on the usage of VMR after progressing the Rel-19 FS_VMR_Ph2 study and documented solution proposals in TR 23.700-06. RAN3 would like to provide the following response to SA2:

- Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases:

- MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

- MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

- RAN3 Answer: RAN3 believe that the core network can refuse the idle/inactive mode WAB-MT to access from a WAB-node during WAB-node integration procedure, or refuse the connected mode WAB-MT being handed over to a WAB-node for both case a) and b).

To prevent the failure of access or handover in advance, the optimization mechanism will be discussed by RAN to make a WAB-MT does not select WAB cells or not report measurement results for WAB cells.

- Issue-2: SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

- RAN3 Answer: RAN3 has discussed this issue and conclude that all three options of the additional ULI are feasible in principle considering that the PLMN serving the WAB-MT and the PLMN broadcasted by the WAB-gNB would be different. It's up to SA2 to decide which option to adopt.

2. Actions:

To SA2 group:

ACTION: 	RAN3 kindly ask SA2 to take RAN3 answer into account.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN3 Meeting	 #126, November 18 to 22, 2024       Orlando, US

TSG-RAN3 Meeting	 #127, February 17 to 21, 2025          Athens, GR

6 Annex B (Proposed LS)

Title:	[DRAFT] LS on questions regarding avoidance of multi-hop WAB topology

Response to:	NA

Release:	Rel-19

Work Item:	FS_NR_WAB_5GFemto

Source:	CATT (to be RAN3)

To:	RAN2

Cc:	SA2

Contact Person:

#### Name:	Yang Lu

Tel. Number:

E-mail Address:	luyang@catt.cn

Attachments:

1. Overall Description:

RAN3 would like to inform RAN2 that RAN3 had progressed the Rel-19 FS_NR_WAB_5GFemto study and documented the conclusions for the WAB study in clause 6 of TR 38.799.

For the issue of multi-hop WAB topology avoidance, i.e., restricting the WAB-MT accessing a WAB-gNB cell, RAN3 conclude that following aspects should be ensured:

- The idle/inactive mode WAB-MT does not (re)select to a WAB-gNB cell

- The connected mode WAB-MT does not report measurement result of WAB-gNB cell(s)

RAN3 assumes how to ensure those aspects need discussion by RAN2, and asks for RAN2 feedback.

2. Actions:

To RAN2 group:

ACTION: RAN3 kindly asks RAN2 to take above into account and provide feedback.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN3 Meeting	 #126, November 18 to 22, 2024       Orlando, US

TSG-RAN3 Meeting	 #127, February 17 to 21, 2025          Athens, GR