3GPP TSG-RAN WG3 Meeting #125bis                                                              R3-245257

Hefei, China, Oct 14th – 19th, 2024

Source:	CATT

Title:	On support on-demand SIB1 for idle UE

Agenda Item:	17.3

Document for:	Discussion and Decision

1 Introduction

In RAN#105 meeting, RAN plenary agreed to turn the study of on-demand SIB1 for idle/inactive UEs to a normative phase from October. The on-demand SIB1 will focus on Case 2 only. And the new WID[1], in which the objectives for on-demand SIB1 include following aspects, has been agreed for Rel-19 NES enhancement.

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

During the former RAN3 meetings, following agreements were achieved for on-demand SIB1:

RAN3#123bis[2]: Scenario1: UE obtains the UL WUS configuration from Cell A, and transmits UL WUS to NES Cell. - UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed RAN3#124[3]: Focus on case2 in RAN3 first. Which node decides the WUS configuration and On-demand SIB1 mode? DU or CU or OAM? NES-cell gNB decides on-demand SIB1 operation activation and deactivation? RAN3#125[4]: UL WUS configuration is decided by NES Cell DU and transfers to NES Cell CU. UL WUS configuration transmission from NES Cell DU to NES Cell CU and to CellA. Whether On-Demand SIB1 activation/deactivation is needed？ Case 2 can be supported with the below details: - UL WUS configuration is decided by NES Cell DU. - UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface. - Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

In this contribution, we will continue to discuss the on-demand SIB1 topic.

2 Discussion

Progress of Other WGs

RAN2#127:

Agreements on OD-SIB1

Scenarios:

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

- RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

RAN1#118:

Agreement For further work on on-demand SIB1 in idle/inactive mode, as a baseline, it is assumed that the transmit power control of UL WUS transmission based on PRACH is applied in the same manner as the legacy PRACH transmission. - FFS: Potential optimization of the power ramp-up procedure. Agreement For further work on on-demand SIB1 in idle/inactive mode related to dedicated PRACH resource usage, RAN1 to study the following options: - Option 1 (shared RO): The dedicated WUS resource shares the same PRACH resource pool with PRACH resource for other usages. - Option 2 (separated RO): The dedicated WUS resource uses an independent RACH resource pool with PRACH resource for other usages. Agreement RAN1 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell. Agreement RAN1 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline. Agreement At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options: - Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission - Option 2: starting time and duration are indicated in the UL WUS configuration Agreement At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, on reference time point to determine the window starting time, RAN1 to down select from the following two options: - Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission - FFS: Definition of RAR reception time - Option 2: The reference time point is defined based on the UL-WUS transmission time - Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission Agreement For on-demand SIB1 in idle/inactive mode, Case 3 (Option 2+B+Y) is feasible from RAN1 perspective for some scenarios. Case 3 (Option 2+B+Y) is lower priority compared to Case 2 from RAN1 perspective. - RAN1 is inconclusive on whether the required specification support is justified by the observed NES gain for Case 3 Agreement RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19. - Note: RAN1 strive to minimize impact to legacy UE. - Note: RAN1 specification impact to support this feature should be minimized. Agreement For Case-2: For type 0 PDCCH monitoring occasions for on demand SIB1, on how the search space zero configuration is provided, RAN1 to down select from the following options: - Option 1: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell - Option 2: searchSpaceZero for on-demand SIB1 is provided from UL WUS configuration - Option 3: searchSpaceZero for on-demand SIB1 is provided from the RAR of UL WUS. Combination of multiple options is not precluded.

2.1 On-demand SIB1 activation

In the last meeting, it was agreed by RAN3 that UL WUS configuration is decided by NES gNB-DU and the UL WUS configuration is delivered to NES gNB-CU over F1 and then to Cell A gNB over Xn. However, it’s not clear how to start on-demand SIB1 operation.

2.1.1 F1AP signalling

Since SIB1 delivery is performed by gNB-DU, the energy used for SIB1 transmission is consumed by gNB-DU. The gNB-DU (instead of the gNB-CU) can be aware of energy consumed for SIB1 transmission. In addition, most of the energy used for base band processing are consumed by gNB-DU, so the energy efficiency of a cell can be obtained by gNB-DU. If the energy efficiency of the NES cell is low due to common signal transmission, e.g., when the user data rate is low or few UEs are connected, and the cell supports OD-SIB1 operations, gNB-DU can initiate operation of OD-SIB1 to the NES cell. When deciding to initiate the operation of OD-SIB1, the gNB-DU can send the UL WUS configuration for the NES cell to the NES gNB-CU, so that the NES gNB-CU delivers the UL WUS configuration to the Cell A gNB-CU.

However, since the cell operating OD-SIB1 cannot access legacy UEs, the normal cell should be deployed covering the area of the NES cell for legacy UEs to access. It is the gNB-CU (rather than gNB-DU) being aware of the network deployment. So, the NES gNB-DU should request for OD-SIB1 operation to NES gNB-CU and then NES gNB-CU confirms whether the request is permitted or refused. For example, the request with the UL WUS configuration can be delivered by NES gNB-DU using gNB-DU Configuration Update message, and the confirm can be sent via gNB-DU Configuration Update Ack message by the NES gNB-CU. New IEs should be introduced in gNB-DU Configuration Update message for containing the request, and new IEs should be introduced in gNB-DU configuration Update Ack message for containing the results.

Further, according to the network deployment, a NES cell may need one or multiple Cell A(s) to broadcast its UL WUS configuration, and the multiple Cell As would be served by the same gNB of the NES cell or by different gNBs. It was agreed by RAN3 that Cell A gNB can provide feedback to NES gNB for whether the WUS configuration broadcast for NES cell is accepted or not. The NES gNB-CU should confirm whether the request of OD-SIB1 operation can be permitted based on the feedbacks on WUS configuration broadcast, and thus it should indicate the result to NES gNB-DU after receiving the feedbacks from Cell A gNBs. If the NES gNB-CU indicates to the NES gNB-DU that OD-SIB1 is permitted for the NES cell, the OD-SIB1 operation in the NES cell is activated.

Proposal 1: NES gNB-DU sends the request for on-demand SIB1 operation for NES cell to NES gNB-CU with the UL WUS configuration by gNB-DU Configuration Update message.

Proposal 2: NES gNB-CU decide whether the request of on-demand SIB1 can be permitted based on the feedbacks from Cell A gNBs on UL WUS configuration broadcast.

Proposal 3: NES gNB-CU indicates whether on-demand SIB1 is permitted in the NES cell to NES gNB-DU by gNB-DU Configuration Update Ack message. If NES gNB-CU indicates on-demand SIB1 is permitted, on-demand SIB1 operation in the NES cell is activated.

It has been agreed that the UL WUS configuration is received by the Cell A gNB from the NES gNB, but it’s not clear how it is transmitted from Cell A gNB-CU to the Cell A gNB-DU. When the Cell A gNB starts to broadcast the UL WUS configuration, the Cell A gNB-CU can deliver the UL WUS configuration to the Cell A gNB-DU. If the ULWUS configuration is broadcasted via a new SIB (SIBx) of Cell A, it can be delivered using the existing gNB-CU System Information IE in the gNB-CU Configuration Update message.

Proposal 4: If the ULWUS configuration is broadcasted via a new SIB (SIBx) of Cell A, it can be transmitted from Cell A gNB-CU to Cell A gNB-DU by the existing gNB-CU Configuration Update message.

2.1.2 XnAP signalling

Per agreements achieved in the last meeting, the NES gNB-CU sends UL WUS configuration, which is received from the NES gNB-DU, to Cell A gNB-CU over Xn. The Cell A gNB can provide feedback to NES gNB whether the UL WUS configuration can be broadcasted in Cell A.

UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface. Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

According to the agreement, the NES gNB should request the Cell A gNB for broadcasting the UL WUS configuration in Cell A for the NES cell. Since the request on broadcasting UL WUS configuration over Xn should target to the Cell A which is served by the Cell A gNB, it’s not suitable to reuse the NG-RAN Node Configuration Update or Xn Setup Request message for that purpose. RAN3 should introduce a new Class-1 procedure.

Proposal 5: Introduce a new Class-2 Xn procedure for request and response on broadcasting the WUS configuration in Cell A.

From the last meeting, there is an FFS for whether Cell A broadcast UL WUS configuration right away upon reception from NES gNB. As we understand, a NES cell may keep in relation with several Cell A, that means NES gNB can request several Cell A gNBs for broadcasting the UL WUS configuration. Since NES gNB need decide whether OD-SIB1 can be operated based on the feedback from Cell A, the OD-SIB1 operation would not be permitted if some Cell A gNBs do not support/accept broadcasting the WUS configuration for NES cell. If the WUS-broadcast-capable Cell A broadcast the UL WUS configuration immediately upon receiving the configuration over Xn, there is the risk that OD-SIB1 is not operated in the NES cell at all. There are two solutions observed as below:

- Solution 1: NES gNB send a on-demand SIB1 activation indication to Cell A over Xn

In Option 1, an indication on OD-SIB1 activation over Xn is sent after the request for UL WUS configuration broadcast over Xn. The WUS-broadcast-capable gNBs which have already received the UL WUS configuration for a NES cell can start to broadcast the WUS configuration at receiving the OD-SIB1 activation indication for the NES cell over Xn.

Thus, two Xn messages are needed for each Cell A to broadcast the UL WUS configuration and the Cell A always need to judge whether the stored UL WUS configuration should be broadcasted based on the OD-SIB1 status.

- Solution 2: NES gNB requests the Cell A to cancel the broadcasting of UL WUS configuration if OD-SIB1 operation is not permitted

On Option 2, Cell A broadcast UL WUS configuration right away upon reception of the WUS configuration broadcast request. If OD-SIB1 operation is unfortunately not permitted due to some Cell A gNBs reject broadcasting the WUS configuration, the NES gNB can request the Cell A which has accepted the UL WUS configuration broadcasting to cancel the broadcasting of UL WUS configuration.

Comparing two solutions, Solution 2 is more efficient. The Cell A shall, if supported, broadcast the UL WUS configuration right away upon reception of the UL WUS configuration from NES gNB.

Proposal 6: Cell A broadcasts the UL WUS configuration right away upon reception.

2.2 On-demand SIB1 deactivation

In the last meeting, it was agreed by RAN3 that UL WUS configuration is decided by NES gNB-DU and the UL WUS configuration is delivered to NES gNB-CU over F1 and then to Cell A gNB over Xn. However, the procedure for deactivation the on-demand SIB1 operation has not been discussed.

2.2.1 F1AP signalling

After OD-SIB1 operation is activated, it’s possible that load of the NES cell increases due to many idle/inactive mode Rel-19 UEs begin to trigger OD-SIB1 or many connected mode UEs are handed over to the NES cell, that the energy saving for the NES cell would not benefit from OD-SIB1 operation any more. So, it’s necessary to deactivate the OD-SIB1 operation in that case, i.e., turn the OD-SIB1 mode to the periodical mode. The NES gNB-DU can decide whether the OD-SIB1 deactivation is needed. When the NES gNB-DU decides to deactivate OD-SIB1, it can send the request to the NES gNB-CU over F1 interface, so that the NES gNB-CU requests the Cell A to cancel broadcasting the UL WUS configuration for the NES cell. The gNB-DU Configuration Update message can be used for requesting OD-SIB1 deactivation.

Proposal 7: NES gNB-DU sends the request for deactivating OD-SIB1 operation to NES gNB-CU by gNB-DU Configuration Update message.

After reception of the OD-SIB1 deactivation request, the NES gNB-CU would request the Cell A gNB(s) to cancel UL WUS configuration broadcast in the Cell A for NES cell. To cancel broadcasting the UL WUS configuration for the NES cell, the Cell A gNB-CU should indicate the Cell A gNB-DU to stop broadcasting the UL WUS configuration. If the ULWUS configuration is broadcasted via a new SIB (SIBx) of Cell A, the Cell A gNB-CU can remove/update the SIBx of Cell A using existing gNB-CU Configuration Update message.

Proposal 8: If the ULWUS configuration is broadcasted via a new SIB (SIBx) of Cell A, to make Cell A gNB-DU stop broadcasting the UL WUS configuration, Cell A gNB-CU can remove/update the SIB1x of Cell A via the existing gNB-DU Configuration Update message.

2.2.2 XnAP signalling

As Section 2.2.1, the NES gNB would request the Cell gNB(s) to cancel the UL WUS configuration broadcasting if the OD-SIB1 operation is deactivated. On the other hand, the NES gNB may decide to change the Cell A for the NES cell based on, e.g, the change of network deployment or operator’s strategy. In that case, some neighbour cells may be selected as the new Cell A for the NES cell, or some neighbour cell may not need to act as Cell A anymore. If the NES gNB selects any neighbour cell(s) as the new Cell A, it can request the neighbour cell(s) to broadcast UL WUS configuration for the NES cell as discussed in Section 2.1. On the contrary, if there is neighbour cell(s) not acting as Cell A anymore, the NES gNB should be able to inform the relevant cell(s) to stop broadcasting the WUS configuration for the NES cell, so as to avoid wasting resource. Also, the cancellation is possible when NES gNB decides the OD-SIB1 activation is not permitted, it need to cancel the WUS configuration broadcast in the Cell A accepted the broadcasting.

The cancellation of UL WUS configuration sent from the NES gNB to the Cell A gNB may share the same Xn message of requesting WUS configuration broadcasting, e.g, the Xn message named as WUS Config Broadcast Request message. The Cell A gNB receiving the information of cancel UL WUS configuration can release the stored UL WUS configuration for the NES cell and stop the transmission immediately.

Proposal 9: The NES gNB can request the Cell A gNB to cancel broadcasting UL WUS configuration for the NES cell.

Proposal 10: The Cell A gNB receives the request of cancel broadcasting UL WUS configuration can remove the stored UL WUS configuration for the NES cell and stop broadcasting it.

2.3 Cell A gNB stops WUS configuration broadcasting

When the Cell A gNB has broadcasted the UL WUS configuration for the NES cell for a period of time, it may decide to stop broadcasting it.

2.3.1 XnAP signalling

The Cell A gNB can send an indication to the NES gNB on stopping WUS configuration transmission for the NES cell. The NES gNB receives the indication should judge whether the NES cell can continue to operate OD-SIB1, as the NES cell may not be permitted to operate OD-SIB1 if one or more Cell A stop broadcasting the WUS configuration. If the NES gNB decides the OD-SIB1 operation cannot be continued in the NES cell, the NES cell should deactivate OD-SIB1.

Proposal 11: Cell A gNB can indicate the NES gNB on stopping UL WUS configuration for the NES cell.

Proposal 12: NES gNB receiving the indication on stopping UL WUS configuration broadcast can determine whether to deactivate on-demand SIB1 for the NES cell.

2.3.2 F1AP signalling

If the NES gNB-CU receives the indication over the Xn interface indicating that the Cell A stops broadcasting the UL WUS configuration for NES cell, and it judges the NES cell is not permitted to continue operating OD-SIB1, it should command the NES gNB-DU to deactivate OD-SIB1 in the NES cell. The gNB-CU configuration Update message can be used for the NES gNB-CU to command the NES gNB-DU to deactivate OD-SIB1.

Proposal 13: NES gNB-CU can command NES gNB-DU to deactivate on-demand SIB1 in the NES cell.

2.4 Exchange on-demand SIB1 cell over Xn

As the OD-SIB1 operation is beneficial for energy saving only in case the load of the NES cell is in a low level, which means OD-SIB1 would not be activated for middle/high load cases, it’s better to keep the load low for the cell supporting OD-SIB1 as long as possible. So, the neighbour cells would need to prevent/deprioritize UEs’ handover to the cell operating or going to operate on-demand SIB1. The OD-SIB1 cell should be known by the neighbour nodes, so that neighbour nodes can apply different policies towards the OD-SIB1 cell to facilitate keeping the load of the cell low. The information of on-demand SIB1 cell can be exchanged over Xn by NG-RAN Node Configuration Update and/or Xn Setup/Response messages.

Observation: The on-demand SIB1 cell need be indicated to neighbour cells over Xn.

Proposal 14: On-demand SIB1 cell can be exchanged over Xn via NG-RAN Node Configuration Update and/or Xn Setup/Response messages.

3 Conclusion

In the previous sections we made the following observations and proposals:

On-demand SIB1 activation

Proposal 1: NES gNB-DU sends the request for on-demand SIB1 operation for NES cell to NES gNB-CU with the UL WUS configuration by gNB-DU Configuration Update message.

Proposal 2: NES gNB-CU decide whether the request of on-demand SIB1 can be permitted based on the feedbacks from Cell A gNBs on UL WUS configuration broadcast.

Proposal 3: NES gNB-CU indicates whether on-demand SIB1 is permitted in the NES cell to NES gNB-DU by gNB-DU Configuration Update Ack message. If NES gNB-CU indicates on-demand SIB1 is permitted, on-demand SIB1 operation in the NES cell is activated.

Proposal 4: If the ULWUS configuration is broadcasted via a new SIB (SIBx) of Cell A, it can be transmitted from Cell A gNB-CU to Cell A gNB-DU by the existing gNB-CU Configuration Update message.

Proposal 5: Introduce a new Class-2 Xn procedure for request and response on broadcasting the WUS configuration in Cell A.

Proposal 6: Cell A broadcasts the UL WUS configuration right away upon reception.

On-demand SIB1 deactivation

Proposal 7: NES gNB-DU sends the request for deactivating OD-SIB1 operation to NES gNB-CU by gNB-DU Configuration Update message.

Proposal 8: If the ULWUS configuration is broadcasted via a new SIB (SIBx) of Cell A, to make Cell A gNB-DU stop broadcasting the UL WUS configuration, Cell A gNB-CU can remove/update the SIB1x of Cell A via the existing gNB-DU Configuration Update message.

Proposal 9: The NES gNB can request the Cell A gNB to cancel broadcasting UL WUS configuration for the NES cell.

Proposal 10: The Cell A gNB receives the request of cancel broadcasting UL WUS configuration can remove the stored UL WUS configuration for the NES cell and stop broadcasting it.

Cell A gNB stops WUS configuration broadcasting

Proposal 11: Cell A gNB can indicate the NES gNB on stopping UL WUS configuration for the NES cell.

Proposal 12: NES gNB receiving the indication on stopping UL WUS configuration broadcast can determine whether to deactivate on-demand SIB1 for the NES cell.

Proposal 13: NES gNB-CU can command NES gNB-DU to deactivate on-demand SIB1 in the NES cell.

Exchange on-demand SIB1 cell over Xn

Observation: The on-demand SIB1 cell need be indicated to neighbour cells over Xn.

Proposal 14: On-demand SIB1 cell can be exchanged over Xn via NG-RAN Node Configuration Update and/or Xn Setup/Response messages.

4 Reference

- Draft Report of 3GPP TSG RAN3 meeting #123-bis, MCC

- Draft Report of 3GPP TSG RAN3 meeting #124, MCC

- Draft Report of 3GPP TSG RAN WG1 #117 v0.3.0, RAN1#117, MCC