3GPP RAN WG3 Meeting #125-bis	R3-245272

# Hefei, China, 14th – 18th October 2024

Source:	CATT

Title:	Discussion on left over issues on DC and split architecture

Agenda Item:	11.4

Document for:	Discussion and decision

# Introduction

This document focuses on how the AI enhancements introduced in Rel-18 may apply in DC and split gNBs.

This document is mainly a revised one based on R3-244510. Some main differences include:

- Removing the part mentioning UL Packet Loss.

- Adding texts proposing UE performance should be provided per group of QoS flow or per DRB.

- Removing the part of PSCell prediction, which is precluded from the scope.

- Adding the proposal of delivering energy cost on F1AP.

# Discussion

## Exchanging UE performance

The IE “UE Performance” is a group of per-UE metric, and may contain three types of measurement results: Average UE throughput, Average Packet Delay, and Average Packet Loss DL. In DC scenario the data for a UE may be split into more than one node, so the MN has to collect both the UE performance measured in the MN itself and the one measured in the SN in order to generate the final “UE Performance” provided toward e.g. the source node.

The question is which node collects these three types of UE performance result in split scenarios—once this is decided the case for multiple bearer types in DC is decided as well. Our preference on this issue is following the legacy mechanism introduced for SON/MDT.

It was ever proposed that gNB-DUs can send the delay of its part toward the gNB-CU-UPs through the F1-U. It is true, but it cannot satisfy the requirement in AI/ML:

- In AI/ML the requesting node usually indicates the periodicity or time window of measurement, so as to provide proper training input (e.g. as an input of “reward”). But the method to report delay over F1-U does not include a configurable periodicity or time window.

- In AI/ML it is assumed that every node may reject data collection requests due to various reasons, and upon such reject further action may be taken, e.g. issuing a reject further back (gNB-DU rejects gNB-CU and then gNB-CU rejects the originating gNB), altering the parameter of request or cancelling requests to other nodes which are deemed to be useless. But the method to report delay over F1-U does not include an outcome IE.

Observation 1: The mechanics of delivering delay over F1-U cannot be configured with reporting periodicity and is without any explicit acceptance or reject indication, therefore not suitable for AI/ML purpose.

The cases on throughput and packet loss are simple:

- Both downlink throughput and uplink throughput are measured by the gNB-DU as defined in Section 6.3.1.4.1 and Section 6.3.1.4.2 of TS 28.558 respectively.

- Downlink packet loss is measured by the gNB-DU as defined in Section 4.2.1.5.1 of TS 38.314.

The case is quite complex on packet delay as shown in Section 4.2.1.2.1 of TS 38.314. In the downlink direction it is defined as the sum of 4 items:

- Average delay DL air-interface in Section 6.3.1.1.1 of TS 28.558, wrongly defined as measured by the gNB-CU-UP but should actually be measured by the gNB-DU (as in Section 5.1.3.1.1 of TS 28.552);

- Average delay in RLC sublayer, measured by the gNB-DU as defined in Section 6.3.1.1.2 of TS 28.558, wrongly defined as measured by the gNB-CU-UP but should actually be measured by the gNB-DU (as in Section 5.1.3.3.3 of TS 28.552);

- Average delay on F1-U, measured by the gNB-CU-UP as defined in Section 6.3.1.1.3 of TS 28.558;

- Average delay DL in CU-UP, measured by the gNB-CU-UP as defined in Section 6.3.1.1.4 of TS 28.558.

In the uplink direction it is defined as the sum of 5 items:

- UL PDCP packet average delay, measured by the UE as defined in Section 4.3.1.1 of TS 38.314 (it is wrongly defined as measured by the gNB-CU-UP in TS 28.558);

- Average over-the-air interface packet delay, measured by the gNB-DU as defined in Section 4.2.1.2.2 of TS 38.314;

- Average RLC packet delay, measured by the gNB-DU as defined in Section 4.2.1.2.3 of TS 38.314;

- Average delay on F1-U, measured by the gNB-CU-UP as defined in Section 6.3.1.1.3 of TS 28.558;

- Average PDCP re-ordering delay, measured by the gNB-CU-UP as defined in Section 4.2.1.2.4 of TS 38.314.

Nevertheless, it is not necessary to report every metric separately. The delay measured in gNB-DU can be combined into one single metric, and so can the delay measured in gNB-CU-UP.

As the summary, there are 5 metrics collected in the gNB-DU whereas 2 metrics collected in the gNB-CU-UP. The remaining one is collected by the UE, which is reported toward the gNB-CU-CP directly and thus has no impact on either E1AP or F1AP.

Proposal 1: The following 5 metrics for UE performance is collected in the gNB-DU (aligned with legacy SON/MDT, and the case in DC should be aligned with this):
-	DL throughput;
-	UL throughput;
-	DL packet loss;
-	Average DL delay air-interface plus Average DL delay in gNB-DU;
-	Average UL delay air-interface plus Average UL delay in gNB-DU.

Proposal 2: The following 3 metrics for UE performance is collected in the gNB-CU-UP (aligned with legacy SON/MDT, and the case in DC should be aligned with this):
-	Average delay F1-U plus Average DL delay in gNB-CU-UP;
-	Average delay F1-U plus Average UL delay in gNB-CU-UP.

The next issue is on granularity. Every UE performance metric introduced in Rel-18 is defined as one value per UE, and the meaning of them is ambiguous (and still in discussion to clarity). No matter what the meaning is, such per-UE metric cannot reflect well how the UE is served. Two examples are here:

- There are two DRBs with vastly different QoS requirements established for a UE. There are temporarily no data on one DRB before handover whereas there are data delivered after handover, or vice versa. The collected per-UE metrics will definitely change drastically before and after handover even if the service for each DRB is kept the same.

- There are two DRBs, one with strict QoS requirement and one with very loose one. Any per-UE average obviously reflects mostly how the loose DRB is served, and provides virtually no information on how the strict one is served.

Therefore we propose introducing per-DRB UE performance IEs into F1AP and E1AP given that the flow-to-DRB mapping is known across all involved nodes: CU-CP, CU-UP, and DU.

In the case of Dual Connectivity, the scenario differs slightly. UE performance is typically configured from the Master Node to the Secondary Node and reported back from the SN to the MN. For most bearer types, the MN is aware of the flow-to-DRB mapping, including all MN-terminated bearers and SN-terminated MCG bearers/Split bearers, making it feasible to support DRB-level UE performance feedback. However, for SN-terminated SCG bearers, the MN is unaware of the flow-to-DRB mapping, necessitating the use of flow list-level reporting as the only viable option.

Proposal 3: To introduce per-DRB UE performance IEs into F1AP and E1AP.

Proposal 4: To introduce per-DRB UE performance IEs and per-QoS-flow-group (i.e. de facto per-DRB) IEs into XnAP for DC scenario. The per-QoS-flow-group one is used for the case that the flow-to-DRB mapping is unknown for the receiving node.

## Delivering measured Energy Cost from gNB-DUs

RAN3 has agreed in the SI phase that the Energy Cost should be provided from the gNB-DUs to the corresponding gNB-CUs. This will surely impact F1AP.

Proposal 5: To enhance F1AP so that the gNB-DU can provide the Energy Cost toward the gNB-CU.

## Introducing new metrics

We observe one type of information which is not introduced in the Rel-18 WI but essential in split scenario: the UE traffic volume. It was ever mentioned in the discussion for the Energy Saving use case, as the “Energy Efficiency” is literally the data volume divided by the energy consumption.

Companies found it hard to describe traffic between neighbouring gNBs, but this is not the case within one gNB: the source gNB-CU-CP knows everything from the QoS requirement toward the UE radio status (i.e. RSRP etc), with the only exception is the data rate for each flow.

Therefore we propose enhancing E1AP so that the gNB-CU-UP can provide the data rate for each flow toward the gNB-CU-CP. (The model for energy saving should surely reside in the gNB-CU-CP.)

Proposal 6: To enhance E1AP so that the gNB-CU-UP can provide the data rate for each QoS flow toward the gNB-CU-CP, in order to cover the use case of energy saving (in which the gNB acts as the source node).

## Message to use

As analysed above, there are many metrics needs to be delivered over XnAP for DC, E1AP and F1AP, many of which the same as the metrics included in XnAP Data Collection Update messages. For XnAP for DC, the Rel-18 XnAP Data Collection Update message can be reused. For E1AP and F1AP, we propose introducing similar procedures for alignment. For measurement objects related to one single UE, a measurement ID pair for reference is added into UE-associated messages, such as XnAP SN Addition Request messages, F1AP UE Context Setup Request messages, E1AP Bearer Context Setup Request messages.

Proposal 7: To introduce new data collection procedures into E1AP and F1AP, similar to the Rel-18 one introduced into XnAP. For measurement objects related to one single UE, a measurement ID pair is added into UE-associated messages for reference.

## New node is configured sometime during the feedback collection window

After a handover take place, the target node may need to collect the feedback upon the prior request from the source node. It is possible that a new node is added to serve the UE while it is still right within the feedback collection window:

- The UE further experiences an intra-CU inter-DU handover (or PSCell change), thus a new gNB-DU (or even gNB-CU-UP) starts to serve the UE, and some performance needs to be collected by the new gNB-DU (or gNB-CU-UP).

- The target node further configures an SCG for the UE, and some performance needs to be collected by the SN or secondary gNB-DU.

- The UE further experience an SN change, and some performance needs to be collected by the new SN.

Generally speaking, the existing data collection mechanism can be reused for such case, except the length of collection time duration: the duration is now a reduced value (the original duration minus the time length between the two RRC reconfigurations), rather than the original one.

The source node expects the feedback one “collection time duration” after the handover toward the target node take places (after which the source node may delete the UE context for data collection), so the target node must collect all information and send it back to the source node in time. Therefore, the new serving node must collect information within the reduced duration and send it back instantly.

Considering that the new serving node has no idea of the time length between the two RRC reconfigurations, the new serving node needs to be provided with a “remaining” collection time duration. Considering that the time length between the two RRC reconfigurations varies from one UE to another, we prefer indicating the “remaining” collection time duration within a UE-associated message, i.e. the message containing the Data Collection ID.

Proposal 8: To introduce a “remaining collection time duration” into the UE-associated message which contains the Data Collection ID, in order to cover the case that a new node is configured sometime during the original data collection window.

# Conclusion

Observation 1: The mechanics of delivering delay over F1-U cannot be configured with reporting periodicity and is without any explicit acceptance or reject indication, therefore not suitable for AI/ML purpose.

Proposal 1: The following 5 metrics for UE performance is collected in the gNB-DU (aligned with legacy SON/MDT, and the case in DC should be aligned with this):
-	DL throughput;
-	UL throughput;
-	DL packet loss;
-	Average DL delay air-interface plus Average DL delay in gNB-DU;
-	Average UL delay air-interface plus Average UL delay in gNB-DU.

Proposal 2: The following 3 metrics for UE performance is collected in the gNB-CU-UP (aligned with legacy SON/MDT, and the case in DC should be aligned with this):
-	Average delay F1-U plus Average DL delay in gNB-CU-UP;
-	Average delay F1-U plus Average UL delay in gNB-CU-UP.

Proposal 3: To introduce per-DRB UE performance IEs into F1AP and E1AP.

Proposal 4: To introduce per-DRB UE performance IEs and per-QoS-flow-group (i.e. de facto per-DRB) IEs into XnAP for DC scenario. The per-QoS-flow-group one is used for the case that the flow-to-DRB mapping is unknown for the receiving node.

Proposal 5: To enhance F1AP so that the gNB-DU can provide the Energy Cost toward the gNB-CU.

Proposal 6: To enhance E1AP so that the gNB-CU-UP can provide the data rate for each QoS flow toward the gNB-CU-CP, in order to cover the use case of energy saving (in which the gNB acts as the source node).

Proposal 7: To introduce new data collection procedures into E1AP and F1AP, similar to the Rel-18 one introduced into XnAP. For measurement objects related to one single UE, a measurement ID pair is added into UE-associated messages for reference.

Proposal 8: To introduce a “remaining collection time duration” into the UE-associated message which contains the Data Collection ID, in order to cover the case that a new node is configured sometime during the original data collection window.

Based on the proposals above, we propose 2 TPs [1][2] accordingly.

# Reference

[1] R3-245273; TP on introducing data collection function into E1; CATT.

[2] R3-245273; TP on introducing data collection function into F1; CATT.