3GPP TSG-RAN WG3 Meeting #125bis	R3-245282

Hefei, China, 14th – 18th, Oct 2024

Agenda item:		20.2

Title:		Discussion on AI/ML based Positioning Accuracy 		Enhancements

Source:		NEC

Document for:		Discussion and Decision

- Introduction

The work items pertaining on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface were officially approved in RAN #102 and updated in RAN #105. The details concerning the positioning use case after RAN #105 have been finalized as follows [1].

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Based on the progress of RAN1 and RAN2, this contribution addresses the following topics:

- Model monitoring for case 3a

- Time stamp in NRPPa

- LOS/NLOS indicator

- Discussion

2.1 Model monitoring for Case 3a

In the context of model monitoring of case 3a, RAN1 #118 has agreed two options for model performance metric calculation for label-based monitoring, and the final selection of Option A and Option B is pending RAN3 confirmation [2].

##### Agreement @RAN1#118 For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

Besides, RAN2 #125b meeting [3] has also made the agreement for model performance monitoring of case 3a:

Agreements: 1 RAN2 to consider an RRC configuration to configure radio measurements and the related reporting to enable data collection for NW-side training 2 For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference. FFS if further enhancements are needed 3 There is no specification impact associated to gNB-side model inference, depending on further RAN1 input. 4 FFS whether there is specification impact associated to gNB-side model monitoring. 5 For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs 6 For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3

Observed from these agreements, it is evident that both Option A and Option B are acceptable to RAN1 and RAN2 for model monitoring of case 3a.

From RAN3’s perspective, the previous RAN1 meeting has agreed that LMF, rather than NG-RAN node, is one of the entities responsible for collecting label and its related data for training data collection of case 3a. This means the LMF is more feasibility than NG-RAN node for generating ground truth label. Furthermore, if the model output is timing information, the LMF can calculate this information based on the known UE location and TRP location. However, the NG-RAN node cannot calculate timing information without the reception of SRS-pos, as it lacks the UE’s location based on current specification.

From the aspect of training data collection, LMF is more feasibility than gNB to generating ground truth label for case 3a.

NG-RAN node cannot calculate timing information without receiving SRS-pos.

Option A is likely to require label transfer from LMF to gNB, while Option B needs model output transfer from gNB to LMF. Since current NRPPa message already supports this kind of model output, i.e., NOL/NLOS indicator or UL RTOA, transfer from gNB to LMF, in order to save standard efforts, Option B should be prioritized for model monitoring of case 3a.

To support Option B, i.e., LMF performs monitoring metric calculation for the model located at the NG-RAN node, for label-based model monitoring for case 3a.

In Annex, we also prepare a draft LS to RAN1 based on Proposal 1.

To sent an LS to RAN1 to inform RAN3 agreement on Option A/Option B selection.

2.2 Time stamp in NRPPa

Agreement @RAN1#116bis For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label

As agreed in RAN1#116bis, "time stamp" in channel measurement and ground truth label is one of the components for a data sample. In RAN1#118 meeting, RAN1 discussed how to design the time stamp of channel measurement. The issue focused on whether the existing IEs (e.g., Rel-16 timestamp related IEs) can be reused to satisfy the range and granularity requirements. For example, RAN1 discussed that if the channel measurement is generated by UE, LPP:NR-TimeStamp-r16 is only limited to SFN, and is not long enough to reflect the time sample information related to minutes, hours, dates, months, etc. Therefore, UTC time is suggested to cover a longer time range at least when UE is responsible for generating channel measurement. However, as the UTC time is absent in NRPPa, the controversy on selecting between "time stamp" and UTC time is also absent. As a result, RAN1 got following agreement when gNB/TRP is responsible for generating channel measurement:

Agreement @RAN1#118 For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

An outstanding issue is whether “Measurement time” (as shown below) should be mandatorily included together with SFN time.

### 9.2.42 Time Stamp This information element contains the time stamp. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. System Frame Number; M; INTEGER(0..1023). CHOICE Slot Index; M. >SCS-15; INTEGER(0..9). >SCS-30; INTEGER(0..19). >SCS-60; INTEGER(0..39). >SCS-120; INTEGER(0..79). Measurement time; O; Relative Time 1900 9.2.36.

In the table above, "SFN and slot index" can indicate a time range of 1023 frames which is not long enough to indicate the data sample which may be collected from different minutes, hours, dates, months and even different years. "Measurement time" has a good time range and granularity based on the definition in clause 9.2.36 in TS38.455, but it's an optional IE.

To address abovementioned issue, the following two options could be considered:

- Option 1: use "UTC time + SFN + slot index" as a baseline, where UTC time can indicate a long enough time period from one year to one second, and "SFN + slot index" can be further used to indicate the slot granularity within one second of UTC.

- Option 2: use "Measurement time" only as a baseline, which can indicate any time relative to Relative Time 1900.

RAN3 discuss following two options as baseline to design the time stamp for channel measurement in case 3b:

- Option 1: use "UTC time + SFN + slot index" as baseline

- Option 2: use "Measurement time" only as baseline

2.3 LOS/NLOS indicator

For the sub-use case of AI/ML assisted positioning (case 2a and case 3a), RAN1 has agreed that LOS/NLOS indicator and/or timing information are supported for reporting. In legacy positioning procedure, the UE/gNB reports measurement of timing information to LMF, the UE/gNB also includes LOS/NLOS indicator associated with the first path, which indicates the likelihood of LOS ray of this measured channel between TRP and UE. However, when this timing measurement is derived from the output of AI/ML model, they represent an estimation based on assumed ‘virtual’ LOS propagation path. In essence, it is not possible to derive timing information under the assumption of an NLOS ray using the deployed AI/ML model. Therefore, the LOS/NLOS indicator is not needed if model output is timing information.

For AI/ML assisted positioning (case 2a and case 3a), there is no need to report LOS/NLOS indicator to LMF if the model output is timing information.

- Conclusion

In this contribution, we discussed the issues of AI/ML for positioning accuracy enhancement. Observations and proposals are summarized as following:

Observation 1:	From the aspect of training data collection, LMF is more feasibility than gNB to generating ground truth label.

Observation 2:	NG-RAN node is incapability to calculate timing information without receiving SRS-pos.

Proposal 1:	To support Option B, i.e., LMF performs monitoring metric calculation for the model located at the NG-RAN node, for label-based model monitoring of case 3a.

Proposal 2:	To sent an LS to RAN1 to inform RAN3 agreement on Option A/Option B selection.

Proposal 3:	RAN3 discuss following two options as baseline to design the time stamp for channel measurement in case 3b:

- Option 1: use "UTC time + SFN + slot index" as baseline

- Option 2: use "Measurement time" only as baseline

Proposal 4:	For AI/ML assisted positioning (case 2a and case 3a), there is no need to report LOS/NLOS indicator to LMF if the model output is timing information.

# References

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #105.

- Chairman’s notes, 3GPP RAN1 #118.

- Chairman’s notes, 3GPP RAN2 #125b.

3GPP TSG-RAN WG3 Meeting #125bis	R3-xxxxxx

Hefei, China, October 14th – 18th 2024

Title:	[draft] LS on AI/ML based positioning accuracy enhancement

Response to:	-

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	NEC (to be RAN3)

To:	RAN1

Cc:	RAN2

Contact person:	Hui Ma

hui.ma@emea.nec.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

RAN3 has discussed the issue for the selection between Option A and Option B for model performance metric calculation of case 3a according to the following agreement from RAN1.

##### Agreement @RAN1#118 For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

RAN3 concluded that Option B should be supported in normative work.

# 2. Actions:

To RAN1

ACTION: 	RAN3 kindly ask RAN1 to take the above information into account .

3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #126             18th – 22nd November 2024, Orlando. FL, USA.

TSG RAN WG3 Meeting #127             17th – 21nd February 2025, Athens, GR.