3GPP TSG-RAN WG3 Meeting #125-bis	R3-245298

Hefei, China, 14th – 28th October 2024

Agenda Item:		12.3

Source:		NEC

Title:			(TP to TS38.300) NR Femto

Document for:		Discussion and decision

# Introduction

In RAN#105 meeting, a new WI on additional topological enhancements for NR was approved based on the study results from SI and the following objectives of 5G Femto are captured [1]:

-	Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3].

-	Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3].

NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective.

In this contribution, we provide our views on the specification impact of NR Femto architecture and NR Femto access control and also TP to TS38.300 correspondingly.

# Discussion

According to the objectives of 5G Femto in WID [1] and the study result in TR38.799 [2], we think only stage 2 impacts are foreseen. For TS 38.300, the following sections need to be added to reflect the study conculsions.

## 4.x	Support of NR Femto

4.x.1	Architecture

4.x.2	Functional Split

4.x.3	Interfaces

4.x.4	Access control

The detailed contents in each sections are described in the TP in Annex.

Proposal: To agree the TP to TS38.300 in Annex.

# Proposals

In this contribution, we provide TP to TS38.300 on NR Femto architecture and NR Femto access control and propose the following.

Proposal: To agree the TP to TS38.300 in Annex.

# Reference

[1] RP-242395, New WID on additional topological enhancements for NR

[2] TR 38.799 V19.0.0 Study on additional topological enhancements for NR

# Annex TP to TS38.300

<Start of change>

## 4.x	Support of NR Femto

### 4.x.1	Architecture

Figure 4.x.1-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto node to the 5GC.

Figure 4.x.1-1: NR Femto node Logical Architecture

The NG-RAN architecture may deploy an NR Femto Gateway (NR Femto GW) to allow the NG interface between the NR Femto node and the 5GC to support a large number of NR Femto nodes in a scalable manner. The NR Femto GW serves as a concentrator for the C-Plane, specifically the NG-C interface.

After introducing NR Femto node and NR Femto GW, the NG interface is defined as the interface:

- Between the NR Femto GW and the Core Network;

- Between the NR Femto node and the NR Femto GW  ;

- Between the NR Femto node and the Core Network.

- Between the gNB and the Core Network.

The Xn interface can be used as the interface:

- Between the NR Femto nodes;

- Between the NR Femto node and the gNB.

- Between the gNBs.

The NR Femto GW appears to the AMF as an gNB. The NR Femto GW appears to the NR Femto node as an AMF. The NG interface between the NR Femto node and the 5GC is the same, regardless whether the NR Femto node is connected to the 5GC via a NR Femto GW or not.

The functions supported by the NR Femto node shall be the same as those supported by a gNB and the procedures run between a NR Femto node and the 5GC shall be the same as those between a gNB and the 5GC.

This version of the specification supports Xn-connectivity between NR Femto nodes.

The overall architecture with deployed NR Femto GW is shown below.

Figure 4.x.1-2: Overall Architecture with deployed NR Femto GW

### 4.x.2	Functional Split

A NR Femto node hosts the same functions as a gNB as described in clause 4.2, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

-	A NR Femto node shall only connect to a single NR Femto GW at one time:

-	The NR Femto node will not simultaneously connect to another NR Femto GW or another AMF.

-	The TAC and PLMN ID used by the NR Femto node shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE attachment is hosted by the NR Femto GW instead of the NR Femto node. Upon reception of the GUMMEI from a UE, the NR Femto node shall include it in the INITIAL UE MESSAGE message; upon reception of the GUMMEI Type from the UE, the NR Femto node shall also include it in the message if supported and supported by the NR Femto GW.

-	NR Femto nodes may be deployed without network planning. A NR Femto node may be moved from one geographical area to another and therefore it may need to connect to different NR Femto GWs depending on its location;

The NR Femto GW hosts the following functions:

-	Relaying UE-associated NG application part messages between the AMF serving the UE and the NR Femto node serving the UE.

-	Terminating non-UE associated NG application part procedures towards the NR Femto node and towards the AMF.

NOTE:	If a NR Femto GW is deployed, non-UE associated procedures shall be run between NR Femto nodes and the NR Femto GW and between the NR Femto GW and the AMF.

-	Optionally terminating NG-U interface with the NR Femto node and with the UPF.

-	Supporting TAC and PLMN ID used by the NR Femto node.

-	Xn interfaces shall not be established between the NR Femto GW and other nodes.

### 4.x.3	Interfaces

#### 4.x.3.1	Protocol Stack for NG User Plane

The NG-U data plane is defined between the NR Femto node, NR Femto GW and the UPF. The figures below show the NG-U protocol stack with and without the NR Femto GW  .

Figure 4.x.3.1-1: User plane for NG-U interface for NR Femto node without NR Femto GW

Figure 4.x.3.1-2: User plane for NG-U interface for NR Femto node with NR Femto GW

The NR Femto GW may optionally terminate the user plane towards the NR Femto node and towards the UPF, and relay User Plane data between the NR Femto node and the UPF.

#### 4.x.3.2	Protocol Stacks for NG Control Plane

The two figures below show the NG-C protocol stacks with and without the NR Femto GW  .

When the NR Femto GW is not present (Fig. 4.x.3.2-1), all the NG-AP procedures are terminated at the NR Femto node and the AMF.

When present (Fig. 4.x.3.2-2), the NR Femto GW shall terminate the non-UE-dedicated procedures – both with the NR Femto node, and with the AMF. The NR Femto GW relays Control Plane data between the NR Femto node and the AMF. The scope of any protocol function associated to a non-UE-dedicated procedure shall be between NR Femto node and NR Femto GW and/or between NR Femto GW and AMF.

Any protocol function associated to an UE-dedicated-procedure shall reside within the NR Femto node and the AMF only.

Figure 4.x.3.2-1: Control plane for NG-C Interface for NR Femto node to AMF without the NR Femto GW

Figure 4.x.3.2-2: Control plane for NG-C Interface for NR Femto node to AMF with the NR Femto GW

#### 4.x.3.3	Protocol Stack for Xn User Plane and Xn Control Plane

The protocol stack for Xn User Plane and Xn Control Plane is reported in clause 7 of TS 38.420 [17].

### 4.x.4	Access control

CAG mechanism is reused for NR Femto nodes deployments. With the existing CAG mechanism as described in clause 16.7.4, the open, hybrid and closed access mode, can be supported as follows:

-	To support the open access mode: The NR Femto node activates a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	To support the hybrid access mode: The NR Femto activates a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	To support the closed access mode: The NR Femto node activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

<End of change>