3GPP TSG-RAN WG3 Meeting #125-bis	R3-245305

Hefei, China, 14th – 18th Oct 2024

Agenda Item:		12.3

Source:		Baicells

Title:		Discussion on NR Femto Architecture and Functionality

Document for:		Discussion and decision

- Introduction

The following objectives regarding 5G Femto are agreed in the new WID (see [2]) at RAN #105:

The objectives of the 5G Femto work are as follows: - Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3]. - Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3]. NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective. NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

The intention of this paper is to discuss the architectural properties associated with NR Femto GW for NG interface, as well as the corresponding functionalities on NR Femto node and NR Femto GW for stage 2.

- Discussion

2.1 Architecture with optional NR Femto GW

In TR 38.799 (see [1]), we have the architecture conclusion agreed as follows:

6.2.1 Architecture for NG interface Option 1, namely direct connection of an NR Femto node to the 5GC via the NG interface, is already possible. In order to maintain the existing infrastructure for an operator who has deployed LTE HeNBs, Option 2 with an optional NR Femto GW is recommended for a normative phase.

Therefore, we recommend reusing option 2 architecture as baseline with a NOTE: NR Femto GW is optional,to support both Option 1 and Option 2:

Figure xxx: NR Femto Logical Architecture NOTE: The NR Femto GW is optional. NOTE: The SeGW and the NR Femto Management System are out of RAN3 scope. The NG-RAN architecture may deploy an NR Femto Gateway (NR Femto GW) to allow the NG interface between the NR Femto node and the 5GC to support a large number of NR Femto nodes in a scalable manner. The NR Femto GW serves as a concentrator for the C-Plane, specifically the NG-C interface. The NG interface is defined as the interface: - Between the NR Femto GW and the 5GC; - Between the NR Femto node and the NR Femto GW; - Between the NR Femto node and the 5GC; The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto node as an AMF. The NG interface between the NR Femto node and the 5GC is the same regardless of whether the NR Femto node is connected to the 5GC via an NR Femto GW or not.

Proposal 1: Reuse Option2 architecture with a NOTE illustrating the NR Femto GW is optional as the NR Femto Logical Architecture, as well as the description above (TR 38.799 5.2.1.2 (see [1])) into TS 38.300.

As justification of the WID (see [2]), one of the motivations is to enable simple end user plug and play. To achieve this target, it is necessary that the NR Femto node should be simple and small.

Meanwhile we have agreed in the TR 38.799 that an NR Femto GW allows the NG interface between the NR Femto node and the 5GC to support a large number of NR Femto nodes in a scalable manner and the NR Femto GW appears to the NR Femto node as an AMF .

From this perspective, it will introduce a tremendous number of NG connections to the NR Femto GW from NR Femto nodes and the support of NG-Flex for NR Femto node will cause even more NG connections.

Therefore, considering reasonable simplifications for the NR Femto node in a network-operation and configuration view, and targeting at a low-complexity implementation of NR Femto GW, the following proposal is envisaged:

Proposal 2: The NR Femto node shall only connect to a single NR Femto GW at one time.

In order to enable NG-Flex connectivity for AMF load balancing, NR Femto GW should support connecting to multiple candidate AMF(s) at one time, namely NG-Flex fucntion shall be used at the NR Femto GW instead of NR Femto nodes.

Proposal 3: The NR Femto GW supports NG-Flex configuration and can simultaneously connect to multiple AMFs.

2.1 Functionalities with NR Femto GW

As the NR Femto GW supports to simultaneously connect with multiple AMFs, the proper AMF selection function, i.e.  the NAS node selection function (NNSF), should be supported by the NR Femto GW. Meanwhile, the NR Femto node is recommended only connecting to a single NR Femto GW at one time, that is the NR Femto node will not simultaneously connect to another NR Femto GW, or another AMF, so it is not necessary for the NR Femto node to host the NNSF. Therefore, the NNSF should be located in the NR Femto GW for the UE instead of NR Femto node.

Proposal 4: The NR Femto GW shall host the NNSF function instead of the NR Femto nodes.

When a UE register itself to the network, the NG-RAN shall use the NAS Node Selection Function (if supported) to select a proper AMF for the UE as specified in the 3GPP TS 38.410 [3],

A NAS node selection function is located in the NG-RAN node to determine the AMF association of the UE, based on the UE's temporary identifier, which was assigned to the UE by the AMF. When the UE's temporary identifier has not been yet assigned or is no longer valid the NG-RAN node may instead take into account other information (e.g. slicing information, onboarding indication) to determine the AMF.

As analysis above, the NNSF shall be host by the NR Femto GW if present instead of NR Femto nodes. Therefore, the assistance information used assisting NR Femto GW to select a proper AMF for the UE, (e.g. UE's temporary identifier,GUAMI, slicing information, onboarding indication), shall be transferred from the NR Femto nodes to the NR Femto GW if present.

Proposal 5: The assistance information for NNSF if present should be transferred over NG interface from NR Femto node to NR Femto GW.

- Proposals

In this contribution, we provide our considerations on NR Femto Architecture, as well as NG connection and interface with NR Femto GW, and the following proposals were drawn from the analysis.

Proposal 1: Proposal 1: Reuse Option2 architecture with a NOTE illustrating the NR Femto GW is optional as the NR Femto Logical Architecture, as well as the description above (TR 38.799 5.2.1.2 (see [1])) into TS 38.300.

Proposal 2: The NR Femto node shall only connect to a single NR Femto GW at one time.

Proposal 3: The NR Femto GW supports NG-Flex configuration and can simultaneously connect to multiple AMFs.

Proposal 4: The NR Femto GW shall host the NNSF function instead of the NR Femto nodes.

Proposal 5: The assistance information for NNSF if present should be transferred over NG interface from NR Femto node to NR Femto GW.

- Reference

- TR 38.799 v19.0.0 Study on additional topological enhancements for NR (Release 19)

- RP-242395 New WID on additional topological enhancements for NR

- 38.410 NG-RAN; Architecture description (Release 18)