3GPP TSG-RAN WG3 Meeting #125-bis                      R3-245306

Hefei, China, 14th – 18th Oct 2024

Agenda Item:		12.3

Source:		Baicells

Title:		TP to 38.300 for NR Femto

Document for:		Discussion and Approval

- Introduction

This TP is based on the R3-xx (Discussion on the NR Femto Architecture and Functionalities)

- TP to 38.300

============================Start of the change ===============================

4.x NR Femto

Figure xxx shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto nodes to the 5GC.

Figure xxx: NR Femto Logical Architecture

NOTE:   The NR Femto GW is optional.

NOTE:	The SeGW and the NR Femto Management System are out of RAN3 scope.

The NG-RAN architecture may deploy an NR Femto Gateway (NR Femto GW) to allow the NG interface between the NR Femto node and the 5GC to support a large number of NR Femto nodes in a scalable manner. The NR Femto GW serves as a concentrator for the C-Plane, specifically the NG-C interface.

The NG interface is defined as the interface:

-	Between the NR Femto GW and the 5GC;

-	Between the NR Femto node and the NR Femto GW;

-	Between the NR Femto node and the 5GC;

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto node as an AMF. The NG interface between the NR Femto node and the 5GC is the same regardless of whether the NR Femto node is connected to the 5GC via an NR Femto GW or not.

An NR Femto node shall only connect to a single NR Femto GW at one time in case of connection to the NR Femto GW:

-	The NR Femto node will not simultaneously connect to another NR Femto GW, or another AMF.

4.x.1 Functionality with NR Femto GW

The NR Femto nodes and NR Femto GW have the following specifics in case of connection with NR Femto GW:

-	NG Flex function shall be used at NR Femto GW instead of the NR Femto nodes.

- 	Selection of an AMF i.e. NAS node selection function (NNSF) at UE registration is hosted by the NR Femto GW instead of the NR Femto node:

-	Upon reception of the assistance information used for assisting NR Femto GW to select an AMF (e.g. UE's temporary identifier, GUAMI, slicing information, onboarding indication), the NR Femto nodes shall transfer them to NR Femto GW through NGAP procedure.