3GPP TSG-RAN WG3 Meeting #125-bis	R3-245310

Hefei, China, 14-18 Oct 2024

Agenda item:	10.2

Source:	Nokia

Title:	Failure handling and optimal use of Subsequent CPAC

Document for:	Discussion

# 1	Introduction

At the RAN3 #123-bis meeting, the discussion on SON for S-CPAC started. It has been proposed to address classic MRO failure events, but also optimisations of the usage of S-CPAC and supportive signalling were considered (e.g. UE History Information). The discussion progresses rather slowly, but at the #124 meeting, following was agreed:

CPAC failure scenarios and detection mechanism captured in stage2 used as baseline.

At the RAN3 #125, the topic was further discussed, but with little progress. In this paper, we first discuss general handling of the S-CPAC failures and then, we remind the details of the solution helping optimise usage of S-CPAC and propose next steps.

# 2	Discussion

## 2.1	Handling S-CPAC failures

The agreement made at #124 can be interpreted in two ways:

- The existing text is adapted to take into account the S-CPAC scenarios.

- The existing text is copied to a new chapter where S-CPAC is described.

To decide between the two options, it is important to consider the background of the decision made at #124 meeting. It was based on the understanding that single execution of S-CPAC is fundamentally the same as execution of CPAC configuration. Thus, possible problems and the needed corrective actions are the same. It is therefore reasonable to adapt the existing text so that it also covers S-CPAC – otherwise the specification work is likely to be duplicated.

The discussion that took place at RAN3 #125 seems to have confirmed this understanding: a TP was nearly agreed where the existing description was only reinterpreted.

Proposal 1.1: The existing stage-2 is edited so that it addresses S-CPAC, too.

The problem was the “reinterpretation”: the existing text refers to “CPA” and “CPC”, while in S-CPAC, the UE executes “initial access” and “subsequent cell change”.

The initial access is the first access to the cell configured for a S-CPAC. The UE may be in single-connectivity, but may also be in DC mode, but without S-CPAC configured. Therefore, both the existing definitions of “CPA” and “CPC” may apply to the initial access – but the latter only if the UE starts S-CPAC operation being already configured with DC mode.

Proposal 1.2: It is clarified that both, CPA and CPC mobility issues may apply to the S-CPAC initial access, the former when the UE starts S-CPAC from single-connectivity mode while the latter when UE starts S-CPAC from dual-connectivity.

The subsequent cell change is any cell change after the initial access. Therefore, CPA problems are not applicable to the subsequent cell change.

Proposal 1.3: It is clarified that only mobility problems of CPC are applicable to the subsequent cell change.

## 2.2	Optimisation of the CPAC / S-CPAC usage

The below solution description was first presented in [1].

The consequences of the use of S-CPAC introduced in Rel-18 (NR_Mob_enh2) [2] have been discussed in [3], here we remind them briefly:

Normal Rel-17 CPAC-configured UE has to release the CPAC configurations when completing random access towards the target PSCell. The advantage of S-CPAC, where multiple cells are prepared, is that an immediate UE autonomous execution for fast subsequent cell changes is possible, since conditional configuration for other candidates are not released after PSCell change. The justification of the feature in [2] was that in the case of frequent SCG changes when operating FR2 delays and signalling overhead can be avoided.

However, if such an immediate cell change is not needed, the resource blockage resulting from the prepared and unused candidate cells can be seen as disadvantage, as well as the increased UE memory needed because of the numerous condConfig options for each potential future hop. Obviously, there are disputable pros and cons for the two CPAC methods (classical vs. subsequent) which depend on dedicated cell layout situations. In case of fast subsequent cell changes the “Subsequent CPAC” is the preferred method.

Observation 2.1: Depending on the cell deployment and UE mobility, usage of either CPAC or S-CPAC may result in more efficient resource utilisation in involved cells.

The optimality of S-CPAC usage depends mainly on the time between needed PSCell changes. The shorter time between two consecutive CPCs, the more preferable is S-CPAC. If time is such short that UE fails with RLF before candidate cell preparation and UE configuration is completed, it is obvious that S-CPAC might be beneficial. Long residence time, on the other hand, hints usage of a the classic CPAC, so that resources are not blocked in prepared cells, while there is enough time for proper CPAC preparation.

Observation 2.2: The main criteria to decide between CPAC and S-CPAC configuration are the time between PSCell changes.

The key aspect is monitoring the time between PSCell changes: it shall not be too short in case of CPAC, while not too long in case of S-CPAC. This must be monitored per-cell pair, since cell change criteria are set per source cell towards each target cell.

Proposal 2.1: RAN3 agrees on enabling of the optimal usage of S-CPAC as part of SON based optimization

In principle, the residence time is logged in the UE history information (UHI). The problem with the UHI is that it does not provide information about the SCG mobility type (i.e. whether the UE may be configured at the same time with CPAC or S-CPAC). And reporting can be requested anytime unsynced with issue the resources where blocked unnecessary for long time..

In theory, the network may monitor the inter-change time as well. The problem with this approach is that the MN, which coordinates S-CPAC (also in case of SN-initiated S-CPAC the MN is the node providing communication with other SNs) may not be aware of intra-SN PSCell changes. Also, XnAP signalling between the MN and involved SNs affects time monitoring. Thus, network-based solutions are not feasible here.

In order to enable optimal usage of S-CPAC, it might be better that the UE is configured to report that suboptimal cases where time passes a threshold which depends on the configured method. In case of CPC configuration, it has to be checked whether the time is shorter than a defined “threshold_min_consecutive_CPC”, and if it is the case, it has to be reported to MRO instance responsible for this cell border where, for instance, a counter is incremented, which expresses that the two consecutive CPC-based cell changes are “tight in succession” indicating that S-CPAC might be preferable.

If UE is however already configured with S-CPAC, the opposite is monitored, namely whether the time is longer than a defined “threshold_max_consecutive_S-CPAC”, and this logged information is to be reported to the MRO instance responsible for this cell border, where a counter is incremented, which expresses that the two consecutive S-CPAC based cell changes are “too widely spaced” and would indicate that S-CPAC might waste network resources and normal CPC would be preferable, since time for preparation is not an issue.

Proposal 2.2: RAN3 shall request RAN2 to enable configuring a time threshold at which the UE is to report to the network either that between two cells time was too short for a classic CPAC or too long for S-CPAC.

# 3	Conclusions

In this paper, we first address the agreement made at the #124 meeting. We propose:

Proposal 1.1: The existing stage-2 is edited so that it addresses S-CPAC, too.

Then, we discuss the problem of selecting the most appropriate mobility scheme for a UE: CPAC or S-CPAC. We make following observations:

Observation 2.1: Depending on the cell deployment and UE mobility, usage of either CPAC or S-CPAC may result in more efficient resource utilisation in involved cells.

Observation 2.2: The main criteria to decide between CPAC and S-CPAC configuration are the time between PSCell changes.

Based on those, we propose:

Proposal 2.1: RAN3 agrees on enabling of the optimal usage of S-CPAC as part of SON based optimization

Proposal 2.2: RAN3 shall request RAN2 to enable configuring a time threshold at which the UE is to report to the network either that between two cells time was too short for a classic CPAC or too long for S-CPAC.

Related TP for stage-2 and a draft LS to RAN2 are proposed in the annexes below.

# References

- R3-243144, RAN3 #124

- RP-223520, RAN #98-e

- R3-241562, RAN3 #123-bis

# Annex 1: Text proposal to TS 37.340

The below text is proposed to be added to the TS 37.340.

First change, ommited text not changed

### 10.18.3	Conditional PSCell addition or change failure and subsequent conditional PSCell addition or change optimisation

Detection of connection failures

One of the functions of self-optimization for CPAC or S-CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases for wrong candidate PSCell list selection:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC or by the MN initiating the CPA, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC or at the MN initiating the CPA.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure. In case of S-CPAC configuration, following understanding applies:

-	The mobility problems related to CPA above apply to the initial PSCell access if the UE is in single-connectivity mode before the initial PSCell access.

-	The mobility problems related to CPC above apply to the initial PSCell access if the UE is in dual-connectivity mode before the initial PSCell access, or to any subsequent cells change.

The MN performs the initial analysis when SCGFailureInformation is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final root cause analysis. In case of no intra-SN PSCell change, the MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC in case of conditional mobility, if CPC whether it is MN initiated or SN initiated.

For CPA or MN initiated CPC, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation.

For SN initiated CPC, the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.

Optimisation of S-CPAC

Another function of self-optimisation for S-CPAC is to coordinate the usage of CPAC and S-CPAC. For that purpose, the UE may be configured to provide the needed information:

-	If the UE is configured with the CPC, it may be requested to report when the time between two consecutive CPC executions is lower than 'threshold_min_consecutive_CPC';

-	If the UE is configured with the S-CPAC, it may be requested to report when the time between two consecutive CPC executions is higher than 'threshold_max_consecutive_S-CPAC'.

Next change, ommited text not changed

# Annex 2: Draft LS to RAN2

3GPP TSG-RAN WG3 #125-bis		R3-24xxxx

Hefei, China, 14-18 Oct 2024

Title:	[DRAFT] LS on MRO for S-CPAC

Response to:

Release:	Release 19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Nokia [to be RAN3]

To:	RAN2

Cc:

Contact Person:	Krzysztof Kordybach (krzysztof.kordybach@nokia.com)

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:	R3-24xxxx

1. Overall Description:

RAN3 would like to inform RAN2 about decisions made in the context of MRO for S-CPAC.

- Regarding handling execution failures during S-CPAC operations, RAN3 assumed the existing framework defined for CPAC will be used.

- RAN3 has also decided to enable a mechanism to support selection of the configuration for the UE: CPAC or S-CPAC. For this purpose, RAN3 agreed a TP, as attached to the LS.

2. Actions:

To RAN2

RAN3 would like to ask RAN2 to enable the needed signalling, as described in the attached TP.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #126	18-22 November 2024			Orlando, USA

TSG-RAN3 Meeting #127	17-21 February 2025			Athens, Greece, EU