3GPP TSG-RAN WG3 Meeting #125-bis	R3-245340

HeiFei, China, Oct 14th – 18th October, 2024

Agenda Item:	14.3

Source:	CSCN

Title:	Discussion on support of regenerative payload for NR NTN

Document for:	Discussion

1	Introduction

In the last RAN3 #125 meeting, there are some agreements as below about discussions on NG interface management solutions and some other open issues of supporting of Regenerative Payload.

NG Interface management Issue: Option 1: Legacy procedure, e.g. SCTP SHUTDOWN Option 2: NG Removal/Setup Option 3: NG suspend/resume Option 4: Both Op2 and Opt3 Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. Further discussions on benefits to introduce Option 3 NG Suspend/Resume for NG interface management are not precluded but there are oppositions of some companies to have 2 methods for NG interface management. Inactive UE support: RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

In this contribution, we will further discuss the following issues on the support of regenerative payload.

Issue 1: NG Interface management

Issue 2: Supported TAI list

2	Discussion

Issue 1: NG Interface management

As mentioned in the contribution R3-244051 last meeting, regression period of LEO satellite is approximately 12 hours, which means the satellite can return to its original orbital position and see again the same location on earth. Different from the concept of regression period, ‘one complete circle’ period of LEO is 90-110 minutes depending on the altitude of the satellite, which means the satellite will return to a position very close to its original position (possibly overlapping the original coverage area) after one circle round the earth due to earth rotation. Actually, the satellite can connect to the original AMF after a regression period, but may not connect to the original AMF after a complete circle period. So, it is possible to resume the connection between satellite and the AMF after a regression period, but may not possible to have the NG resume procedure after a complete circle.

Observation 1: It is possible to resume the connection between satellite and the AMF after a regression period, but may not possible to have the NG resume procedure after a complete circle.

There are also some proposals that gNB can configure either a timer or some other NG resume conditions for NG suspend procedure, which can save the NG resume procedure, then the gNB can resume automatically with the AMF. However, the TAI list is different and updated when the satellite move into and move out of the AMF serving area. In this way, gNB does not actually save NG resume signal.

For example, the gNB on board send NG suspend message to the AMF when the satellite and AMF is about to disconnect (i.e., the satellite moves out of the AMF service area), and the TAI list for the gNB and AMF is TAI list_1 at this NG suspend time. The gNB try to reconnect with the same AMF again when the satellite moves into the AMF service area, and the TAI list will be the TAI list_3 (i.e. NG resume time) after a circle period. Then the gNB still needs NG resume to update the TAI list saved by gNB and AMF in the NG suspend procedure. Besides TAI list, there are also many others that are also needed to be updated after a period of time. So NG suspend/resume procedure actually doesn't save too much NG signaling as it claims.

Observation 2: The TAI list may be different and updated when the satellite moves into and moves out of the AMF serving area.

Although whether the TNL layer remains connected or not is out of RAN3 scope, TNL connectivity have influence on the NG interface. If TNL is always connected, then TNL needs to be maintained for approximately 12 hours, in which case NG suspend may have some benefit, but it also consume some of the satellite's valuable storage. If TNL cannot maintain a continuous connection (legacy procedure), which means the higher-layer NG connection should followed by the lower-layer TNL connection, and the NG suspend procedure won’t have much actual benefit. Considering the TNL connection is not always present, NG resume procedure will be affected.

Observation 3: Considering the legacy TNL connection is not always present, NG connection will be affected.

At the last meeting, some companies mentioned to support both option2&3 at the same time. Based on the above content, we think there is no need to introduce the NG suspend/resume procedure.

Proposal 1: No need to introduce NG suspend/resume procedure.

Issue 2: Supported TAI list

So far there are two methods for providing supported TAI list:

Signalling based solution

OAM based solution

Considering the NGSO is moving at about 7.56km/s and satellite beam diameter of LEO-600 is 50km (LEO-1200 is 90km), no matter what moving cell or quasi earth-fixed cell, the supported TAI list changes over a few seconds or a few tens of seconds. As a result, the gNB on board need to frequently notify the AMF of the supported TAI list which the current gNB belongs, so that CN can paging UEs correctly.

In fact, the supported TAI list of gNB and AMF is known very well to the OAM according to the implementation and preliminary designing. OAM can pre-configure the following supported TAI list for both gNB on board and the AMF without frequent NG signalling to configure and update, e.g. NG setup and RAN Configuration Update signal.

So we prefer to have a OAM method to configure the AMF and gNB on board to avoid information interaction during supported TAI list changing.

Proposal 2: OAM method can be used to configure supported TAI for gNB and AMF to avoid frequent signaling interactions including NG setup and RAN Configuration Update signalling.

3	Conclusion

In this contribution, we provide considerations on the NTN regenerative payload in RAN3, following observations and proposals are made:

Observation 1: It is possible to resume the connection between satellite and the AMF after a regression period, but may not possible to have the NG resume procedure after a complete circle.

Observation 2: The TAI list may be different and updated when the satellite moves into and moves out of the AMF serving area.

Observation 3: Considering the legacy TNL connection is not always present, NG connection will be affected.

Proposal 1: No need to introduce NG suspend/resume procedure.

Proposal 2: OAM method can be used to configure supported TAI for gNB and AMF to avoid frequent signaling interactions including NG setup and RAN Configuration Update signalling.

4	Reference

- 3GPP TR 38.821, Solutions for NR to support non-terrestrial networks (NTN)

- R3-244051 Regenerative payload considerations, Eutelsat Group, RAN WG3 Meeting #125