3GPP TSG-RAN WG3 Meeting #125-bis                                                        R3-245354

Hefei, CH, 14 - 18 October 2024

Agenda item:	 11.3

Source:		Rakuten

Title:		Discussion on AI/ML based CCO

Document for:		Discussion and Decision

# 1	Introduction

RAN3#125 meeting agreed that:

Predicted CCO Issue (including predicted affected cells/beams) from CU to DU, in the following two scenarios:

- CU1 predicts the CCO issue for DU1 and sends to DU1

- Based on the received future CCO state (including an associated coverage modification cause) from CU1, CU2 will deduce for DU2 the predicted CCO issue and affected cells/beams, and send it to DU2.

For split arch, gNB-CU sends the future coverage status to neighbour gNBs, together with an associated coverage modification cause.

For non-split arch, gNB sends the future coverage status to neighbour gNBs, together with an associated coverage modification cause.

In TP, capture whether the CU1 generates the suggested future CCO state to DU1 as assistance information is left to normative discussion.

Future CCO state and predicted CCO issue will be provided together with timing information.

In this contribution, we would further discuss details of AI/ML based CCO.

# 2	Discussion

# In the legacy CCO mechanism (for the case of split architecture):

Step 1: The gNB-CU detects the CCO issue (i.e., either coverage issue or cell edge capacity issue) and affected cells/SSBs;

Step 2: The gNB-CU sends the CCO Assistance Information IE (within the GNB-CU CONFIGURATION UPDATE message over F1) containing the CCO issue type and affected cells/SSBs to the gNB-DU;

Step3: The gNB-DU uses the received CCO Assistance Information IE to determine the new coverage state (CCO state) for the affected gNB-DU’s cells/SSBs;

Step 4: The gNB-DU sends the Coverage Modification Notification IE (within the GNB-DU CONFIGURATION UPDATE message over F1) containing the new CCO state for the affected gNB-DU’s cells/SSBs to the gNB-CU;

Step 5: The gNB-CU sends the Coverage Modification List IE (within the NG-RAN NODE CONFIGURATION UPDATE message over Xn) containing the detected CCO issue type and the new CCO state for the affected gNB-DU’s cells/SSBs to the neighbour gNB to allow for the neighbour gNB to determine a new CCO state for its own cells/SSBs that best matches the new CCO state of the gNB-CU’s cells/SSBs resulting from the detected CCO issue.

And in the AIML-based CCO mechanism (for the case of split architecture):

-	Step 0: AIML training entity generates predicted CCO issue

-	Step 1: gNB-CU predicts the CCO issue.

-	Step 2: gNB-CU sends the predicted CCO issue to gNB-DU.

-	Step 3: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether only local information is used can be further discussed.

-	Step 4: gNB-DU sends the future coverage status to gNB-CU.

-	Step 5: gNB-CU sends the future coverage status to neighbour gNBs.

The main issue with the legacy mechanism is that the gNB-DU in the source cell, where the CCO problem is first detected, makes coverage modifications autonomously which addresses only the locally detected issue. It does not consider what impact this change could have on the neighbouring gNBs in particular, or on the network in general.

Similar problem occurs even in the case of AIML based CCO, where the information from AIML training entity assists the gNB-CU in inferring the CCO issue, but there is not enough information provided to the gNB-DU about the impacted neighbouring gNBs to make a globally optimized coverage modification configuration change. Moreover, the neighbouring gNBs are not aware of the issue faced by the source gNB until Step#5.

Observation 1: Both the legacy and the AI/ML based CCO mechanism have the limitation that the changes made by the source cell gNB-DU does not have the visibility of the impact the changes made by it causes in the neighbouring gNBs. It only benefits the local issue faced by the source cell. This could results in unwanted and unintended issues in the neighbouring gNBs. Hence there is a need for a solution which addressses the CCO problem from a global perspective.

This limitation can be addressed with the help of enhanced AIML capabilities at the AI/ML entity residing at the OAM. In the following sections, we discuss the proposed changes at each step to ensure a globally optimal network behaviour.

  - 2.1 AI/ML Model Training Output

AI/ML-based CCO model in a gNB can generate following information as output [1]:

-	Predicted CCO issue (including predicted affected cells/beams)

-	Future CCO State

NOTE 1:	Future CCO State can also be derived by legacy means. Signalling future CCO state will not be described as prediction over Xn.

NOTE 2:	Future CCO state and predicted CCO issue will be provided together with timing information.

NOTE 3:	Future CCO state can be derived based on the predicted CCO issue and other information.

The AIML entity outputs the predicted CCO issue and future CCO state which is sent to the source gNB-CU along with the timing information (Step#0). The neighbouring gNBs remain unaware of the predicted CCO issue of the source gNB, till Step#5 described above.

To address the above concern, the AI/ML entity should be enhanced to additionally output the following supplementary information:

- List of neighbouring gNBs (including affected cells/beams) which might be impacted by the CCO issue of the source gNB

- Future CCO state of the neighbouring gNBs indicating the cell/SSB coverage modification

All this information along with the original information of the predicted CCO issue and the future CCO state is provided to the neighbouring gNB-CUs which are impacted by the predicted CCO issue together with the timing information. The list of neighbouring gNBs is also shared with the source gNB which can then effectively send GNB-CU CONFIGURATION UPDATE to these gNBs when the source gNB-DU has changed the coverage configuration.

In summary, the source gNBs should receive the following output from the AI/ML training entity:

- Predicted CCO issue (including predicted affected cells/beams)

- Future CCO State of source gNB

- Timing information of the predicted CCO issue of the source gNB

- List of the neighbouring gNBs (including affected cells/beams) which might be impacted by the CCO issue/action of the source gNB

And all the identified neighbouring gNBs which might be impacted by the CCO issue/action of the source gNB should receive the following output from the AI/ML training entity:

- Predicted CCO issue (including predicted affected cells/beams)

- Future CCO State of source gNB

- Timing information of the predicted CCO issue of the source gNB

- List of the neighbouring gNBs (including affected cells/beams) which might be impacted by the CCO issue/action of the source gNB

- Future CCO state of the neighbouring gNBs (including affected cells/beams) indicating the cell/SSB coverage modification

Since the AIML entity is outputting the future CCO states for both the source gNB and the neighbouring gNBs (impacted by the CCO issue of the source gNBs), this will ensure that the coverage modification changes made at the source gNB-DU will have a suitable reaction by the neighbouring cell gNB-DUs which have been apriori informed of the predicted issue, and the likely changes it will incur at the source gNB. This further prevents multiple neighbouring gNBs from making a reactive change in their coverage configuration which might end up causing new unintended CCO issues.

The proposed enhancement to the of output from an AI/ML entity also ensures a joint optimization across multiple gNBs for potentially multiple CCO issues existing at the same time, and hence the greater need of this information from the AI/ML entity not just at the source cells which are predicted to experience the CCO issue, but also to all their impacted neighbouring gNBs. The collective use of this information by the source and the neighbouring gNBs ensures a solution of the predicted CCO issue(s) which ensures global optimality.

Proposal 1: We propose the source gNBs should receive the following output from the AI/ML training entity:

- Predicted CCO issue (including predicted affected cells/beams)

- Future CCO State of source gNB

- Timing information of the predicted CCO issue of the source gNB

- List of the neighbouring gNBs (including affected cells/beams) which might be impacted by the CCO issue/action of the source gNB

Proposal 2: We propose all the identified neighbouring gNBs which might be impacted by the CCO issue/action of the source gNB should receive the following output from the AI/ML training entity:

- Predicted CCO issue (including predicted affected cells/beams)

- Future CCO State of source gNB

- Timing information of the predicted CCO issue of the source gNB

- List of the neighbouring gNBs (including affected cells/beams) which might be impacted by the CCO issue/action of the source gNB

- Future CCO state of the neighbouring gNBs (including affected cells/beams) indicating the cell/SSB coverage modification

  - 2.3 AI/ML Model Inference

As proposed above, the source gNBs receiving the following output from the AI/ML training entity:

- Predicted CCO issue (including predicted affected cells/beams)

- Future CCO State of source gNB

- Timing information of the predicted CCO issue of the source gNB

- List of the neighbouring gNBs (including affected cells/beams)  which might be impacted by the CCO issue/action of the source gNB

Once the source gNB-CU receives the output of AI/ML training entity, it uses other inputs from neighbouring NG RAN nodes and UEs to infer a CCO issue. That brings us to discuss the following:

What are the relevant inputs required at the source gNB-CU for an accurate inference?

The source gNB can make an inference with some degree of accuracy based on information it receives from the local node itself, from the neighbouring gNBs and from the UEs. This information is dynamic and keeps changing over time. In addition to this, the gNB can also leverage the use of the configuration information from OAM which is more static in nature.

Configuration information from the OAM:

- Overlapping cell/beam information of the source and the neighbouring gNBs

Inputs from source cell:

- Resource Utilization

- HARQ KPIs

- HO KPIs

- QoS KPIs

Inputs from UEs:

- UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

- UE location information (e.g., coordinates, serving cell ID, moving velocity)

- UE Mobility History Information

- SON (CEF/RA/RLF) reports from the UE

Inputs from neighboring node:

- UE trajectory

- Resource Utilization

- Assistance Information based on UE measurement reports indicating a confirmation of the CCO issue

- Assistance information of neighboring cells/beams overlapping with the source cells

Proposal 3: The source gNB-CU takes the inputs from the OAM configuration, the source cell KPIs, the UE measurements s and assistance information from the neighbouring nodes to infer the CCO issue.

At what point in time does the source gNB-CU perform the inference?

The source gNB receives the prediction of the CCO issue and the timing of the predicted CCO issue as the output from the AIML entity. If the predicted CCO issue is a long way off in the future (in order of 10s of minutes or more), then the source gNB-CU may not have relevant information to help it make an accurate inference. For example, the predictive output from the AI/ML entity says that the source gNB will have a capacity issue at time to+t in the future (t0 being the current time). Then we propose that the gNB-CU uses the data from the network and the UEs to make periodic “intermediate” inferences between t0 and t.

From the future CCO state output from the AI/ML entity, the gNB-CU can also infer the “severity” of the problem. In the above example, if the intermediate inferences show increasing trend of the UE traffic in the source cell, then it is likely that the prediction from the AI/ML entity is accurate. However, if there is little or no increase in UE traffic in the source cell, then either the prediction is incorrect, or perhaps the degree of coverage modification required is lesser than the prediction. This information is shared periodically with the source gNB-DU as CCO Assistance Information IE (within the GNB-CU CONFIGURATION UPDATE message over F1) (Step#2) which can in turn make decisions to change the coverage modification. The gNB-DU sends the Coverage Modification Notification IE (within the GNB-DU CONFIGURATION UPDATE message over F1) containing the new CCO state for the affected gNB-DU’s cells/SSBs to the gNB-CU. The gNB-CU sends the Coverage Modification List IE (within the NG-RAN NODE CONFIGURATION UPDATE message over Xn) containing the new CCO state for the affected gNB-DU’s cells/SSBs with the neighbouring gNBs (Step#3).

Proposal 4: The source gNB-CU may make intermediate inferences to ensure the accuracy of the predicted CCO issue. This information is shared with the source gNB-DU which makes the appropriate coverage modification if needed, which is then communicated to the impacted neighbouring gNB-CUs.

  - 2.4 Action at Source gNB and Neighboring gNBs

Once the source gNB-CU infers a CCO issue, it sends to its DU the CCO Assistance Information IE within the GNB-CU CONFIGURATION UPDATE message over F1. The source gNB-CU may indicate to the gNB-DU the desired setting for Cell Coverage State and/or SSB Coverage State based on the future CCO status from the AI/ML entity and the resulting inference at the gNB-CU. This will help the gNB-DU select the appropriate values in alternate coverage modification configuration.

Proposal 5: The source gNB-CU may indicate to the gNB-DU the desired setting for Cell Coverage State and/or SSB Coverage State based on the future CCO status from the AI/ML entity and the resulting inference at the gNB-CU.

As proposed above, the neighbouring gNBs receiving the following output from the AI/ML training entity:

- Predicted CCO issue (including predicted affected cells/beams)

- Future CCO State of source gNB

- Timing information of the predicted CCO issue of the source gNB

- List of the neighbouring gNBs (including affected cells/beams) which might be impacted by the CCO issue/action of the source gNB

- Future CCO state of the neighbouring gNBs (including affected cells/beams) indicating the cell/SSB coverage modification

Based on this information, and the NG-RAN NODE CONFIGURATION UPDATE message received from the source gNB-CU over Xn, the neighbouring gNB-CU sends to its DU the CCO Assistance Information along with the desired setting for Cell Coverage State and/or SSB Coverage State. This enables the neighbouring impacted gNB-DUs to make appropriate coverage modifications in response to the coverage modifications made by the source gNB-DU.

2.5 Reversal of the CCO based coverage modifications

The basic understanding is that with use of cell planning tools, drive tests, inventory management etc., the operators set the cell’s coverage and capacity configurations to meet their requirements. These default configurations suffice the average needs anticipated by the operator, but these might be insufficient to address dynamic changes in the traffic.

The long-term cell RF parameters tuning, in order to meet the operator’s requirement on coverage and capacity, and the prioritization between them, is usually hosted by OAM and relies on UE radio measurements, e.g. RSRP, RSRQ, call drops statistics, etc which are collected by MDT and monitored using performance management function. This is not relevant to the scope of the current discussion.

The short-term cell coverage switch is implemented by the NG-RAN nodes in response to dynamic changes in the traffic conditions/requirements. The cell coverage configuration changes, assisted by the AI/ML entities, are described in the sections above. However, there has been no discussions so far in RAN3 on how long these changes are valid for. Should the NG-RAN nodes take explicit steps to reverse the changes in response to the predicted CCO issue when the “cause” of the issue goes away. Or should subsequent predictions from the AI/ML entity and the input data from the network and UEs result in reverting to a default setting.

Proposal 6: RAN3 to discuss the methods needed for reversal of the CCO changes to the default setting.

# 3	Conclusion

In this contribution, AI/ML based CCO are discussed. We have the following observations and proposals:

Proposal 1: We propose the source gNBs should receive the following output from the AI/ML training entity:

- Predicted CCO issue (including predicted affected cells/beams)

- Future CCO State of source gNB

- Timing information of the predicted CCO issue of the source gNB

- List of the neighbouring gNBs (including affected cells/beams) which might be impacted by the CCO issue/action of the source gNB

Proposal 2: We propose all the identified neighbouring gNBs which might be impacted by the CCO issue/action of the source gNB should receive the following output from the AI/ML training entity:

- Predicted CCO issue (including predicted affected cells/beams)

- Future CCO State of source gNB

- Timing information of the predicted CCO issue of the source gNB

- List of the neighbouring gNBs (including affected cells/beams) which might be impacted by the CCO issue/action of the source gNB

- Future CCO state of the neighbouring gNBs (including affected cells/beams) indicating the cell/SSB coverage modification

Proposal 3: The source gNB-CU takes the inputs from the OAM configuration, the source cell KPIs, the UE measurements s and assistance information from the neighbouring nodes to infer the CCO issue.

Proposal 4: The source gNB-CU may male intermediate inferences to ensure the accuracy of the predicted CCO issue. This information is shared with the source gNB-DU which makes the appropriate coverage modification if needed, which is then communicated to the impacted neighbouring gNB-CUs.

Proposal 5: The source gNB-CU may indicate to the gNB-DU the desired setting for Cell Coverage State and/or SSB Coverage State based on the future CCO status from the AI/ML entity and the resulting inference at the gNB-CU.

Proposal 6: RAN3 to discuss the methods needed for reversal of the CCO changes to the default setting.

# Reference

[1] TR 38.743

[2] TS 38.473