3GPP TSG-RAN WG3 #125bis	R3-245395

Hefei, PRC, Oct 14–18, 2024

Agenda Item:	10.2

Source:	Ericsson

Title:	(TP for BLCR for SON for TS 38.300) MRO enhancements for LTM and CHO with candidate SCGs

Document for:	Discussion and Decision

# 1	Introduction

One of the main objectives of the SONMDT Rel-19 WID [RP-234038] is to enhance the MRO features to optimize the LTM cell switch procedure.

MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

This contribution discusses considerations regarding MRO feature enhancements for LTM and for and CHO with candidate SCGs.

# 2	MRO enhancements for LTM

During RAN3#123-bis and RAN3#124, the following agreements were taken on MRO for LTM:

- Work on scenarios of near failure LTM

- Work on scenarios for the differentiation of too early LTM, too late LTM and LTM to wrong cell

- RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study.

During RAN3#125, the following agreements were taken on MRO for LTM:

Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover.

FFS to enhance failure scenario.

Take the existing detection mechanism descriptions as baseline.

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed.

The next sections will discuss the scenarios to be discussed in RAN3.

## 2.1	LTM scenarios

During RAN3#125, the following text proposal was agreed:

##### [1]5.5.2.2.2 Connection failure due to intra-system mobility One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows: - Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell. - Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell. - Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell. In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure. In case of LTM cell switch, the "successful handover" refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM. In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell. In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell. Editor’s note 1: FFS if LTM recovery needs to be captured in the definitions. Editor’s note 2: For the too late LTM cell switch, FFS if the re-establishment cell is a candidate cell, a cell not selected by CU, or both.

The Editor’s notes deal with LTM recovery and cell selection by the CU. The LTM recovery procedure can be performed only when the selected cell is a LTM candidate cell. When the selected cell is not a LTM candidate cell, the UE has to rely on the re-establishment procedure. Annex 1 contains a text proposal capturing this aspect. The same approach can be used for the detection mechanism, if the LTM preparation (i.e. cell selection by the CU) is taken into account.

Agree clarification on LTM recovery and LTM re-establishment, in TP in Annex 1.

Agree clarification on detection mechanisms, in TP in Annex 1.

- There are two ways for the UE to perform RRC re-establishment instead of LTM Recovery after a first RLF/HOF. Either the selected cell is not an LTM candidate cell, or the network did not allow the UE to perform LTM Recovery (attemptLTM-Switch-r18 in RRC). Therefore, 3GPP needs to differentiate (e.g. by means of information contained in RLF Report) cases where the selected cell was not an LTM candidate, or if LTM Recovery was not allowed by the network,

For LTM, in case of RLF/HOF, and for root cause analysis, the network needs to know the reason if and why LTM Recovery was not attempted (i.e. the selected cell was not a candidate cell, or LTM Recovery was not allowed).

## 2.2 	Successive failure cases.

During the RAN3#124 meeting, successive failure cases were discussed. The following figure was contributed to discuss further scenarios:

We analyse the figure as follows:

- Case a is already covered, as it is a failure with reestablishment.

- Case b is already covered, as it is a failure with a failed recovery.

- Case c is already covered, as it is a failure with a successful recovery.

- Case d has not been discussed in the context of LTM. It consists of a failure with a successful recovery, quickly followed by another failure and a reestablishment. This case has been discussed for CHO, and there is a solution defined for CHO. In this solution, the second RLF Report does not log the source cell ID, the time to connection failure and the last handover type. The reason for not logging those is that the report could then mistakenly be interpreted as describing a too early cell switch failure.

As case d has very similar properties to the same case for CHO, we propose to copy the solution defined for CHO (i.e. keep logging the source cell ID, time to connection failure and last handover type from the first failure).

For the case of LTM failure with a subsequent successful LTM recovery, do not log the source cell ID, the time to connection failure and the last handover type.

## 2.3 	Beam Failure Recovery after successful LTM cell switch

In LTM, the LTM Cell Switch command from the network to the UE includes a beam indication and an indication of an LTM candidate cell, based on which the UE accesses the indicated LTM candidate cell. That “beam indication” is specified as a Transmission Configuration Identifier (TCI) State Identity (ID) associated to the LTM candidate cell.

In a typical case, the network determines the beam to be included based on lower layer measurement reports from the UE (also called L1 measurement reports for LTM), which includes measurement per beam (SSB measurements, such as L1 RSRP for one or more SSBs). Since these measurements may not be layer 3 (L3) filtered, the radio conditions may change quite quickly, so it might not always be simple for the network to determine the optimal beam to indicate to the UE in an LTM cell switch. TCI states are sent from target DU to source DU, via the CU, at LTM preparation. There can be up to 128 TCI states signaled to the source DU. For DL pre-sync, the source DU can pick at most 16 of them. But in the LTM cell switch command, only one TCI state from the 16 can be chosen by the source DU

One problem that may occur is that the network may include a beam indication (e.g. TCI State ID=X) and, shortly after a successful LTM cell switch to an LTM candidate cell, the UE declares a Beam Failure Detection (BFD) for the beam indicated in the LTM cell switch for the indicated LTM candidate cell (let’s call it the target cell once the UE has performed the LTM cell switch there). Upon BFD in the target cell, the UE triggers a Beam Failure Recovery (BFR), selects another beam (e.g. corresponding to TCI State ID=Y) and continues the communication with the target cell without a Radio Link Failure (RLF).

RAN3 to support the optimization of BFR after successful LTM cell switch.

In order to avoid these failures, it would be beneficial for the source DU to understand that it selected the wrong TCI state, and to know the TCI state in which the UE successfully recovered. It is therefore proposed to signal this information from target DU to CU and to CU to source DU.

Target DU to signal BFD and BFR information (e.g. recovery TCI state) to the CU, which will forward this information to source DU.

## 2.4 	RACH-less and RACH-based optimization.

In an LTM cell switch attempt, the UE may be configured to obtain uplink sync in a candidate cell in two different ways.

- The first way is that early UL sync is achieved prior to the LTM cell switch execution to support a RACH-less procedure. Configuration for this procedure is optional. Early UL sync can be achieved in two different ways:

  - The network determines the TA based on a PDCCH ordered RA preamble transmission in the candidate cell.

  - The UE autonomously determines the TA based on measurements of the difference in time of arrival for DL signals in the candidate cell and the source cell.

- The second way is to use a RACH-based procedure, much like legacy handover. Configuration for this procedure is mandatory.

The RACH-less procedure may fail for a number of reasons, for example the TA configuration becoming stale after some time, or that the UE failed to autonomously calculate a TA. In this case the UE may attempt a RACH-based procedure. However, attempting a RACH-based procedure is not mandatory. If neither a RACH-based nor a RACH-less procedure succeeded by the UE, the UE will declare an RLF.

When performing an LTM cell switch, and if RACH-less access is configured in the UE, the conditions for RACH-less access may not be fulfilled. In that case, the UE may attempt a RACH-based access.

When performing an LTM cell switch, and if no RACH-less access is configured in the UE (i.e. no TA is available for that candidate), the UE will attempt a RACH-based access.

For LTM, we need to distinguish between a set of failure cases:

- The UE was configured for both RACH-less and RACH-based access. The RACH-less access cannot be executed, and the UE attempted a RACH-based access. The RACH-based access succeeded. This case should not result in a RLF Report, as the access finally succeeded.

- The UE was configured for both RACH-less and RACH-based access. The RACH-less access cannot be executed, and the UE attempted a RACH-based access. The RACH-based access failed as well. This case should result in a RLF Report.

- The UE was configured for only a RACH-based access. The RACH-based access failed. This case should result in a RLF Report.

Case 1 above should not result in an RLF. However, cases 2 and 3 should generate an RLF. As it is now, there is no possibility of separating these failures.

For current RLF Reports, there is no possibility to observe whether an LTM cell switch failure attempted both RACH-less and RACH-based access, or only RACH-based access.

However, for root cause analysis, it is important to distinguish these cases. Therefore, it is proposed that the RLF Report is enhanced with information indicating that RACH-less access was attempted but failed.

Enhance RLF Report with information indicating that RACH-less access was attempted but failed.

Using early UL sync, the network can instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE. In the LTM cell switch command, if a TA value is included or if the UE has successfully measured the TA, then RACH-less LTM can be executed. If RACH-less LTM cell switch cannot be executed, the UE may attempt a RACH-based LTM. However, if the RACH-based LTM is successful, the interruption time will be larger than with RACH-less LTM. Hence it is necessary to log the early UL sync related information to optimize this undesired behavior.

If the UE fails RACH-less LTM, and subsequentially performs a RACH-based LTM, this will increase the LTM interruption time. This is a sub-optimal scenario.

In order to log the early UL sync related information in the SHR, a new triggering condition can be considered, e.g., SHR can be triggered when the UE attempts a RACH-based LTM after a failed RACH-less LTM.

Introduce a new trigger to log SHR indicating that RACH-based LTM was attempted after RACH-less LTM failed.

Enhance SHR with information indicating that RACH-less access was attempted but failed.

Similar observations and enhancements can be made for the RA Report. As described above, in the LTM cell switch command, if a TA value is included or if the UE has successfully measured the TA, then the RA procedure can be skipped and RACH-less LTM can be performed.

Failure of the RACH-less LTM cell switch will lead to an LTM recovery procedure or an RRC connection re-establishment procedure, with an increased interruption time for the UE. With enhancement to RLF Report proposed above, the RACH-less failure can be known to the network.

However, if the RACH-less procedure cannot be executed, the UE may attempt a RACH-based procedure before expiry of T304. In order to detect if there is a previous RACH-less failure before the RA procedure, an indication of the failed RACH-less procedure can be included in RA-Report when the RACH procedure is successfully completed. In case that the RACH-based procedure also fails, this indication is then useful in the RLF-Report for the network to detect the root cause for the LTM cell switch failure. Furthermore, in a near-failure case, this indication is also beneficial in SHR for the network to optimize the LTM cell switch procedure.

RA-InformationCommon is included in the RA Report, the RLF Report and the SHR. It is simple and straightforward to log the fact that a failed RACH-based attempt was followed by RACH-based LTM in RA-InformationCommon.

Include an indication that a failed RACH-less attempt was followed by a RACH-based LTM access (e.g. TA validity timer has expired or that UE failed to autonomously calculate TA) inside RA-InformationCommon to enable logging it in the RA-report, the RLF-Report and the SHR.

## 2.5	Optimization of early UL synchronization

Early UL synchronization is an important part of LTM, since it is one of the mechanisms that allows the LTM cell switch execution to take place with really short interruption. To achieve this benefit, the source DU has to ensure that the early UL synchronization takes place with some margin before the LTM cell switch execution is actually triggered. On the other hand, if the source DU lets the TA obtained through early UL synchronization become too old, there is a risk that it is outdated and incorrect at the time of the LTM cell switch execution, potentially leading to an LTM failure. How fast a TA obtained through early UL synchronization becomes outdated is highly dependent on the circumstances, e.g. if UEs generally move fast or slow in the source cell or in certain beams in the source cell. This implies that it would be useful for the source DU to receive feedback on how well the early UL synchronization it triggers perform, e.g. how large are the UEs' transmission errors when they access the LTM target cell? Was it close to being so large that successful reception/decoding would have been precluded in the target DU? It is therefore proposed to acknowledge this scenario and to propose solutions to optimize early UL synchronization.

RAN3 to study mechanisms to optimize early UL synchronization e.g. target DU signals feedback on early UL synchronization performances to source DU.

## 2.6	Optimization triggered by RLF Report and its impact on F1

In case of LTM handover failure, there are at least two types of handover optimisation that the source gNB can perform:

Firstly, for failures due to the wrong selection of candidates, during RAN3#124 it was agreed that:

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

During RAN3#125, it was agreed that:

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed.

The FFS was discussed during RAN3#125 and some companies proposed that the CU sends the failure type to the DU together with the RLF Report. However, the CU does not need to identify the failure type when forwarding the RLF Report to the DU. The initial analysis consists in understanding if the fault is due to the CU or to the DU. And this can be done by comparing the re-establishment cell with the list of candidate cells. If the re-establishment cell is part of the candidate cells, then the RLF Report is sent to the DU. Otherwise, the CU is in fault and it should not forward the RLF Report. Having the failure type determined by the CU before forwarding the RLF Report to the DU will therefore add an unnecessary burden on the CU.

There is no need for the CU to send the failure type to the DU together with the RLF Report.

# 3	MRO enhancements for CHO with candidate SCGs

## 3.1	Case 9

- During RAN3#125, RAN3 agreed to study case 9, where either CHO or CPAC condition was fulfilled at any point in time before the failure, but not at failure.

| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |
| --- | --- | --- |

- It was clarified that it is beneficial for the network to understand that CHO or CPAC condition was fulfilled at some point. Some interesting solutions, however complicated (e.g. how many times the condition was fulfilled), were proposed. However, and to limit the impact on the UE, it is proposed to simply add a flag indicating that CHO or CPAC condition was fulfilled at any point in time before the failure.

Add a flag indicating that CHO or CPAC condition was fulfilled at any point in time before the failure.

## Reducing the time between associated conditions

One of the main differences when we compare CHO with candidate SCGs to plain-old CHO+CPAC is that one CHO configuration and one CPAC configuration can now been linked together. In fact, for the UE to apply the PCell and PSCell configuration, both conditions (the one related to PCell and the one related to PSCell) need to be fulfilled. The goal for an optimized feature is then to have both conditions fulfilled at the same time, or close. If one of the conditions is not fulfilled, the CHO with candidate SCGs will not be executed at all. Which is sub-optimal, to say the least, or can even lead to failure:

- RLF due to UE waiting for the CPAC condition to be fulfilled.

- SCG Failure due to UE waiting for the CHO condition to be fulfilled.

Therefore, one important point to be discussed and optimized is how to reduce the time between fulfilment of CHO execution condition and CPAC execution condition.

Study solutions to reduce the time between fulfilment of CHO execution condition and CPAC execution condition.

# 4	References

R2-125bis_Maint_eRedCap_SON-MDT_(Mattias)_20240419_Final.docx

3GPP TS 38.300, "NR and NG-RAN Overall Description; Stage 2"

R3-243846, Summary of Offline Discussion on MRO for LTM

# 5	Conclusion

- For MRO enhancements for LTM:

- Agree clarification on LTM recovery and LTM re-establishment, in TP in Annex 1.

Agree clarification on detection mechanisms, in TP in Annex 1.

For LTM, in case of RLF/HOF, and for root cause analysis, the network needs to know the reason if and why LTM Recovery was not attempted (i.e. the selected cell was not a candidate cell, or LTM Recovery was not allowed).

For the case of LTM failure with a subsequent successful LTM recovery, do not log the source cell ID, the time to connection failure and the last handover type.

RAN3 to support the optimization of BFR after successful LTM cell switch.

Target DU to signal BFD and BFR information (e.g. recovery TCI state) to the CU, which will forward this information to source DU.

- When performing an LTM cell switch, and if RACH-less access is configured in the UE, the conditions for RACH-less access may not be fulfilled. In that case, the UE may attempt a RACH-based access.

When performing an LTM cell switch, and if no RACH-less access is configured in the UE (i.e. no TA is available for that candidate), the UE will attempt a RACH-based access.

For current RLF Reports, there is no possibility to observe whether an LTM cell switch failure attempted both RACH-less and RACH-based access, or only RACH-based access

Enhance RLF Report with information indicating that RACH-less access was attempted but failed.

If the UE fails RACH-less LTM, and subsequentially performs a RACH-based LTM, this will increase the LTM interruption time. This is a sub-optimal scenario.

Introduce a new trigger to log SHR indicating that RACH-based LTM was attempted after RACH-less LTM failed.

Enhance SHR with information indicating that RACH-less access was attempted but failed.

Include an indication that a failed RACH-less attempt was followed by a RACH-based LTM access (e.g. TA validity timer has expired or that UE failed to autonomously calculate TA) inside RA-InformationCommon to enable logging it in the RA-report, the RLF-Report and the SHR.

RAN3 to study mechanisms to optimize early UL synchronization e.g. target DU signals feedback on early UL synchronization performances to source DU.

There is no need for the CU to send the failure type to the DU together with the RLF Report.

- For MRO enhancements for CHO with candidate SCGs:

Add a flag indicating that CHO or CPAC condition was fulfilled at any point in time before the failure.

Study solutions to reduce the time between fulfilment of CHO execution condition and CPAC execution condition.

# Annex 1:	TP to 38.300 on MRO Use Cases

First change

##### 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell. In these definitions, the UE can attempt LTM recovery if the selected cell is a LTM candidate, or re-establishment is the cell is not a candidate.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO is configured but the CHO execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell is the cell that served the UE at the last handover initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell detection mechanisms refers to Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell. For the Too Late LTM cell switch, the network can determine if the re-establishment cell is an LTM candidate cell or a cell not selected by CU at LTM preparation.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO execution until connection failure.

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

For MRO analysis, in case of RLFs or handover failures, the source gNB may take into account the information regarding the LBT failures occurred in UL and DL for a specific UE. For UL, the information may be collected and provided by the UE to the network in the RLF report. For DL, the information may be collected at the source and target gNBs. The target gNB may provide the information to the source gNB via the Access and Mobility Indication procedure over Xn, if requested by the source gNB during the handover preparation.

Retrieval of information needed for problem analysis

In order to retrieve relevant information collected at the network side as part of the UE context, the UE provides C-RNTI used in the last serving cell. If the cause for the failure is identified as a "Too Early HO" or a "HO to Wrong Cell", the NG-RAN node controlling the last serving cell shall, include in the HANDOVER REPORT message the C-RNTI used in the source cell of the last completed handover before the failure. If the NG RAN node controlling that source cell provided the Mobility Information, it is also included in the HANDOVER REPORT message. If used, the Mobility Information is prepared at the source NG RAN node of a handover and may refer to or identify any handover-related data at this NG RAN node. In the Handover Preparation procedure, the source gNB can request the target gNB to provide information on DL LBT failures at the target gNB during handover execution.

Handling multiple reports from a single failure event

In case the RRC re-establishment fails and the RRC connection setup succeeds, MRO evaluation of intra-RAT mobility connection failures may be triggered twice for the same failure event. In this case, only one failure event should be counted.

End of changes