3GPP TSG-RAN3 Meeting #125bis	R3-245415

Hefei, China, 14th – 18th October, 2024

Title:	[Draft] LS on per-UE UE performance metrics

Response to:

Release:	Release 18

Work Item:	NR_AIML_NGRAN

Source:	Huawei [to be RAN3]

To:	SA5

Cc:	RAN2

Contact person:	Damiano Rapone

damiano.rapone@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

As part of the Rel-18 Work Item on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, RAN3 agreed to specify UE performance metrics in TS 38.423 to be exchanged over the Xn interface from target gNB to source gNB after a handover event occurs. The UE performance metrics are collected by the target gNB and reported to the source gNB upon request.

The specified UE performance metrics are Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay and Average Packet Loss DL. In Rel-18 these UE performance metrics are collected and reported by the target gNB on a per-UE granularity.

RAN3 common understanding is that such per-UE performance metrics can be calculated by averaging the corresponding per-DRB measurements over all configured DRBs of the concerned UE. To this end, RAN3 considers per-DRB measurements as specified in TS 28.558.

However, RAN3 understanding of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558 for the Average UE Throughput DL and Average UE Throughput UL, respectively, is that such measurements are already specified to be on a per-UE granularity as it seems by referring to formulas in fields c) of the mentioned clauses, and not on a per-DRB granularity as it can be deduced by checking the measurement type in field e) of the same mentioned clauses.

If the RAN3 understanding is correct, RAN3 intends to refer to the content already specified by SA5 in clauses 6.3.1.4.1 and 6.3.1.4.2 of TS 28.558 for deriving the per-UE Average UE Throughput DL and Average UE Throughput UL.

For the remaining UE performance metrics, that is, Average Packet Delay and Average Packet Loss DL, the corresponding per-UE measurements lack of a clear indication from the standard as for the Average UE Throughput DL/UL. Therefore, RAN3 kindly asks SA5 to provide feedback on whether such indication for deriving per-UE Average Packet Delay and Average Packet Loss DL metrics could be provided by SA5 as already done for the per-UE Average UE Throughput DL/UL.

# 2	Actions

To SA5

ACTION: 	RAN3 respectfully asks SA5 to clarify whether the Average UE Throughput DL/UL as specified in clauses 6.3.1.4.1 and 6.3.1.4.2 of TS 28.558 are on a per-UE granularity.

RAN3 also asks SA5 to provide feedback on how per-UE Average Packet Delay and Average Packet Loss DL metrics could be derived.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#126				2024-11-18 – 2024-11-22				Orlando, US

RAN3#127				2025-02-17 – 2025-02-21				Athens, GR