3GPP TSG-RAN WG3 Meeting #125bis	R3-245418

Hefei, China, 14th – 18th October, 2024

Title: 	AI/ML-based Coverage and Capacity Optimization

Source: 	Huawei

Agenda item:	11.3

Document Type:	Discussion and Decision

# 1. Introduction

In RAN#105 the Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” was approved in [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (latest SID in [2]) whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Coverage and Capacity Optimization (CCO), as excerpted from [1] and highlighted in yellow below:

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743. The detailed objectives of the WI are listed as follows: - Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing and AI/ML based CCO. [RAN3] - Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]: - Mobility Optimization for NR-DC - Split architecture support for Rel-18 use cases - Continuous MDT collection targeting the same UE across RRC states Note: Coordination with RAN2, SA5 when needed if any.

This contribution analyzes the technical enablers for the AI/ML-based CCO use case to be introduced over F1 and Xn interfaces, and provides proposals for discussion.

# 2. Discussion

As stated before, RAN3 in Rel-19 will specify data collection enhancements and signaling support in NG-RAN for the AI/ML-based CCO use case, among others. As clearly stated in the WID, the specification work in Rel-19 for AI/ML for NG-RAN is based on the conclusions captured in TR 38.743, which are excerpted from [3] and reported below:

# 6 Conclusion The new use cases and the Rel-18 leftover cases follow Rel-18 AI/ML framework. The following new use cases are recommended by RAN3 to be specified in the Rel-19 normative phase: - AI/ML-based Network Slicing - AI/ML-based Coverage and Capacity Optimization Recommended solutions and standard impacts for each use case are based on section 4.1.2 and section 4.2.2. For each use case above, it is recommended to take the corresponding section “Solutions and standard impacts” (section 4.1.2 and 4.2.2) as basis during the normative work. The following Rel-18 leftovers are recommended by RAN3 to be specified in Rel-19 normative phase: - Mobility Optimization for NR-DC - Split architecture support for Rel-18 use cases - Continuous MDT collection targeting the same UE across RRC states The corresponding descriptions and potential standard impacts for each Rel-18 leftovers above shall be taken as baseline during the normative phase.

Prior discussing the technical solution to enable the AI/ML-based CCO use case in Rel-19 and the corresponding standard impacts the legacy CCO mechanism is recalled.

For the split architecture:

- the gNB-CU detects the CCO issue (i.e., either coverage issue or cell edge capacity issue) and affected cells/SSBs;

- the gNB-CU sends the CCO Assistance Information IE (within the GNB-CU CONFIGURATION UPDATE message over F1) containing the CCO issue type and affected cells/SSBs to the gNB-DU;

- the gNB-DU uses the received CCO Assistance Information IE to determine the new coverage state (CCO state) for the affected gNB-DU’s cells/SSBs;

- the gNB-DU sends the Coverage Modification Notification IE (within the GNB-DU CONFIGURATION UPDATE message over F1) containing the new CCO state for the affected gNB-DU’s cells/SSBs to the gNB-CU;

- the gNB-CU sends the Coverage Modification List IE (within the NG-RAN NODE CONFIGURATION UPDATE message over Xn) containing the detected CCO issue type and the new CCO state for the affected gNB-DU’s cells/SSBs to the neighbour gNB to allow for the neighbour gNB to determine a new CCO state for its own cells/SSBs that best matches the new CCO state of the gNB-CU’s cells/SSBs resulting from the detected CCO issue.

For the non-split architecture, instead, steps b. to d. above are internal to the gNB, hence there is no standard impact, and steps a. and e. still hold (with gNB replacing gNB-CU / gNB-DU).

For the AI/ML-based CCO use case, the input, output and feedback data for the AI/ML model deployed in the gNB(-CU) have been identified and listed in TR 38.743 clauses 4.2.2.2, 4.2.2.3 and 4.2.2.4, respectively.

Concerning the input data, and focusing on the information from both the local node and the neighbour nodes, the following information have been agreed during the SI and reported in TR 38.743:

#### 4.2.2.2 Input data of AI/ML based CCO For a proactive prediction and resolution of a CCO issue, a gNB may need the following information as input data for AI/ML-based CCO: From local node: - Measured/Predicted radio resource status - Current CCO State From neighbouring gNBs: - Measured/Predicted radio resource status From the UE: - UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements - SON Reports (e.g., RLF, CEF, RA)

The transfer of both measured and predicted radio resource status is already supported by the standard over the Xn interface, by means of the Radio Resource Status Reporting (Initiation) and Data Collection Reporting (Initiation) procedures, respectively. While for the current CCO state (of the local node), this input information is an internal input of the local node, and in case of split gNB it is already supported by the standard over F1 (transfer of the CCO state from the gNB-DU to gNB-CU as in step d. above), hence there is no standard impact as well.

Observation 1: For the AI/ML-based CCO, there is already standard support for providing the agreed input information for the AI/ML model in the local node – i.e., measured/predicted radio resource status (from both local node and neighbour nodes) and current CCO state (of the local node).

For the feedback information, instead, the following information have been agreed by RAN3 during the SI phase to be collected from neighbour gNBs, as reflected in TR 38.743:

#### 4.2.2.4 Feedback of AI/ML based CCO To optimize the performance of AI/ML-based CCO model, following feedback can be considered to be collected from gNBs: - Measured radio resource status - Legacy UE performance feedback for those UEs handed over from the source gNB - SON Reports (e.g., RLF, CEF, RA)

Also in this case, the transfer over Xn of all the pieces of information listed above is already supported by the standard.

Observation 2: For the AI/ML-based CCO, there is already standard support for providing the agreed feedback information for the AI/ML model in the local node collected from neighbour gNBs – i.e., measured radio resource status, legacy UE performance feedback, SON Reports.

With regards to the output data, RAN3 agreed in the SI phase the following information are generated by the A/ML model, as reflected in TR 38.743:

#### 4.2.2.3 Output data of AI/ML based CCO AI/ML-based CCO model in a gNB can generate following information as output: - Predicted CCO issue (including predicted affected cells/beams) - Future CCO State NOTE 1: Future CCO State can also be derived by legacy means. Signalling future CCO state will not be described as prediction over Xn. NOTE 2: Future CCO state and predicted CCO issue will be provided together with timing information. NOTE 3: Future CCO state can be derived based on the predicted CCO issue and other information.

As indicated in NOTE 1 within clause 4.2.2.3 of TR 38.743, the future CCO state can be derived also via legacy means, depending on the implementation, for both non-split and split architecture. In particular for the split architecture case, and following the legacy CCO mechanism where the CCO state is always decided by the gNB-DU based on the information provided by the gNB-CU, the future CCO state could also be derived via an AI/ML model deployed in the gNB-DU for which, however, there is no standard support as agreed by RAN3 during the SI phase.

Both pieces of output information listed above are not supported in current XnAP and F1AP specifications, and the NOTE 2 within clause 4.2.2.3 of TR 38.743 also provides further indications concerning relevant timing information of such AI/ML model’s output data.

Finally, TR 38.743 clause 4.2.2.5 lists the standards impacts for both Xn and F1 interfaces as follows:

#### 4.2.2.5 Potential standard impacts Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18 based on the CCO framework described in TS 38.300 [2] and TS 38.401 [3]. Xn interface: Enhance existing procedure to collect information between gNBs: - Future CCO State together with an associated coverage modification cause - Timing Information F1 interface: - Predicted CCO Issue (including predicted affected cells/beams) from gNB-CU to gNB-DU. - Future CCO State from gNB-DU to gNB-CU - Timing Information NOTE 4: Whether gNB-CU generates the suggested future CCO state and sends it to gNB-DU as assistance information is left to normative discussion. NOTE 5: For split architecture, future CCO State is derived based on the predicted CCO issue signaled from gNB-CU and other information in the gNB-DU.

Observation 3: For the AI/ML-based CCO, there are standard impacts for both Xn and F1 interfaces for transferring the agreed output information – i.e., predicted CCO issue, future coverage state and relevant timing information.

## 2.1 Timing information for AI/ML-based CCO

Prior discussing the standard impact for transferring the agreed AI/ML output information over Xn and F1 interfaces, it is worth discussing the relevant timing information of predicted CCO issue and future CCO state.

In the legacy CCO mechanism, after a certain CCO issue has been detected by the gNB(-CU), the gNB(-DU) should react basically immediately by selecting a new CCO state being able to cope with detected CCO issue. For the AI/ML-based CCO use case, instead, a more proactive approach is used to prevent (or limit at an early stage) the occurrence of a CCO issue resulting in a degradation of network (and UE) performance. This implies that, after a CCO issue is predicted by the gNB(-CU), the gNB(-DU) is requested to select a new CCO state to be applied in the future, hence not immediately because the CCO issue did not occur yet.

Observation 4: Differently from the legacy CCO mechanism, in the AI/ML-based CCO use case after a CCO issue has been predicted by the gNB(-CU), the gNB(-DU) will select a new CCO state to be applied in the future and not immediately.

In the last RAN3 meeting there was an initial discussion on the following options concerning the timing information for AI/ML-based CCO:

- Option A: the timing information of the future CCO state (i.e., the time point in the future where the future CCO state is activated) corresponds to the timing information of the predicted CCO issue (i.e., the time point in the future where the CCO issue is expected to occur);

- Option B: the timing information of the future CCO state is a separate piece of information with respect to the timing information of the predicted CCO issue – this means that the future CCO state can be activated before the time point at which the CCO issue has been predicted to occur. It is our understanding that activating the future CCO state after the CCO issue has been predicted to occur (in case it actually occurs) is out of AI/ML discussions since it basically corresponds to the legacy CCO mechanism.

To better understand the implications of both options above let’s discuss them one by one.

Figure 1 below is the call-flow for Option A and it shows the interaction between two neighbor gNBs, i.e., gNB1 and gNB2 (where gNB1 is a split gNB) for the purpose of AI/ML-based CCO; in this figure the timing information of the predicted CCO issue is indicated as predicted CCO issue time and, as per Option A, it also corresponds to the timing information of the future CCO state, i.e., the time point in the future where the future CCO state is activated.

Figure 1 - Call flow for Option A (future CCO state activation time corresponds to the predicted CCO issue time).

Here follows a list of steps describing the nodes’ interaction as for Option A:

- gNB-CU1 predicts a CCO issue (coverage, cell edge capacity, …) for affected cells/SSBs and the corresponding timing information related to the time point in the future where the predicted CCO issue is expected to occur (labelled as predicted CCO issue time);

- gNB-CU1 sends the predicted CCO issue for affected cells/SSBs and the corresponding predicted CCO issue time to gNB-DU1 in order for gNB-DU1 to determine a future CCO state being able to cope with the predicted CCO issue;

- based on at least the received information from gNB-CU1, the gNB-DU1 determines the future CCO state and provides it to the gNB-CU1;

- gNB-CU1 notifies neighbor gNB2 that a CCO issue is predicted to occur (by means of the coverage modification cause) at a point in time in the future (i.e., at the predicted CCO issue time) for which a future CCO state will be activated at predicted CCO issue time;

- gNB2 determines a future CCO state for its own cells/SSBs that best matches the future CCO state of the gNB-CU1’s cells/SSBs resulting from the predicted CCO issue that will occur at a point in time indicated in predicted CCO issue time.

Figure 2 shows how the network situation evolves over time in case of Option A.

Figure 2 - Graphical representation of the network situation over time in case of Option A.

At time t0 (current time) the CCO states for gNB(-DU)1 and gNB2 are indicated as CCO1A and CCO2A, respectively, and the AI/ML model in gNB(-CU)1 predicts a CCO issue to occur at time t2 (time point in the future). The gNB(-DU)1 determines its own future CCO state (indicated as CCO1B) to be activated at t2 (i.e., when the CCO issue is predicted to occur, as per Option A) and gNB(-CU)1 notifies gNB2 so that gNB2 can also determine its own “best matching” future CCO state (indicated as CCO2B) to be also activated at t2. In this way, at time t2 and following the nodes’ coordination activity which occurred within the time interval (t0 .. t2), the actual network situation results in such a way that the predicted CCO issue has been proactively avoided.

Lets’ now discuss Option B. Figure 3 below is the call-flow for this option and it shows the interaction between two neighbor gNBs, i.e., gNB1 and gNB2 (where gNB1 is a split gNB) for the purpose of AI/ML-based CCO; in this figure the timing information of the predicted CCO issue is still indicated as predicted CCO issue time but, as per Option B, the timing information of the future CCO state, i.e., the time point in the future where the future CCO state is activated, is distinct from the predicted CCO issue time and it is indicated as future CCO state activation time.

Figure 3 - Call flow for Option B (future CCO state activation time is different with respect to the predicted CCO issue time).

Here follows a list of steps describing the nodes’ interaction as for Option B (changes with respect to Option A is highlighted in green):

- gNB-CU1 predicts a CCO issue (coverage, cell edge capacity, …) for affected cells/SSBs and the corresponding timing information related to the time point in the future where the predicted CCO issue is expected to occur (labelled as predicted CCO issue time);

- gNB-CU1 sends the predicted CCO issue for affected cells/SSBs and the corresponding predicted CCO issue time to gNB-DU1 in order for gNB-DU1 to determine a future CCO state being able to cope with the predicted CCO issue;

- based on at least the received information from gNB-CU1, the gNB-DU1 determines the future CCO state and the corresponding future CCO state activation time, and provides them to the gNB-CU1;

- gNB-CU1 notifies neighbor gNB2 that a CCO issue is predicted to occur (by means of the coverage modification cause) at a point in time in the future (i.e., at the predicted CCO issue time) for which a future CCO state will be activated at a time corresponding to future CCO state activation time;

- gNB2 determines a future CCO state for its own cells/SSBs that best matches the future CCO state of the gNB-CU1’s cells/SSBs resulting from the predicted CCO issue that will occur at a point in time indicated in predicted CCO issue time.

Figure 4 shows how the network situation evolves over time in case of Option B.

Figure 4 - Graphical representation of the network situation over time in case of Option B.

At time t0 (current time) the CCO states for gNB(-DU)1 and gNB2 are indicated as CCO1A and CCO2A, respectively, and the AI/ML model in gNB(-CU)1 predicts a CCO issue to occur at time t2 (time point in the future). The gNB(-DU)1 determines its own future CCO state (indicated as CCO1B) to be activated at the future CCO state activation time indicated as t1 (i.e., before the time point in the future t2 when the CCO issue is predicted to occur, as per Option B) and gNB(-CU)1 notifies gNB2 so that gNB2 can also determine its own “best matching” future CCO state (indicated as CCO2B). Differently from Option A, with Option B the future CCO state in gNB(-DU)1 is activated at a time t1 < t2 and, depending on the activation time of the future CCO state in gNB2, a CCO issue might occur in t1 if the CCO state in gNB2 remains the same as it was in t0 (i.e., CCO2A). Therefore, in order to minimize such risk of a CCO issue at the t1 (i.e., at the activation time of the future CCO state in gNB(-DU)1) it needs to be ensured that the future CCO state in gNB2 is also activated at t1, i.e., at the same time point in the future where the future CCO state in gNB(-DU)1 is activated. In this way, at time t2 and following the nodes’ coordination activity which occurred within the time interval (t0 .. t1), the actual network situation results in such a way that the predicted CCO issue has been proactively avoided.

Based on the above analysis of both Options, the following conclusions can be drafted:

- Option B requires gNBs to coordinate within a time interval (t0 .. t1) which is shorter than the time interval (t0 .. t2) as in Option A – this poses a requirement in the neighbor gNB (gNB2 in the above analysis) to determine a “best matching” future CCO state to be activated at a point in time in the future which depends not only on the predicted CCO issue time (as in Option A) but also on the future CCO state activation time of the local gNB (gNB(-DU)1 in the above analysis);

- by allowing the gNB-DU to decide when the future CCO state is activated (as in Option B), there could be no means to deal with the case where multiple gNB-DUs (controlled by the same gNB-CU) determine, for the same predicted CCO issue, their own future CCO states which are to be activated at different activation times. This scenario may result in CCO issues similar to the one represented in Figure 4 case “B.”. In order to avoid such kind of scenarios it would be better that the activation time of future CCO state is decided by the gNB-CU (as in Option A);

- the higher flexibility in determining the activation time of the future CCO state of the local gNB (gNB(-DU)1 in the above analysis) offered by Option B comes at higher complexity of the overall AI/ML-based CCO use case compared to Option A – this because, in Option A, the activation time of the future CCO states in the gNBs is only determined by the predicted CCO issue time (i.e., by the time point in the future where the CCO issue is predicted to occur);

- the standard impact for supporting Option B is higher than the one needed to support Option A – this because, in Option A, there is the need to transfer less information than Option B (namely, the future CCO state activation time does not need to be transferred over both F1 and Xn, and only the predicted CCO issue time is needed).

Observation 5: Compared to Option B, Option A is a simpler (in terms of both complexity and standard impact) and more robust solution for the AI/ML-based CCO.

Based on the above we prefer Option A but we are open to discuss Option B as well.

Proposal 1: RAN3 to discuss and down-select between the following options concerning the timing information for the AI/ML-based CCO use case:

- Option A: the time point in the future where the future CCO state is activated corresponds to the time point in the future where the CCO issue is expected to occur;

- Option B: the future CCO state can be activated before the time point in the future at which the CCO issue is predicted to occur.

## 2.2 On recommended future CCO state from gNB-CU to gNB-DU

TR 38.743, clause 4.2.2.5, contains also a note (NOTE4) related to the possibility for the gNB-CU to generate a recommended/suggested future CCO state and send it to the gNB-DU as assistance information, so that the gNB-DU could take this information into account in order to determine the actual future CCO state in response of the predicted CCO issue. Below the note mentioned above is excerpted from in the TR [3]:

#### 4.2.2.5 Potential standard impacts Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18 based on the CCO framework described in TS 38.300 [2] and TS 38.401 [3]. **** Skip non-relevant text **** F1 interface: **** Skip non-relevant text **** NOTE 4: Whether gNB-CU generates the suggested future CCO state and sends it to gNB-DU as assistance information is left to normative discussion. **** Skip non-relevant text ****

In our previous paper in [4] we already provided our view on such scenario. From the legacy CCO mechanism, it could be noted that the gNB-CU actually has visibility of the coverage state and corresponding coverage modification cause information from the gNB-DU (i.e., Coverage Modification Notification IE within the GNB-DU CONFIGURATION UPDATE message) and, if available, from the neighbour gNBs (i.e., Coverage Modification List IE within the NG-RAN NODE CONFIGURATION UPDATE message) related to either another CCO issue detected in the neighbour gNB itself or as a consequence of a coverage configuration change in the concerned gNB-CU. In our understanding, this makes it possible for the AI/ML model in the gNB-CU to infer not only the predicted CCO issue but also the corresponding future coverage state based on historical information such as the previous coverage state from the gNB-DU and neighbour gNB. In this way, the gNB-DU could obtain richer and more meaningful information from the gNB-CU (i.e., not only the predicted CCO issue but also the future coverage state) to solve or mitigate the predicted CCO issue in advance. It should be noted that we are not excluding the possibility that the gNB-DU determines the actual coverage configuration and coverage state for its own cells/SSBs, that is, the gNB-DU could either directly consider the recommended future coverage state as determined by the gNB-CU or determine the actual future coverage state on its own based on the predicted CCO issue from the gNB-CU and by also taking the recommended future coverage state from the gNB-CU as a reference.

Observation 6: For the AI/ML-based CCO use case, both predicted CCO issue and corresponding future coverage state from the gNB-CU could represent richer and more meaningful prediction information for the gNB-DU to solve or mitigate the predicted CCO issue in advance.

Observation 7: gNB-CU could provide to the gNB-DU not only the predicted CCO issue but also the corresponding recommended future coverage state to be either directly considered by the gNB-DU or to be used as a reference by the gNB-DU in determining the actual future coverage state.

Proposal 2: RAN3 to agree that the gNB-CU could provide to the gNB-DU not only the predicted CCO issue but also the corresponding future coverage state as a recommendation: the gNB-DU could either confirm the recommendation from the gNB-CU or use it as a reference in determining the actual future coverage state.

## 2.2 On F1/Xn procedures for transferring predicted CCO issue and future CCO state

Other than the timing information aspects as analysed in Section 2.1, the predicted CCO issue as well as the future CCO state need to be introduced in F1AP and XnAP specifications.

To this end we think that the existing procedures – i.e., gNB-CU Configuration Update and gNB-DU Configuration Update over F1, and NG-RAN node Configuration Update over Xn – can be enhanced to transfer the new agreed information, and this allows to minimize the specification impact for supporting the AI/ML-based CCO use case.

Proposal 3: RAN3 to agree that existing procedures – i.e., gNB-CU Configuration Update and gNB-DU Configuration Update over F1, and NG-RAN node Configuration Update over Xn – can be enhanced to enable transferring the agreed information in support of the AI/ML-based CCO use case.

Over F1 interface:

- gNB-CU Configuration Update procedure:

  - Within the GNB-CU CONFIGURATION UPDATE message (from gNB-CU to gNB-DU) introduce a new IE Predicted CCO Assistance Information carrying information related to the predicted CCO issue and associated affected cells/SSBs, as well as its timing information (predicted CCO issue time).

- gNB-DU Configuration Update procedure:

  - Within the GNB-DU CONFIGURATION UPDATE message (from gNB-DU to gNB-CU) introduce a new IE Future Coverage Modification Notification carrying information related to the list of cells/SSBs with the corresponding coverage configuration selected by the gNB-DU as part of the future coverage state, the coverage modification cause and, if Option B is agreed, also the future CCO state activation time.

Over Xn interface:

- NG-RAN node Configuration Update procedure:

  - Within the NG-RAN NODE CONFIGURATION UPDATE message (from NG-RAN node1 to NG-RAN node2) introduce a new IE Future Coverage Modification carrying information related to the list of cells/SSBs with the corresponding coverage configuration selected by the NG-RAN node1 as part of the future coverage state, the coverage modification cause, the predicted CCO issue time and, if Option B is agreed, also the future CCO state activation time.

TPs for Xn and F1 BLCRs (covering both Option A and Option B related to the timing information) can be found in the contribution in [5].

# 3. Conclusion

Based on the above discussion, we have the following observations and proposals:

Observation 1: For the AI/ML-based CCO, there is already standard support for providing the agreed input information for the AI/ML model in the local node – i.e., measured/predicted radio resource status (from both local node and neighbour nodes) and current CCO state (of the local node).

Observation 2: For the AI/ML-based CCO, there is already standard support for providing the agreed feedback information for the AI/ML model in the local node collected from neighbour gNBs – i.e., measured radio resource status, legacy UE performance feedback, SON Reports.

Observation 3: For the AI/ML-based CCO, there are standard impacts for both Xn and F1 interfaces for transferring the agreed output information – i.e., predicted CCO issue, future coverage state and relevant timing information.

On timing information

Observation 4: Differently from the legacy CCO mechanism, in the AI/ML-based CCO use case after a CCO issue has been predicted by the gNB(-CU), the gNB(-DU) will select a new CCO state to be applied in the future and not immediately.

Observation 5: Compared to Option B, Option A is a simpler (in terms of both complexity and standard impact) and more robust solution for the AI/ML-based CCO.

Proposal 1: RAN3 to discuss and down-select between the following options concerning the timing information for the AI/ML-based CCO use case:

- Option A: the time point in the future where the future CCO state is activated corresponds to the time point in the future where the CCO issue is expected to occur;

- Option B: the future CCO state can be activated before the time point in the future at which the CCO issue is predicted to occur.

On recommended future CCO state from gNB-CU to gNB-DU

Observation 6: For the AI/ML-based CCO use case, both predicted CCO issue and corresponding future coverage state from the gNB-CU could represent richer and more meaningful prediction information for the gNB-DU to solve or mitigate the predicted CCO issue in advance.

Observation 7: gNB-CU could provide to the gNB-DU not only the predicted CCO issue but also the corresponding recommended future coverage state to be either directly considered by the gNB-DU or to be used as a reference by the gNB-DU in determining the actual future coverage state.

Proposal 2: RAN3 to agree that the gNB-CU could provide to the gNB-DU not only the predicted CCO issue but also the corresponding future coverage state as a recommendation: the gNB-DU could either confirm the recommendation from the gNB-CU or use it as a reference in determining the actual future coverage state.

On F1/Xn procedures for transferring predicted CCO issue and future CCO state

Proposal 3: RAN3 to agree that existing procedures – i.e., gNB-CU Configuration Update and gNB-DU Configuration Update over F1, and NG-RAN node Configuration Update over Xn – can be enhanced to enable transferring the agreed information in support of the AI/ML-based CCO use case.

TPs for Xn and F1 BLCRs (covering both Option A and Option B related to the timing information) can be found in the contribution in [5].

# 4. Reference

- RP-242385, New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#105

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- R3-243467, (TP to TR 38.743) AI/ML-based Coverage and Capacity Optimization, Huawei

- R3-245419, TPs for AI/ML BLCR to TS 38.743 and AI/ML BLCR to TS 38.423 for AI/ML-based Coverage and Capacity Optimization, Huawei