3GPP TSG-RAN WG3 Meeting #125bis									R3-245423

Hefei, China, October 14th - 18th, 2024

Agenda Item:	20.2

Source:	Huawei

Title:	Discussion on RAN3 impacts for Direct AI/ML positioning

Document for:	Discussion and decision

# Introduction

At the RAN#102 meeting, a new Rel-19 WI on AI for air interface [1]  was approved and the corresponding WID has been revised in RAN#105 meeting [2]. One objective which has RAN3 impacts is the specification support for positioning accuracy enhancements as follows:

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: **** Skip non-relevant text **** - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

This is the first meeting for RAN3 to discuss about the RAN3 impacts for supporting AI/ML-based positioning use cases and, in this contribution, we focus on Direct AI/ML positioning use cases and provide proposals for discussion accordingly. The contribution in [3], instead, focuses on RAN3 impacts for supporting AI/ML Assisted positioning use cases.

# Discussion

The following sections will discuss the potential RAN3 impacts for Direct AI/ML positioning use cases, which are listed in the WID [2] – along with the corresponding prioritization – as excerpted below:

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: **** Skip non-relevant text **** - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning **** Skip non-relevant text **** - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

For AI/ML positioning, the preliminary Rel-18 study phase – whose outcome is summarized in TR 38.843 [4] – identified two possible approaches depending on the type of AI/ML model output as follows:

## 5.3 Positioning accuracy enhancements Finalization of representative sub-use cases: The following are selected as representative sub-use cases: - Direct AI/ML positioning: - AI/ML model output: UE location - e.g., fingerprinting based on channel observation as the input of AI/ML model - AI/ML assisted positioning: - AI/ML model output: new measurement and/or enhancement of existing measurement - e.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement **** Skip non-relevant text ****

Based on the definition in TR 38.843 [4], for Direct AI/ML positioning use cases the output of the AI/ML model is an estimate (prediction) of the UE location.

Observation 1: For Direct AI/ML positioning use cases the output of the AI/ML model is an estimate (prediction) of the UE location.

Moreover, depending on the location of the AI/ML model used to directly infer the UE location, there can be multiple cases which have been agreed to be specified according to a clear prioritization as indicated in the WID [2].

Observation 2: Direct AI/ML positioning use cases are distinguished based on the location of the AI/ML model used to directly infer the UE location.

As indicated in the latest version of the WID in [2], a Note has been added stating that RAN1 will not discuss any proposals for AI/ML positioning cases which have been identified as 2nd priority cases in Q4 of 2024. Therefore, we start focusing our attention on the following two 1st priority use cases highlighted in yellow below:

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: **** Skip non-relevant text **** - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning **** Skip non-relevant text **** - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

In our understanding, RAN3 should focus on Case 3b above for Direct AI/ML positioning since Case1 does not have any RAN3 impact. Case 1, in fact, is a Direct AI/ML positioning case where the UE hosts an AI/ML model which is used to directly infer the UE location. Meanwhile, for Case 3b, an AI/ML model is hosted in the Location Management Function (LMF) which infers the UE location by means of measurements provided by the NG-RAN, meaning that some RAN3 impacts for signalling such measurements from the NG-RAN to the LMF are foreseen, also considering that the format of the measurement report is still under discussion in RAN1.

For the sake of completeness, we also think that Case 2b does not have any RAN3 impacts: in fact, such case uses an AI/ML model hosted in the LMF which directly infers the UE location by means of measurements provided by the UE.

Observation 3: Among the 1st priority cases for Direct AI/ML positioning, Case 1 does not have any RAN3 impact.

Based on the above Observations we provide the following:

Proposal 1: RAN3 to discuss for Direct AI/ML positioning only the impacts of Case 3b.

In the following section 2.1 we discuss aspects related to model training, model inference and model monitoring for Direct AI/ML positioning Case 3b.

## 2.1 RAN3 impacts for Case 3b

Case 3b is the Direct AI/ML positioning case of NG-RAN node assisted positioning with LMF-side model. A graphical representation of Case 3b can be seen in Figure 1.

Figure 1 - Direct AI/ML positioning Case 3b.

In order to assess the RAN3 impacts for supporting Direct AI/ML positioning Case 3b we will recall in below sections the relevant RAN1 agreements achieved so far concerning model training, model inference and model monitoring aspects.

### Model training considerations

In previous RAN1 meetings the following agreements were reached:

******** RAN1#116bis ******** Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: "Part A" and "Part B" terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. ******** RAN1#117 ******** Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed. Note: Part A and Part B may be generated by the same or different entities, depending on the cases. ******** RAN1#118 ******** Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective. Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications.

For Direct AI/ML positioning Case 3b our understanding is that the AI/ML model training is located at the LMF, which is the termination point of training data. This is based on what it is stated in TR 38.843 clause 7.2.4, where the training data termination point is identified for the LMF-based model as follows [4]:

### 7.2.4 Positioning accuracy enhancements **** Skip non-relevant text **** For data collection, model transfer/delivery, and function-to-entity mapping analysis, various scenarios unfold when the data generation and termination entities differ. For instance, for: - Model Training: **** Skip non-relevant text **** o For LMF-side model, the LMF is the termination point for training data.

Observation 4: For Direct AI/ML positioning Case 3b the AI/ML model training is located in the LMF.

According to previous RAN1 agreement cited above, it can be seen that a training data sample is composed of two parts, a “measurement” (Part A in RAN1 discussions) and a “ground-truth label” (Part B in RAN1 discussions). For the “measurement” RAN1 agreed that it is generated by the TRP/gNB, that is, NG-RAN; while the “label” can be generated by PRU/Non-PRU UE with estimated location or directly at the LMF. Therefore, it can be acknowledged that there is RAN3 impact in providing the “measurement” part of the training data sample from NG-RAN to LMF. For the “label”, instead, if it is generated by PRU/Non-PRU UE with estimated location, there is no RAN3 impact since this information can be provided via LPP protocol to the LMF. While if the “label” is generated by the LMF there is as well no RAN3 impact since this information is already available in the entity performing AI/ML model training, i.e., LMF.

In addition, RAN1 agreed that the purpose for which a certain information is provided does not need to be indicated in the specifications. In our view, this is in line with the principle which drove the specification of “AI/ML for NG-RAN” since Rel-18, for which the following was specified in TS 38.300 clause 16.20.2 [5]:

### 16.20.2 Principles **** Skip non-relevant text **** Signalling procedures used for the exchange of information to support AI/ML for NG-RAN, are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

and, therefore, it is suggested that the same principle is also considered during the discussions in RAN3 about the impacts for supporting Direct AI/ML positioning Case 3b. Based on the above, we have the following proposal:

Proposal 3: For the discussion of RAN3 impacts of Direct AI/ML positioning Case 3b, RAN3 to focus on the “measurement” part of the training data collection, depending on further RAN1 progress.

- The “label” part of the training data collection has no RAN3 impact;

- Purpose, such as above “training data collection”, will not necessarily be specified in RAN3 specification.

Still based on RAN1 agreements, the “measurement” part of the training data sample includes a timestamp of the channel measurement. For this piece of information RAN1 also agreed in the last meeting that the existing IE “Time Stamp” in TS 38.455 (NRPPa protocol) can be reused from RAN1 perspective, hence there is no additional RAN3 impacts on how to transfer the timestamp information associated to the channel measurement to the LMF.

Proposal 4: There is no RAN3 impact in providing the timestamp of the channel measurement included in the “measurement” part of the training data sample to the LMF.

Within both the “measurement” and the “label” parts of a training data set an associated quality indicator is also included. Before discussing this aspect in RAN3, i.e., how to convey this information from NG-RAN to LMF, it would be better to wait for further RAN1 progress on its structure.

Proposal 5: RAN3 to wait for further RAN1 progress on the structure of the quality indicator associated to the “measurement” part of the training data sample, so that RAN3 can decide how to convey this information to the LMF.

### Model inference considerations

To enable the AI/ML model in the LMF to directly infer the UE location some input data needs to be provided to the AI/ML model itself. In previous RAN1 meetings the following agreements were achieved concerning the AI/ML model input information:

******** RAN1#116 ******** Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. Note: The use of SRS for MIMO resource is transparent to UE. Agreement For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation. Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc. ******** RAN1#116bis ******** Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT. ******** RAN1#117 ******** Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed). ******** RAN1#118 ******** Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples, - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples. - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

RAN1 agreed that channel measurements supported for reporting can be either timing information or paired timing information and power information.

In our understanding the power information associated to channel measurements that needs to be provided from NG-RAN to the LMF for determining model input for Case 3b is based on the existing UL SRS (Uplink Sounding Reference Signal). Therefore, we think there is no standard impact in reporting such information to from NG-RAN to LMF.

Proposal 6: RAN3 to confirm that for Case 3b the power information associated to channel measurements to be provided from NG-RAN to LMF which can be used as model input are based on the existing UL SRS. There is no standard impact for reporting the power information of UL SRS-RSRPP from NG-RAN to LMF.

- The purpose of the report such as “model input” will not necessarily be specified in RAN3 specification.

Moreover, for the timing information, RAN1 agreed that the timing information is represented relative to a reference time which is the existing UL RTOA (Uplink Relative Time of Arrival) reference time T0+tSRS as defined in TS 38.215. Hence in our view there is no RAN3 impact in reporting timing information of channel measurements from NG-RAN to LMF.

Proposal 7: RAN3 to confirm that for Case 3b there is no standard impact for reporting the timing information associated to channel measurements from NG-RAN to LMF.

Regarding the time domain channel measurements, RAN1 is also investigating two alternatives:

- Sample-based measurements, where the timing information is an integer multiple of sampling periods.

- Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods, and for which RAN1 also agreed that it refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

For the sample-based measurements RAN1 agreed some aspects such as its definition, the number of samples of the estimated channel response in time domain and the time granularity for the reporting. It is still under discussion whether sample-based measurements will be supported in Rel-19. It is then suggested waiting for further RAN1 progress before starting discussions in RAN3 on how to enable signalling from LMF to gNB of sample-based measurements configuration parameters (Nt, Nt', k as agreed by RAN1).

Proposal 8: RAN3 to wait RAN1 confirmation on whether sample-based channel measurements will be supported in Rel-19 before starting discussions in RAN3 on how to enable signalling from LMF to gNB of configuration parameters (Nt, Nt', k as agreed by RAN1).

For the path-based measurements, instead, they follow the existing specifications as per RAN1 agreement, and the only potential enhancements could be on the number of reported paths. Also in this case, it is suggested waiting for further RAN1 progress before RAN3 makes changes in NRPPa to support an increased number of paths to be reported to the LMF.

Proposal 9: RAN3 to wait further RAN1 progress on whether enhancements on the number of reported paths in path-based channel measurements are needed before RAN3 makes changes in NRPPa to support an increased number of paths to be reported to the LMF.

Other than timing and power information, RAN1 is also investigating on whether also phase information should be included in the channel measurements reported from NG-RAN to LMF. Again, the actual RAN3 impact could be evaluated only after RAN1 made further progress on this aspect.

Proposal 10: RAN3 to wait further RAN1 progress on whether phase information should be included in the channel measurements (along with timing and power information).

### Model monitoring considerations

Another important aspect for a proper AI/ML model’s handling is the model monitoring. In previous RAN1 meetings the following agreement was reached:

******** RAN1#116 ******** Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

Based on the above, our understanding is that RAN1 is still studying whether and what information needs to be provided from the NG-RAN to the LMF for model monitoring in Case 3b. Therefore, it is suggested waiting for further RAN1 progress before starting the discussion in RAN3.

Proposal 11: RAN3 to wait further RAN1 progress on whether and what information needs to be provided from the NG-RAN to the LMF for model monitoring in Case 3b.

# Conclusions

General aspects of Direct AI/ML positioning

Observation 1: For Direct AI/ML positioning use cases the output of the AI/ML model is an estimate (prediction) of the UE location.

Observation 2: Direct AI/ML positioning use cases are distinguished based on the location of the AI/ML model used to directly infer the UE location.

Observation 3: Among the 1st priority cases for Direct AI/ML positioning, Case 1 does not have any RAN3 impact.

Proposal 1: RAN3 to discuss for Direct AI/ML positioning only the impacts of Case 3b.

Model training considerations

Observation 4: For Direct AI/ML positioning Case 3b the AI/ML model training is located in the LMF.

Proposal 3: For the discussion of RAN3 impacts of Direct AI/ML positioning Case 3b, RAN3 to focus on the “measurement” part of the training data collection, depending on further RAN1 progress.

- The “label” part of the training data collection has no RAN3 impact;

- Purpose, such as above “training data collection”, will not necessarily be specified in RAN3 specification.

Proposal 4: There is no RAN3 impact in providing the timestamp of the channel measurement included in the “measurement” part of the training data sample to the LMF.

Proposal 5: RAN3 to wait for further RAN1 progress on the structure of the quality indicator associated to the “measurement” part of the training data sample, so that RAN3 can decide how to convey this information to the LMF.

Model inference considerations

Proposal 6: RAN3 to confirm that for Case 3b the power information associated to channel measurements to be provided from NG-RAN to LMF which can be used as model input are based on the existing UL SRS. There is no standard impact for reporting the power information of UL SRS-RSRPP from NG-RAN to LMF.

- The purpose of the report such as “model input” will not necessarily be specified in RAN3 specification.

Proposal 7: RAN3 to confirm that for Case 3b there is no standard impact for reporting the timing information associated to channel measurements from NG-RAN to LMF.

Proposal 8: RAN3 to wait RAN1 confirmation on whether sample-based channel measurements will be supported in Rel-19 before starting discussions in RAN3 on how to enable signalling from LMF to gNB of configuration parameters (Nt, Nt', k as agreed by RAN1).

Proposal 9: RAN3 to wait further RAN1 progress on whether enhancements on the number of reported paths in path-based channel measurements are needed before RAN3 makes changes in NRPPa to support an increased number of paths to be reported to the LMF.

Proposal 10: RAN3 to wait further RAN1 progress on whether phase information should be included in the channel measurements (along with timing and power information).

Model monitoring considerations

Proposal 11: RAN3 to wait further RAN1 progress on whether and what information needs to be provided from the NG-RAN to the LMF for model monitoring in Case 3b.

# References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#105

- R3-245424, Discussion on RAN3 impacts for AI/ML assisted positioning, Huawei

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (v18.0.0), Rel-18

- TS 38.300, NR and NG-RAN Overall Description; Stage 2, Rel-18