3GPP TSG-RAN3 Meeting #125bis	R3-245431

Heifei, China, Oct 14-18, 2024

Agenda Item:	11.3

Source: 	Qualcomm Incorporated

Title: 	Discussion on AI/ML enabled CCO

Document for:	Discussion/Decision

# Background

In RAN3#125 we completed the study item phase for AI/ML based CCO and the below potential standard impacts were agreed in TR 38.743.

TR 38.743 #### 4.2.2.5 Potential standard impacts Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18 based on the CCO framework described in TS 38.300 [2] and TS 38.401 [3]. Xn interface: Enhance existing procedure to collect information between gNBs: - Future CCO State together with an associated coverage modification cause - Timing Information F1 interface: - Predicted CCO Issue (including predicted affected cells/beams) from gNB-CU to gNB-DU. - Future CCO State from gNB-DU to gNB-CU - Timing Information NOTE 4: Whether gNB-CU generates the suggested future CCO state and sends it to gNB-DU as assistance information is left to normative discussion. NOTE 5: For split architecture, future CCO State is derived based on the predicted CCO issue signaled from gNB-CU and other information in the gNB-DU.

In this paper we discuss our views on the how to capture the standard impacts normatively.

# Discussion

## Predicted CCO Issue and Future CCO State

It is agreed in the SI phase that gNB-CU generates the predicted CCO issue and forwards the information to gNB-DU. gNB-CU-CP uses information like L3 measurements, SON reports, Neighbour CCO configuration change as input to derive the predicted CCO issue.

Currently the AI/ML based predictions are introduced in the new Data Collection Reporting Initiation procedure to be exchanged over Xn. Similarly to maintain the modularity of AI/ML functionality, we propose to introduce Data Collection Reporting Initiation procedure over F1 to request and report Predicted CCO issue from gNB-CU-CP to gNB-DU

Introduce Data Collection Reporting Initiation procedure over F1 similar to Xn, to maintain modularity and similarity of AI/ML functionality across Xn and F1 interfaces and to request and report Predicted CCO issue and future CCO state between gNB-CU-CP and gNB-DU.

The Predicted CCO issue and future CCO state can be added as bits in the bitmap of Report Characteristics for Data Collection.

To add Predicted CCO issue and future CCO state as bits in the bitmap of Report Characteristics for Data Collection IE in the F1AP Data Collection Request message and future CCO state in the bitmap of Report Characteristics for Data Collection IE in the XnAP Data Collection Request message.

The Predicted CCO issue and future CCO state along with time information can be provided in the Data Collection Update message over F1. Future CCO state with timing information and Coverage Modification cause can be exchanged over Xn in Data Collection Update message.

Predicted CCO issue and future CCO state along with time information can be exchanged between gNB-CU-CP and gNB-DU in F1AP Data Collection Update message. Future CCO state with timing information and coverage modification cause can be exchanged between neighbouring gNBs in XnAP Data Collection Udpate message.

The associated time information for predicted CCO issue and future CCO state can be a relative time of certain duration in seconds calculated from the time of receiving the Data Collection Update message.

The associated time information for predicted CCO issue and future CCO state can be a relative time of certain duration (e.g 0 to 3600) in seconds calculated at the receiving node from the time of receiving the Data Collection Update message.

## Future CCO State Assistance Information

A Note was added in the TR 38.743 to further discuss if gNB-CU-CP can generate the future CCO state as assistance information and send it to gNB-DU. On this Note, we are unclear on what are the input parameters which gNB-CU-CP will use to generate future CCO state. Moreover OAM provides the CCO state index configuration to gNB-DU. If gNB-CU-CP needs to generate the future CCO state, OAM also needs to configure gNB-CU-CP with CCO state index values which would involve changes to the configuration model and SA5 impact. This is also not a legacy behaviour. Hence we propose, that gNB-DU generates the future CCO state on its own and future CCO state need not be provided as assistance information from gNB-CU-CP.

The input parameters used by gNB-CU-CP to generate future CCO state is unclear. OAM provides the CCO state index configuration to gNB-DU. If gNB-CU-CP needs to generate the future CCO state, OAM also needs to configure gNB-CU-CP with CCO state index values which would involve changes to the configuration model and SA5 impact.

Future CCO state proposed to be generated by gNB-CU-CP as assistance information to gNB-DU is not needed as it involves changes to OAM to configure CCO index value to gNB-CU-CP similar to that of gNB-DU and duplicating functionality across gNB-CU-CP and gNB-DU.

## Assistance Information for Prediction of CCO Issue

It was agreed the gNB-CU will predict the CCO issue and forward it to the gNB-DU. gNB-CU predicts the CCO issue based on L3 measurements. gNB-DU has L1 measurements and beam level load information. Hence gNB-DU has a better visibility of the beam level information for detecting CCO issue. We therefore propose that gNB-DU can send affected beams based on L1 measurements as assistance information to gNB-CU.

gNB-CU uses L3 measurement information as one of the inputs to predict CCO issue. However gNB-CU lacks insight into the beam level information which is available in the gNB-DU. gNB-DU has better visibility of beam level information based on L1 measurements for detecting CCO issue.

For CCO issue prediction, gNB-DU can send assistance information to gNB-CU on affected beams based on L1 measurements. RAN3 to discuss including “Assistance Information on Affected beams” over F1 as input data for CCO issue prediction.

## Predicted CCO Configuration Feedback

During the SI phase Predicted CCO issue and future CCO state feedback was not discussed due to time constraints. We therefore propose to discuss the feedback for Predicted CCO issue and future CCO state in normative phase.  Once the predicted coverage pattern is applied, the neighboring gNBs can send feedback on Cell performance like throughput, packet loss, packet delay, RLF reports etc to the source gNB to inform the effects of applying predicted coverage pattern. The Feedback can be transferred from gNB-CU to gNB-DU to correct the CCO configuration.

RAN3 to discuss the following parameters as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn.

- Cell throughput (DL/UL)

- Cell level Packet loss

- Cell level Packet delay

# Summary

Based on the discussion above, we have following observations:

Observation 1.	The input parameters used by gNB-CU-CP to generate future CCO state is unclear. OAM provides the CCO state index configuration to gNB-DU. If gNB-CU-CP needs to generate the future CCO state, OAM also needs to configure gNB-CU-CP with CCO state index values which would involve changes to the configuration model and SA5 impact.

Observation 2.	gNB-CU uses L3 measurement information as one of the inputs to predict CCO issue. However gNB-CU lacks insight into the beam level information which is available in the gNB-DU. gNB-DU has better visibility of beam level information based on L1 measuremens for detecting CCO issue.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Introduce Data Collection Reporting Initiation procedure over F1 similar to Xn, to maintain modularity and similarity of AI/ML functionality across Xn and F1 interfaces and to request and report Predicted CCO issue and future CCO state between gNB-CU-CP and gNB-DU.

Proposal 2:	To add Predicted CCO issue and future CCO state as bits in the bitmap of Report Characteristics for Data Collection IE in the F1AP Data Collection Request message and future CCO state in the bitmap of Report Characteristics for Data Collection IE in the XnAP Data Collection Request message.

Proposal 3:	Predicted CCO issue and future CCO state along with time information can be exchanged between gNB-CU-CP and gNB-DU in F1AP Data Collection Update message. Future CCO state with timing information and coverage modification cause can be exchanged between neighbouring gNBs in XnAP Data Collection Udpate message.

Proposal 4:	The associated time information for predicted CCO issue and future CCO state can be a relative time of certain duration (e.g 0 to 3600) in seconds calculated at the receiving node from the time of receiving the Data Collection Update message.

Proposal 5:	Future CCO state proposed to be generated by gNB-CU-CP as assistance information to gNB-DU is not needed as it involves changes to OAM to configure CCO index value to gNB-CU-CP similar to that of gNB-DU and duplicating functionality across gNB-CU-CP and gNB-DU.

Proposal 6:	For CCO issue prediction, gNB-DU can send assistance information to gNB-CU on affected beams based on L1 measurements. RAN3 to discuss including “Assistance Information on Affected beams” over F1 as input data for CCO issue prediction.

Proposal 7:	RAN3 to discuss the following parameters as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn.

	Cell throughput (DL/UL)

	Cell level Packet loss

	Cell level Packet delay