3GPP TSG-RAN WG3 #125-bis	R3-245498

# Hefei, P. R. China, 14th-18th October 2024

Agenda Item:	17.3

Source:	Ericsson

Title: 	Xn impact of On-demand SIB1 for UEs in idle/inactive mode

Document for:	Discussion and Approval

# 1	Background

A work item on “Enhancements of network energy savings for NR” was agreed in [1]. The Work Item Description (WID) was revised in [2]. According to the revised WID, the objectives of the work item are the following:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we discuss issues and solutions regarding Objective 2 of the work item (see highlighted text above), focusing on the impact on Xn.

A companion contribution discussing intra NG-RAN node signalling and the impact on F1 is provided in [4].

# 2	Discussion

In the following, we discuss the main principles of the inter NG-RAN node signaling to enable on-demand SIB1 (OD-SIB1) transmission, considering non-split architecture.

## 2.1	General scenario

Figure 1 illustrates an example deployment scenario for OD-SIB1 transmission, where the “NES Cell” of gNB1 is meant to operate in OD-SIB1 transmission mode and the “Cell A” of gNB2 provides the UL Wake-Up Signal (WUS) configuration (that is needed to request SIB1 transmission in the NES Cell) to UEs. The scenario corresponds to Case 2 as defined by RAN1, assumed for normative work according to the WID.

Figure 1: An example deployment scenario for OD-SIB1 transmission (corresponding to Case 2).

We assume that, before the NES Cell operates in OD-SIB1 transmission mode, gNB1 provides the UL WUS configuration of its NES Cell to gNB2. In step 2, gNB2 provides the UL WUS configuration of the NES Cell to UEs camping on (or connected to) Cell A. In step 3, when needed, a UE (in RRC Idle/Inactive mode) sends the UL WUS to the NES Cell and, in response, receives SIB1 from the NES Cell in step 4.

Agreement from the previous meeting, refer to [3]

At the RAN3#125 meeting, the following was agreed:

Case 2 can be supported with the below details:

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

RAN3 has agreed that: in response to receiving the UL WUS configuration (of the NES Cell) from the NES-Cell gNB (here also referred to as gNB1), a Cell-A gNB (here also referred to as gNB2) shall indicate to the NES-Cell gNB whether it agrees (i.e., can and will ) or not to provide the UL WUS configuration to UEs via Cell A. It is understood that a feedback mechanism is crucial for the NES-Cell gNB to ensure that, when the NES Cell operates in OD-SIB1 transmission mode, the NES Cell is still accessible for idle/inactive UEs.

## 2.2	Issues when reusing the existing procedures

In our view, reusing the existing Xn SETUP REQUEST message or NG-RAN NODE CONFIGURATION UPDATE message for the NES-Cell gNB to provide the UL WUS configuration to the Cell A gNB are not good solutions for the following three reasons:

- First, the corresponding response messages are not suitable for providing the necessary feedback from Cell-A gNB to NES-Cell gNB. Our reasoning is the following. On one side, a request to provide an UL WUS configuration for the sake of OD-SIB1 operation may be part of a procedure to reduce energy consumption of certain RAN nodes but is not of principal operational system relevance (neither for the NES nor the Cell A gNB). On the contrary, any failing of an Xn Setup procedure or the NG-RAN Node Configuration Update procedure may cause critical system impact. Conveying an UL WUS configuration, and related request for provisioning, as part of an Xn Setup or NG-RAN node Configuration Update procedure would allow that the failing of a non-critical system feature (the OD-SIB1 support by the Cell A gNB) to cause a critical system procedure failure response (failure of the complete Xn Setup or NG-RAN Node Configuration Update). Practical consequences would be that Xn connection establishment is hindered, in case of Xn Setup failure, or it prevents other (non-OD-SIB1 transmission related) configuration changes to be applied, in case of NG-RAN node configuration update failure.

- Second, the NES-Cell gNB (gNB1) should be able to request (or to indicate to) the Cell-A gNB (gNB2) to provide the UL WUS configuration in one or multiple specific cells of gNB2, which is not supported when using Xn Setup or NG-RAN node Configuration Update for this purpose. It goes beyond the NES Cell’s gNB configuration information to indicate a cell of another node (the Cell A gNB) requesting a provisioning of an UL WUS configuration of the NES Cell gNB.

On the other hand, a potential Cell-A gNB cannot choose the assisting cell(s) on its own. We explain an example based on the Figure 2 scenario, where using OD-SIB1 transmission at gNB1 requires assistance from cells of two neighboring gNBs (here gNB2 and gNB3). Both, gNB2 and gNB3, operate two cells of different coverage. If either one, gNB2 or gNB3, would choose the cell with smaller coverage (in Figure 2 the Cell 2-2 or Cell 3-2) as Cell A, the UL WUS configuration is only partially provided in the coverage area of the NES Cell. Also, neither gNB2 nor gNB3 can know how the cells of the respective other gNB overlap the NES Cell in terms of coverage and which of the cells of the respective other gNB was/were chosen to function as Cell A for the NES Cell gNB. Therefore, only the NES-Cell gNB itself can choose the assisting cells.

Figure 2: An example deployment scenario for OD-SIB1 transmission, where using OD-SIB1 transmission at gNB1 requires assistance from cells of two neighboring gNBs.

Observation 1: Only the NES-Cell gNB can reliably determine the set of assisting cells needed to activate OD-SIB1 transmission.

- Third, returning to the example in Figure 1, gNB2 (providing the Cell A) may at some point suspend the UL WUS configuration provisioning in its Cell A (e.g., if it wants to shut down Cell A, or experiences very high load, or an unexpected failure in Cell A). In that case, gNB2 must inform gNB1 about the stop of UL WUS configuration provisioning, so that gNB1 can switch back the NES Cell operation to regular periodic SIB1 transmission mode to prevent unfavorable system and service impact. However, there is no apparent way for providing such UL WUS configuration provisioning status indication reusing legacy procedures.

Observation 2: Feedback (from Cell A gNB to NES Cell gNB) regarding UL WUS configuration provisioning status changes is needed while a NES Cell is operating in OD-SIB1 transmission mode.

In summary, reusing the Xn Setup procedure or the NG-RAN node Configuration Update procedure for exchanging the UL WUS configuration between NG-RAN nodes has several disadvantages, including:

- no suitable means to get feedback from a neighboring NG-RAN node on the status of the UL WUS configuration provisioning (before and during OD-SIB1 transmission mode), and

- no suitable means to indicate to a neighboring NG-RAN node in which cell(s) of the neighboring NG-RAN node the UL WUS configuration should be provided.

Observation 3: Reusing Xn Setup or NG-RAN node Configuration Update procedures for exchanging the UL WUS configuration has several disadvantages, including: no feedback on the status of the UL WUS configuration provisioning and no means to indicate in which cell(s) of a potential Cell A gNB the UL WUS configuration should be provided.

As legacy procedures seem unfit to be adapted to support the required functionality, we propose to discuss the introduction of new procedure(s) to enable inter-node coordination for UL WUS configuration provision. For example, RAN3 could introduce a Class 1 procedure for a NG-RAN node requesting another NG-RAN node to start (or stop) providing an UL WUS configuration in certain cell(s), and a Class 2 procedure for the other NG-RAN node to provide UL WUS configuration provision status updates to the NG-RAN node, as described below.

Proposal 1: RAN3 to introduce (a) new procedure(s) to enable inter-node coordination for UL WUS configuration provision.

## 2.3	Further discussion on introducing new procedures

In the following, we introduce potential new procedures for exchanging an UL WUS configuration and coordinating the provisioning of the UL WUS configuration. Specifically, we propose a new Class 1 procedure for a gNB requesting another gNB to start (or stop) providing an UL WUS configuration in certain cell(s) (i.e., Cell A(s)), and a new Class 2 procedure for the other gNB to provide UL WUS configuration provision status updates to the gNB. We first present an example of how these procedures work and then discuss further why they are necessary.

Figure 3 shows how the procedures can be used by a NES Cell gNB (gNB1) for activating OD-SIB1 transmission and temporary fallback to periodic SIB1 transmission. In a step 1, gNB1 generates an UL WUS configuration. In step 2, gNB1 sends to gNB2 an UL WUS configuration provision request message comprising the UL WUS configuration as well as an indication of cell(s) of gNB2 in which gNB1requests the UL WUS configuration to be provided. Subsequently, gNB1 receives from gNB2 an UL WUS configuration provision response message with an accept/reject indication concerning the request. In case of an accept indication, gNB2 starts providing the UL WUS configuration in requested cell(s) in a step 3. In a step 4, gNB2 sends to gNB1 an UL WUS configuration provision status message indicating that it is now providing the UL WUS configuration in requested cell(s). In a step 5, gNB1 starts operating the NES Cell in OD-SIB1 transmission mode.

Figure 3: An example of how new procedures can be used for activating OD-SIB1 transmission and temporary fallback to periodic SIB1 transmission.

We believe, it is sufficient, and in fact most appropriate, that gNB2 responds with accept only if it can provide the UL WUS configuration in all cells to be Cell A (as per request by gNB1) and reject otherwise.

If at some point the UL WUS configuration provisioning at gNB2 is temporarily interrupted (potential reasons mentioned above), in a step 6, gNB2 sends to gNB1 an UL WUS configuration provision status message indicating that it is not providing the UL WUS configuration in the requested cell(s). After receiving a message as in step 6 (Cell A not providing the UL WUS configuration anymore), in a step 7, gNB1 may switches the NES Cell back to periodic SIB1 transmission mode, assuming that the loss of (any) Cell A(s) at gNB2 leads to a lack of Cell-A coverage for the NES Cell (meaning no Cell-A redundancy).

Figure 4 shows how the proposed procedures can be used to deactivate OD-SIB1 transmission and revert the NES Cell to periodic SIB1 transmission (until further notice). If gNB1 decides to stop using OD-SIB1 transmission in the NES Cell for a longer period, it can inform gNB2 that the provisioning of the UL WUS configuration is no longer needed. Thus, it can send to gNB2 an UL WUS configuration provision request message with a corresponding indication, to relieve gNB2 from the additional provisioning of the UL WUS configuration to UEs at times when not needed. Unnecessary provisioning of an UL WUS configuration means both an unnecessary occupying of radio resources as well as an continuously increased energy by Cell A.

Figure 4: An example of how new procedures can be used to deactivate OD-SIB1 transmission and revert to periodic SIB1 transmission (until further notice).

A Class 1 procedure, e.g., UL WUS Configuration Provision procedure (FFS on the name), is needed to enable a NES Cell gNB to request a Cell A gNB to start (or stop) providing the UL WUS configuration at times when it intends (or no longer intends) to use OD-SIB1 transmission in a NES Cell. For reasons discussed above, the NES Cell gNB must be able to indicate the cell(s) of a designated Cell A gNB that should assist. To highlight it again, only the NES Cell gNB can choose a set of cells as candidate Cell A to back up the entire coverage area of the NES Cell, at least in cases where several neighbor gNBs must assist. Some companies proposed that such procedure is only used to start/stop the UL WUS configuration provision in the said cell(s), and that the UL WUS configuration could be exchanged via legacy procedures in advance. We, however, believe that there is no advantage in such two-step approach, which adds complexity and potentially delay. The UL WUS configuration can simply be included in this new procedure.

We have discussed that feedback regarding the UL WUS configuration provision is needed before OD-SIB1 transmission is activated in a NES Cell, but also while it is active, e.g., in case the UL WUS configuration provision is interrupted. To support this, another procedure is needed, one that a Cell-A gNB can initiate. Hence, a Class 2 procedure, e.g., UL WUS Configuration Provision Change procedure (FFS on the name), is needed to allow a CellA gNB to inform a NES-Cell gNB about changes in the UL WUS configuration provision status in the Cell A(s). It must be possible for a Cell-A gNB to, e.g., stop the provisioning of an UL WUS configuration in a certain Cell A depending on Cell A operation conditions (e.g., traffic load), and such decision must be shared with the corresponding NES-Cell gNB to avoid negative system impact.

Observation 4: Feedback from a Cell A gNB to a NES Cell gNB regarding the UL WUS configuration provision is needed also while OD-SIB1 transmission is activated.

Besides, there is another reason why a procedure is needed for a Cell-A gNB to indicate to a NES-Cell gNB status changes concerning the UL WUS configuration provisioning. This has to do with how fast the provisioning at gNB2 can be ensured from the time of receiving the UL WUS configuration provision request message from gNB1. Any SI update in Cell A requires at least one modification period.

As specified in TS 38.331 (see IE DownlinkConfigCommonSIB), the modification period, expressed in number of radio frames, is the following: m = modificationPeriodCoeff * defaultPagingCycle. The modificationPeriodCoeff is between 2, and 16 and the defaultPagingCycle is between 32 and 256 radio frames. Hence, the modification period m can be up to 4096 radio frames (≈ 41 seconds), and this value should not be optimized just for the sake of OD-SIB1 transmission. For defaultPagingCycle, for example, large values are common, since they improve UE energy efficiency.

The Class 1 procedure to enable a NES-Cell gNB requesting a Cell-A gNB to start providing the UL WUS configuration should not be kept open for long, certainly not as long as 40 seconds or more (or until the SI update has reached all UEs and the UL WUS configuration provisioning is ensured). The response message in this procedure from gNB2 to gNB1 should be sent promptly. The status change/update message can be sent (i.e., above Class 2 procedure can be initiated) once the provisioning in Cell A is ensured.

Observation 5: The response to a request for an UL WUS configuration provisioning cannot be delayed until the UL WUS configuration provisioning is ensured; it must be signaled promptly.

Observation 6: Inter-node signalling for OD-SIB1 transmission must include a procedure, that can be initiated by a Cell-A gNB, to indicate, to a NES-Cell gNB, status changes regarding the UL WUS configuration provision.

Proposal 2: RAN3 to agree the introduction of new procedures to enable inter-node coordination for UL WUS configuration provision, including support for sending, to another node, a request to start (or stop) UL WUS configuration provision in certain cell(s) and receiving, from the other node, UL WUS configuration provision status updates.

## 2.4	Discussion on indication of missing UL WUS configuration

Figure 5 visualizes an example scenario, where a part of the coverage area of the NES Cell is not covered by Cell A. However, it is covered by another cell, here called Cell X. In this particular case, the UE finds the NES Cell but cannot find (any) Cell A. Hence, it cannot camp on the NES Cell. Such incomplete Cell-A coverage effectively reduces the coverage area of a NES Cell, causing suboptimal performance and a waste of energy for both network and UE, since the UE cannot connect to the nearest cell when needed.

Observation 7: A lack of Cell-A coverage in the coverage area of the NES Cell causes suboptimal performance and a waste of energy for both network and UE.

Figure 5: An example of how a missing UL WUS configuration indication can reach the NES-Cell gNB.

It is thus very important that 3GPP provides means to detect and inform about missing provisioning of UL WUS configuration, so that such problem can be resolved by the network. For this purpose, the UE needs to report a missing UL WUS configuration to the network, either via Cell X, or later via the NES Cell or Cell A, in case it moves into the coverage area of Cell A. Note that there are other types of problems that may arise with OD-SIB1 transmission. For example, the case where the UE has an UL WUS configuration and sends an UL WUS but does not receive OD-SIB1.

To support detection of issues with UL WUS configuration provisioning, it is essential that a UE can detect that a cell is a NES Cell (i.e., is using OD-SIB1 transmission) from the SSB/MIB of the cell itself. This is in RAN1 scope. In addition, RAN2 must specify means for a UE to inform the network about a missing UL WUS configuration.

However, a complete solution also requires some RAN3 work. Specifically, a UE, not able to camp on the NES Cell and out of coverage of a Cell A,  can only report the observation of a missing UL WUS configuration to the network via Cell X (gNB3 in Figure 5), even though only gNB1 can improve the situation, e.g., by making the gNB3 providing the UL WUS configuration in Cell X.

As depicted in Figure 5, the Cell-X gNB can send the indication of a missing UL WUS configuration to the NES-Cell gNB, so that the NES-Cell gNB can take necessary action to improve the situation, e.g., by selecting more cells to act as Cell A, or turning off OD-SIB1 transmission. Such inter-node signalling is in RAN3 scope and should be already addressed in Rel-19. If not, the OD-SIB1 feature could in practice do more harm than good and would be difficult to deploy properly.

Furthermore, through such inter-node signalling, automatic Cell-A relations may be enabled. For example, an operator may deliberately deploy NES cells without manually configuring the NES-Cell/Cell-A relations, and through the missing UL WUS configuration reporting, the relations are automatically set up by the different NG-RAN nodes. This can simplify NES-Cell deployment for the operator.

Observation 8: Inter-node signaling concerning missing UL WUS configuration is needed for a NES-Cell gNB to detect and remedy lack of Cell A coverage. This is in RAN3 scope and should be addressed in Rel-19.

Observation 9: Inter-node signaling concerning missing UL WUS configuration can also be used for automatic NES-Cell/Cell-A relation establishment and can simplify NES-Cell deployment for the operator.

Proposal 3: RAN3 to discuss inter-node signaling for indicating to a NES-Cell gNB that a missing UL WUS configuration event has occurred.

# 3	Proposals

In this contribution, we discussed issues and high-level solutions for Objective 2 of the work item mentioned above and made the following observations and proposals:

Observation 1: Only the NES-Cell gNB can reliably determine the set of assisting cells needed to activate OD-SIB1 transmission.

Observation 2: Feedback (from Cell A gNB to NES Cell gNB) regarding UL WUS configuration provisioning status changes is needed while a NES Cell is operating in OD-SIB1 transmission mode.

Observation 3: Reusing Xn Setup or NG-RAN node Configuration Update procedures for exchanging the UL WUS configuration has several disadvantages, including: no feedback on the status of the UL WUS configuration provisioning and no means to indicate in which cell(s) of a potential Cell A gNB the UL WUS configuration should be provided.

Proposal 1: RAN3 to introduce (a) new procedure(s) to enable inter-node coordination for UL WUS configuration provision.

Observation 4: Feedback from a Cell A gNB to a NES Cell gNB regarding the UL WUS configuration provision is needed also while OD-SIB1 transmission is activated.

Observation 5: The response to a request for an UL WUS configuration provisioning cannot be delayed until the UL WUS configuration provisioning is ensured; it must be signaled promptly.

Observation 6: Inter-node signalling for OD-SIB1 transmission must include a procedure, that can be initiated by a Cell-A gNB, to indicate, to a NES-Cell gNB, status changes regarding the UL WUS configuration provision.

Proposal 2: RAN3 to agree the introduction of new procedures to enable inter-node coordination for UL WUS configuration provision, including support for sending, to another node, a request to start (or stop) UL WUS configuration provision in certain cell(s) and receiving, from the other node, UL WUS configuration provision status updates.

The Text Proposal for the above is included in Chapter 4.

Observation 7: A lack of Cell-A coverage in the coverage area of the NES Cell causes suboptimal performance and a waste of energy for both network and UE.

Observation 8: Inter-node signaling concerning missing UL WUS configuration is needed for a NES-Cell gNB to detect and remedy lack of Cell A coverage. This is in RAN3 scope and should be addressed in Rel-19.

Observation 9: Inter-node signaling concerning missing UL WUS configuration can also be used for automatic NES-Cell/Cell-A relation establishment and can simplify NES-Cell deployment for the operator.

Proposal 3: RAN3 to discuss inter-node signaling for indicating to a NES-Cell gNB that a missing UL WUS configuration event has occurred.

# 4	Text Proposal for BL CR for TS 38.423

*************** Start of TP ***************

### 8.x.x	UL WUS Configuration Provision

#### 8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node to request a neighbouring NG-RAN node to provide an UL WUS configuration in one or more cells.

The procedure uses non UE-associated signalling.

#### 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

The NG-RAN node1 initiates the procedure by sending the UL WUS CONFIGURATION PROVISION REQUEST message to the peer NG-RAN node2 to start a provisioning, stop a provisioning, of an UL WUS configuration in one or more cells of the NG-RAN node2. Upon receipt, the NG-RAN node2:

-	shall initiate the requested provisioning according to the parameters given in the request in case the Request Type IE set to "start"; or

-	shall terminate the indicated (previously admitted) provisioning in case the Request Type IE is set to "stop".

The NG-RAN node2 shall, if supported, provide the UL WUS configuration included in the UL WUS CONFIGURATION PROVISION REQUEST message in the cells indicated in the UL WUS CONFIGURATION PROVISION REQUEST message and shall respond with the UL WUS CONFIGURATION PROVISION RESPONSE message.

#### 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision, unsuccessful operation

If the NG-RAN node2 cannot provide the UL WUS configuration included in the UL WUS CONFIGURATION PROVISION REQUEST message in the cells indicated in the UL WUS CONFIGURATION PROVISION REQUEST message, it shall respond with the UL WUS CONFIGURATION PROVISION FAILURE message with an appropriate cause value.

#### x.x.x.4	Abnormal Conditions

Void.

### 8.x.y	UL WUS Configuration Provision Change

#### 8.x.y.1	General

This procedure is initiated by an NG-RAN node to inform a neighbouring NG-RAN node about status changes concerning an admitted UL WUS configuration provision.

It is also used to indicate that the UL WUS configuration provisioning is ongoing or interrupted.

The procedure uses non UE-associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Provision Change, successful operation

The NG-RAN node2 shall indicate the status of the admitted UL WUS configuration provision in the UL WUS CONFIGURATION PROVISION STATUS message.

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Void.

#### 9.x.x.x	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request provisioning of an UL WUS configuration in one or more cells.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Request Type | M |  | ENUMERATED(start, stop, …) | Type of request concerning the provisioning of the UL WUS Configuration. | YES | reject |
| UL WUS Configuration | M |  | OCTET STRING | Includes the xyz message as defined in subclause a.b.c of TS 38.331. | YES | reject |
| Assisting Cells List |  | 1 |  | List of cells of the NG-RAN node2 requested to provide the UL WUS configuration. | YES | reject |
| >Assisting Cells Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 | Indicates the NR_CGI at the NG-RAN node2 | – |  |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

#### 9.x.x.y	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the UL WUS configuration will be provided in the requested cells.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.x.x.z	UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the UL WUS configuration cannot be provided in the requested cells.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | ignore |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | ignore |

#### 9.x.x.t	UL WUS CONFIGURATION PROVISION STATUS

This message is sent by NG-RAN node2 to NG-RAN node1 to report a status change concerning an admitted UL WUS configuration provision.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Provision Status |  |  | ENUMERATED (providing, not providing, …) | Indicates the provision status of the UL WUS configuration in the assisting cells. |  |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

*************** End of TP ***************

# 5	References

[1]	RP-234065	New WID: Enhancements of network energy savings for NR

[2]	RP-242354	Revised WID: Enhancements of network energy savings for NR

[3] 	R3-244708	Summary of offline disc, Ericsson (Moderator)

[4]	R3-245517	F1 impact of On-demand SIB1 for UEs in idle/inactive mode, Ericsson