3GPP TSG-RAN WG3 Meeting #125-bis	R3-245501

Hefei, P.R.C, 14th Oct – 18th Oct, 2024

Title: 	AI/ML support for CCO

Source: 	Ericsson, Deutsche Telekom, InterDigital

Agenda item:	11.3

Document Type:	Discussion and agreement

# 1. Introduction

At RAN3#125 the SI on AI/ML in RAN in Rel-19 was completed, and one of the conclusions captured in the TR 38.743 is to proceed with normative work for AI/ML based Coverage and Capacity Optimization (CCO) use case. In this contribution, we discuss aspects related to the enhancements needed in NG-RAN interfaces (for split and non-split architecture) to support this use case.

# 2. Discussion

## Background for AI/ML based CCO solution

During the study phase, it was clarified that AI/ML based CCO uses a proactive approach to prevent or limit at an early stage the rise of a CCO issue. To maintain the same separation of concerns as in the legacy CCO, where the gNB-CU detects a CCO issue and the gNB-DU attempts to resolve or mitigate it, it has been agreed that in case of split-architecture deployment, the AI/ML model inference is at gNB-CU, and the main inference output is a predicted CCO issue. The standard impact for NG-RAN interfaces as captured in the TR 38.473 is reported below.

For Xn interface:

-	Future CCO State together with an associated coverage modification cause

-	Timing Information

For F1 interface:

-	Predicted CCO Issue (including predicted affected cells/beams) from gNB-CU to gNB-DU.

-	Future CCO State from gNB-DU to gNB-CU

-	Timing Information

In relation to potential impact over Xn, it is also worth to mention that at last meeting, RAN3 agreed the following: Based on the received future CCO state (including an associated coverage modification cause) from CU1, CU2 will deduce for DU2 the predicted CCO issue and affected cells/beams, and send it to DU2.

## Discussion on future CCO state

When the source gNB-DU receives a predicted CCO issue, it attempts to resolve it and generates a future CCO state, i.e., it generates a combination of Cell Coverage State(s) and/or SSB Coverage State(s) to apply at some point in the future. The timing information indicating when the future CCO state will be applied is an important piece of the overall solution. In particular, we note that the timing information for predicted CCO issue and the timing information for future CCO state serve different purposes, and we think it is appropriate to make them different.

In a split gNB, the source gNB-DU is responsible for deriving the future CCO state and executing changes reflecting it, i.e., it is the source gNB-DU that decides when to apply it. Therefore, in a split gNB, the timing information for future CCO state is decided by the gNB-DU. In non-split gNB the timing information for future CCO state is decided by the gNB. The only way to make the solution consistent for a split gNB and a non-split gNB is that the timing information for future CCO state sent over Xn reflects the timing information for the future CCO state sent over F1 (from source gNB-DU to source gNB-CU).

Observation 1: To make AI/ML based CCO solution consistent, timing information for future CCO state signalled over Xn should reflect the timing information for the future CCO state signalled over F1 (from source gNB-DU to source gNB-CU).

In principle, the value of the timing information for the future CCO state (from source gNB-DU to source gNB-CU) could be the same as the timing information for predicted CCO issue (from source gNB-CU to source gNB-DU). However, this option shall not be mandated by the standard as there could be cases where these two timing information are different.

It should be noted that, if we consider the overview of the solution depicted in Figure1, if no timing information for future CCO state is sent from gNB1 from gNB2, then the CCO state signalled is the legacy one, i.e.  the non-AI/ML based CCO solution, where the gNB2 is just informed about a coverage modification applied at gNB1.

Observation 2: If no timing information for future CCO state is signalled over Xn, there is no difference between legacy CCO and AI/ML based CCO.

When a timing information for future CCO state is sent from source gNB-DU to source gNB-CU, it becomes also possible to handle situations where, for various reasons (changes in the network configurations, traffic distribution, etc.) a predicted CCO issue becomes outdated. In this case no coverage modification is needed, and if the source gNB-CU knows that the future CCO state has not been applied yet, it can inform the source gNB-DU that the coverage change can be avoided altogether. Similar possibility is applicable in the signalling between gNBs.

One reasonable constraint to establish about the predicted CCO issue and future CCO state timing is that the future CCO state time shall occur before  the CCO issue time.

The figure below shows three scenarios that can be supported when both the timing information for a predicted CCO issue and the timing information for a future CCO state are included in the signalling design.

Figure 2 – Scenarios for timing information related to Future CCO state

In scenario 1, at t0 the gNB-CU1 sends to the gNB-DU1 a predicted CCO issue and a reference prediction time (T). At t1 the gNB-DU1 applies the “future” CCO state to address the predicted CCO issue and it informs the gNB-CU1. In this case there is no timing information for the Future CCO state. The gNB-CU1 sends the “future” CCO state to the gNB-CU2 with no timing information (i.e., the gNB-CU2 knows that the new CCO state has already been applied).

In scenario 2, at t0 the gNB-CU1 sends to the gNB-DU1 a predicted CCO issue and a reference prediction time (T) as in scenario 1. At time t1 the gNB-DU1 indicates to the gNB-CU1 the future CCO state and that the future CCO state will take effect from t3 (let’s call it the “future CCO state time”). At t2 the gNB-CU1 sends to the gNB-CU2 the future CCO state together with the “future CCO state time”. Finally, at t3, the future CCO state is applied.

In scenario 3, up to the time t1 the first steps are the same as for scenario 2. At time t2 (before t3), the gNB-CU1 realizes that the predicted CCO issue is outdated (e.g., the traffic in the cells have changed considerably), and the coverage modification can be avoided. The gNB-CU1 informs the gNB-DU1, and no coverage modification is applied

Based on the presented analysis, we think it is appropriate to have two different timing information, one for predicted CCO issue and one for future CCO state. So, we propose to confirm the presence of both timing information in the signalling and that the two timing information can be different. We also propose that the gNB-CU can signal a “cancel” indication for the predicted CCO issue.

Proposal 1: Confirm that the gNB-CU can send timing information for predicted CCO issue to the gNB-DU.

Proposal 2: Confirm that the gNB-DU can send timing information for future CCO state to the gNB-CU.

Proposal 3: The values of the timing information for predicted CCO issue and the timing information for future CCO state can be different.

Proposal 4: The gNB-CU informs the gNB-DU if a predicted CCO issue is not valid anymore.

Let’s now consider the following notes captured in the TR 38.743:

NOTE 1:	Future CCO State can also be derived by legacy means. Signalling future CCO state will not be described as prediction over Xn.

NOTE 4:	Whether gNB-CU generates the suggested future CCO state and sends it to gNB-DU as assistance information is left to normative discussion.

We note that in legacy (non-AI/ML based) CCO, the OAM may configure alternative coverage configurations at the gNB-DU so that the gNB-DU can appropriately select a CCO state on the basis of detected CCO issue, affected cells and beams, CCO states of neighbour cells.  This is captured in TS38.300 as follows:

#### 15.5.5 Support for Coverage and Capacity Optimisation […] #### 15.5.5.2 OAM requirements Each NG-RAN node may be configured with alternative coverage configurations by OAM. The alternative coverage configurations contain relevant radio parameters and may also include a range for how each parameter is allowed to be adjusted.

Therefore, in legacy CCO, the set of information “CCO issue, affected cells and beams, alternative coverage configurations” is enough for the gNB-DU to generate the new CCO state. 
In AI/ML based CCO, the set of information available at the gNB-DU is equivalent to the information available for legacy CCO. Hence, the future CCO state for cells and beams served by the gNB-DU can be derived by legacy means as the type of information available at the gNB-DU is the same as in legacy CCO. Therefore we think there is no need for the gNB-CU to suggest a future CCO state as assistance information to the gNB-DU.

It is worth noting that, in case the gNB-CU signalled to the gNB-DU a “suggested future CCO state”, such suggestion could even contradict the alternative coverage configurations provided by the OAM to the gNB-DU, thereby creating a conflict that will have to be further resolved by means of new polisies and configurations (to assign priority to one or the other information).

Proposal 5: Future CCO state for cells and beams controlled by a given gNB-DU is not sent from gNB-CU to the gNB-DU as assistance information.

## Predicted CCO issue and future CCO state for neighbour gNB

The solution for AI/ML based CCO needs to consider the impacts on Xn signalling towards a neighbour gNB, and the impacts on F1 signalling between the target gNB-CU and the target gNB-DU. In this section we discuss such impacts looking at how the AI/ML based CCO solution could work for the more complex case where both source nd target gNBs are split gNBs. Solution for non-split gNBs can be easily derived. We will call the source gNB, gNB1 (including gNB-CU1 and gNB-DU1). This is the node where the CCO issue is predicted. The target gNB is called gNB2, which includes gNB-CU2 and gNB-DU2.

The gNB-CU1 indicates the predicted CCO issue and predicted issue timing to gNB-DU1 and obtains from gNB-DU1 the future CCO state together with a timing information for the future CCO state (of gNB1). According to the agreed impact in TR38.743, gNB-CU1 sends via Xn the future CCO State (of gNB1), an associated (future) coverage modification cause, and “timing information”. An excerpt from TR38.743 is reported below:

Xn interface: Enhance existing procedure to collect information between gNBs: - Future CCO State together with an associated coverage modification cause - Timing Information

But what is this timing information?

To answer this question we need to also recall the Impact on the F1 agreed in TR38.743, as follows:

F1 interface: - Predicted CCO Issue (including predicted affected cells/beams) from gNB-CU to gNB-DU. - Future CCO State from gNB-DU to gNB-CU - Timing Information

In the impact above, it is stated that gNB-CU2 shall send to gNB-DU2 a Predicted CCO Issue (including predicted affected cells and beams of gNB1) and timing information for the Predicted CCO Issue.

With respect to this aspect we would like to clarify that the predicted CCO issue gNB-CU2 signals to gNB-DU2 is not a “new” issue, but it is the same issue predicted by gNB-CU1 and signalled by gNB-CU-1 to gNB-CU2 in the (predicted) Coverage modification Cause over Xn.

Therefore, to enable gNB2 to take appropriate actions following a Future CCO State derived at gNB1 and to respect the agreements from the TR above, gNB2 needs to receive the following over Xn:

- The predicted Coverage Modification Cause, which reflects the Predicted CCO Issue at gNB1

- The timing at which the  CCO issue derived by gNB1is predicted to occur

- The CCO changes will be applied at gNB1 (future CCO state at gNB1)

- When such changes will be applied, namely the timing of the Future CCO State at gNB1.

Therefore, both the timing information related to predicted CCO issue and the timing information for future CCO state at gNB1 need to be sent via Xn.

Proposal 6: For AI/ML based CCO, a gNB1 sends to a neighbour gNB2:

- Future CCO state of gNB1

- Timing information for future CCO state of gNB1

- Predicted CCO issue at gNB1 (in the form of the Coverage Modification Cause)

- Timing information of predicted CCO issue in gNB1

Upon receiving such information over Xn, gNB-CU2 should signal to gNB-DU2 the following information over F1:

- Affected cell as and beams of gNB1 (namely affected by the future CCO state of gNB1)

- Timing information of the Future CCO State at gNB1 (as per agreements in TR 38.743)

- Predicted CCO Issue derived at gNB1 (as per agreements in TR 38.743)

- Predicted CCO Issue timing derived at gNB1 (as per agreements in TR 38.743).

Proposal 7: For AI/ML based CCO, upon receiving at gNB2 an indication for Future Coverage State and predicted Coverage Modification Cause over Xn from gNB1 gNB-CU2 signals to gNBDU2 the following information over F1:

- Predicted CCO issue at gNB1

- Predicted affected cells and beams of gNB1

- Timing information of predicted CCO issue in gNB1

- Timing information for future CCO state of gNB1

In light of the above, and to prevent further confusion, we also propose to amend the following agreement with the suggestions below:

Proposal 8: amend the agreement taken in RAN3-125 as follows

Predicted CCO Issue (including predicted affected cells/beams) from CU to DU, in the following two scenarios:

- CU1 predicts the CCO issue for DU1 and sends to DU1

- Based on the received future CCO state (including an associated coverage modification cause) from CU1, CU2 will deduce for DU2 the predicted CCO issue and affected cells/beams, and send it to DU2.

- CU2 will derive the predicted CCO issue and affected cells and beams to be signalled to DU2, from the future CCO state (including an associated coverage modification cause) received from CU1. Namely, the predicted CCO issue and the affected cells and beams CU2 signals TO DU2 are the same as the Coverage Modificaiton Cause and cells/beams affected by Future CCO state changes received from CU1.

## Overview of the solution for AI/ML based CCO

Considering the proposals above, we present an overview of the solution for AI/ML based CCO in the flow chart below.

Figure 1 – Overview of the solution for AI/ML based CCO

## Data collection for AI/ML based CCO

For AI/ML based CCO, data can be collected from UEs or neighbor gNBs, e.g., as input to infer a predicted CCO issue, or as feedback after a certain future CCO state is deployed.

For example, it has been agreed that UE measurement reports can be used as input data, including cell level and beam level UE measurements.

However, measurement reports of UEs served by neighbour nodes are not exchanged between gNBs today. They can help to reveal trends that can support the inference of a predicted CCO issue, e.g., a CCO issue due to coverage. 
For example, a coverage degradation is experienced by UEs served by a gNB2 and moving in the vicinity of the border between a cell2 of gNB2 and a cell1 of gNB1. The gNB1, although it is a neighbor of gNB2, is not able to help gNB2 before a real coverage issue is detected, as it is unaware of the situation, since it does not receive UE measurement reports for the UE served by cell2.

To fulfill the main purpose of AI/ML based CCO, i.e., to prevent CCO issues to occur, the gNB1 could ask the gNB2 to report information revealing coverage degradation at cell edge (for specific cells), and with that, predict a CCO issue and try to help gNB2. We need to consider that the gNB1 knows that certain alternative configurations could be used to address a certain CCO issue affecting certain cells or beams. gNB1 also knows that cell1 under its control is a neighbor of cell2 in gNB2, and it knows e.g., that if a coverage issue at cell border between cell1 and cell2 occurs, the coverage of cell1 will be stretched towards cell2. Hence, to prevent the CCO issue, the gNB1 can request the gNB2 to report information for the UEs experiencing coverage degradation in cell2 and that could be served by cell1 instead (e.g., in case the coverage of cell1 is stretched). Various type of information can be used, and a simple request from gNB1 to gNB2 can be to forward to gNB1 those UE measurement reports which indicate an RSRP of cell1 above a certain threshold (namely, reporting measurement reports of UEs in cell2 that detect cell1 with sufficient strength).

Perhaps even more important is the case where the same information can be used as feedback. For example, it would be very useful for gNB1 to receive measurement reports of UEs in cell2 (that detect cell1 with sufficient strength) before and after a CCO state change is applied by gNB1 for cell1. With that, gNB1 can understand if the CCO state change action has resolved the coverage issue or if the issue persists. Namely, AI/ML models for issue prediction at gNB1 can be improved based on this feedback information.

Based on the analysis above, we propose that a gNB can request to a neighbor gNB some UE measurement reports that can help the gNB to predict a CCO issue and that can be used as feedback information to determine whether a CCO issue was resolved or not by means of applied CCO state changes.

Proposal 9: To support CCO issue prediction and reporting of feedback information for predicted CCO issue detection, a gNB can request information related to UE measurement reports collected by a neighbour gNB.

# 3. Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Observation 1: To make AI/ML based CCO solution consistent, timing information for future CCO state signalled over Xn should reflect the timing information for the future CCO state signalled over F1 (from source gNB-DU to source gNB-CU).

Observation 2: If no timing information for future CCO state is signalled over Xn, there is no difference between legacy CCO and AI/ML based CCO.

Proposal 1: Confirm that the gNB-CU can send timing information for predicted CCO issue to the gNB-DU.

Proposal 2: Confirm that the gNB-DU can send timing information for future CCO state to the gNB-CU.

Proposal 3: The values of the timing information for predicted CCO issue and the timing information for future CCO state can be different.

Proposal 4: The gNB-CU informs the gNB-DU if a predicted CCO issue is not valid anymore.

Proposal 5: Future CCO state for cells and beams controlled by a given gNB-DU is not sent from gNB-CU to the gNB-DU as assistance information.

Proposal 6: For AI/ML based CCO, a gNB1 sends to a neighbour gNB2:

- Future CCO state of gNB1

- Timing information for future CCO state of gNB1

- Predicted CCO issue at gNB1 (in the form of the Coverage Modification Cause)

- Timing information of predicted CCO issue in gNB1

Proposal 7: For AI/ML based CCO, upon receiving at gNB2 an indication for Future Coverage State and predicted Coverage Modification Cause over Xn from gNB1 gNB-CU2 signals to gNBDU2 the following information over F1:

- Predicted CCO issue at gNB1

- Predicted affected cells and beams of gNB1

- Timing information of predicted CCO issue in gNB1

- Timing information for future CCO state of gNB1

Proposal8: amend the agreement taken in RAN3-125 as follows

Predicted CCO Issue (including predicted affected cells/beams) from CU to DU, in the following two scenarios:

- CU1 predicts the CCO issue for DU1 and sends to DU1

- Based on the received future CCO state (including an associated coverage modification cause) from CU1, CU2 will deduce for DU2 the predicted CCO issue and affected cells/beams, and send it to DU2.

- CU2 will derive the predicted CCO issue and affected cells and beams to be signalled to DU2, from the future CCO state (including an associated coverage modification cause) received from CU1. Namely, the predicted CCO issue and the affected cells and beams CU2 signals TO DU2 are the same as the Coverage Modificaiton Cause and cells/beams affected by Future CCO state changes received from CU1.

Proposal 9: To support CCO issue prediction and reporting of feedback information for predicted CCO issue detection, a gNB can request information related to UE measurement reports collected by a neighbour gNB.