3GPP TSG-RAN WG3 Meeting #125-bis	R3-245505

# Hefei, China, 14th – 18rd October, 2024

Title: 	Split architecture support for Release 18 use cases

Source: 	Ericsson, Jio, InterDigital

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In this contribution, we discuss the details of split architecture use-cases, primarily in the context of Load Balancing, Energy Saving, and Mobility Optimization use-case, based on the conclusions reached during the SI [1]. Release 19 normative discussions shall propose solutions for the above use-cases for split architecture based on the principles and agreements made for the non-split architecture cases.

# 2. Discussion

The discussion and proposals in this paper are organized in terms of the different use-cases to which split architecture leftovers are applicable, followed by the required signalling enhancements and normative proposals.

## 2.1 Measured UE performance

#### 2.1.2 Delay

The following was agreed as part of the meeting outcomes from RAN3 #125:

The details of signaling measured UE performance metrics from DU to CU-CP and/or from CU-UP To CU-CP needs further discussion during normative phase.

The above overrides the agreement “Transferring Measured UE performance from CU-UP to CU-CP.” from RAN3#124.

During discussions in Rel-19 study item phase, different proposals for collecting UE performance feedback metrics, e.g., only from the gNB-CU-UP, from both the gNB-CU-UP and the gNB-DU, and other variants were discussed. RAN3 agreed that, for the metrics to be collected in accordance with the general principles of AI/ML data collection, i.e., where the requesting node can configure a reporting periodicity, where the responding node may accept/reject a request, etc., a part of the metrics would have to be collected from the gNB-CU-UP and another from the gNB-DU.

In the context of downlink UE delay measurements, this corresponds to exchange of the following metrics:

- D1: Average delay in DL over air-interface (available at gNB-DU)

- D2: Average delay in DL in gNB-DU (available at gNB-DU)

- D3: Average DL delay over F1-U (available at gNB-CU-UP)

- D4: Average DL delay in CU-UP (available at gNB-CU-UP)

In the uplink direction, the delay components consist of the following:

- D1: UL PDCP packet average delay (available at gNB-CU-CP, no spec impact)

- D2.1: Average over-the-air interface UL packet delay (available at gNB-DU)

- D2.2: Average UL RLC packet delay (available at gNB-DU)

- D2.3: Average UL delay over F1-U (available at gNB-CU-UP)

- D2.4: Average UL PDCP re-ordering delay (available at gNB-CU-UP)

The metrics listed above are measured with a granularity of per DRB per UE. The gNB-CU-CP, which is the recipient of the above information from the UE, gNB-CU-UP, and the gNB-DU has sufficient knowledge to remap the per DRB per UE granularity to per S-NSSAIs per UE (see Proposal 1), and it may therefore be beneficial to signal these metrics at the finest available granularity.

Proposal 1: RAN3 to agree to the exchange of RAN delay measurements for UL and DL from the gNB-CU-UP and gNB-DU to the gNB-CU-CP with a per-DRB-level granularity.

In the context of collecting UE performance at the gNB-CU-CP, which is then sent to a neighbour gNB, the granular components of the delay measurements, e.g., delay over the air interface, delay in the gNB-DU, and others listed above are not individually relevant for the gNB-CU-CP or the UE performance measurements at the neighbour node. It is only the total end-to-end delay in the UL or DL that is of interest in this context. It is therefore efficient to signal a compound measurement that is generated by summing up the cumulative measurements from the gNB-CU-UP or the gNB-DU to the gNB-CU-CP.

While granular details on the components of the UL and DL may not be necessary to have at the gNB-CU-CP, it is vital that the different component measurements measured at the UE (UL PDCP packet average delay already available at the gNB-CU), gNB-DU and the gNB-CU-UP:

- are measured over the same time period, and

- are aligned with one another when a cumulative UL/DL performance measurement is generated at the gNB-CU-CP, i.e., the sample from the gNB-CU-UP and the gNB-DU are available when a cumulative measurement is calculated.

For the alignment of measurements, the gNB-CU-CP, which configures the reporting of the metrics from the UE, gNB-DU and the gNB-CU-UP, must ensure that the measurements are configured with the same reporting periodicity. Whilst to ensure that the measurements reported are measured in the same time period as the reporting, the specifications (RRC, F1AP, and E1AP) must specify that a reported measurement shall correspond exactly to the recorded values in the previous time period and no period before or after it. With these conditions, the gNB-CU-CP would be able to collate delay measurements coming from the UE, gNB-DU and from the gNB-CU-UP and received within the same time period, thereby forming a total DL value representing the delay experienced across the configured time period.

Proposal 2: The gNB-DU and the gNB-CU-UP shall signal a UL or DL delay metric to the gNB-CU-CP that is generated by summing up the different delay components at gNB-DU and gNB-CU-UP respectively. The gNB-CU ensures that the measurements are configured with the same measurement and reporting periodicity. The gNB-CU-CP adds together the delay measurements received (from the UE, gNB-CU-UP, and the gNB-DU) within the same reporting period and uses this sum as the UL/DL delay metric to report as UE performance information.

#### 2.1.3 Packet loss

In Rel18 it was agreed to include Packet Loss as a parameter for UE performance feedback. It is important to note that UE performance feedback is collected at a target node to then provide to the source RAN information about the service level performance with which a UE is served after a handover. This information may then be used to evaluate the performance of mobility actions derived from AI/ML-based inference, in terms of how well services are served at the target node. However, we observe and describe in detail in the subsequent paragraphs that the Packet Loss parameter introduced in Rel18 may not represent the information originally desired and that it is challenging to collect it in a way that is meaningful and such that it covers all cases.

TS 28.552 defines measurement of DL Drop rate in gNB-CU-UP as the fraction of DL user plane packets that are dropped in relation to the total number of packets that entered the queue in the corresponding DRB over a period of time. The above measurement may be performed either at the PDCP level or at the RLC level to derive the packet loss rate at different levels in the protocol stack. In the context of UE performance reporting for packet losses, a value derived at a higher protocol layer, i.e., the measurements on the PDCP level packet loss rate, is considered more relevant as it better reflects the performance at service-level. Indeed, the packet loss at e.g. MAC level does not necessarily carry information about the quality of the service because a dropped MAC PDU may end up being successfully retransmitted with no negative effects on the service. Besides, a dropped MAC PDU may be due to aggressive modulation and coding scheme, which is an implementation/operational choice and not a sign of good or bad performance.

It is also understood that, for a DRB delivered over RLC Unacknowledged mode, computing a PDCP packet loss metric as defined above is not possible. This is due to the fact that the PDCP entity cannot keep track of packets that were acknowledged in RLC UM but can only keep track of packets submitted to lower layers for transmission. Therefore, PDCP does not know whether a RLC UM packet has been successfully delivered or not. On the contrary, it is possible to compute packet ross rate under RLC unacknowledged mode at lower layers, i.e., at the RLC entity in the gNB-DU. However, as explained above, the service level performance is not necessarily reflected by packet loss at RLC layer.

In addition to the above, packet loss rate at RLC layer is not available for uplink traffic. To make this measurement available for UL traffic, a change at the UE would be needed, implying high impact on UEs and standard specifications and also implying that the measurement would not be available for legacy UEs.

In summary, UL Packet Loss Rate is NOT available.

DL packet loss rate at RLC indicates loss of packets at lower layers, which may not at all reflect in application packet losses, hence this metric is not useful to drive the performance with which the UE bearers in use are served.  Note that DL packet losses might be purposely accounted for in implementations that adopt aggressive modulation and coding, hence such packet losses are not necessarily an indication of low performance.

DL packet loss at PDCP cannot monitor for loses in RLC Unacknowledged Mode

The table below summarizes the drawbacks of Packet Loss rate collection at different protocol levels:

|  | Advantages | Disadvantages |
| --- | --- | --- |
| Packet Loss Rate Calculation at PDCP | It represents service level performance as it reflects IP packet drop rate | It cannot be calculated for RLC UM |
| Packet Loss Rate Calculation at RLC | It could be derived for RLC UM (from H-ARQ retransmissions result) | It is not available for UL unless new UL packet loss measurement reporting is introduced (this has UE impacts). It does not necessarily map to Application packet losses and it might simply be the result of aggressive modulation and coding. |

As it can be seen from the above, the existing Packet Loss measurements are all affected by rather major drawbacks.

Observation 1: Computing packet loss rate may not always be possible, and it may not provide full information on service level performance.

Furthermore, there are some important distinctions to note when discussing the relationship between packet loss measurement at a lower layer and the service or application-level performance. Firstly, compared to other performance metrics, such as throughput and delay, the packet loss rate measured in the RAN is not necessarily reflected by the packet loss rate experienced at a higher layer.

Delay and throughput, on the other hand, are metrics directly linked to service performance at the higher layers, i.e., there is no way an application can improve the packet delay or throughput limitations caused by the RAN. Based on the above, it may be argued that while delay and throughput are absolute measures of service level performance, packet loss is not.

Furthermore, it is also the case that compared to throughput and delay, packet loss is a much more volatile metric. 
In most situations, the choice of a modulation scheme is made in relation to a target BLER, and as a consequence a certain base packet loss rate is the consequence of a design choice of the BLER target. BLER targets are implementation specific and may change in relation to, e.g. the current SNR, the overall amount of resources available, the scheduling policy of a RAN node. Therefore, changes in the packet loss rate may not give information of whether the target cell is a good mobility target or not. The mobility target cell may be a perfectly good cell, but due to implementation choices the target RAN node may decide to e.g. apply an aggressive modulation and coding scheme to the UE and end up in higher packet loss rates, while the performance at the application layer is still good.

Hence the question is whether the impact to the standard of providing this partial and somewhat misleading parameter is worthy of the advantages that it may bring, given that the Packet Loss metric does not cover all use cases and it may not represent performance at service level. On the above basis, we propose the following:

Proposal 3: RAN3 to agree that packet loss is not a suitable candidate metric for AI/ML UE performance feedback in split architecture use cases.

Based on the above discussion, it is easy to deduce that packet loss rate may not be a good choice as a UE performance feedback metric even for a gNB in non-split architecture. Even if a new UL packet loss UL measurement was introduced in Rel19 (with associated UE impacts), such measurement would not be applicable to legacy UEs. On the other hand, lack of applicability of PDCP level packet loss to RLC UM transmission mode means that such feedback may only be obtained for a fraction of the UEs (i.e. those not using RLC UM bearers).

Additionally, we reflect on the fact that release 18 discussions on packet loss and the inclusion of this IE in the UE performance feedback was largely derived from the QoS settings for packet loss rate. The specified range for the packet loss rate is an integer between 0 to 1000 with steps of 0.1%. In realistic scenarios, as specified for NR QoS parameters, the expected packet loss/error rate that the network is expected to offer to services is at most on the order of 10-2. Therefore, the Average Packet Loss DL IE, in its current standardised form, would not even be able to represent the measured packet loss rate in the most typical case (as the value range of the IE is too coarse). In other words, the Average Packet Loss DL IE introduced in TS38.423 is of limited use and ignoring this IE would bring no impact to the overall functionality.

On the basis of this, we conclude that fixing the shortfalls to enable measurements of the packet loss rate in all use cases implies big impacts to the standard and in the end, it does not deliver information that are crucial to understand whether a mobility target is good or bad. We therefore propose the following:

Proposal 4: RAN3 to agree on a correction to release 18 UE performance feedback, where the Average Packet Loss DL IE is not used, and it is ignored.

Proposal 5: RAN3 to not continue further discussion on Average Packet Loss for UE performance in the context of split architecture.

A discussion may be taken in RAN3 on whether packet losses could be re-interpreted as PDCP level losses and whether they can be calculated in UL and DL.

#### 2.1.4 Throughput

As discussed in the previous section, and in Ericsson’s contribution to R3#125 (R3-244417) any performance measure computed at a higher layer is generally considered closer to the application-level or service-level perceived performance. TS 28.552 defines throughput measurements, however, the Average DL and UL UE throughput in gNB is defined as the ratio of successfully delivered RLC SDU data volume over time. This measure is available at the gNB-DU.

An alternative to the above is to obtain the UE throughput measurements from the gNB-CU-UP, which is the termination point for PDCP. While there are no measurements defined for PDCP throughput measurements at the gNB-CU-UP, the specifications imply that such functionality is available at the gNB-CU-UP as part of the traffic policing functionality. This functionality is likely up to the implementation. However, if such a metric were to be used in inter-node signalling, this would require a more rigorous definition of the measurement. It is likely preferred to avoid such discussions as another defined metric is available at the gNB-DU already.

Based on the above, we make the following proposal:

Proposal 6: RAN3 to agree using downlink and uplink throughput as measured by the gNB-DU (defined in Sec. 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558) as the downlink and uplink throughput for UE performance feedback in split architecture.

## 2.2 Energy Saving

During RAN3 #121, the below agreement was reached, although further discussions in the context of split architecture were suspended to future releases, and the same is in scope in Release 19.

Work on the measured EC transmission from gNB-DU to gNB-CU over F1 in R18. Whether reusing the current F1AP procedures or defining new procedures needs to be further discussed.

The following in RAN3 #124 confirms the earlier agreement:

Measured Energy Cost can be transferred from DU to CU.

In addition to signalling the above, agreements in Rel-18 and Rel-19 do not mention exchange of EC from the gNB-CU-UP to the gNB-CU-CP or if it suffices to report EC from only the gNB-DU for use as node-level EC. Herein, we describe our views and argumentation in this matter.

It is common understanding that a significant portion of the energy consumed at a gNB in split architecture is expected to be at the different gNB-DUs, as it is the logical entity that hosts the baseband and radios. The energy consumed on a gNB-CU-UP, on the other hand, is expected to be negligible if compared to that of the gNB-DU. Another complication worth considering in the generation of an EC associated to the gNB-CU-UP is that the gNB-CU-UP may be realized on a virtual/cloud environment hosting multiple instances of gNB-CU-UPs and either together with or independent of the gNB-CU-CP. In such cases, accurately determining the EC associated with a single instance of a gNB-CU-UP function may not be feasible.

Furthermore, there is precedence in NR specs when it comes to determining the energy consumption in split architecture, shown by the extract below from TS 28.310, where it is considered reasonable to compute the gNB’s energy consumption as just the energy consumption of the gNB-DU.

When gNBCU/gNBCU-CP/gNBCU-UP energy consumption is assumed to be very small compared to gNBDU and given that, in some cases, the gNBCU/gNBCU-CP/gNBCU-UP may be virtualized, the present document only considers the energy consumed in gNBDU(s) (in case of split scenarios) and in non-split gNBs (see clause 4.2.1 of 3GPP TS 28.541 [11] and clause 6.1.1 of 3GPP TS 38.401 [12]). There might be a need for some correction in KPI between the different deployment scenarios.

Based on the above, we have the following proposals:

Proposal 7: There is no need to signal EC from gNB-CU-UP to the gNB-CU-CP.

Proposal 8: RAN3 to only discuss signalling of EC from gNB-DU to gNB-CU-CP.

## 2.3 Signalling aspects

In light of what is discussed in this paper it is straight forward that some UE performance metrics in the split architecture case shall be signaled from the gNB-CU-UP to the gNB-CU-CP and some other metrics shall be signaled from the gNB-DU to gNB-CU-CP.

In this section, we tackle the signalling aspects of the aforementioned metrics.

#### 2.3.1 UE performance

In the non-split architecture use-case, the request for UE performance metrics is implicitly carried over a handover request and it is indicated by the presence of a DataCollectionID in the handover request. The node collecting the UE performance transmits the collected information to the requesting node over the corresponding Data Collection Reporting procedure identified by the DataCollectionID. The configuration for the UE performance reporting is determined by the UE Performance Feedback Collection Configuration IE present in the Data Collection Request.

In the split architecture however, the UE performance metrics must be retrieved from the gNB-CU-UP or the gNB-DU of the target node, which does not receive the HANDOVER REQUEST from the source gNB. Furthermore, the DATA COLLECTION REPORTING procedure, which is used to signal back the collected UE performance across gNBs, is not defined for the F1 or the E1 interface. Hence, signaling extensions are required to facilitate exchange of the UE Performance Feedback Collection Configuration and UE performance feedback reporting. Below we analyze the need and the required extensions.

First, it is important to note that the Data Collection Reporting procedure (over Xn) is envisioned to exchange/fetch both measured and predicted information between NG RAN nodes, where either of the nodes may be providing inference, input, feedback, or predicted data to the other node simultaneously.

On the other handfor all the use cases addressed by RAN3 for AI/ML, it has been agreed that the AI/ML inference functions will only be hosted at the gNB-CU-CP. Therefore, the gNB-DU or the gNB-CU-UP only report measured values (either used for training, or feedback, or as input for inference) to the AI/ML functions at the gNB CU-CP, i.e., there is no AI/ML-based inference/predictions signaled from the gNB-DU or the gNB-CU-UP to the gNB-CU-CP.

Observation 2:  In the context of exchanging UE performance metrics, compared to the Xn interface, E1 and F1 interfaces have different characteristics, namely there is no signaling of AI/ML inference results over F1 or E1. E1 and F1 are used purely to exchange measurements.

Currently over F1 and E1 interfaces, there already exists Resource Status Reporting procedure which is used by a gNB-CU to request from the gNB-DU or the gNB-CU-UP reporting of measurements which may be used for various purpose, e.g. as AI/ML inference inputs or feedback information. From an operational perspective, the gNB-CU-CP uses this information for actions such as load balancing, resource planning, etc. While UE performance feedback is intended to be used as feedback to AI/ML derived actions. Obtaining data to validate, or train, or obtain feedback on performed actions and its impact on load and other operational parameters is an integral aspect of efficient networks and should be considered as another dimension in the resource/load optimization and other operational functions, e.g., taking into account the UE performance as part of the mobility, load balancing, and resource optimization problems. It is therefore better to include signaling of the UE performance metrics from the gNB-DU and the gNB-CU-UP to the gNB-CU-CP as part of the Resource Status Reporting procedure.

Observation 3: Based on the above, there is no strong need to add a new procedure over the F1 or the E1 interface to accomplish exchange of UE Performance information from the gNB-DU or the gNB-CU-UP to the gNB-CU, in fact extending the Resource Status Reporting procedure results in more efficient interface design where the different relevant metrics are signaled over the same procedure and where the Resource Status Update carries purely measurements.

Proposal 9: Extend the F1 AP RESOURCE STATUS REPORTING procedure’s specification with the requesting and reporting of UE performance metrics.

Proposal 10: Extend the E1 AP RESOURCE STATUS REPORTING procedure’s specification with the requesting and reporting of UE performance metrics.

#### 2.3.1.1 Enabling triggering of UE performance collection at the CU-UP

Another aspect of the design over the XN interface for UE performance feedback is that the request for feedback for the UE is carried over the Handover Request message. In split architecture however, the proposed extensions to Resource Status Reporting are signaled between RAN functions of the same gNB. In order to collect feedback for specific UEs, appropriate messages to trigger reporting have to be identified over the E1 and F1 interfaces.

The E1 AP Bearer Context Setup procedure is performed once per DRB for a UE that is setup at a target node. By taking advantage of a procedure that runs per DRB implies that a finer granularity of feedback can be obtained, however, additional information, with regards to the DRB structure, is required at a neighbor node that received per-DRB-level measurements. The gNB-CU also has the possibility to selectively enable UE performance feedback on specific DRBs if needed. The gNB-CU-UP, which collects the measurements at a per-DRB-level may signal the measurements as such to the gNB-CU-CP.

Proposal 11: RAN3 to discuss and agree to obtaining UE performance feedback at DRB-level granularity between a gNB-CU-UP and a gNB-CU-CP.

Based on the above, UE performance feedback over E1 interface in split architecture will operate as the following description:

- A target gNB-CU-CP that receives a HANDOVER REQUEST with an implicit indication to collect UE performance for the corresponding handover,

- There exists a pre-configured Resource Status Reporting procedure between the gNB-CU-CP and the gNB-CU-UP, as part of which the UE Performance Collection Configuration was signaled. The DataCollectionID of this reporting procedure is identified as the combination of NG-RAN node 1 and NG-RAN node 2 measurement IDs.

- The BEARER CONTEXT SETUP REQUEST message from the gNB-CU-CP to the gNB-CU-UP contains a DataCollectionID IE corresponding to an already configured E1 AP Resource Status Reporting Procedure between the gNB-CU-UP and the gNB-CU-CP.

- The gNB-CU-UP receiving the above request may collect the specified UE performance depending on the measurement and exit criteria and report the metrics back to the gNB-CU-UP in the next transmission instance of a RESOURCE STATUS UPDATE message after the UE performance collection has terminated. The above measurement and reporting are performed per DRB configured for the UE

The figure below represents the full flow chart.

Step 1: Two neighbour gNBs have already been configured with a XN AP: Data Collection Reporting procedure between them, which also has a corresponding configuration for the UE performance collection configuration.

Step 2: gNB-CU-CP2 and gNB CU-UP2 also have a configured E1 AP: Resource Status Reporting procedure between them with a corresponding UE performance collection configuration.

Step 3a: gNB1 determines to handover one or more UEs to gNB2. This may be based on any AI/ML algorithm connected to use cases such as mobility optimization, load balancing, or energy savings.

Step 3b: gNB handovers UEs to the neighbour node. In the handover request message, the Data Collection ID corresponding to the Data Collection Reporting procedure (Step 1) is provided to the target gNB.

Step 4a: The UE connects to gNB2.

Step 4b: As part or continuation of the UE connecting to gNB2, DRBs are setup for the UE through the E1 AP: Bearer Context Setup Request message. This message also includes the ID of the E1 AP: Resource Status Reporting procedure over which the UE performance feedback shall be sent.

Step 5: Data delivered to the UE and legacy UP metrics (e.g. RAN level delay) are collected and reported

Step 6a: The exit criteria as specified in the UE performance collection configuration is fulfilled. This could be based on a timer, a handover, or other criteria.

Step 6b: The gNB-CU-UP reports the collected UE performance on the E1 AP: Resource Status Reporting procedure.

Step 7: Upon receiving the collected UE Performance, the gNB-CU-CP2 generates per S-NSSAI, per UE performance metrics and transmits the information to the neighbour gNB-CU-CP1 over the XN AP: Data Collection Reporting procedure.

#### 2.3.1.2 Enabling triggering of UE performance collection at the DU

Extensions similar to the ones discussed above for the E1interface are also required over the F1 interface to trigger the collection of UE performance metrics at the DU when a UE is handed over to a new gNB. The F1 AP UE CONTEXT SETUP procedure is performed between the gNB-CU-CP and the gNB-DU to establish a UE context at the gNB-DU and the associated configuration for the different SRB, DRB, etc. By taking advantage of a procedure that runs per DRB implies that a finer granularity of feedback can be obtained, however, additional information, with regards to the DRB structure, is required at a neighbor node that received per-DRB-level measurements. The gNB-CU also has the possibility to selectively enable UE performance feedback on specific DRBs if needed. The gNB-DU, which collects the measurements at a per-DRB-level may signal the measurements as such to the gNB-CU-CP.

Proposal 12: RAN3 to discuss and agree to obtaining UE performance feedback at DRB-level granularity between a gNB-DU and a gNB-CU-CP.

Based on the above, UE performance feedback over the F1 interface in split architecture will operate as the following description:

- A target gNB-CU-CP that receives a HANDOVER REQUEST with an implicit indication to collect UE performance for the corresponding handover,

- There exists a pre-configured Resource Status Reporting procedure between the gNB-CU-CP and the gNB-DU, as part of which the UE Performance Collection Configuration was signaled. The DataCollectionID of this reporting procedure is identified as the combination of NG-RAN node 1 and NG-RAN node 2 measurement IDs.

- The UE CONTEXT SETUP REQUEST message from the gNB-CU-CP to the gNB-DU contains a DataCollectionID IE corresponding to an already configured F1 AP Resource Status Reporting Procedure between the gNB-DU and the gNB-CU-CP.

- The gNB-DU receiving the above request may collect the specified UE performance depending on the measurement and exit criteria and report the metrics back to the gNB-CU-UP in the next transmission instance of a RESOURCE STATUS UPDATE message after the UE performance collection has terminated. The above measurement and reporting are performed per DRB configured for the UE

The figure below represents the full flow chart.

Step 1: Two neighbour gNBs have already been configured with a XN AP: Data Collection Reporting procedure between them, which also has a corresponding configuration for the UE performance collection configuration.

Step 2: gNB-CU-CP2 and gNB CU-UP2 also have a configured F1 AP: Resource Status Reporting procedure between them with a corresponding UE performance collection configuration.

Step 3a: gNB1 determines to handover one or more UEs to gNB2. This may be based on any AI/ML algorithm connected to use cases such as mobility optimization, load balancing, or energy savings.

Step 3b: gNB handovers UEs to the neighbour node. In the handover request message, the Data Collection ID corresponding to the Data Collection Reporting procedure (Step 1) is provided to the target gNB.

Step 4a: The UE connects to gNB2.

Step 4b: As part or continuation of the UE connecting to gNB2, DRBs are setup for the UE through the F1 AP: UE CONTEXT SETUP Request message. This message also includes the ID of the F1 AP: Resource Status Reporting procedure over which the UE performance feedback shall be sent.

Step 5: Data delivered to the UE and legacy UP metrics (e.g. RAN level delay) are collected and reported

Step 6a: The exit criteria as specified in the UE performance collection configuration is fulfilled. This could be based on a timer, a handover, or other criteria.

Step 6b: The gNB-DU reports the collected UE performance on the F1 AP: Resource Status Reporting procedure.

Step 7: Upon receiving the collected UE Performance, the gNB-CU-CP2 generates per S-NSSAI, per UE performance metrics and transmits the information to the neighbour gNB-CU-CP1 over the XN AP: Data Collection Reporting procedure.

#### 2.3.2 Energy Saving

Over F1-C, there already exists a Resource Status Reporting procedure which is used by a gNB-CU to request from the gNB-DU reporting of measurements such as PRB usage, TNL available capacity, etc., which may be used to understand the load and resource related situation on the gNB-DU. From an operational perspective, the gNB-CU uses this information for actions such as load balancing, resource planning, etc. Energy saving is another integral aspect of efficient networks and should be considered as another important dimension in the resource optimization and other operational functions. It is therefore better to include signaling of the EC from the gNB-DU to the gNB-CU-CP as part of the Resource Status Reporting procedure.

Observation 5: Based on the above, there is no strong need to add a new procedure over the F1 interface to accomplish exchange of EC from the gNB-DU to the gNB-CU, in fact extending the Resource Status Reporting procedure results in more efficient interface design where the different relevant metrics are signaled over the same procedure.

Proposal 13: The required extensions to F1-C to facilitate the exchange of the EC from the gNB-DU to the gNB-CU-CP can be achieved by extending legacy procedures, i.e., Resource Status Reporting procedure.

# 3. Conclusion

Based on the discussion in this paper, we make the following observation and proposals.

Proposal 1: RAN3 to agree to the exchange of RAN delay measurements for UL and DL from the gNB-CU-UP and gNB-DU to the gNB-CU-CP with a per-DRB-level granularity.

Proposal 2 4: The gNB-DU and the gNB-CU-UP shall signal a UL or DL delay metric to the gNB-CU-CP that is generated by summing up the different delay components at gNB-DU and gNB-CU-UP respectively. The gNB-CU ensures that the measurements are configured with the same measurement and reporting periodicity. The gNB-CU-CP adds together the delay measurements received (from the UE, gNB-CU-UP, and the gNB-DU) within the same reporting period and uses this sum as the UL/DL delay metric to report as UE performance information.Proposal 3: RAN3 to agree that packet loss is not a suitable candidate metric for AI/ML UE performance feedback in split architecture use cases.

Proposal 4: RAN3 to agree on a correction to release 18 UE performance feedback, where the Average Packet Loss DL IE is not used, and it is ignored.

Proposal 5: RAN3 to not continue further discussion on Average Packet Loss for UE performance in the context of split architecture.

Proposal 6: RAN3 to agree using downlink and uplink throughput as measured by the gNB-DU (defined in Sec. 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558) as the downlink and uplink throughput for UE performance feedback in split architecture.

Proposal 7: There is no need to signal EC from gNB-CU-UP to the gNB-CU-CP.

Proposal 8: RAN3 to only discuss signalling of EC from gNB-DU to gNB-CU-CP.

Proposal 9: Extend the F1 AP RESOURCE STATUS REPORTING procedure’s specification with the requesting and reporting of UE performance metrics.

Proposal 10: Extend the E1 AP RESOURCE STATUS REPORTING procedure’s specification with the requesting and reporting of UE performance metrics.

Proposal 11: RAN3 to discuss and agree to obtaining UE performance feedback at DRB-level granularity between a gNB-CU-UP and a gNB-CU-CP.

Proposal 12: RAN3 to discuss and agree to obtaining UE performance feedback at DRB-level granularity between a gNB-DU and a gNB-CU-CP.

Proposal 13: The required extensions to F1-C to facilitate the exchange of the EC from the gNB-DU to the gNB-CU-CP can be achieved by extending legacy procedures, i.e., Resource Status Reporting procedure.

Proposal 14: Agree on the TPs provided in R3-245506 (F1) and R3-245507 (E1).

# 4. References

- R3-244810 (TP to 38.473) Conclusion on AI/ML NG-RAN Enhancements