3GPP TSG-RAN WG3 Meeting #125bis	R3-245571

Hefei, China, October 14th – 18th, 2024

Agenda item:	14.3

Source: 	LG Electronics Inc.

Title: 	Discussions on NG management and Inactive support

Document for:	Discussion and Decision

# Introduction

The last RAN3-125 meeting has progressed as follows [1]:

NG Interface management Issue:

Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management.

Inactive UE support

RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

Feeder link switch issue

RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

Regarding the NG interface management, we discuss the OAM aspects to finalize and complement the agreed solution. For the Inactive UE support, we highlight some potential issues for further discussions.

# Discussion

## OAM requirement for NG connection management

In the last meeting, we proposed for AMF to initiate the graceful removal of NG interface when an NG-connected satellite on-board gNB leaves its service area [2], based on the following reason:

- AMF can better predict when a NG-connected satellite will leave its service area based on dynamic TAI updates. Over NGAP, tracking area information is uni-directional from gNB to AMF, i.e. earth-fixed TACs supported by a gNB is only reported/updated to AMF, while the service area of a specific AMF, in terms of tracking areas, has never been exposed to a gNB over NGAP.

However, after discussions, RAN3 decided to approach this differently. Option 2 was agreed that the satellite will now initiate the removal and setup of NG interfaces as it moves in and out of AMF services areas:

NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management.

This requires that a satellite on-board gNB be aware of the service areas of the AMFs with which it should have NG connections along its orbit. That service area information could be pre-configured by specifying when to initiate NG removal or NG setup toward specific AMFs (e.g. timetable). Or, it could be pre-configured by providing a mapping between the (earth-fixed) tacking areas and AMF service areas, so that the satellite can determine the appropriate actions as it moves along.

Regardless of how, we believe that it should be captured as an OAM requirement to a gNB hosted by the regenerative payload. Now, frequent changes in NG connectivity are expected with different AMFs over time, which was never the case with Rel-17/18 NTN gNB on the ground for transparent payloads, whose NG connectivity was observed to be no different from that of a gNB in the terrestrial network. To implement Option 2, it is critical that a satellite on-board gNB knows the service areas of the AMFs with which it should have NG connections – a new requirement that was never considered for a gNB in the previous releases.

Moreover, we think OAM based configuration is appropriate for this new requirement. Since each satellite follows the pre-determined orbits and visits a specific geographical area for specific time duration as pre-planned, such AMF service area information would not change dynamically. But this does not mean it will never change once configured (e.g. a new AMF site could be added). Any updates should be able to be configured on the fly – such non-dynamic nature well suits OAM based configuration.

Proposal 1: The agreed Option 2 requires a satellite on-board gNB be aware of the coverage areas of the AMFs with which it should have NG connections along its orbit, for which their service areas (in terms of tracking areas) has never been exposed to a gNB over NGAP.

Proposal 2: Capture this as an additional OAM requirement for a gNB hosted by the regenerative payload, to be configured with the service areas of the AMFs for NG connection management.

## OAM requirement for dynamic TAI updates

In the last meeting, some companies raised a good discussion point regarding the dynamic TAI updates from the satellite on-board gNB [3][4]:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

R3-244182 [3] OAM requirement for supported TAI list of a gNB on board:

The gNB provides the supported TA list information to the AMF, and based on the TA list of the UE registration area and the supported TA list of gNB, the AMF selects the appropriate gNB to send paging messages to the UE. For ground gNB, the coverage area of the Tracking Area remains fixed. Once the gNB is deployed onto a satellite, such as MEO or LEO satellites, the mobility of the on board gNB causes the coverage of the gNB to change over time, and the supported TAIs list also changes accordingly. For traditional procedure, frequent updates of the supported TAIs list will cause a large number of NG interface signaling interactions.

SA2 has already discussed how to manage the dynamic list of supported TAIs. The solutions are as follows in current TR 23.700-29:

- TA "timetable“ is calculated per RAN node by AMF based on satellite ephemeris (Solution #2)

- TA "timetable“ is provided to AMF per RAN node ID (Solution #4)

Observation 6: The solutions of supported TAIs management are as follows in current TR 23.700-47:

- Signalling based solution

- OAM based solution

In TR 23.700-29, Solution #2 mentioned that the on-board RAN nodes may not notify the AMF or may periodically notify the AMF of the supported TA List and the AMF should be configured with ephemeris information of all on-board eNB by OAM. Solution #4 mentioned that RAN only updates the TAI list at the quantified time point or TA "timetable“ is provided to AMF per RAN node ID by preconfiguring. In R18, we discussed the served cells and neighbor relations. It is not need to exchange the served cells and neighbor relations in case of earth moving cells are deployed. The served cell information and its neighbour relations could be left to OAM. Similar as the above problem, we have a preference on OAM based solution to reduce the frequency of interactions. The information of supported TAIs is provided to AMF per RAN node ID by preconfiguring. In addition, it is proposed to send LS to SA2 and SA5 on OAM requirement for supported TAIs and the draft LS is provided in appendix 5.

Proposal 4: To avoid frequently signalling interactions, it is proposed to adopt OAM based solution, which means the supported TAIs of an on-board gNB is provided to AMF by OAM/preconfiguration.

Proposal 5:  It is proposed to send LS to SA2 and SA5 on OAM requirement for supported TAIs and the draft LS is provided in appendix 5.

R3-244182 [4] TAI update:

The Supported TAI List of onboard gNB is change over time for the NGSO. This requires pre-configuring mapping relationship among Global RAN Node ID, supporting TAI list and valid period. In the context of R17/R18, when RAN updates the TAI list, RAN will inform AMF of the updates. Even though there are frequent interactions between RAN and AMF, the cost of interactions is limited because the interactions are via ground link (e.g. fibre, cable, etc.). However, such interactions will be over feeder link in R19 and may be more frequent, whose bandwidth is scarce. OAM based solution is preferred to exchange the supported TAI list of the gNB.

Proposal 3: The exchange of Supported TAI List of onboard gNB can be based on OAM.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

We think their concerns are valid. Although dynamic updates to the supported TAI list toward AMF (caused by changes in satellite coverages over time) were also present in Rel-17/18 for NTN gNBs on the ground for transparent payloads, but we agree with their points that such frequent updates from a gNB at satellite in Rel-19 are not desirable due to their power/processing constraints and limited feeder link transport capability (compared to the on-the-ground transport network’s capability, e.g. fiber).

The solution doesn’t have to be based on OAM, but it seems to be tied up with ephemeris information of the satellite and also with the NTN parameters / tracking areas managed by OAM. From this sense, we also prefer the OAM based solution.

Proposal 3: Discuss OAM support and requirement for dynamic updates of the supported TAIs to AMF from a satellite on-board gNB in Rel-19, considering such frequent updates are not desirable due to their power/processing constraints and limited feeder link transport capability (compared to Rel-17/18 NTN gNBs on the ground for transparent payloads).

## RAN transitioning

The conclusion from SA2 suggests that a UE’s serving gNB change within the regenerative architecture can be leveraged by the existing procedures [5]:

## 8.1 Conclusion for KI#1: Support of Regenerative-based satellite access The conclusion for supporting gNB/eNB on-board the satellite is based on the following principles: - When the eNB/gNB leaves the service area of an AMF/MME (e.g. when setting over the horizon) any new procedures or updates to existing procedures to handle the N2/S1 connection will be defined by RAN WG3. SA WG2 will align its specifications once this has been determined by RAN WG3. - gNB /eNB IP address change due to feeder link switch can be supported using existing N2 and S1 procedures. There are no normative impacts to SA WG2 specifications due to this. SA WG2 will align its specifications if new solution is developed by RAN WG3. - RAN transitioning between different NTN GWs, TAIs and potentially between MMEs/AMFs can be supported by using existing procedures, e.g. the eNB/gNB releasing UEs to (E)CM_IDLE before changing NTN gateway, or by using existing load balancing, trigger handover when it realizes it is going to leave a tracking area due to its own movement, etc. - AMF/MME can treat the Mapped Cell IDs as for Rel-17. - Knowledge of when regenerative payload type could be used to assist with CN PDB determination and policy decisions. Introducing new RAT types is not required and it is assumed deployments can determine the usage of regenerative payload type if needed.

The above highlighted part is basically saying that an already connected UE can be handed over to another incoming gNB or released to IDLE by the time that the serving gNB at satellite moves out of its area. HO may not be the best solution, but switching the UE’s RAN termination point to different gNB will be better than moving an innocent UE to IDLE (re-establishing RRC again with new incoming gNB). For an already IDLE UE, the existing CN paging and cell (re-)selection were expected to be re-used.

While we are not saying that the existing procedures won’t work, but we observe that RRC Inactive was not well considered in the SA2’s conclusion.

From RAN perspective, waking up UEs to move to IDLE or CONNECTED requires explicit Uu signalling exchanges (e.g. RAN paging, resume request/complete or release). This could cause a large burden on service links, especially, if there are many INACTIVE UEs with no active data exchanges, but nonetheless forced to update their RRC states every time the serving satellite is switched over their earth-fixed RNAs. In fact, such updates are unnecessary as these RRC state changes were not caused by the UE’s mobility.

There are also several issues with applying the existing INACTIVE mobility framework that RAN3 already confirmed (“subject to implementation and deployment”). For example, the serving gNB will be sooner or later moving out the service area of the AMF serving the INACTIVE UEs. RAN3 has already agreed to support Option 2 for NG connection management, meaning that the NG interface will be torn down if the last serving gNB moves out of the AMF’s service area. The stored UE context would be released, and RAN paging would not work, even if there is Xn connectivity between the last serving gNB and the incoming gNB. Moreover, Xn connectivity over ISL is not guaranteed, making the Xn-based context retrieval mechanism unreliable for MO-traffic.

One potential solution could be to allow AMF to temporarily assume the role of “last serving gNB” for those INACTIVE UEs. The problem is caused due to a moving satellite, while the UE and CN on the ground remains still when MO/MT-traffic occurs for the INACTIVE UEs. Therefore, we think the right solution is to fix the point of “last serving gNB” to be reachable during the transient period.

For that, when the serving gNB moves out of the RNA of an INACTIVE UE, it can inform the serving AMF and provide the UE’s RAN UE context information (e.g. I-RNTI, AS context, security context, etc.). This information can be temporarily stored in the serving AMF. When MO-traffic occurs, the UE requests resume to a new gNB serving that area. The new gNB then can request UE context retrieval from the connected AMF. Using the received I-RNTI, the AMF can look up the stored RAN UE context, verify MAC-I, and reply with the UE context response (as if performed at the last serving gNB), which then establishes NGAP connection for the UE with the new gNB.

For MT-traffic, DL traffic/signalling for the UE can be buffered (similar to MT-communication handling), and CN paging can be subsequently triggered to the new gNB serving that area to make the UE request resume. Then, the mobile originated INACTIVE mobility with AMF described above can follow. After the RAN UE context is successfully established in the new gNB, AMF can release the temporarily stored context.

Proposal 4: Discuss potential enhancements for the IANCTIVE mobility framework caused due to a moving satellite gNB (while the UE and CN on the ground remains still).

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

OAM requirement for NG connection management

Proposal 1: The agreed Option 2 requires a satellite on-board gNB be aware of the coverage areas of the AMFs with which it should have NG connections along its orbit, for which their service areas (in terms of tracking areas) has never been exposed to a gNB over NGAP.

Proposal 2: Capture this as an additional OAM requirement for a gNB hosted by the regenerative payload, to be configured with the service areas of the AMFs for NG connection management.

The TP for Proposal 2 can be found in [6].

OAM requirement for dynamic TAI updates

Proposal 3: Discuss OAM support and requirement for dynamic updates of the supported TAIs to AMF from a satellite on-board gNB in Rel-19, considering such frequent updates are not desirable due to their power/processing constraints and limited feeder link transport capability (compared to Rel-17/18 NTN gNBs on the ground for transparent payloads).

RAN transitioning

Proposal 4: Discuss potential enhancements for the IANCTIVE mobility framework caused due to a moving satellite gNB (while the UE and CN on the ground remains still).

# Reference

[1] R3-244748, “Discussion on Support of Regenerative Payload”, THALES (moderator)

[2] R3-244479, “Discussion on N2/S1 connection management”, LG Electronics Inc.

[3] R3-244182, “(TP to NTN BL CRs) Support of regenerative payload”, CATT

[4] R3-244247, “Further discussion on support of regenerative payload”, China Telecom

[5] TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3 (Release 19)

[6] R3-245572, “(TP for NR_NTN_Ph3 TS 38.300)”, LG Electronics Inc.