3GPP TSG-RAN WG3 Meeting #125bis	R3-245592

Hefei, China, 14 - 18, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | draftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of SDT network-based solution [SDTnetworkSolution] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The network-based solution was discussed in RAN3 to support the scenario in which the network moves the UE to RRC_CONNECTED mode by triggering RRC setup towards the UE, in case e.g., DL non-SDT data arrive during an on-going RA-SDT without anchor relocation (after SDT partial context transfer). In this network-based solution, there is a need to add new indication in XnAP: RETRIEVE UE CONTEXT RESPONSE to trigger the receiving gNB to send RRCSetup towards the UE, and update procedural text and stage2 call flows accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description of network-based solution into the existing RACH based SDT section. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it introduces network-based solution for SDT, to allow the gNB to use another way to terminate the ongoing SDT procedure and moves the UE to RRC_CONNECTED mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support network-based solution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS 38.423 CR 1407 TS 38.401 CR 0435 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ------------Start of the Change---------------

## 18.3	SDT without UE context relocation

The overall procedure for SDT procedure over RACH without UE context relocation is illustrated in the figure 18.3-1.

Figure 18.3-1. RA-based SDT without UE context relocation

1/2. The steps 1/2 are as defined in steps 1/2 in Figure 18.2-1.

3.	The last serving gNB decides not to relocate the full UE context for SDT. In case that the Last Serving gNB has requested AMF for CN Buffering as part of step 0 for the UE in RRC INACTIVE state with eDRX cycle longer than 10.24 seconds, reachability of the UE is indicated to the AMF by the last serving gNB using the MT Communication Handling procedure as described in TS 23.501 [3] and TS 38.413 [26].

4.	The last serving gNB transfers a partial UE context including the SDT related RLC context.

5.	The receiving gNB acknowledges receiving the partial UE context and provides associated DL TNL address. The UE context is kept at the last serving gNB and the SDT related RLC context is established at the receiving gNB. Then UL/DL GTP-U tunnels are established for DRBs configured for SDT, if any, and the UL SDT data and/or signalling, if any, are forwarded to the last serving gNB, and then delivered to the core network.

NOTE 1:	The DL signalling from the last serving gNB, if any, is forwarded to the receiving gNB via the RRC TRANSFER message, for which the receiving gNB delivers it to the UE.

NOTE 1a:	In case UE assistance information (i.e. UL non-SDT data arrival indication) is received from the UE, the last serving gNB terminates the SDT procedure by sending the RRCRelease message as defined in step 8/9.

NOTE 1b:	The last serving gNB may terminate the SDT procedure by sending the RRCRelease message as defined in step 8/9, or by providing full UE context via RETRIEVE UE CONTEXT RESPONSE message to the receiving gNB to trigger the receiving gNB sending RRCSetup message to move the UE to RRC_CONNECTED state, based on (e.g. large size of) DL SDT data or DL SDT signalling.

NOTE 1c:	In case DL non-SDT data or DL non-SDT signalling arrives, the last serving gNB terminates the SDT procedure, either by sending the RRCRelease message as defined in step 8/9, or by providing full UE context via RETRIEVE UE CONTEXT RESPONSE message to the receiving gNB to trigger the receiving gNB sending RRCSetup message to move the UE to RRC_CONNECTED state.

6.	The receiving gNB detects the end of SDT session and sends the RETRIEVE UE CONTEXT CONFIRM message including whether this is a "normal" end of SDT transaction or a radio link problem, or large SDT volume from BSR.

7.	Upon receiving the RETRIEVE UE CONTEXT CONFIRM message and deciding to terminate the SDT, the last serving gNB responds to the receiving gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRCRelease message. The receiving gNB shall release the established partial UE context.

NOTE 2:	Void.

NOTE 3:	Void.

8.	The receiving gNB sends the RRCRelease message to the UE.

NOTE 4:	Void.

NOTE 5:	The last serving gNB may terminate the SDT procedure by sending the RRCRelease message upon receiving the indication about large uplink SDT data from the BSR from the receiving gNB in step 6.

NOTE 6:	The Last Serving gNB may send the NGAP MT Communication Handling Request message to the AMF to trigger CN buffering when the UE is put again into RRC_INACTIVE state with eDRX cycle longer than 10.24 seconds.

9.	The UE moves to RRC_INACTIVE state if the suspend indication is included in the RRCRelease message, or the UE may initiate RRC resume procedure to transition to RRC_CONNECTED state if the resume indication is included in the RRCRelease message. Or else, the UE moves to RRC_IDLE state.

### ------------End of the Change---------------