3GPP TSG-RAN WG3 Meeting #125bis	R3-245608

Hefei, China, 14 – 18 October, 2024

Agenda item:	11.4

Source:	Nokia, Jio

Title:	Procedures for AI/ML Mobility Optimization in NR-DC

Document for:	Discussion and Decision

# 1	Introduction

In TR 38.743, we have captured that the potential standards impacts from Mobility Optimization for NR-DC relate to enhancing Dual Connectivity procedures (e.g., SN Addition, MN-initiated SN Change) to trigger the collection of measured UE performance. In this contribution we provide our views on the introduced procedures to enable the collection and reporting of UE Performance measurements in NR-DC.

# 2	Discussion

## 2.1	Procedures to trigger the Collection and Reporting of UE Performance in Dual Connectivity

In TR 38.743 we have agreed that Mobility Optimization for NR-DC only assumes inference in the MN. This means that the main use case is limited to Dual Connectivity procedures such as SN Addition and MN-Initiated SN Change, without taking into account Conditional Dual Connectivity procedures. The two scenarios are illustrated, as a matter of example, in Figure 1. In the example of Figure 1, a UE is connected to gNB 2. Even though for simplicity the figure doesn’t illustrate such details, a UE may be connected to gNB 2 after it has been handed over from another gNB 1. gNB 2 may initiate SN Addition by requesting gNB 3 to be added as an SN node. Upon the completion of the SN Addition procedure, the relationship between gNB2 and gNB3 will change to MN and SN respectively. The MN (gNB 2) may also decide subsequently to change SN (gNB3) to another gNB 4 by initiating SN Change procedures. Upon completion of the SN Change procedure the new SN for MN (gNB 2) will be gNB4.

Figure 1 Example of SN Addition Request and SN Addition Change

According to TR 38.743, the standards impacts of the NR-DC Mobility Optimization topic is focused on enhancing Dual Connectivity procedures e.g., SN Addition and MN-initiated SN Change in order to trigger the collection of measured UE Performance. In the sequel, we provide some example scenarios when a UE is handed over. In the example of Figure 2, gNB1 has sent a DATA COLLECTION REQUEST message to gNB 2. The request may include for instance UE Performance measurements. gNB2 has acknowledged the request by sending a DATA COLLECTION RESPONSE to gNB1. In the course of time, a UE served by gNB 1 is handed over to gNB 2. In this example, after the handover, gNB2 sends a DATA COLLECTION UPDATE message with measurements, that may include for instance UE Performance measurements and subsequently decides to initiate SN Addition. This means that subsequent reports from gNB 2 will include measurements measured under Dual Connectivity.

Figure 2 Example of UE Handover where Dual Connectivity is Initiated by gNB 2 at a later point time after a UE is handed over to gNB 2.

Another example is illustrated in Figure 3. In this example, gNB 2 establishes Dual Connectivity to serve immediately the UE when it is handed over to its cell. The UE is served in Dual Connectivity in gNB 2 throughout the time it is served by the target cell.

Figure 3 Example where Dual Connectivity is initiated immediately when UE is handed over to gNB 2.

As shown in the above examples, UE Performance will need to be collected by all gNBs serving the UE, including MN and SN nodes. Several issues make the problem of UE Performance measurement collection different in Dual Connectivity:

- UE Performance measurement collection needs to be configured by MN to the SN that collects UE Performance measurements. As shown in the example of Figure 1 above, it is possible that the chosen SN by the MN changes over time.

- The trigger to start Measurement Collection is not only a successful Handover event for an incoming Handover, but also successful SN Addition completion or successful SN Change completion.

In the rest of the paper, we provide our views with respect to the procedures that we think are needed to address the above aspects. The following two approaches could be considered in order to configure the reporting of UE Performance measurements from SN serving a UE:

- Reuse Data Collection Reporting Initiation and Data Collection Reporting procedures defined over Xn.

- Reuse existing Dual Connectivity (SN Addition and SN Change) procedures for UE Performance Configuration and a new Class 2 procedure for the reporting of the measurements.

In our view, reusing Data Collection Reporting procedures would be an overly complex way of configuring UE Performance measurements from SN. This is because an NR cell, e.g. the target cell served by gNB 2 in Figure 2 and Figure 3, may serve a wide geographical area and hence there may be a high number of candidate SNs, depending on geographical location of the served UEs or the UEs handed over to this cell. Furthermore, since the UE performance collection parameters (reporting duration, reporting periodicity) will be those provided by the source node (gNB 1), the gNB 2 may need to handle UE performance reporting configured by a possibly large number of neighbour nodes, providing possibly different reporting configurations. It seems not feasible to preconfigure each of the SNs under the gNB 2’s coverage with these different configuration parameters. Additionally, the MN-SN relationship may be dynamic and change with time. Configuring Data Collection procedures from a gNB to all possible SNs that the gNB may choose for Dual Connectivity may be wasting resources, especially since UE Performance is UE-associated and the SN decision may be taken by the MN on a per UE-basis.

Observation 1: Configuring Data Collection procedures from a gNB to all possible SNs that the gNB may choose for Dual Connectivity may be wasting resources because UE Performance is UE-associated and the MN may select the SN on a per UE-basis.

On the other hand, reusing existing Dual Connectivity procedures such as SN Addition and SN Change could be a simple way in which an MN, that needs to receive UE Performance related to a specific UE, can configure, in a UE-associated way, UE Performance measurements from the SN that it decides to serve the UE.

Proposal 1: MN configures the collection of UE Performance measurements to an SN by reusing existing Dual Connectivity procedures, such as SN Addition and SN Change procedures.

Another aspect to address is how to enable the actual reporting of UE Performance measurements from SN to MN. We see it as a simple solution to introduce a new UE-associated Class 2 procedure to be used to transfer UE Performance measurements from Target SN to Target MN, and hence avoiding the need for instantiation of specific context for the UE performance measurement in this case (meaning that usage of Measurement ID or Data Collection ID will not be needed).

Proposal 2: Introduce a new UE-associated Class 2 procedure for the reporting of UE Performance measurements from Target SN to Target MN.

Another aspect that needs to be considered is the actual trigger at the MN to start the UE Performance measurement collection. Even though for single connectivity we introduced the handover event trigger this trigger is not suitable for Dual Connectivity, since, as it can be seen in Figure 2, gNB 2 where UE is handed over does not immediately start Dual Connectivity. We think that similarly to the successful handover trigger we can introduce successful SN Addition completion trigger or successful SN change completion triggers to start the collection of UE Performance measurements at the SN node.

Proposal 3: Introduce successful SN Addition completion trigger or successful SN change completion triggers to start the collection of UE Performance measurements at the SN node.

## 2.2	UE(s) with Dual Connectivity in a Target Node

In NR-DC, after a gNB hands over a UE to another node (target node) and if the latter configures Dual Connectivity, the UE is served by two legs, the Master Leg and the Secondary Leg. If the target node does not configure Dual connectivity, the UE is served by only one leg. With MN-Terminated Dual Connectivity, the Core Network Termination and PNR-DCP layer is hosted in MN and data is transmitted over both Master and Secondary legs. Accordingly, the measured UE Performance (e.g, Average Throughput DL) varies depending on whether UE is configured with Dual Connectivity or not.

The collection of UE Performance measurements is started at the target gNB after a successful handover to enable the source node to evaluate the performance of a handover decision based on the UE Performance at the target node. The target may configure Dual Connectivity immediately after a UE is handed over to the target node or at a later point in time. Clearly, the reported UE Performance measurements at the target node will be impacted by the configuration of Dual Connectivity.

Observation 2: The measured (reported) UE Performance varies depending on whether the target node configures Dual Connectivity for a UE that is handed over to it.

Since the decision to configure Dual Connectivity is totally under the control of the target node, the latter may decide to only configure some of the handed over UEs with Dual Connectivity and the rest with Single Connectivity. In this case, the source will observe a difference in the UE Performance feedback for the set of UE(s) configured with Dual Connectivity. Since the source node is completely unaware of those configurations at the target node, it cannot evaluate correctly the different UE Performance results received from the target. In MN-Terminated Dual Connectivity, the reporting node will always include the UE Performance measurements from both MN and SN.

Observation 3: In MN-Terminated Dual Connectivity, the reporting node will always include the UE Performance measurements from both MN and SN.

Good values of measured UE performance may imply that the handover decision was indeed a good one, but it may also be that the handover decision was not optimal, though still the target node improved UE Performance by configuring Dual Connectivity. Clearly, the latter handover would be inferior to a handover with the same performance where the target does not configure Dual Connectivity, since no additional resources would need to be configured at the target for the same observed UE Performance.

Observation 4: Aggregating UE Performance measurements from MN and SN may hide from the receiving source node important information that can be used to determine the performance of a handover decision.

Consider the following scenarios:

- No NR-DC is configured in source node, but there is NR-DC configured in the target node: If a target node configures Dual Connectivity for a certain UE, there will be a change in the measured UE Performance at the target compared to the one at the source due to the additional resources that the target configures to serve the UE. In this case, the source may decide it needs UE Performance measurements only from MN (from MCG bearer).

- NR-DC is configured in both the source node and in the target node: If the UE was configured with Dual Connectivity in the source node (prior to the handover), the source node may only be interested in the UE Performance measurement for the UE only after the Dual Connectivity configuration is successfully completed. Otherwise, part of the measurements are collected from MN only while the rest include both MN  and SN.

Based on the above, we propose the following.

Proposal 4: Enable the reporting of UE Performance measurements on different node granularities, e.g., per a) Target MN only, b) per Target SN only, c) overall UE Performance, or d) overall UE Performance with information per MCG bearers, SCG bearers, and split bearers.

# 3	Conclusion

In this paper we provide the following observations and proposals:

Observation 1: Configuring Data Collection procedures from a gNB to all possible SNs that the gNB may choose for Dual Connectivity may be wasting resources because UE Performance is UE-associated and the MN may select the SN on a per UE-basis.

Proposal 1: MN configures the collection of UE Performance measurements to an SN by reusing existing Dual Connectivity procedures, such as SN Addition and SN Change procedures.

Proposal 2: Introduce a new UE-associated Class 2 procedure for the reporting of UE Performance measurements from Target SN to Target MN.

Proposal 3: Introduce successful SN Addition completion trigger or successful SN change completion triggers to start the collection of UE Performance measurements at the SN node.

Observation 2: The measured (reported) UE Performance varies depending on whether the target node configures Dual Connectivity for a UE that is handed over to it.

Observation 3: In MN-Terminated Dual Connectivity, the reporting node will always include the UE Performance measurements from both MN and SN.

Observation 4: Aggregating UE Performance measurements from MN and SN may hide from the receiving source node important information that can be used to determine the performance of a handover decision.

Proposal 4: Enable the reporting of UE Performance measurements on different node granularities, e.g., per a) Target MN only, b) per Target SN only, c) overall UE Performance, or d) overall UE Performance with information per MCG bearers, SCG bearers, and split bearers.