3GPP TSG RAN3 meeting #125bis  	         	                                 R3-245617

14th – 18th Oct 2024  Hefei, China

Agenda item: 	    10.2

Source: 	China Unicom

Title: 	Discussion on MRO enhancements

Document for:	Discussion

1	Introduction

At RAN#102, the new Work Item on Data Collection for SON_MDT in NR standalone and MR-DC phase 4 was agreed [1] which introduced the following objectives in the core part:

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure If needed, co-operate with RAN1, SA2, SA5, CT4.

This contribution provides our views on the scenarios of MRO enhancements which can be supported in Rel-19.

2	Discussion

2.1	LTM

Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover. FFS to enhance failure scenario. Take the existing detection mechanism descriptions as baseline. For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure. CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed.

At the RAN3#125 meeting, we have been made above agreements and introduced stage-2 definition of LTM failure cases.

Due to time constraints, we didn't discuss RACH-less and RACH-based access related topic at the last meeting, following the agreement from RAN3#124 meeting stated below.

RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

Regarding enhancements related to the information of RACH-less access and RACH-based access for LTM, it will be helpful for network to distinguish whether UE perform RACH-less LTM or RACH-based LTM for further optimization which can be included in the RLF report and SHR report.

Proposal 1: Introduce an indication indicating the RACH-based or RACH-less in RLF report and SHR report.

2.2	CHO with candidate SCGs

In last RAN3#125 meeting, we have the following agreements and FFS to be further discussed.

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed.

MRO for case 4/5/6 is in the scope.

FFS whether there is any specification impact for case 4/5/6.

Case 1c, 2c, 2c, 3c, 7c, 8c are FFS.

The RLF report includes

- The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met.

- Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered

- FFS whether to optimize the association of the PCell and PSCell.

In terms of the failure scenarios on MRO for CHO with candidate SCGs, as illustrated in paper R3-244223 listing 9 failure cases for CHO with candidate SCGs. While we validated some of the failure scenarios with some sub-case need further discussion. As in case 1c, 2c, 3c, 7c, 8c, which is a combined concurrent failure scenario that a MCG failure happens then a SCG failure followed. We think that such a complex failure case can be discussed after the MCG failure case is confirmed if time allows, which is rather a corner and severe case in the network. RAN3 can firstly focus on case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9.

| Cases | Figure: CHO with candidate SCG(s) is configured | Brief Description |
| --- | --- | --- |
| Case 1 |  | Failure happens before CHO and/or CPAC execution. 1a) RLF@srcPCell 1b) SCG failure 1c) MCG failure+SCG failure |
| Case 2 |  | CPC/CPA execution condition is met. Failure happens before CHO execution condition is met. 2a) RLF@srcPCell 2b) SCG failure 2c) MCG failure + SCG failure |
| Case 3 |  | CHO execution condition is met. No CHO only or CHO with SCG configuration. Failure happens before CPAC execution condition is met. 3a) RLF@srcPCell 3b) SCG failure 3c) MCG failure + SCG failure |
| Case 4 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. CHO execution to a candidate PCell fails. |
| Case 5 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. RLF shortly after CHO execution success |
| Case 6 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. SCG failure shortly after CHO execution success |
| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| Case 8 |  | Failure happens shortly after CHO and CPAC execution success 8a) RLF@targetPCell 8b) SCG failure 8c) MCG failure + SCG failure |
| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |

Proposal 2: Case 1c, 2c, 2c, 3c, 7c, 8c can be discussed after stabilizing MRO for MCG failure and SC failure only.

As MRO for CHO and MRO for CPAC are supported in previous release, which can be  considered as base line for R19 MRO for CHO with candidate SCG(s). RAN3 can reuse RRC recovery procedure, RRC reestablishment procedure, or RRC re-connected procedure and SCGFailureInformation message for R19 MRO for CHO with candidate SCG(s). To distinguish it from the previous cases, new IE can be introduced referring to the previous spec design.

Proposal 3: It is proposed to reuse same procedure and message for R19 MRO for CHO with candidate SCG(s) from MRO for CHO and MRO for CPAC.

2.3	Subsequent CPAC

Reusing SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN. For the CPAC failures which occurred during an S-CPAC procedure, CPC Execution above refers to the initial CPC execution or the subsequent CPC execution. Editor’s note: FFS whether the differentiation between initial and following CPC is needed.

A conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change, or a SCG release, based on pre-configured subsequent CPAC configuration of candidate PSCell(s). UE would keep the subsequent CPAC configuration after a PSCell change, and if the execution condition of other one candidate PSCell is satisfied, UE may apply the stored configuration and synchronises to the candidate PSCell.

In that case, subsequent CPAC failure may occur at the first configured SN modification, as well as subsequent PSCell changes scenario. When subsequent CPAC fails, SCG FAILURE INFORMATION REPORT can be further enhanced to record the corresponding configurations for root cause analysis. Meanwhile, RAN3 needs to discuss how to trigger the forwarding of failure report to the configured node with the potential impacts on UE recording the mobility history information.

And regarding the differentiation between initial and following CPC, we don’ t see the strong intention to separate the two case.

Proposal 4: There is no need to differentiate between initial and following CPC.

For subsequent CPAC, network cannot log the UHI since no more SN adding procedure during the mobility phase based on the existing UHI mechanism,

Proposal 5: UHI mechanism can be enhanced for subsequent CPAC.

3	Conclusion

Proposal 1: Introduce an indication indicating the RACH-based or RACH-less in RLF report and SHR report.

Proposal 2: Case 1c, 2c, 2c, 3c, 7c, 8c can be discussed after stabilizing MRO for MCG failure and SC failure only.

Proposal 3: It is proposed to reuse same procedure and message for R19 MRO for CHO with candidate SCG(s) from MRO for CHO and MRO for CPAC.

Proposal 4: There is no need to differentiate between initial and following CPC.

Proposal 5: UHI mechanism can be enhanced for subsequent CPAC.

4   References

- RP-234038: New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4; TSG RAN Meeting #102, Edinburgh, Scotland, Dec 11-15, 2 023