3GPP TSG-RAN WG3 Meeting #125-bis	R3-245621

Hefei , CN, 14 - 18 Oct, 2024

Title:	(TP for MDT BLCRs for TS38.413, TS38.423, TS 37.320) SON for Intra-NTN mobility

Agenda Item:	10.3.1

Source:	Huawei

Document for:	Discussion

1 Introduction

The following progress were made at RAN3 #124 meeting for SON for intra-NTN mobility.

Take the connection failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point with necessary update, if needed, in MRO for intra-NTN mobility.

MRO for intra-NTN mobility shall consider the following trigger conditions:

- Measurement-based trigger condition;

- A time-based trigger condition;

- A location-based trigger condition;

- A time-based and Measurement-based combined trigger condition;

- A location-based and Measurement-based combined trigger condition;

Do not consider MRO for feeder link switch.

MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover.

FFS on whether there are any enhancements to RLF report due to support for multiple trigger conditions in intra-NTN mobility.

FFS on whether to consider any enhancement for MDT configuration e.g., area scope and trigger condition for NTN.

Discuss MRO for RACH-less HO of NTN, only for those different issues identified compared with MRO for LTM.

Time information when RLF occurs needs to be included in the RLF report, other information to be included can be checked as well in next meeting.

Further progress was made at RAN3 #125 meeting,

MRO for CHO with a time-based trigger condition:

Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell.

- Distinguish if the RLF happens before or after T1

Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

- FFS on whether it matters if RLF happens within the duration or after the duration period.

The RLF report includes for single time-based trigger condition:

-      Absolute time (measured UTC time) when RLF occurred

-      Measured UTC time when CHO is executed

Whether to consider the fallback issue from RACH-less HO to RACH-based HO for NTN?

Whether to introduce any enhancement for the Area Scope of logged MDT for NTN over NG interface?

In this contribution, we continue on this topic and present our further proposals.

2 Discussion

2.1 MRO for intra-NTN mobility

Time-based trigger condition:

The following agreements are made at last meeting:

Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell.

- Distinguish if the RLF happens before or after T1

Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

- FFS on whether it matters if RLF happens within the duration or after the duration period.

The RLF report includes for single time-based trigger condition:

-      Absolute time (measured UTC time) when RLF occurred

-      Measured UTC time when RLF CHO is executed

Regarding the FFS on case 2, whether it matters if RLF happens within the duration or after the duration period, since case 2 is about too early CHO execution or CHO execution to wrong cell, meaning that the UE has already executed the CHO towards the target cell. The network may identify the failure use case and decide how to tune the T1 without such information. And even for the RLF shortly after successful CHO, the UE may already discard the CHO configuration.

Proposal 1: There is no need to distinguish whether the RLF happens within the duration or after the duration.

There were some discussions at last meeting on UE reporting absolute UTC time when CHO is executed and when RLF is occurred to enable the network distinguish some sub use cases. The received comments were that the network may deduce such absolute time by using existing relative timers in the UE RLF report, e.g., the timeSinceCHO-Reconfig, and the timeConnFailure. Technically, this is feasible if assuming the network will record the absolute time of the sending the CHO configuration to the UE. However, this brings a new requirement to the network implementation.

For the sake of progress, we may accept and go with the majority views on this issue.

Proposal 2: Reusing existing relative timers (e.g., the timeSinceCHO-Reconfig, and the timeConnFailure.) in the UE RLF report for the network to identify the use cases when time based CHO is configured.

Location based trigger condition:

Figure 1 shows the possible RLF/HOF happening places after the location-based trigger condition is configured to the UE.

Figure 1: Failure scenarios in location-based condition

- Case 1: RLF happens in the source cell before D1/D2 is triggered. The UE re-establishes in the target cell.

- Case 2: HOF happens in the target cell after D1/D2 is triggered. The UE re-establishes in the source cell or a third cell.

- Case 3: RLF happens in the target cell short after successful HO. The UE re-establishes in the source cell or a third cell.

Case 1 is a typical too-late CHO. In this case, neither of or not both the trigger conditions are fulfilled.

Case 2 and 3 imply either a too-early CHO or a wrong-cell CHO depending on the re-establishment cell.

In order for the network to identify the failure types and perform root cause analysis, the following additional information related to distance measurement needs to be reported in the UE RLF report:

- The measured distanceFromReference1 when the condition to the candidate cell is fulfilled, or the measured distanceFromReference2 when the condition to the source cell is fulfilled. This may help the network to adjust the threshold of the unfulfilled condition in D1 and D2.

- The indicator to indicate which threshold, the distanceThreshFromReference1 or distanceThreshFromReference2 is fulfilled.

- The time duration between the two thresholds fulfilled. This will imply the extent how the thresholds are set improperly.

With above information, the network is able to determine the failure causes of the location-based CHO and further to tune either one or both distance thresholds in D1/D2.

There were views that no any distance measurement is needed as the locationInfo-r16 is already included in the RLF report. The network may calculate the distance between the UE and the reference location with the locationInfo-r16. This is true when D1 is configured. But the network implementation would be quite difficult and complex if D2 is configured in earth moving case. Because the network needs to deduce the absolute time firstly of the RLF occurred based on the received RLF report , and then determine the location of the satellite based on the satellite ephemeris.  In the third step, the distance to the reference location can be calculated out.

While reporting the measured distance information when RLF occurs does not need any effort for the UE, as those measurements are already available in the UE. Therefore, it is proposed:

Proposal 3: The following information needs to be collected into the UE RLF report for the location based CHO optimisation:

- An indicator to indicate which threshold is fulfilled, the distanceThreshFromReference1, the distanceThreshFromReference2, or neither.

- The measured distanceFromReference1 when the condition to the candidate cell is fulfilled, or the measured distanceFromReference2 when the condition to the source cell is fulfilled.

- The time duration between the two thresholds fulfilled

RRM-based trigger condition:

The analysis of RRM-based CHO can be carried out in a similar way as above.

- Case 1: RLF happens in the source cell before A3/A4/A5 is triggered. The UE re-establishes in the target cell.

- Case 2: HOF happens in the target cell after A3/A4/A5 is triggered. The UE re-establishes in the source cell or a third cell.

- Case 3: RLF happens in the target cell short after successful HO. The UE re-establishes in the source cell or a third cell.

Case 1 is a typical too-late CHO. Case 2 and 3 imply either a too-early CHO or a wrong-cell CHO depending on the reestablishment cell. In order for the network to identify the failure types and perform root cause analysis, the following RRM-related information needs to be reported in the UE RLF report:

- RRM measurement results when A3/A4/A5 is satisfied (already contained in the RLF report).

- Specifically, for A5, the indicators that indicate whether threshold1 and threshold2 are satisfied, respectively.

- Proposal 3: The indicators that indicate whether threshold1 and threshold2 for A5 shall be included in RLF report for optimization of RRM-based CHO.

Joint trigger condition:

Figure 2: Failure scenarios in joint condition CHO

As illustrate in Figure 3, the failure scenarios for joint trigger condition could be:

- Case 1: RLF happens in the source cell when only one condition is satisfied, or neither is satisfied. The UE re-establishes in the target cell.

- Case 2: HOF happens in the target cell after both conditions are triggered. The UE re-establishes in the source cell or a third cell.

- Case 3: RLF happens in the target cell short after successful HO. The UE re-establishes in the source cell or a third cell.

Case 1 is a typical too-late CHO since at least one condition is set too hard to be reached. Case 2 and 3 imply either a too-early CHO or a wrong-cell CHO depending on the reestablishment cell. In order for the network to identify the failure types and perform root cause analysis, the following conditional information needs to be reported in the UE RLF report:

Both

- The time duration between the two conditions fulfilled (for case 2 and case3). This will imply the extent how the thresholds are set improperly. The existing timeBetweenEvents -r17 in the MeasResultNR can be reused.

- Time+RRM

- For case 1 handover too late, the following information from UE RLF report can help the network to determine the range to be adjusted for the unfulfilled event.

- The measured UTC time when the RRM condition are fulfilled.

- The RRM measurement results when the time condition is fulfilled.

- The fulfilled event.

- For case 2 and case 3, The existing timeBetweenEvents -r17 and the firstTriggeredEvent-r17 in the MeasResultNR can be reused to determine which event or both events are the main reason to cause the RLF.

- Location+RRM

- For case 1 handover too late, the following information from UE RLF report can help the network to determine the range to be adjusted for the unfulfilled event.

- The RRM measurement results when the location conditions are fulfilled.

- The measured distance (with serving cell and neighbour cell) when the RRM condition are fulfilled.

- The fulfilled event.

- For case 2 and case 3, The existing timeBetweenEvents -r17 and the firstTriggeredEvent-r17 in the MeasResultNR can be reused to determine which event or both events are the main reason to cause the RLF.

In summary, it is proposed:

- Proposal 4: For joint-condition CHO, at least the following information shall be reported in RLF report.

  - measured UTC time in T1 or distance in D1/D2 when the RRM condition is fulfilled;

  - RRM measurement results when the time condition or the location condition is fulfilled.

  - The fulfilled event.

2.2 MDT enhancements for intra-NTN mobility

Whether to introduce any enhancement for the Area Scope of logged MDT for NTN over NG interface?

There were proposals in past meetings to introduce a geographical area based area scope especially for logged MDT. We also see some benefits to have the geographical area based area scope, as for earth moving cell case, the legacy area scope becomes not fixed anymore. Although as specified in section 16.14 in TS 38.300, the tracing area in NTN corresponding to a fixed geographical area, it is not applicable to idle and inactive mode UEs. As the UE does not know the exact tracking area that it locates in an NTN cell. Another usage of the geographical area is to avoid the UE logging in a different country, but within the same NTN cell.

The definition of the geographical area could be a reference location plus a radius to be in line with RRC protocol.

- Proposal 5: RAN3 to agree to introduce a geographical area as the area scope for MDT in NTN.

Legacy logged MDT enables the UE to perform periodic and event triggered measurement logging in idle and inactive state. The main purpose is to produce a coverage map for the configured area scope and further evaluate the potential coverage issues.

In order to produce the coverage map of an NTN network, it is beneficial to introduce new trigger events for logged MDT, including time based trigger event and  location based trigger event.

Furthermore, to assist the network to optimize the measurement conditions through UE’s logged MDT, RAN3 should also discuss the contents of the UE’s logged MDT records. For example, the UE may at least additionally record the measured distance if the trigger condition is location based and the reference location.

- Proposal 6: RAN3 to agree to introduce time based and location based trigger events for logged MDT, and the additional measurements to record: the measured distance and the reference location.

- Proposal 7: RAN3 to send a LS to inform RAN2 about the progress on this topic.

3 Conclusion

This document proposes:

Proposal 1: There is no need to distinguish whether the RLF happens within the duration or after the duration.

Proposal 2: Reusing existing relative timers (e.g., the timeSinceCHO-Reconfig, and the timeConnFailure.) in the UE RLF report for the network to identify the use cases when time based CHO is configured.

Proposal 3: The indicators that indicate whether threshold1 and threshold2 for A5 shall be included in RLF report for optimization of RRM-based CHO.

Proposal 4: For joint-condition CHO, at least the following information shall be reported in RLF report.

- measured UTC time in T1 or distance in D1/D2 when the RRM condition is fulfilled;

- RRM measurement results when the time condition or the location condition is fulfilled.

- The fulfilled event.

- Proposal 5: RAN3 to agree to introduce a geographical area as the area scope for MDT in NTN.

- Proposal 6: RAN3 to agree to introduce time based and location based trigger events for logged MDT, and the additional measurements to record: the measured distance and the reference location.

- Proposal 7: RAN3 to send a LS to inform RAN2 about the progress on this topic.

The draft TPs for MDT BLCRs for TS38.413, TS38.423, TS 37.320 are provided in the annex.

The drat LS to RAN2 is also provided in the annex.

Annex – TP for MDT BLCR for TS38.413

/*********************Start of changes************************************/

9.3.1.169	MDT Configuration-NR

This IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …) |  | - |  |
| CHOICE Area Scope of MDT | M |  |  |  | - |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | - |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TA List for MDT |  | 1..<maxnoofTAforMDT> |  |  | - |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - |  |
| >PLMN wide |  |  | NULL |  |  |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | - |  |
| >>>TAI | M |  | 9.3.3.11 |  | - |  |
| >PNI-NPN Based MDT |  |  |  |  | YES | ignore |
| >>CAG List for MDT |  |  | 9.3.3.65 |  | - |  |
| >SNPN Cell Based MDT |  |  |  |  | YES | ignore |
| >>SNPN Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | - |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity in the NR CGI IE. | - |  |
| >SNPN TAI Based MDT |  |  |  |  | YES | ignore |
| >>SNPN TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | - |  |
| >>>TAI | M |  | 9.3.3.11 |  | - |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity in the TAI IE. | - |  |
| >SNPN Based MDT |  |  |  |  | YES | ignore |
| >>SNPN List for MDT |  | 1..<maxnoofMDTSNPNs> |  |  | - |  |
| >>>PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity IE. | - |  |
| >GeoLocation Based MDT |  |  |  |  | YES | ignore |
| >>Reference Location | M |  | OCTET STRING | This IE is used as a reference location and includes a Ellipsoid-Point IE as defined in TS37.355 [x]. | - |  |
| >> Distance Radius | M |  | INTEGER(0..65535) | Distance from the reference location. | - |  |
| CHOICE MDT Mode | M |  |  |  | - |  |
| >Immediate MDT |  |  |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [41]. First Bit = M1, Second Bit= M2, Third Bit = M4, Fourth Bit = M5, Fifth Bit = M6, Sixth Bit = M7, Seventh Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, other bits reserved for future use. Value “1” indicates “activate” and value “0” indicates “do not activate”. | - |  |
| >>M1 Configuration | C-ifM1 |  | 9.3.1.171 |  | - |  |
| >>M4 Configuration | C-ifM4 |  | 9.3.1.172 |  | - |  |
| >>M5 Configuration | C-ifM5 |  | 9.3.1.173 |  | - |  |
| >>M6 Configuration | C-ifM6 |  | 9.3.1.174 |  | - |  |
| >>M7 Configuration | C-ifM7 |  | 9.3.1.175 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>MDT Location Information | O |  | 9.3.1.176 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >Logged MDT |  |  |  |  |  |  |
| >>Logging Interval | M |  | ENUMERATED (320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms, 20480ms, 30720ms, 40960ms, 61440ms, infinity, …) | Corresponds to the LoggingInterval IE as defined in TS 38.331 [18]. | - |  |
| >>Logging Duration | M |  | ENUMERATED (10, 20, 40, 60, 90,120, …) | Corresponds to the LoggingDuration IE as defined in TS 38.331 [18]. Unit: [minute]. | - |  |
| >>CHOICE Report Type | M |  |  |  | - |  |
| >>>Periodical |  |  | NULL |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Event Trigger Logged MDT Configuration | M |  | 9.3.1.180 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >>Area Scope of Neighbour Cells | O |  | 9.3.1.182 |  | - |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [18]. | YES | ignore |
| Signalling Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 |  | - |  |
| PNI-NPN Area Scope of MDT |  | 0..1 |  | This IE is ignored if the Area Scope of MDT IE is set to "PLMN Wide" | YES | ignore |
| >CAG List for MDT | M |  | 9.3.3.65 |  | - |  |

/*********************End of changes************************************/

Annex – TP for MDT BLCR for TS38.423

/*********************Start of changes************************************/

9.2.3.126	MDT Configuration-NR

The IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Immediate MDT and Trace, Logged MDT only, ...) |  | – |  |
| CHOICE Area Scope of MDT-NR | O |  |  |  | – |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>Cell ID List for MDT-NR |  | 1 .. <maxnoofCellIDforMDT> |  |  | – |  |
| >>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  | – |  |
| >>>TAC | M |  | 9.2.2.5 | The TAI is derived using the current serving PLMN. | – |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TAI List for MDT |  | 1 |  |  | – |  |
| >>>TAI List for MDT Item |  | 1 .. <maxnoofTAforMDT> |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| >>>>TAC | M |  | 9.2.2.5 |  | – |  |
| >PNI-NPN Based MDT |  |  |  |  | YES | ignore |
| >>CAG List for MDT |  |  | 9.2.3.191 |  | – |  |
| >SNPN Cell Based MDT |  |  |  |  | YES | ignore |
| >>SNPN Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | – |  |
| >>>NR CGI | M |  | 9.2.2.7 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity in the NR CGI IE. | – | - |
| >SNPN TAI Based MDT |  |  |  |  | YES | ignore |
| >>SNPN TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | – | - |
| >>>PLMN Identity | M |  | 9.2.2.4 |  | – | - |
| >>>TAC | M |  | 9.2.2.5 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity IE. | – | - |
| >SNPN Based MDT |  |  |  |  | YES | ignore |
| >> SNPN List for MDT |  | 1..<maxnoofMDTSNPNs> |  |  | – | - |
| >>>PLMN Identity | M |  | 9.2.2.4 |  | – | - |
| >>>NID | M |  | 9.2.2.65 | Identifies an SNPN together with the PLMN Identity IE. | – | - |
| >GeoLocation Based MDT |  |  |  |  | YES | ignore |
| >>Reference Location | M |  | OCTET STRING | This IE is used as a reference location and includes a Ellipsoid-Point IE as defined in TS37.355 [x]. | - |  |
| >> Distance Radius | M |  | INTEGER(0..65535) | Distance from the reference location. | - |  |
| CHOICE MDT Mode | M |  |  |  | – |  |
| >Immediate MDT-NR |  |  |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [43]. First Bit = M1, Second Bit= M2, Fourth Bit = M4, Fifth Bit = M5, Sixth Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, Seventh Bit = M6, Eighth Bit = M7. Value "1" indicates "activate" and value "0" indicates "do not activate". This version of the specification does not use bits 3. | – |  |
| >>M1 Configuration | C-ifM1 |  | 9.2.3.128 |  | – |  |
| >>M4 Configuration | C-ifM4 |  | 9.2.3.129 |  | – |  |
| >>M5 Configuration | C-ifM5 |  | 9.2.3.130 |  | – |  |
| >>MDT Location Information | O |  | BITSTRING(SIZE(8)) | Each position in the bitmap represents requested location information as defined in TS 37.320 [43]. First Bit = GNSS Other bits are reserved for future use and are ignored if received. Value "1" indicates "activate" and value "0" indicates "do not activate". The eNB shall ignore the first bit unless the Measurements to Activate IE has the first bit or the sixth bit set to "1". | – |  |
| >>M6 Configuration | C-ifM6 |  | 9.2.3.131 |  | – |  |
| >>M7 Configuration | C-ifM7 |  | 9.2.3.132 |  | – |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.134 |  | – |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.135 |  | – |  |
| >>Sensor Measurement Configuration | O |  | 9.2.3.136 |  | – |  |
| >Logged MDT-NR |  |  |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960 , ms61440, infinity, ...) | Corresponds to information provided in the LoggingInterval IE as defined in TS 38.331 [10]. The value "infinity" represents one shot logging, i.e., only one log per event in the logged MDT report. | – |  |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90, 120) | Corresponds to information provided in the LoggingDuration IE as defined in TS 38.331 [10]. Unit: [minute]. | – |  |
| >>CHOICE Report Type | M |  |  |  | – |  |
| >>>Periodical |  |  |  |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Logged Event Trigger Config | M |  | 9.2.3.137 |  | – |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.134 |  | – |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.135 |  | – |  |
| >>Sensor Measurement Configuration | O |  | 9.2.3.136 |  | – |  |
| >>Area Scope of Neighbour Cells | O |  | 9.2.3.140 |  | – |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [10]. | – |  |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | – |  |
| PNI-NPN Area Scope of MDT |  | 0..1 |  |  | YES | Ignore |
| >CAG List for MDT | M |  | 9.2.3.191 |  | – |  |

/*********************End of changes************************************/

Annex – TP for MDT BLCR for TS37.320

/*********************Start of changes************************************/

5.1.1.1.1	Configuration parameters

The logged measurement configuration consists of:

-	configuration of downlink pilot strength measurements logging for (E-)UTRA and NR.

-	configuration of MBSFN measurement logging for E-UTRA.

-	configuration of the triggering of logging events:

-	for (E-)UTRAN:

-	periodic measurement trigger is supported, for which the logging interval is configurable. The parameter specifies the periodicity for storing MDT measurement results. It should be configured in seconds in multiples of the applied IDLE mode DRX, i.e. multiples of 1.28s which is either a factor or multiple of the IDLE mode DRX. The UE behaviour is unspecified when the UE is configured with a DRX cycle larger than the logging interval.

-	for NR:

-	periodic measurement trigger is supported, for which the logging interval is configurable. The parameter specifies the periodicity for storing MDT measurement results.

-	Event measurement trigger is supported, for which the logging interval is configurable. Two types of events are supported: time based event, and location based event.

-	for E-UTRAN and NR:

-	event-based trigger is supported, for which the logging interval is configurable, which determines periodical logging of available data (e.g. time stamp, location information), and the following two types of events are supported:

-	measurement quantity-based event L1, for which the event threshold, hysteresis, and time to trigger are configurable. If the configured time to trigger is not a multiple of the DRX cycle, then the UE uses the next multiple of DRX cycle duration that is larger than the time to trigger for evaluating the event L1;

-	out-of-coverage detection trigger.

NOTE:	The logging configuration for event-based and periodical DL pilot strength logged measurements can be configured independently. Only one type of event can be configured to the UE.

-	configuration of the logging duration. This configuration parameter defines a timer activated at the moment of configuration, that continues independent of state changes, RAT or RPLMN change. When the timer expires the logging is stopped and the configuration is cleared (except for the parameters that are required for further reporting e.g. network absolute time stamp, trace reference, trace recording session reference and TCE Id).

-	network absolute time stamp to be used as a time reference to UE.

-	Trace Reference parameter as indicated by the OAM configuration as specified in TS 32.422 [6].

-	Trace Recording Session Reference as indicated by the OAM configuration as specified in TS 32.422 [6].

-	TCE Id as indicated by the OAM configuration as specified in TS 32.422 [6].

-	(optionally) MDT PLMN List, indicating the PLMNs where measurement collection and log reporting is allowed. It is either the Management Based MDT PLMN List or the Signalling Based MDT PLMN List, depending on how the Logged MDT task was initiated (see 5.1.3).

-	(optionally) configuration of a logging area. A UE will log measurements as long as it is within the configured logging area. The scope of the logging area may consist of one of:

-	a list of up to 32 global cell identities for PLMN, and, for NR, additionally a list of up to 144 PNI-NPNs with a maximum of 12 different PLMN identities. If one or both of these lists are configured, the UE will only log measurements when camping in any of the cells belonging to the list of global cell identities, or in any of the cells belonging to the listed PNI-NPNs.

-	a list of up to 8 TAs or 8 LAs or 8 RAs for PLMN, and, for NR, additionally a list of up to 256 PNI-NPNs. If one or both of these lists are configured, the UE will only log measurements when camping in any cell belonging to the preconfigured TA/LA/RAs, or in any of the cells belonging to the listed PNI-NPNs.

-	for NR, a list of inter-frequency neighbouring cells per frequency.

-	for NR, a list of up to 144 PNI-NPNs with a maximum of 12 different PLMN identities.

-	for NR, a list of up to 16 SNPNs.

-	for NR, a list of up to 32 global cell identities for SNPN. If this list is configured, the UE will only log measurements when camping in any of these cells.

-	for NR, a list of up to 8 TAs for SNPN. If this list is configured, the UE will only log measurements when camping in any cell belonging to the configured TAs.

-	for NR, a Geographical location area. If this geographical location area is configured, the UE ill only log measurements when comping in any cell belongs to the configured location area.

-	The configured logging area can span one of:

-	PLMNs in the MDT PLMN List. If no area is configured, the UE will log measurements throughout the PLMNs of the MDT PLMN list.

-	Any configured SNPN area.

-	(optionally) for NR, configuration of a list of neighbouring frequencies and/or cells, indicating the UE to include neighbouring cell's measurements as indicated in the list in the logged MDT report.

-	(optionally) for E-UTRA, configuration of target MBSFN area(s) for MBSFN measurement logging. If target MBSFN area(s) is configured, UE applies it in addition to other restrictions such as the logging area. The UE will log measurements as long as it receives MBMS service from an indicated target MBSFN area and is within the configured logging area. The target MBSFN area(s) is defined by a list of up to 8 entries, where each entry indicates a carrier frequency and optionally indicates a specific MBSFN area on a carrier frequency.

-	(optionally) configuration of the WLAN access point names, indicating the UE to attempt to obtain WLAN measurements associated to these access points.

-	(optionally) configuration of the Bluetooth beacon names, indicating the UE to attempt to obtain Bluetooth measurements associated to these beacons.

-	(optionally) for NR, configuration of the sensor names, indicating the UE to attempt to obtain sensor measurements.

-	(optionally) for E-UTRA, configuration indicating the UE to attempt to obtain uncompensated barometric pressure measurements.

-	(optionally) for NR, the network can use a flag to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes, indicating the UE is allowed to log the measurement results related to early measurement frequencies in the logged MDT report.

-	(optionally) for NR and E-UTRA, logged MDT type flag, indicating the logged measurement configuration is the signalling based MDT (see 5.4.0).

Annex - Draft LS

3GPP TSG-RAN WG3 Meeting #125-bis	TDoc <TDoc#>

Hefei , CN, 14 - 18 Oct, 2024

Title:	[DRAFT] LS on SON for intra-NTN mobility

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2

Cc:	SA5

Contact person:	Zhang Hongzhuo

Zhanghongzhuo(at)Huawei(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN3 discussed the support of SON for intra-NTN mobility. In order for MRO analysis in network, RAN3 thinks that it is beneficial to enhance the UE RLF report with following information.

For location based CHO,

- An indicator to indicate which threshold is fulfilled, the distanceThreshFromReference1, the distanceThreshFromReference2, or neither.

- The measured distanceFromReference1 when the condition to the candidate cell is fulfilled, or the measured distanceFromReference2 when the condition to the source cell is fulfilled.

- The time duration between the two thresholds fulfilled.

For time and RRM based CHO, or location and RRM based CHO,

- The measured UTC time in T1

- The measured distance in D1/D2 when the RRM condition is fulfilled;

- RRM measurement results when the time condition or the location condition is fulfilled.

- The fulfilled event.

RAN3 also discussed the potential enhancement to MDT feature in NTN. RAN3 made the following agreements which may have RRC impact:

- introduce a geographical area as the area scope for MDT in NTN (i.e., a reference location plus a radius).

- introduce time based and location based trigger events for logged MDT, and the additional measurements to record: the measured distance and the reference location.

2	Actions

To RAN2

ACTION: 	RAN3 kindly asks RAN2 to take above RAN3 conclusions into account and feedback if any.

3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#126	2024-11-18 – 2024-11-22		Orlando, US

RAN3#127	2025-02-17 - 2025-02-21		Athens, GR