3GPP TSG-RAN WG3 Meeting #125-bis	R3-245828

Hefei , CN, 14 - 18 Oct, 2024

Title:	(TP to TR 38.769) A-IoT RAN Architecture aspects

Agenda Item:	16.2

Source:	Huawei

Document for:	other

- Introduction

This paper provides the TP based on the discussion during RAN3#125 meeting on RAN Architecture apects.

- Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

6.4	RAN architecture aspects

This clause attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

This chapter also attempts to identify a functional split between RAN and CN.

The logical system architecture for A-IoT consists of the following architectural elements:

A-IoT device:	Equipment with characteristics outlined e.g. in TS 22.369 [10].

Editor’s Note 3: Further details FFS, if any.

A-IoT RAN:	Hosts certain functions for A-IoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding A-IoT functions hosted in the A-IoT RAN and the respective functional split to be decided by RAN2, RAN3 and SA2.

A-IoT radio:	Radio interface between A-IoT device and A-IoT RAN node in topology 1 and between A-IoT device and A-IoT-enabled UE in topology 2.

Editor’s Note 5: Further details on A-IoT radio to be discussed by RAN1 and RAN2.

A-IoT CN:	Hosts certain functions for A-IoT as of the functional split between RAN and CN.

NOTE: the details of A-IoT CN are subject to SA2.

Editor’s Note 6: Further details regarding A-IoT functions hosted in the A-IoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface:	Interface between the A-IoT RAN and the A-IoT CN on which certain A-IoT specific functions are performed.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between A-IoT RAN and A-IoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between A-IoT CN and A-IoT RAN, or one instance for NG alone.

Common reader function:	A function that communicates with the A-IoT device by means of A-IoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

A-IoT RAN node function:	A function that contains e.g. the control of the A-IoT radio resources used towards the A-IoT device.

Editor’s Note 9: further details are FFS. Note that “control of A-IoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function may need further discussion.

6.4.1	Support of Topology 1

Figure 6.4.1-1 depicts a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN. The A-IoT RAN consists of one or multiple Readers.

Figure 6.4.1-1 Logical system architecture for topology 1

For Toploogy 1, architecture and protocol aspects of split RAN architecture are not studied.

In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface.

Figure 6.4.1-2 shows the Protocol stack for Topology 1, assuming a SCTP-based transport:

Figure 6.4.1-2. Protocol Stack for Topology 1

Editor’s Note 1: The protocol stack in Figure 6.4.1-2 does not illustrate how A-IoT upper layer information, if any, is transported over XXAP. Details are subject to SA2 agreements.

For topology 1, the XXAP is terminated at an A-IoT RAN node.

The signalling transport for XXAP at the AIoT RAN node is SCTP/IP. Other options of signalling transport for XXAP at the A-IoT RAN node (e.g. HTTP/2/TLS/TCP) were discussed, but will not be pursued.

Editor’s Note 3: The A-IoT CN may include AMF and A-IoT related functions which is up to SA2 decision.

In Topology 1, an A-IoT RAN node may serve one or more readers.

The A-IoT RAN node should enable the coordination of the usage of the A-IoT radio resources among readers.

Reader selection may need coordination between A-IoT RAN node and A-IoT CN.

6.4.2	Support of Topology 2

Figure 6.4.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB.

The following definitions apply:

A-IoT-enabled gNB:	A gNB supporting A-IoT RAN node function in topology 2, which is able to communicate with the A-IoT enabled UE via NR Uu interface.

A-IoT-enabled UE:	A UE supporting Common reader function, which is able to communicate with the A-IoT device via the A-IoT radio interface.

Figure 6.4.2-1 Logical system architecture for topology 2

Editor’s Note 1:	Figure 6.4.2-1 doesn’t illustrate the protocol between A-IoT enabled UE and A-IoT CN, if needed, the figure needs to be revised in case such is defined by SA2.

Editor’s Note 2:	In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS.

Editor’s Note 3:	The A-IoT CN could include AMF and A-IoT related functions, which is up to SA2 decision.

Editor’s Note 4:	The A-IoT enabled gNB performs radio resource management for A-IoT related radio resources, details are pending on RAN1 and RAN2 mechanisms.

Editor’s Note 5:	In Topology 2, it is FFS whether and in which way the A-IoT CN and the A-IoT enabled gNB are involved in the selection of the readers.

In Topology 2, the RAN architecture should enable the coordination of the usage of the A-IoT radio resources among readers.

An AIoT-enabled gNB could support both topology 1 and topology 2, this is an implementation matter.

6.4.2.1	Solutions for Topology 2

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

Editor’s note 1: How to enable radio resource control, i.e. trigger A-IoT RAN node functions for above solutions is FFS.

Editor’s note 2: Depiction and further details of the options above are FFS

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>