3GPP TSG-RAN WG3 Meeting #126	R3-247142

Orlando, USA, 18-22 Nov 2024

Agenda Item:	11.4

Source:	ZTE Corporation

Title:	(TP to 38.473, 37.483) Discussion on split architecture

Document for:	Discussions & Approval

1	Introduction

This contribution aligns with the objectives [1] of the Rel-19 WI on 'Enhancements for AI/ML in NG-RAN.' Based on the conclusions and progress made in Rel-18 Leftovers, as captured in TR 38.743 [2], we present our analysis of the normative work for NR-DC Mobility Optimization, Support of split architecture, and Continuous MDT Collection.

The detailed objectives of the WI are listed as follows:

- Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing and AI/ML based CCO. [RAN3]

- Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]:

  - Mobility Optimization for NR-DC

  - Split architecture support for Rel-18 use cases

  - Continuous MDT collection targeting the same UE across RRC states

Note: Coordination with RAN2, SA5 when needed if any.

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743. Additionally, the corresponding TP is included.

2	Discussion

2.1 Measured EC over F1 interface

It was concluded in TR 38.743 [2] that Measured Energy Cost (EC) is reported upon request from the gNB-DU to the gNB-CU. In reviewing progress on the Xn interface, it was agreed to introduce new Class 1 and Class 2 procedures to facilitate the transfer of AI/ML-related information, which includes predicted resource status, UE performance feedback, measured EC, and more.

To differentiate between the existing resource status procedure and the one used for AI/ML data collection, we propose introducing a new process specifically for reporting the measured EC over F1AP. Following the subscription principle of measured EC, gNB-CU can request gNB-DU to report its measured EC, and then gNB-DU can report the measured EC via one-time reporting or periodically reporting if configured successfully.

- Introduce the new class 1 procedure Data Collection Reporting Initiation procedure over F1AP used by gNB-CU to request from gNB-DU the reporting of information to support, e.g., AI/ML in NG-RAN. The procedure applies the same reporting mechanisms of resource status reporting initiation procedure.

- Introduce the new class 2 procedure Data Collection Reporting procedure over F1AP to report information accepted by the gNB-DU. The procedure applies the same reporting mechanisms of resource status report procedure.

- Using the new class 1& class 2 procedure to report the measured EC from gNB-DU to gNB-CU.

The encoding format of measured energy cost over F1 interface shall follow the agreement of measured energy cost over Xn interface.

- Define the Energy Cost IE over F1 interface as an INTEGER (0..10000, ...).

2.1 UE performance feedback over E1 and F1

Following is the conclusion on UE performance feedback:

The details of signaling measured UE performance metrics from gNB-DU to gNB-CU-CP and/or from gNB-CU-UP to gNB-CU-CP need further discussion during normative phase.

We need to discuss the types of UE performance metrics that should be reported over the F1 and E1 interfaces. Currently, the UE performance metrics agreed upon during the Rel-18 phase include average UE throughput (UL/DL), average packet delay (UL/DL), and average packet loss (DL).

As outlined in TS 28.558 and TS 38.314, the average UE throughput (UL/DL) and average packet delay (DL) are measured and calculated at the gNB-DU.

- Agree to report the average UE throughput (UL/DL) and average packet loss from the gNB-DU to the gNB-CU-CP through a new procedure over the F1 interface.

However, regarding packet delay, it can be seen from the TS 38.314 that some portions of the packet delay can be measured by the DU, while others can be measured by the gNB-CU-UP.

Packet delay includes RAN part of delay and CN part of delay.

The RAN part of DL packet delay measurement comprises:

-	D1 (DL delay in over-the-air interface), referring to Average delay DL air-interface in TS 28.552 [2] 5.1.1.1.1.

-	D2 (DL delay on gNB-DU), referring to Average delay in RLC sublayer of gNB-DU in TS 28.552 [2] 5.1.3.3.3.

-	D3 (DL delay on F1-U), referring to Average delay on F1-U in TS 28.552 [2] 5.1.3.3.2.

-	D4 (DL delay in CU-UP), referring to Average delay DL in CU-UP in TS 28.552 [2] 5.1.3.3.1.

The DL packet delay measurements, i.e. D1 (the DL delay in over-the-air interface ), D2 (the DL delay in gNB-DU), D3 (the DL delay on F1-U) and D4 (the DL delay in CU-UP), should be measured per DRB per UE.

NOTE:	The delay measurements D1, D2 and D4 are also applicable for EUTRA in case of EN-DC related DL delay measurements on the MN side.

The RAN part (including UE) of UL packet delay measurement comprises:

-	D1 (UL PDCP packet average delay, as defined in clause 4.3.1.1).

-	D2.1 (average over-the-air interface packet delay, as defined in 4.2.1.2.2).

-	D2.2 (average RLC packet delay, as defined in 4.2.1.2.3).

-	D2.3 (average delay UL on F1-U, it is measured using the same metric as the average delay DL on F1-U defined in TS 28.552 [2] clause 5.1.3.3.2).

-	D2.4 (average PDCP re-ordering delay, as defined in 4.2.1.2.4).

The UL packet delay measurements, i.e. D1(UL PDCP packet average delay), D2.1(average over-the-air interface packet delay), D2.2(average RLC packet delay), D2.3(average delay UL on F1-U) and D2.4(average PDCP re-ordering delay), should be measured per DRB per UE. The unit of D1, D2.1, D2.2, D2.3 and D2.4 is 0.1ms.

Therefore, we propose that the DU and gNB-CU-UP each report their respective average packet delay, and then the CU-CP can calculate the overall average.

- The gNB-DU and gNB-CU-UP each report their respective average packet delays to gNB-CU-CP through a new procedure, allowing the gNB-CU-CP to calculate the overall average.

The corresponding TP is provided in the Annex.

3	Conclusion

Following is our proposals and observations:

- Introduce the new class 1 procedure Data Collection Reporting Initiation procedure over F1AP used by gNB-CU to request from gNB-DU the reporting of information to support, e.g., AI/ML in NG-RAN. The procedure applies the same reporting mechanisms of resource status reporting initiation procedure.

- Introduce the new class 2 procedure Data Collection Reporting procedure over F1AP to report information accepted by the gNB-DU. The procedure applies the same reporting mechanisms of resource status report procedure.

- Using the new class 1& class 2 procedure to report the measured EC from gNB-DU to gNB-CU.

- Define the Energy Cost IE over F1 interface as an INTEGER (0..10000, ...).

- Agree to report the average UE throughput UL/DL and average packet loss from gNB-DU to gNB-CU-CP via new procedure over F1.

- The gNB-DU and gNB-CU-UP each report their respective average packet delays to gNB-CU-CP through a new procedure, allowing the gNB-CU-CP to calculate the overall average.

4	Reference

[1] RP-242385, New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, ZTE Corporation, NEC

[2] TR 38.743

5	Text Proposal to 38.473

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| gNB-DU Resource Coordination | GNB-DU RESOURCE COORDINATION REQUEST | GNB-DU RESOURCE COORDINATION RESPONSE |  |
| F1 Removal | F1 REMOVAL REQUEST | F1 REMOVAL RESPONSE | F1 REMOVAL FAILURE |
| BAP Mapping Configuration | BAP MAPPING CONFIGURATION | BAP MAPPING CONFIGURATION ACKNOWLEDGE | BAP MAPPING CONFIGURATION FAILURE |
| GNB-DU Resource Configuration | GNB-DU RESOURCE CONFIGURATION | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE | GNB-DU RESOURCE CONFIGURATION FAILURE |
| IAB TNL Address Allocation | IAB TNL ADDRESS REQUEST | IAB TNL ADDRESS RESPONSE | IAB TNL ADDRESS FAILURE |
| IAB UP Configuration Update | IAB UP CONFIGURATION UPDATE REQUEST | IAB UP CONFIGURATION UPDATE RESPONSE | IAB UP CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Positioning Measurement | POSITIONING MEASUREMENT REQUEST | POSITIONING MEASUREMENT RESPONSE | POSITIONING MEASUREMENT FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| Broadcast Context Setup | BROADCAST CONTEXT SETUP REQUEST | BROADCAST CONTEXT SETUP RESPONSE | BROADCAST CONTEXT SETUP FAILURE |
| Broadcast Context Release | BROADCAST CONTEXT RELEASE COMMAND | BROADCAST CONTEXT RELEASE COMPLETE |  |
| Broadcast Context Modification | BROADCAST CONTEXT MODIFICATION REQUEST | BROADCAST CONTEXT MODIFICATION RESPONSE | BROADCAST CONTEXT MODIFICATION FAILURE |
| Multicast Context Setup | MULTICAST CONTEXT SETUP REQUEST | MULTICAST CONTEXT SETUP RESPONSE | MULTICAST CONTEXT SETUP FAILURE |
| Multicast Context Release | MULTICAST CONTEXT RELEASE COMMAND | MULTICAST CONTEXT RELEASE COMPLETE |  |
| Multicast Context Modification | MULTICAST CONTEXT MODIFICATION REQUEST | MULTICAST CONTEXT MODIFICATION RESPONSE | MULTICAST CONTEXT MODIFICATION FAILURE |
| Multicast Distribution Setup | MULTICAST DISTRIBUTION SETUP REQUEST | MULTICAST DISTRIBUTION SETUP RESPONSE | MULTICAST DISTRIBUTION SETUP FAILURE |
| Multicast Distribution Release | MULTICAST DISTRIBUTION RELEASE COMMAND | MULTICAST DISTRIBUTION RELEASE COMPLETE |  |
| PDC Measurement Initiation | PDC MEASUREMENT INITIATION REQUEST | PDC MEASUREMENT INITIATION RESPONSE | PDC MEASUREMENT INITIATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |
| Timing Synchronisation Status | TIMING SYNCHRONISATION STATUS REQUEST | TIMING SYNCHRONISATION STATUS RESPONSE | TIMING SYNCHRONISATION STATUS FAILURE |
| Multicast Context Notification | MULTICAST CONTEXT NOTIFICATION INDICATION | MULTICAST CONTEXT NOTIFICATION CONFIRM | MULTICAST CONTEXT NOTIFICATION REFUSE |
| Multicast Common Configuration | MULTICAST COMMON CONFIGURATION REQUEST | MULTICAST COMMON CONFIGURATION RESPONSE | MULTICAST COMMON CONFIGURATION REFUSE |
| Data Collection Reporting Initiation | DATA COLLECTION REQUEST | DATA COLLECTION RESPONSE | DATA COLLECTION FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Context Release Request | BROADCAST CONTEXT RELEASE REQUEST |
| Multicast Context Release Request | MULTICAST CONTEXT RELEASE REQUEST |
| PDC Measurement Report | PDC MEASUREMENT REPORT |
| PDC Measurement Failure Indication | PDC MEASUREMENT FAILURE INDICATION |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| Measurement Activation | MEASUREMENT ACTIVATION |
| QoE Information Transfer | QOE INFORMATION TRANSFER |
| Positioning System Information Delivery | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |
| DU-CU Cell Switch Notification | DU-CU CELL SWITCH NOTIFICATION |
| CU-DU Cell Switch Notification | CU-DU CELL SWITCH NOTIFICATION |
| DU-CU TA Information Transfer | DU-CU TA INFORMATION TRANSFER |
| CU-DU TA Information Transfer | CU-DU TA INFORMATION TRANSFER |
| QoE Information Transfer Control | QOE INFORMATION TRANSFER CONTROL |
| RACH Indication | RACH INDICATION |
| Timing Synchronisation Status Report | TIMING SYNCHRONISATION STATUS REPORT |
| Mobile IAB F1 Setup Triggering | MIAB F1 SETUP TRIGGERING |
| Mobile IAB F1 Setup Outcome Notification | MIAB F1 SETUP OUTCOME NOTIFICATION |
| Broadcast Transport Resource Request | BROADCAST TRANSPORT RESOURCE REQUEST |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| DU-CU Access And Mobility Indication | DU-CU ACCESS AND MOBILITY INDICATION |
| Data Collection Reporting | DATA COLLECTION UPDATE |

8.2.xx	Data Collection Reporting Initiation

8.2.xx.x1	General

This procedure is used by an gNB-CU to request to gNB-DU the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

8.2.xx.x2	Successful Operation

Figure 8.2.9.2-1: Data Collection Reporting Initiation, successful operation

The procedure is initiated with a DATA COLLECTION REQUEST message sent from gNB-CU to gNB-DU to start a measurement or stop a measurements.

If gNB-DU is capable to provide all requested information, it shall initiate the measurement as requested by gNB-CU, and respond with the RESOURCE STATUS RESPONSE message.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the DATA COLLECTION REQUEST indicates the type of objects gNB-DU shall perform measurements on. The gNB-CU-UP shall include in the DATA COLLECTION UPDATE message:

-	the Average UE Throughput DL IE, if the first bit, "Average UE Throughput DL" of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to "1".

-	the Average UE Throughput UL IE, if the second bit, "Average UE Throughput UL" of the Report Characteristics included in the DATA COLLECTION REQUEST message is set to "1".

-	the Average Packet Delay IE, if the third bit, "Average Packet Delay" of the Report Characteristics for included in the DATA COLLECTION REQUEST message is set to "1".

-	the Average Packet Loss DL IE, if the forth bit, "Average Packet Loss DL" of the Report Characteristics included in the DATA COLLECTION REQUEST message is set to "1".

-	the Energy Cost IE, if the fifth bit, "Energy Cost" of the Report Characteristics included in the DATA COLLECTION REQUEST message is set to "1".

If the Reporting Periodicity IE is included in the DATA COLLECTION REQUEST message, this indicates the periodicity for the reporting of periodic measurements. The gNB-DU shall report only once, unless otherwise requested within the Reporting Periodicity IE.

8.2.xx.x3	Unsuccessful Operation

Figure 8.2.9.3-1: Data Collection Reporting Initiation, unsuccessful operation

If any of the requested measurements cannot be initiated, gNB-DU shall send a DATA COLLECTION FAILURE message with an appropriate cause value.

8.2.xx.x4	Abnormal Conditions

If the initiating gNB-CU does not receive either DATA COLLECTION RESPONSE message or DATA COLLECTION FAILURE message, the gNB-CU may reinitiate the Data Collection Reporting Initiation procedure towards the same gNB-DU, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message with the same Transaction ID.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the DATA COLLECTION REQUEST message then gNB-DU shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the gNB-DU receives a DATA COLLECTION REQUEST message which includes the Registration Request IE set to "start" and the gNB-CU Measurement ID IE corresponding to an existing on-going load measurement reporting, for which a different Transaction ID is used, then gNB-DU shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

8.2.yy	DATA COLLECTION Reporting

8.2.yy.y1	General

This procedure is initiated by an gNB-DU to report information accepted by the gNB-DU following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

8.2.yy.y2	Successful Operation

Figure 8.2.10.2-1: Data Collection Reporting, successful operation

The gNB-DU shall report the results of the admitted measurements in DATA COLLECTION UPDATE message. The admitted measurements are the measurements that were successfully initiated during the preceding Data Collection Initiation procedure.

8.2.yy.y3	Unsuccessful Operation

Not applicable.

8.2.yy.y4	Abnormal Conditions

Void.

9.2.1.x	DATA COLLECTION REQUEST

This message is sent by an gNB-CU-CP to gNB-DU to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(36)) | Each position in the bitmap indicates measurement object the gNB-DU is requested to report. First Bit = Average UE Throughput DL, Second Bit = Average UE Throughput UL, Third Bit = Average Packet Delay, Forth Bit = Average Packet Loss DL Fifth Bit = Energy Cost . Other bits shall be ignored by the gNB-DU. | YES | reject |
| Reporting Periodicity | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, 20000ms, 30000ms, 40000ms, 50000ms, 60000ms, 70000ms, 80000ms, 90000ms, 100000ms, 110000ms, 120000ms, …) | Periodicity that can be used for reporting. Also used as the averaging window length for all measurement object if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value “stop” |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

9.2.1.yy	DATA COLLECTION RESPONSE

This message is sent by the gNB-DU to indicate that the requested measurement, for all the measurement objects included in the measurement is successfully initiated.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.1.mm	DATA COLLECTION FAILURE

This message is sent by the gNB-DU to indicate that for any of the requested measurement objects the measurement cannot be initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | O |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.1.nn	DATA COLLECTION UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Energy Cost | O |  | INTEGER (0..10000,…) |  | YES | ignore |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | OCTET STRING (SIZE(8)) |  | – |  |
| >>UE Performance | O |  | 9.3.1.x |  | – |  |

9.3.1.x	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  |  |  |
| Average UE Throughput UL | O |  |  |  |
| Average Packet Delay | O |  |  |  |
| Average Packet Loss DL | O |  |  |  |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

5	Text Proposal to 37.483

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

8.1	List of E1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| gNB-CU-UP E1 Setup | GNB-CU-UP E1 SETUP REQUEST | GNB-CU-UP E1 SETUP RESPONSE | GNB-CU-UP E1 SETUP FAILURE |
| gNB-CU-CP E1 Setup | GNB-CU-CP E1 SETUP REQUEST | GNB-CU-CP E1 SETUP RESPONSE | GNB-CU-CP E1 SETUP FAILURE |
| gNB-CU-UP Configuration Update | GNB-CU-UP CONFIGURATION UPDATE | GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU-UP CONFIGURATION UPDATE FAILURE |
| gNB-CU-CP Configuration Update | GNB-CU-CP CONFIGURATION UPDATE | GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU-CP CONFIGURATION UPDATE FAILURE |
| E1 Release | E1 RELEASE REQUEST | E1 RELEASE RESPONSE |  |
| Bearer Context Setup | BEARER CONTEXT SETUP REQUEST | BEARER CONTEXT SETUP RESPONSE | BEARER CONTEXT SETUP FAILURE |
| Bearer Context Modification (gNB-CU-CP initiated) | BEARER CONTEXT MODIFICATION REQUEST | BEARER CONTEXT MODIFICATION RESPONSE | BEARER CONTEXT MODIFICATION FAILURE |
| Bearer Context Modification Required (gNB-CU-UP initiated) | BEARER CONTEXT MODIFICATION REQUIRED | BEARER CONTEXT MODIFICATION CONFIRM |  |
| Bearer Context Release (gNB-CU-CP initiated) | BEARER CONTEXT RELEASE COMMAND | BEARER CONTEXT RELEASE COMPLETE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| IAB UP TNL Address Update | IAB UP TNL ADDRESS UPDATE | IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE | IAB UP TNL ADDRESS UPDATE FAILURE |
| BC Bearer Context Setup | BC BEARER CONTEXT SETUP REQUEST | BC BEARER CONTEXT SETUP RESPONSE | BC BEARER CONTEXT SETUP FAILURE |
| BC Bearer Context Modification (gNB-CU-CP initiated) | BC BEARER CONTEXT MODIFICATION REQUEST | BC BEARER CONTEXT MODIFICATION RESPONSE | BC BEARER CONTEXT MODIFICATION FAILURE |
| BC Bearer Context Modification Required (gNB-CU-UP initiated) | BC BEARER CONTEXT MODIFICATION REQUIRED | BC BEARER CONTEXT MODIFICATION CONFIRM |  |
| BC Bearer Context Release (gNB-CU-CP initiated) | BC BEARER CONTEXT RELEASE COMMAND | BC BEARER CONTEXT RELEASE COMPLETE |  |
| MC Bearer Context Setup | MC BEARER CONTEXT SETUP REQUEST | MC BEARER CONTEXT SETUP RESPONSE | MC BEARER CONTEXT SETUP FAILURE |
| MC Bearer Context Modification (gNB-CU-CP initiated) | MC BEARER CONTEXT MODIFICATION REQUEST | MC BEARER CONTEXT MODIFICATION RESPONSE | MC BEARER CONTEXT MODIFICATION FAILURE |
| MC Bearer Context Modification Required (gNB-CU-UP initiated) | MC BEARER CONTEXT MODIFICATION REQUIRED | MC BEARER CONTEXT MODIFICATION CONFIRM |  |
| MC Bearer Context Release (gNB-CU-CP initiated) | MC BEARER CONTEXT RELEASE COMMAND | MC BEARER CONTEXT RELEASE COMPLETE |  |
| Data Collection Reporting Initiation | DATA COLLECTION REQUEST | DATA COLLECTION RESPONSE | DATA COLLECTION FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| Bearer Context Release Request (gNB-CU-UP initiated) | BEARER CONTEXT RELEASE REQUEST |
| Bearer Context Inactivity Notification | BEARER CONTEXT INACTIVITY NOTIFICATION |
| DL Data Notification | DL DATA NOTIFICATION |
| UL Data Notification | UL DATA NOTIFICATION |
| Data Usage Report | DATA USAGE REPORT |
| gNB-CU-UP Counter Check | GNB-CU-UP COUNTER CHECK REQUEST |
| gNB-CU-UP Status Indication | GNB-CU-UP STATUS INDICATION |
| MR-DC Data Usage Report | MR-DC DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Early Forwarding SN Transfer | EARLY FORWARDING SN TRANSFER |
| GNB-CU-CP Measurement Results Information | GNB-CU-CP MEASUREMENT RESULTS INFORMATION |
| IAB PSK Notification | IAB PSK NOTIFICATION |
| BC Bearer Context Release (gNB-CU-UP initiated) | BC BEARER CONTEXT RELEASE REQUEST |
| MC Bearer Context Release (gNB-CU-UP initiated) | MC BEARER CONTEXT RELEASE REQUEST |
| MC Bearer Notification | MC BEARER NOTIFICATION |
| Data Collection Reporting | DATA COLLECTION UPDATE |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.2.xx	Data Collection Reporting Initiation

8.2.xx.x1	General

This procedure is used by an gNB-CU-CP to request to gNB-CU-UP the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

8.2.xx.x2	Successful Operation

Figure 8.2.9.2-1: Data Collection Reporting Initiation, successful operation

The procedure is initiated with a DATA COLLECTION REQUEST message sent from gNB-CU-CP to gNB-CU-UP to start a measurement or stop a measurements.

If gNB-CU-UP is capable to provide all requested information, it shall initiate the measurement as requested by gNB-CU-CP, and respond with the RESOURCE STATUS RESPONSE message.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the DATA COLLECTION REQUEST indicates the type of objects gNB-CU-UP shall perform measurements on. The gNB-CU-UP shall include in the DATA COLLECTION UPDATE message:

-	the Average Packet Delay IE, if the first bit, "Average Packet Delay" of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to "1".

If the Reporting Periodicity IE is included in the DATA COLLECTION REQUEST message, this indicates the periodicity for the reporting of periodic measurements. The gNB-CU-UP shall report only once, unless otherwise requested within the Reporting Periodicity IE.

8.2.xx.x3	Unsuccessful Operation

Figure 8.2.9.3-1: Data Collection Reporting Initiation, unsuccessful operation

If any of the requested measurements cannot be initiated, gNB-CU-UP shall send a DATA COLLECTION FAILURE message with an appropriate cause value.

8.2.xx.x4	Abnormal Conditions

If the initiating gNB-CU-CP does not receive either DATA COLLECTION RESPONSE message or DATA COLLECTION FAILURE message, the gNB-CU-CP may reinitiate the Data Collection Reporting Initiation procedure towards the same gNB-CU-UP, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message with the same Transaction ID.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the DATA COLLECTION REQUEST message then gNB-CU-UP shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the gNB-CU-UP receives a DATA COLLECTION REQUEST message which includes the Registration Request IE set to "start" and the gNB-CU-CP Measurement ID IE corresponding to an existing on-going load measurement reporting, for which a different Transaction ID is used, then gNB-CU-UP shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

8.2.yy	DATA COLLECTION Reporting

8.2.yy.y1	General

This procedure is initiated by an gNB-CU-UP to report information accepted by the gNB-CU-UP following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

8.2.yy.y2	Successful Operation

Figure 8.2.10.2-1: Data Collection Reporting, successful operation

The gNB-CU-UP shall report the results of the admitted measurements in DATA COLLECTION UPDATE message. The admitted measurements are the measurements that were successfully initiated during the preceding Data Collection Initiation procedure.

8.2.yy.y3	Unsuccessful Operation

Not applicable.

8.2.yy.y4	Abnormal Conditions

Void.

9.2.1.x	DATA COLLECTION REQUEST

This message is sent by an gNB-CU-CP to gNB-CU-UP to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(36)) | Each position in the bitmap indicates measurement object the gNB-CU-UP is requested to report. First Bit = Average Packet Delay. Other bits shall be ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, 20000ms, 30000ms, 40000ms, 50000ms, 60000ms, 70000ms, 80000ms, 90000ms, 100000ms, 110000ms, 120000ms, …) | Periodicity that can be used for reporting. Also used as the averaging window length for all measurement object if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value “stop” |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

9.2.1.yy	DATA COLLECTION RESPONSE

This message is sent by the gNB-CU-UP to indicate that the requested measurement, for all the measurement objects included in the measurement is successfully initiated.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.1.mm	DATA COLLECTION FAILURE

This message is sent by the gNB-CU-UP to indicate that for any of the requested measurement objects the measurement cannot be initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | O |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.1.nn	DATA COLLECTION UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the results of the requested measurements.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | OCTET STRING (SIZE(8)) |  | – |  |
| >>UE Performance | O |  | 9.3.1.x |  | – |  |

9.3.1.x	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average Packet Delay | O |  | 9.3.1.y |  |

9.3.1.y	Average Packet Delay

This IE indicates the RAN part of the average packet delay in the UL and DL directions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Packet Delay UL | M |  | INTEGER (0.. 10000) | Defined in TS 38.314 [42]. Unit: 0.1 millisecond |
| Average Packet Delay DL | M |  | INTEGER (0.. 10000) | Defined in TS 38.314 [42]. Unit: 0.1 millisecond |