3GPP TSG-RAN WG3 Meeting #126	R3-247161

Orlando, US, 18 - 22 Nov, 2024

Title:	(TP for BLCR for SON for 38.413, 36.423) Rel-18 SON and MDT leftovers

Agenda Item:	10.4

Source:	Huawei

Document for:	other

# 1. Introduction

This paper discusses the Rel-18 SON and MDT leftovers

# 2. Discussion

## 2.1	RACH optimization for SDT

From our point of view, at least the following information are helping the network to analyze the issues causing SDT failure and optimize the related condition parameters to maximize the performance of SDT:

- Data volume at the time of initiating the SDT or normal RA

- DL RSRP at the time of initiating the SDT or normal RA

- Configured RSRP and data volume via SIB1

The first two seems to be agreed in RAN2 and for the 3rd one we have the previous agreement to not report this from the UE. In turn, the RAN2#123 meeting has confirmed the agreent achieved by RAN3 that there is no need to add logging of sdt-RSRP-Threshold. For enabling the network to retrieve the 3rd, the last RAN3 meeting has decided to report the time between the SDT and the SDT report.  We propose that, RAN3 inform RAN2 of the RAN3’s decision.

Send our agreement to RAN2 regarding logging the time from the start of RA-SDT to the reporting of RA report.

Furthermore, we analyse the necessity of other SDT related information as below:

- CG-SDT related information if the UE is configured with CG-SDT but fails to initiate the CG-SDT

- indication regarding that the UE failed to initiate RA-SDT or CG-SDT

When UL data arrives in the UE in RRC inactive, the UE could first check whether the CG-SDT is configured and thus the conditions for CG-SDT are fulfilled, and then check if the RA-SDT is fulfilled if CG-SDT is not configured or the conditions for CG-SDT are not fulfilled. In our point of view, the network may need to know why CG-SDT is not initiated when the CG is reserved and configured for the UE. The UE also compares the DL RSRP and buffered data volume to CG-SDT condition parameter to derive the fulfilment status. Regarding the fallback from CG-SDT to RA-SDT, it may help the network to optimize the CG-SDT related parameters and improve the utilization probability of CG resources. Since the DL RSRP and buffered data are already agreed to be reported by UE and the UE-sepecific CG-SDT config is known from the UE context, we understand that an indication of whether CG-SDT is configured in RA report, when RA-SDT is initiated, enables the network to know that it fails to initiate CG-SDT.

The UE indicates in the RA-Report if the CG-SDT initiation was evaluated.

## 2.3 UHI and MHI enhancement for SCG deactivation/activation

The potential anhancement of MHI/UHI was put on hold in RAN3 while waiting for RAN2 progress. This was discussed in Last RAN2 meeting with the following conclusions

It is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state.

Send an LS to RAN3 to ask if a NW solution is good enough to achieve the agreement above, or if a UE based solution is needed.

The hierarchical structure of the UHI is a list of 16 PCells where each PCell can include a list of 16 PSCells. This IE is defined as below in NGAP:

#### 9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. |

As can be seen above, the IE already contains the stay time. It is also worth noting that the stay time is capped at 4096 seconds. Hence, the most straightforward solution would be to add an optional IE representing the ratio of time the SCG is in activated state, i.e. as indicated below:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| ... |  |  |  |  |  |  |
| Relative time in SCG activated state | O |  | INTEGER(0..100) |  | YES | ignore |

Agree to include the relative time in SCG activated state as an optional IE in Last Visited PSCell Information.

## 2.2	MRO for MR-DC SCG failure

To facilitate the UE with EN-DC, the LTM MN is capable of selecting a target PSCell for PSCell addition purpose based on SSB RRM measurement. SSB RRM measurement is sufficient for the mobility decision made by the MN for PSCell addition or PSCell change or SCG failure handling procedure.

For MRO analysis, the measurement result of SCG frequencies configured by the MN for SSB RRM measurement may not be as complete as the one configured by the SN.

When the last serving SN receives the SCGfailureInformation and analyses that it is not its fault, the SN could indicate the suitable PSCell in the SCG failure transfer to help the MN identify why the sutiable cell is not selected as candidate cell, which may be the wrong cell selection of initiating node or candidate SN. If it is the candidate SN’s wrong selection, the MN could forward to the candidate SN which the suitable PSCell belongs to.

Therefore, we propose:

Include the suitable PSCell ID in SCG failure transfer message.

Agree to update the TP as included in the annex.

# 3	Conclusion

This document proposes:

Proposal 1:	Send our agreement to RAN2 regarding logging the time from the start of RA-SDT to the reporting of RA report.

Proposal 2:	The UE indicates in the RA-Report if the CG-SDT initiation was evaluated.

Proposal 3:	Agree to include the relative time in SCG activated state as an optional IE in Last Visited PSCell Information.

Proposal 4:	Include the suitable PSCell ID in SCG failure transfer message.

Proposal 5:	Agree to update the TP as included in the annex.

# Annex	LS to RAN2 on SDT

Title:	[DRAFT] LS on SDT enhancements for SON

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2

Cc:

Contact person:	Henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 has discussed SDT enhancements for SON. RAN3 noted that RAN2 already made some progress and agreed that in addition to agreements in RAN2, RAN3 sees benefit in including the time between the RA event and the report in the RA report. This could be used by the network to determine the configuration that was used at the time the RA event occurred.

RAN3 also agreed on the benefit to include an indication whether the CG-SDT initiation was evaluated or not in the RA-Report.

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly ask RAN2 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#125-bis	2024-10-14 – 2024-10-18 	China, CN

RAN3#126	2024-11-18  – 2024-11-22 	Orlando, US

# Annex	TP for UHI for 38.413

#### 9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. | - |  |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. | - |  |
| Relative time in SCG activated state | O |  | INTEGER(0..100) |  | YES | ignore |

[snip]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

[snip]

id-UserPlaneFailureIndication,

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-RelativeTimeInSCGActivatedState,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

maxnoofAoIMinusOne,

[snip]

LastVisitedPSCellInformation ::= SEQUENCE {

pSCellID			NGRAN-CGI	 			OPTIONAL,

timeStay			INTEGER (0..40950),

iE-Extensions		ProtocolExtensionContainer { {LastVisitedPSCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedPSCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-RelativeTimeInSCGActivatedState	CRITICALITY ignore	EXTENSION RelativeTimeInSCGActivatedState		PRESENCE optional

...

}

[snip]

RelativeAMFCapacity ::= INTEGER (0..255)

RelativeTimeInSCGActivatedState ::= INTEGER (0..100)

ReportArea ::= ENUMERATED {

cell,

...

}

# Annex	TP for MR-DC SCG failure for 36.423

### 8.7.y	SCG Failure Transfer

#### 8.7.y.1	General

The purpose of the SCG Failure Transfer procedure is to indicate to the MeNB that the root cause of the SCG failure may have occurred in the other nodes.

The procedure uses UE-associated signalling.

#### 8.7.y.2	Successful Operation

Figure 8.7.y.2-1: SCG Failure Transfer, successful operation

en-gNB initiates the procedure by sending the SCG FAILURE TRANSFER message to MeNB.

If received, MeNB uses the information according to TS 36.300 [15].

#### 8.7.y.3	Unsuccessful Operation

Not applicable.

#### 8.7.y.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.4.y	SCG FAILURE TRANSFER

This message is sent by the en-gNB to the MeNB to indicate that the root cause of the SCG failure may have occurred in the other nodes.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE XnAP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| Suitable PSCell ID | O |  | NR CGI 9.2.111 |  | YES | ignore |

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

[snip]

id-IABAuthorized,

id-SourcePSCellCGI,

id-FailedPSCellCGI,

id-SCG-FailureReportContainer,

id-TimeSCG-Failure,

id-SuitablePSCellID,

maxCellineNB,

maxnoofBearers,

maxnoofPDCP-SN,

maxFailedMeasObjects,

[snip]

-- **************************************************************

--

#### -- SCG FAILURE TRANSFER

--

-- **************************************************************

ScgFailureTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ ScgFailureTransfer-IEs}},

...

}

ScgFailureTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID					CRITICALITY ignore		TYPE UE-X2AP-ID								PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID					CRITICALITY ignore		TYPE SgNB-UE-X2AP-ID						PRESENCE mandatory}|

{ ID id-SuitablePSCellID					CRITICALITY ignore		TYPE NRCGI									PRESENCE mandatory},

...

}

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

[snip]

id-IABAuthorized															ProtocolIE-ID ::= 449

id-SourcePSCellCGI															ProtocolIE-ID ::= xx1

id-FailedPSCellCGI															ProtocolIE-ID ::= xx2

id-SCG-FailureReportContainer												ProtocolIE-ID ::= xx3

id-TimeSCG-Failure															ProtocolIE-ID ::= xx4

id-SuitablePSCellID															ProtocolIE-ID ::= xx5