3GPP TSG RAN WG3 Meeting #126	R3-247182

Orlando, U.S.A., 18 - 22 November, 2024

Agenda Item:	13.2

Source:	ZTE Corporation

Title:	TP to SN initiated inter-CU SCG LTM

Document for:	Discussions & Approval

- Introduction

In this meeting, RAN3 has received an LS (R3-247111 / R2-2409378) from RAN2 as below.

- RAN2#127: Agreements on inter-CU LTM Reference configuration: - Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported. CSI resource and report configuration: - The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration. - The source CU is responsible to generate the common CSI resource configuration. Early DL sync: - For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused. Inter-CU LTM switch: - For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell. RLC and PDCP re-establishment in inter-CU LTM: - If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it. Handling of candidate configuration after inter-CU LTM cell switch: - As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network. DRB/PDU session mismatch in subsequent inter-CU LTM: - RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421. LTM and L3 HO: - L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM): - When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network. - The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations. - For the execution order between CHO and LTM, Rel-18 principle is applied. Inter-CU SCG LTM: - Inter-CU SCG LTM preparation can be initiated by source SN. - The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. - For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN. - RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works. - Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19. Inter-CU MCG LTM: - SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). - Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied). - For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure. - RAN2#127bis: Agreements on inter-CU LTM - The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs). - For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18. - The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3. - Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline: - The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part. - FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations. - The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message. - Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed. - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs. 5. For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN. 6. The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells. 7. The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN. 8. In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN. 9. Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. 10. Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM. 11. RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen: - Inter-MN LTM and intra-SN LTM - Inter-SN LTM and intra-MN LTM

This paper discusses SN initiated inter-CU SCG LTM, based on the RAN2 progress.

- Discussion

Issue 2: Stage-2 signaling flows and procedures

Taking the stage-2 flow chart for SN initiated inter-SN SCPAC (i.e. Figure 10.20-2 in TS 37.340 [2]) as reference, an example of the stage-2 signaling flows for SN initiated inter-CU SCG LTM is shown as below:

Figure 1. SN initiated inter-CU SCG LTM procedure

LTM Preparation

In step 1~3a, the source SN initiates the inter-CU SCG LTM preparation procedure by sending a SN Change Required message to the MN. Then the MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedure. Within the list of cells suggested by the source SN, the candidate SN decides the list of PSCell(s) to prepare for SCG LTM, considering of the maximum number indicated by the MN. For each prepared PSCell, the candidate SN provides the corresponding SCG radio resource configuration, and may also provide the L1 RS (e.g. SSB or CSI-RS) configuration for L1 measurement, early UL sync configuration, or TCI-state configuration. The candidate SN sends those configurations to the MN via SN Addition Request Acknowledge message. And then the MN may send the Xn-U Address Indication message to the candidate SN(s) to provide the information for data forwarding and/or trigger the early data forwarding.

5.	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

Proposal 1: For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending a SN Change Required message to the MN.

Proposal 2: Then the MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

Proposal 3: Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

In step 4~5a, the MN initiate the SN modification procedure towards the source SN and candidate SN(s), to transfer the LTM related configuration of candidate cells.

For inter-CU LTM in NR SA, we have agreed that the source CU to generate a common CSI resource configuration for L1 measurement on candidate cells, based on the L1 RS configuration from each candidate cell. Besides, in order to support subsequent LTM, the source CU shall also send the common CSI resource configuration and the information for candidate cells from multiple candidate CUs to each candidate CU, to allow the candidate CU updating the configurations of candidate cells, e.g. the L1 CSI report configuration. The similar principle can be reused for SN initiated inter-CU SCG LTM. Considering that only the SN initiated inter-SN LTM (including LTM configuration, early DL/UL sync and LTM execution) is supported in Rel-19, the source SN needs to know the L1 measurement configuration and early sync configuration of candidate cells.

- For SN initiated inter-SN LTM, the LTM configuration, early DL/UL sync and LTM execution are triggered by the source SN, so the source SN needs to know the L1 measurement configuration and early sync configuration of candidate cells.

Thus, it’s natural to let the source SN generate the common CSI resource configuration for candidate cells. The MN can transfer the received L1 RS (e.g. SSB or CSI-RS) configuration, early UL sync configuration, or TCI-state configuration to the source SN via MN initiated SN modification procedure, as the step 4 in Figure 1. Based on the L1 RS configuration for each candidate cell, the source SN can generate the common CSI resource configuration, as well as the L1 CSI report configuration provided within the source SCG configuration. Then the source SN transfers the generated common CSI resource configuration and the updated source SCG configuration (e.g. including the L1 CSI report configuration) to the MN via SN modification request acknowledge message, i.e. as the step 5 above.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7.	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

Proposal 4: The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN, e.g. via SN modification request message, and the source SN responds with the common CSI resource configuration to the MN via SN modification request ACK message.

In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), e.g. via MN initiated SN modification procedure. And then the candidate SN can update the candidate SCG configuration (e.g. generating the L1 CSI report configuration), accordingly.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer ,during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

Proposal 5: In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

In step 6/7, the MN sends the RRCReconfiguration message to the UE including the SCG LTM configuration. And the UE responses an RRCReconfigurationComplete message to the MN. The MN RRCReconfigurationComplete message includes an SN RRCReconfigurationComplete message to the source SN.

In step 8, the MN informs the source SN with the SN RRCReconfigurationComplete message via SN Change Confirm message, to indicate that SCG LTM is prepared.

Early sync

In step 9a/9b, the UE performs DL/UL sync with the LTM candidate cell(s) before receiving the cell switch command. For the PDCCH order triggered early RACH, the candidate SN(s) needs to transfer the calculated TA value of candidate cell(s) to the source SN via the MN. The existing TA information transfer procedure via F1 interface can be extended to Xn interface to transfer the TA information between the MN and the source/candidate SN.

- The existing TA information transfer procedure via F1 interface can be extended to Xn interface to transfer the TA information from the candidate SN to the source SN via the MN.

LTM cell switch execution

In step 10~11, the UE sends the L1 measurement report(s) to the source SN (i.e. source SN DU). The source SN decides to execute LTM cell switch to the target cell by sending an LTM cell switch command MAC CE to the UE.

In step 12, the source SN sends a notification message to the target SN via the MN, to indicate the triggering of LTM to the UE. The message may include the target cell ID, the selected TCI-state ID(s) of the target cell. We can simply extend the existing cell switch notification procedure via F1 interface to Xn interface.

- The existing cell switch notification procedure via F1 interface can be extended to Xn interface to inform the selected target cell from the source SN to the target SN via the MN.

LTM cell switch completion

In step 13/14, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. And then the MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete message.

Proposal 6: Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message, pending to RAN2.

In step 15, the UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell. If the UE has valid TA of the target cell, the UE skips random access procedure towards the target cell, i.e. RACH-less LTM. The order the UE sends the MN RRCReconfigurationComplete message and performs the Random Access procedure towards the target SCG is not defined. If the UE has performed a RA procedure in step 15, the UE considers that LTM cell switch execution is successfully completed when the random access procedure towards the target SCG is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the target SN has successfully received its first UL data.

In step 16, the MN initiates the SN modification or release procedure to the source SN, to inform the execution of SCG LTM. The detailed procedure is similar to SN initiated inter-SN SCPAC.

In step 17~18a/b, the data forwarding procedure is performed.

In step 19, if applicable, the update of the UP path towards the core network is performed via a PDU session path update procedure.

Based on the description of each step above, we proposed to take flow charts and procedures for SN initiated inter-CU SCG LTM in Figure 1 as baseline.

Proposal 7: The flow charts and procedures in Figure 1 are taken as baseline for SN initiated inter-CU SCG LTM.

- Conclusion

After the above analysis, we provide the following proposals.

Observation 1: 	For SN initiated inter-SN LTM, the LTM configuration, early DL/UL sync and LTM execution are triggered by the source SN, so the source SN needs to know the L1 measurement configuration and early sync configuration of candidate cells.

Observation 2: 	The existing TA information transfer procedure via F1 interface can be extended to Xn interface to transfer the TA information from the candidate SN to the source SN via the MN.

Observation 3: 	The existing cell switch notification procedure via F1 interface can be extended to Xn interface to inform the selected target cell from the source SN to the target SN via the MN.Proposal 7: Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message, pending to RAN2.

Proposal 1: For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending a SN Change Required message to the MN.

Proposal 2: Then the MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

Proposal 3: Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

Proposal 4: The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN, e.g. via SN modification request message, and the source SN responds with the common CSI resource configuration to the MN via SN modification request ACK message.

Proposal 5: In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

Proposal 6: Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message, pending to RAN2.

Proposal 7: The flow charts and procedures in Figure 1 are taken as baseline for SN initiated inter-CU SCG LTM.

- Text Proposal to TS37.340

Figure 1. Signalling procedure for SN initiated inter-CU SCG LTM

The procedure for SN initiated inter-CU SCG LTM is as follows:

1. For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending a SN Change Required message to the MN.

2. The MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

3. Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

3a. The MN may send the Xn-U Address Indication message to the candidate SN(s) to provide the information for data forwarding and/or trigger the early data forwarding.

4/5. The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN, e.g. via SN modification request message. The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells, and responds with the common CSI resource configuration to the MN via SN modification request ACK message.

5a. In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

6/7. [RAN2] The MN sends the RRCReconfiguration message to the UE including the SCG LTM configuration. And the UE responses an RRCReconfigurationComplete message to the MN. The MN RRCReconfigurationComplete message includes an SN RRCReconfigurationComplete message to the source SN.

8. The MN informs the source SN with the SN RRCReconfigurationComplete message via SN Change Confirm message, to indicate that SCG LTM is prepared.

9a/9b. If early TA acquisition is performed to the candidate SN(s), the candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source SN via the MN.

10/11. [RAN2]The UE sends the L1 measurement report(s) to the source SN. The source SN decides to execute LTM cell switch to the target cell by sending an LTM cell switch command MAC CE to the UE.

12. The source SN sends a CELL SWITCH NOTIFICATION message to the target SN via the MN, to indicate the triggering of LTM to the UE. The message may include the target cell ID, the selected TCI-state ID(s) of the target cell. We can simply extend the existing cell switch notification procedure via F1 interface to Xn interface.

13/14. [RAN2]The UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. And then the MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete message.

15. [RAN2]The UE performs the random access procedure towards the target cell,

16. The MN initiates the SN modification or release procedure to the source SN, to inform the execution of SCG LTM. The detailed procedure is similar to SN initiated inter-SN SCPAC.

17 and 18a/b. The data forwarding procedure is performed.

19. If applicable, the update of the UP path towards the core network is performed via a PDU session path update procedure.