3GPP TSG-RAN WG3 Meeting #126                                                                 R3-247268

Orlando, USA, November 18th– 22nd, 2024

Source:	CATT

Title:	(draft Reply LSs to SA2) On support of WAB

Agenda Item:	12.2

Document for:	Discussion and Decision

1 Introduction

In RAN3#125bis meeting [1], additional topological enhancements for NR are studied. According [1], the following of Wireless Access Backhaul (WAB) is captured in stage-2 spec.

RAN3 to capture the following in the stage2 spec based on TR 38.799. FFS on which spec. - The definitions and terminology - Architecture and protocol stack for WAB. - WAB-node integration procedure. - Configuration of WAB-gNB - Concerning protocol stacks: only Figure 4.2.4 and 4.2.5 are relevant for normative text - Concerning the architecture: only figure 4.2.1 is relevant for normative text Note: other types of backhaul, e.g. non 3GPP, could be also used. No impact on the specifications and no further RAN3 discussions are expected concerning such other types of backhauls

In this contribution, we will continue our further discussion about the basic functions of WAB.

2 Discussion

2.1 WAB authorization

The behavior of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes should be specified. The TR 38.799 [2] has captured the WAB authorization part, where authorization of the WAB-MT is independent with the WAB-gNB service authorization. However, the TR only includes the behavior of WAB-node when the status of WAB-gNB authorization is changed from “authorized” to “not authorized”. The behaviors of WAB-node in case the authorization status is changed from “not authorized” to “authorized” and the authorization status of WAB-MT changes are not included. If the authorization status of WAB-gNB is changed from “authorized” to “not authorized”, the WAB-gNB can remove the NG/Xn connections after releasing/handing over all the UEs, but keep the SCTP connections alive, as the BH PDU session providing the IP connectivity is still available. If the authorization status of WAB-gNB is changed from “not authorized” to “authorized”, the WAB-gNB should re-establish NG connections (and Xn connections if needed).

The behaviors of WAB-node for the authorization status change of WAB-MT are similar with those in case authorization status of WAB-gNB is changed. The difference is that the WAB-node needs to release the BH PDU session(s) of the WAB-MT if the authorization status of WAB-MT is changed to “not authorized”, and thus the WAB-node should shut down the SCTP connections after removing the NG/Xn connections. Accordingly, the WAB-node should re-establish the BH PDU session(s) if the authorization status of WAB-MT is changed to “authorized”.

TS 38. 401 should capture the behavior of WAB-node for authorization status of WAB-gNB/WAB-MT changing.

Proposal 1-1: Capture the WAB authorization function in TS 38.401 including the WAB-node behaviors in case authorization status of WAB-gNB/WAB-MT changes.

Proposal 1-2: The WAB-node releases the SCTP connections and BH PDU session(s) of the WAB-MT if the authorization status of WAB-MT is changed to “not authorized”, and re-establish the BH PDU session(s) if the authorization status of WAB-MT is changed back to “authorized”.

2.2 Avoid multi-hop WAB topology

According to the WID [3], the mechanism to avoid multi-hop WAB topology should be specified. And one LS [4] was sent from SA2 for asking RAN to discuss several issues that included the issue to restrict WAB-MT accessing a WAB-gNB cell:

S2-2407345 Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

In the last meeting, four solutions are produced to solve the issue. The solutions are listed as below:

RAN3 to consider the following RAN based solutions to avoid multi hop WAB: Solution 1: The WAB-gNB uses dedicated frequencies and/or PCIs. FFS on any other legacy OTA parameters. Solution 2: Use the slice dedicated for backhauling, i.e. use a list of S-NSSAIs in RRCsetupcomplete to do access control and/or use a list of S-NSSAIs in handover signalling. No involvement of 5GC is expected Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator. Solution5: In case of handover for a WAB-node, the WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node.

For solution 1, it’s possible that the WAB-gNB uses dedicated frequencies and/or PCIs for restricting WAB-MT accessing its cell to avoid multi-hop WAB topology. However, this solution was costly, as it requires the different frequencies and/or PCIs to be assigned to each WAB-gNB, which is clearly not what operators want. Thus, solution 1 is feasible but not sufficient.

Solution 2 can be used to avoid multi-hop for connected WAB-MT being handed over to WAB-gNB. By including the S-NSSAIs specific for WAB in HO request message, the WAB-gNB can refuse a HO request for connected WAB-MT. For idle mode WAB-MT, a list of WAB specific S-NSSAIs can be included in RRCSetupComplete message, and the WAB-gNB can refuse initial access of a WAB-MT. Solution 2 requires no enhancement to spec. However, if the WAB-gNB rejects the initial access from the idle mode WAB-MT, the WAB-MT does not know the reason why it is rejected and would probably (re-)select the WAB cell again, which will lead to consecutive access failures. This issue cannot be resolved by existing specs.

Solution 3 can address the issue of solution 2 for idle mode WAB-MT. It prevents the WAB-MT accessing a WAB cell before it initiating RRC setup through WAB-MT’s cell (re)selection. WAB-MT will consider the cell broadcasting the new indicator as barred and not select to the cell. Although solution 3 has RAN2 impacts, the impacts only apply to the WAB-node. So, solution 3 can be adopted for avoiding multi-hop for idle mode WAB-MT. The name of the new indicator broadcasted by WAB cells can be left to RAN2.

Solution 5 was proposed to avoid handing over connected WAB-MT to a WAB-node. If the solution is adopted, the HO request will introduce a new indication, which is not supported by the legacy gNB. The application of WAB in the legacy NG-RAN would be significantly restricted. Secondly, as solution 2 can be used to avoid connected WAB-MT being handed over to WAB-gNB by including the WAB specific S-NSSAIs as the implicit indication of WAB-node in HO request, including explicit indication of WAB-node in HO request is not needed. Thus, solution 5 should not be adopted.

In summary, solution 2 can be used for connected mode WAB-MT and solution 3 can be used for idle mode WAB-MT.

Observation 1: Multi-hop WAB topology can be avoided though solution 2 and solution 3. Solution 2 can be used for connected mode WAB-MT and solution 3 can be used for idle mode WAB-MT.

Proposal 2-1: Multi-hop topology for connected mode WAB-MT can be avoided by solution 2. Multi-hop topology for idle mode WAB-MT can be avoided by solution 3.

RAN3 should send the response LS to SA2 on the conclusion for multi-hop topology avoidance.

Proposal 2-2: RAN3 send response LS to SA2 on multi-hop topology avoidance as Annex A.

2.3 Additional ULI

According to SA2’s conclusion [5], indication of an additional ULI from the WAB-gNB to the AMF for UE is required. One LS [4] was sent from SA2 for asking RAN to discuss the issue of additional ULI and feedback:

S2-2407345 Issue-2: SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

According to clause 8.5 of TR 23.700-06 [5], SA2 has some progress on the issue of additional ULI. It was agreed that TAC/Cell ID the cell serving WAB-MT is reported as the additional ULI of UE when the PLMN serving the WAB-MT and that serving the UE is the same, whereas, 3 potential options are listed for the content of additional ULI in case the PLMN serving the WAB-MT and the PLMN serving the UE are not the same PLMN.

clause 8.5 of TR 23.700-06 - The MWAB-gNB provides to the AMF an Additional ULI. The Additional ULI is: - TAC/Cell ID of the cell serving the MWAB-UE if the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are the same PLMN. - If the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are not the same PLMN, which below option used for the Additional ULI will be determined in normative in phase. - option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM. - option 2: geo-location of the MWAB. - option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN. - The AMF may provide the received Additional ULI to LMF in LCS request. NOTE: If option 2 is selected in normative phase, support of passing Additional ULI to LMF needs to be reviewed.

In Rel-18, due to the cell ID/TAC broadcasted by the mobile IAB cell may kept unchanged during the IAB-node’s movement over a long distance, it may not reflect the real location of UE. So, there is problem to support the services that use cell ID/TAC as location reference, e.g., regulatory services. In Rel-18, the IAB-donor-CU can provide an Additional ULI which includes TAI/NCGI of the cell serving the mIAB-MT in addition to the ULI of UE, then the AMF can determine UE’s location according to the additional ULI.

The same problem is applied to WAB, i.e., the cell ID/TAC of the WAB-node cell may not be able to reflect the real location of UE. The method of reporting the additional ULI in mobile IAB can be reused for WAB. However, the AMF of UE may not recognize the cell ID/TAC of a PLMN which is not the PLMN of UE, that the AMF would not be able to derive the UE’s location if the additional ULI pertains to cell ID/TAC of a different PLMN to the UE’s PLMN. Above 3 options are presented to address this issue.

On option 1, additional ULI includes the mapped ULI information. OAM can provide the WAB-gNB the configuration of mapped TAC/Cell ID according to different geographic locations.

On option 2, additional ULI includes geolocation of WAB-node. NGAP has to introduce the new IE of WAB-node’s geolocation in UE’s ULI.

Option 3 is workable from RAN3 perspective, but it’s not clear how does AMF of a PLMN identify the cell ID of other PLMNs. The normative work is same with option 1.

From RAN3's point of view, all 3 options are feasible. It’s up to SA2 to determine which option to adopt.

Proposal 3-1: All three options for the WAB node providing additional ULI are feasible, it's up to SA2 to determine which option to use.

Proposal 3-2: RAN3 send response LS to SA2 on additional ULI as Annex A.

2.4 Handling the change of BH PDU session

According to WID [3], handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes, needs to be specified. However, this aspect has not been discussed during SI stage.

During the WAB-node’s movement, the WAB-MT’s BH PDU session used for backhauling the WAB-gNB’s traffic may be re-established, for example, due to moving to a new region or moving between TN and NTN, that will cause the (outer) IP addressed used for transmitting the WAB-gNB’s traffic being changed. The existing mechanisms defined in TS 23.501/TS 23.502 can be used for re-establishment of BH PDU session(s).

WAB-gNB need not know the reason why the BH PDU session is re-established. However, the procedures for TNL address update for SCTP connection of the WAB-gNB’s would be needed. The WAB-gNB may have to establish new SCTP connections using the new TNL address for NG/Xn connection. Besides that, on NG interface, the WAB-gNB initiates RAN Configuration Update procedure/PDU Session Resource Modify Indication procedure to update the TNL addresses for NG-C and NG-U respectively. On Xn interface, the WAB-gNB can initiate NG-RAN Node Configuration Update procedure to update the TNL addresses for Xn-C and Xn-U.

Proposal 4: WAB-gNB may update the TNL addresses used for NG or Xn connection using the new IP address of the BH PDU session when the BH PDU session is re-established. The TNL addresses for NG or Xn connection can be updated using existing mechanism.

2.5 Support the UE’s AMF change

The following is captured in SA2’s TR for Rel-19 MWAB [5]:

… - It is assumed that when MWAB-gNB changes the AMF it connects due to mobility, the TAC(s) and Cell ID(s) it announces also change. NOTE 1: The change of TAC triggers MRU of CM-IDLE UE(s) and change of Cell ID (including gNB-ID) triggers N2 handover of CM-CONNECTED UE(s) (the source AMF decides the new AMF during a N2 HO preparation). … - The MWAB instantiates a new logical MWAB gNB when it is configured with a new TAC (and possibly Cell ID ) and with a different gNB ID. This may be used to support migrating / handing over of UEs served by the MWAB to the new logical gNB. Once all UEs have been migrated, the MWAB removes the old logical MWAB-gNB instance. This can happen e.g. when AMF, serving the UEs of the MWAB, change is required based on the operator policy. NOTE 4: This aspect will be coordinated with RAN WG3. Whether the gNB ID will be changed will be reviewed in normative phase based on RAN WG3 input taking into account potential AMF impacts. NOTE 5: The solution based on instantiating multiple virtual cells by the same MWAB-gNB needs to wait for RAN WG3 confirmation.

It can be observed that SA2 has agreed NG handover is used for changing the AMF of connected UEs, and the change of Cell ID of WAB-gNB triggers the NG handover.

Observation 2: SA2 has agreed to use NG handover for changing the AMF of connected mode UEs.

However, other solutions for changing AMF of connected UEs are proposed in TR 38.799 [2].

Option 1: Single WAB-gNB with a single cell using mobility registration update due to TAC change.

Option 2: Single WAB-gNB with two cells with different TACs, using NG-based HO.

Option 3: Single WAB-gNB single cell without TAC change.

Noted that Option 1 has been discussed by SA2 during the SI phase, but it was not adopted. Option 3 declares TAC is unchanged in cased of AMF reselection, however, SA2 has concluded the TAC(s) the WAB-gNB broadcasts should change in addition to the Cell ID in that case as the new AMF(s) control different TAs compared to the old AMF(s). So, neither Option 1 nor Option 3 is aligned with SA2’s conclusion. What’s more, Option 1 and Option 3 are NAS based solutions. They are out of RAN scope, so it’s not appropriate to discuss them in RAN3.It can be observed the solution captured by SA2 for changing UE’s AMF is based on the scenario where the gNB ID is configured with a different gNB-ID if the AMF connecting WAB-gNB changes. There is a note saying that SA2 needs to check with RAN3 if it is possible to keep the gNB-ID unchanged. So, the leftover issue for RAN3 is whether the NG-handover solution for changing AMF of connected UEs is feasible when gNB-ID is unchanged, i.e., whether Option 2 is workable.

Observation 3: The leftover issue is whether the NG-handover solution is workable when the gNB-ID is unchanged for WAB-gNB.

Someone would concern about that old AMF cannot locate the new AMF during NG handover, when gNB-ID of source node is the same as that of target node. Actually, the old AMF does not locate the new AMF based on gNB-ID of the target node, as AMF does not know which gNB is connecting with other AMF(s). Therefore, if there is problem on selecting target AMF for the same gNB-ID case, the same issue is valid when gNB-ID is different between the source and target nodes.

In legacy NG handover, the source AMF uses the target TAI to select the target AMF. “Selected TAI” of Target ID IE in the Handover Required message is set by the source gNB as the TAI broadcasted in the target cell. The TAI broadcasted by target cell belongs to the target AMF, thus, the source AMF can locate the target AMF based on this “selected TAI” IE. Following is the definition for Target ID IE in TS 38.413 [6]:

#### 9.3.1.25 Target ID This IE identifies the target for the handover. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. CHOICE Target ID; M; -. >NG-RAN. >>Global RAN Node ID; M; 9.3.1.5; -. >>Selected TAI; M; TAI 9.3.3.11; -. >>Selected Target SNPN Identity; O; NID 9.3.3.42; This IE together with the PLMN Identity IE included in the Selected TAI IE indicates the target SNPN.; YES; reject. >E-UTRAN. >>Global eNB ID; M; Global ng-eNB ID 9.3.1.8; -. >>Selected EPS TAI; M; EPS TAI 9.3.3.17; -.

In the WAB mobility scenario, as the WAB-node moves to a new area controlled by the new AMF, the new cell of WAB-gNB should broadcast a new TAI corresponding to the new area. AMF of the connected UEs should be changed. The old AMF can locate the new AMF based on the new TAI (i.e., target TAI of UE) controlled by the new AMF. This new TAI is related to the WAB-gNB’s current location. Given the new TAI and new AMF are configured to WAB-gNB according to its geo-location, after the WAB-node enters a new area of a new AMF and sets up NG connection with the new AMF, the WAB-gNB can initiate a NG handover to the old AMF and set the “selected TAI” IE in Handover Required message as the new TAI belonging to the new AMF. Then the old AMF can locate the new AMF based on the “selected TAI”.

Proposal 5-1: When WAB-gNB initiates NG handover to change AMF of UE, it sets the “selected TAI” IE in Handover Required message as the new TAI corresponding to its current location for the old AMF to select the target AMF.

As selection of the target/new AMF during NG handover is based on the target TAI of UE, rather than the gNB-ID of target node, unchanging gNB-ID of WAB-gNB will not impact new AMF selection during NG handover for changing UE’s AMF.

Observation 4: Unchanging gNB-ID of WAB-gNB will not impact new AMF selection during NG handover for changing UE’s AMF.

The solution for changing UE’s AMF using NG based handover in NTN scenario is captured by TS 38.300 [7]:

16.14.6 AMF (Re-)Selection The gNB implements the NAS Node Selection Function specified in TS 38.410 [16]. For an RRC_CONNECTED UE, when the gNB is configured to ensure that the UE connects to an AMF that serves the country in which the UE is located, if the gNB detects that the UE is in a different country to that served by the serving AMF, then it should perform an NG handover to change to an appropriate AMF, or initiate an UE Context Release Request procedure towards the serving AMF (in which case the AMF may decide to de-register the UE). For the purpose of selecting an appropriate AMF, the 5GC may verify the UE location according to TS 23.501 [3] and TS 38.305 [42] after the UE has attached to the network. NOTE: UE location verification for AMF selection should not be necessary if NTN cell(s) do not extend across countries.

It can be observed that, the UE’s serving gNB can initiate a legacy NG handover to change the UE’s AMF without changing the serving gNB. That means no enhancement to legacy NG handover is needed for changing the UE’s AMF under the same gNB, and there is no potential impact to AMF to do that.

It is concluded that NG handover can be used to change AMF for the connected UEs in case gNB-ID of WAB-gNB is not changed.

Proposal 5-2: NG handover can be used to change AMF for the connected UEs in case gNB-ID of WAB-gNB is not changed.

Per SA2’s conclusion, the WAB-gNB can power up two cells corresponding to the old and new configurations respectively, i.e., WAB-gNB powers up the new cell after setting up NG connection with the new AMF and shut down the old cell after UEs’ handovers. From RAN point of view, that is beneficial for performing the interruption-less HO for UE.

However, some WAB-gNBs would not support multiple cells in parallel, which means those WAB-gNBs cannot keep multiple physical cells activated simultaneously.

For WAB-nodes not capable of multiple physical cells, the WAB-gNB cannot keep the old and the new cells activated at the same time. The WAB-gNB can deactivate the old cell after all the HO commands are sent to UEs, and then activate the new cell. The UEs which have received the HO command can search the new cell when the new cell(s) is activated and connect with the new cell.

Proposal 5-3: WAB-gNB can instantiate multiple cells or it can activate the new cell(s) after deactivating the old cell(s). It depends on WAB-gNB’s capability/implementation whether to instantiate multiple cells simultaneously.

Proposal 5-4: RAN3 send response LS to SA2 on the change of UE’s AMF as Annex B.

2.6 NG connection management

During the WAB-node movement, if the AMF(s) that WAB-gNB should connect and the TAI(s) broadcasted by the WAB-gNB change, the WAB-gNB will not serve the old TAI(s) and the old AMF(s) any more. The WAB-gNB may stop using the NG connection with the old AMF(s) after all the UEs connected to the old AMF(s) are handed over to the new AMF(s). The old AMF(s) are still reachable because the BH PDU session providing the IP connection to NG connections with the old AMF(s) is available. The WAB-gNB can remove the NG connections and keep the underlying SCTP connection alive. The removal of NG connection during WAB-node’s movement should be specified. Noted that BL CRs to TS 38.300 and TS 38.413 on NG connection removal have been approved by NTN. We can specify NG removal for WAB scenario in TS 38.300.

In past meeting, the introduction of NG suspend/resume is proposed. Since the WAB-node does not serve the old AMF(s) anymore, the old AMF(s) should not use the corresponding NG connections, e.g., sending paging messages to the WAB-node. Therefore, the WAB-node has to notify the old AMF(s) that the NG interface is not available through either new NGAP signalling or existing NGAP signalling (i.e., implicitly). Also, the WAB-node has to inform those AMF(s) to resume the NG connection when the WAB-node moves back. From our perspective, given the removal of NG connection is adopted, there is no need to introduce NG connection suspension.

But NG connection suspend/resume has not been discussed in the last meeting, it will be further checked in the coming RAN3 meetings. We can wait for NTN whether to introduce NG suspend/resume procedures.

Proposal 6-1: Specify NG removal for WAB scenario in TS 38.300.

Proposal 6-2: Whether to introduce NG suspend/resume can wait for NTN's conclusion.

2.7 PCI collision avoidance

For PCI collision avoidance, as the scenario is quite similar with mobile IAB, the method of mobile IAB can be used as baseline for WAB. And it has only impact to stage-2 spec.

Proposal 7: The mechanism for PCI collision avoidance in mobile IAB can be reused for WAB.

3 Conclusion

In the previous sections we made the following proposals:

WAB authorization

Proposal 1-1: Capture the WAB authorization function in TS 38.401 including the WAB-node behaviors in case authorization status of WAB-gNB/WAB-MT changes.

Proposal 1-2: The WAB-node releases the SCTP connections and BH PDU session(s) of the WAB-MT if the authorization status of WAB-MT is changed to “not authorized”, and re-establish the BH PDU session(s) if the authorization status of WAB-MT is changed back to “authorized”.

Avoid multi-hop WAB topology

Observation 1: Multi-hop WAB topology can be avoided though solution 2 and solution 3. Solution 2 can be used for connected mode WAB-MT and solution 3 can be used for idle mode WAB-MT.

Proposal 2-1: Multi-hop topology for connected mode WAB-MT can be avoided by solution 2. Multi-hop topology for idle mode WAB-MT can be avoided by solution 3.

Proposal 2-2: RAN3 send response LS to SA2 on multi-hop topology avoidance as Annex A.

Additional ULI

Proposal 3-1: All three options for the WAB node providing additional ULI are feasible, it's up to SA2 to determine which option to use.

Proposal 3-2: RAN3 send response LS to SA2 on additional ULI as Annex A.

Handling the change of BH PDU session

Proposal 4: WAB-gNB may update the TNL addresses used for NG or Xn connection using the new IP address of the BH PDU session when the BH PDU session is re-established. The TNL addresses for NG or Xn connection can be updated using existing mechanism.

Support the UE’s AMF change

Observation 2: SA2 has agreed to use NG handover for changing the AMF of connected mode UEs.

Observation 3: The leftover issue is whether the NG-handover solution is workable when the gNB-ID is unchanged for WAB-gNB.

Proposal 5-1: When WAB-gNB initiates NG handover to change AMF of UE, it sets the “selected TAI” IE in Handover Required message as the new TAI corresponding to its current location for the old AMF to select the target AMF.

Observation 4: Unchanging gNB-ID of WAB-gNB will not impact new AMF selection during NG handover for changing UE’s AMF.

Proposal 5-2: NG handover can be used to change AMF for the connected UEs in case gNB-ID of WAB-gNB is not changed.

Proposal 5-3: WAB-gNB can instantiate multiple cells or it can activate the new cell(s) after deactivating the old cell(s). It depends on WAB-gNB’s capability/implementation whether to instantiate multiple cells simultaneously.

Proposal 5-4: RAN3 send response LS to SA2 on the change of UE’s AMF as Annex B.

NG connection management

Proposal 6-1: Specify NG removal for WAB scenario in TS 38.300.

Proposal 6-2: Whether to introduce NG suspend/resume can wait for NTN's conclusion.

PCI collision avoidance

Proposal 7: The mechanism for PCI collision avoidance in mobile IAB can be reused for WAB.

4 Reference

- RAN3#125bis agenda, RAN3 Chair

- R3-244849, 3GPP TR 38.799,  Study on additional topological enhancements for NR, v1.2.0 (2024-08)

- RP-242359, New WID on additional topological enhancements for NR, NTT DOCOMO, INC., AT&T, RAN#105

- S2-2407345, LS on questions regarding FS_VMR_Ph2, SA2#163

- 3GPP TR 23.700-06, Study on architecture enhancements for vehicle-mounted relays - Phase 2, v 19.0.0 (2024-06)

- 3GPP TS 38.413,  NG Application Protocol (NGAP), V18.3.0 (2024-09)

- 3GPP TS 38.300, NR and NG-RAN Overall Description, Stage-2, v18.3.0 (2024-09)

5 Annex A (Proposed response LS)

Title:	[DRAFT] Response LS on questions regarding FS_VMR_Ph2

Response to:	Reply LS on questions regarding FS_VMR_Ph2 (S2-2407345)

Release:	Rel-19

Work Item:	FS_NR_WAB_5GFemto

Source:	CATT (to be RAN3)

To:	SA2

Cc:	RAN2

Contact Person:

#### Name:	Yang Lu

Tel. Number:

E-mail Address:	luyang@catt.cn

Attachments:

1. Overall Description:

RAN3 thanks SA2 for providing issues on the usage of VMR after progressing the Rel-19 FS_VMR_Ph2 study and documented solution proposals in TR 23.700-06. RAN3 would like to provide the following response to SA2:

- Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases:

- MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

- MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

- RAN3 Answer:

RAN3 has discussed the issue and have following conclusions:

1. By including the S-NSSAIs specific for backhauling in HO request message, a WAB-gNB can refuse the HO request for a connected mode WAB-MT.

2. WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, that the idle mode WAB-MTs can avoid (re)selecting to the cells broadcasting this indicator.

- Issue-2: SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

- RAN3 Answer:

RAN3 has discussed this issue and conclude that all three options of the additional ULI are feasible in principle considering that the PLMN serving the WAB-MT and the PLMN broadcasted by the WAB-gNB would be different. It's up to SA2 to decide which option to adopt.

2. Actions:

To SA2 group:

ACTION: 	RAN3 kindly ask SA2 to take RAN3 answer into account.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN3 Meeting	 #127, February 17 to 21, 2025          Athens, GR

TSG-RAN3 Meeting	 #127bis, April 7 to 11, 2025          China, CN

6 Annex B (Proposed response LS)

Title:	[DRAFT] Response LS on questions regarding the change of UE’s AMF

Response to:	LS on FS_VMR_Ph2 solution impacts to RAN (R3-243021)

Release:	Rel-19

Work Item:	FS_NR_WAB_5GFemto

Source:	CATT (to be RAN3)

To:	SA2

Cc:	RAN2

Contact Person:

#### Name:	Yang Lu

Tel. Number:

E-mail Address:	luyang@catt.cn

Attachments:

1. Overall Description:

RAN3 thanks SA2’s consulting after concluding the Rel-19 FS_VMR_Ph2 study and documented solution proposals in TR 23.700-06. RAN3 would like to provide the following response to SA2:

- Question 3: To support mobility of the MWAB, some solutions assume that the MWAB-gNB can instantiate two cells (with same gNB ID or different gNB ID), and handover connected UEs between the two cells. The different gNB IDs use case is driven by the need to change AMF if the MWAB moves into a geographic area where a different AMF must be chosen to serve UEs. SA2 would like to ask RAN3 to confirm if this can be supported or not.

- RAN3 Answer:

RAN3 believe that the NG-handover solution that SA2 mentioned in Q3 for changing AMF for connected UEs is also workable when the gNB-ID is unchanged for WAB-gNB.

Besides, RAN3 assume that the MWAB-gNB can instantiate multiple cells or it can activate the new cell(s) after deactivating the old cell(s). It depends on MWAB-gNB’s capability/implementation whether to instantiate multiple cells simultaneously.

2. Actions:

To RAN2 group:

ACTION:  RAN3 kindly ask SA2 to take RAN3 answer into account.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN3 Meeting	 #127, February 17 to 21, 2025          Athens, GR

TSG-RAN3 Meeting	 #127bis, April 7 to 11, 2025          China, CN