3GPP TSG-RAN WG3 Meeting #126                                                                    R3-247269

Orlando, USA, November 18th– 22nd, 2024

Source:	CATT

Title:	Discussion on enhancements for WAB

Agenda Item:	12.2

Document for:	Discussion and Decision

1 Introduction

In RAN3#125bis meeting, some aspects on support of WAB were discussed. According to [1] and [2], some basic aspects for WAB are proposed to be captured in stage-2 spec.

In this contribution, we will discuss potential enhancements to WAB.

2 Discussion

2.1 Xn connection management

2.1.1 Dynamic Xn setup

Based on the WID approved during RAN#105 [3], Xn interface setup for the WAB-gNB with BH-RAN-node or surrounding NG-RAN nodes should be specified, which has not been captured in [3]. Different to normal gNB, Xn setup is dynamic for WAB-gNB, due to the neighbour relationship may change with the movement of WAB-node.

As a legacy mechanism, UEs served by WAB-node can detect the PCI of the neighbour cell of WAB-gNB, and UE’s CGI reporting procedure can be used to derive the RAN-node ID of the neighbour nodes. However, the CGI acquisition is optionally supported by UE. Other method should be studied. Basically, the cell serving the WAB-MT under the BH-RAN-node is probably the nearest neighbour cell of WAB-gNB. As the RAN-node ID of the BH-RAN-node is broadcasted in SIB1, the WAB-gNB can recognize the BH-RAN-node after the WAB-MT obtaining SIB1 of the serving cell. Then, the WAB-gNB obtains IP address of the BH-RAN-node and sets up Xn connection with the BH-RAN-node. IP address of the BH-RAN-node can be obtained by the legacy Xn-C TNL address discovery mechanism. This procedure only impacts stage-2.

Proposal 1-1: WAB-gNB can recognize BH-RAN-node via SIB1 of the cell serving the WAB-MT.

Proposal 1-2: WAB-gNB obtains IP address of the BH-RAN-node via legacy Xn-C TNL address discovery mechanism and sets up Xn connection with it.

If setting up Xn with the BH-RAN-node, the WAB-gNB may discover some neighbour cells’ CGI from the serving cell information delivered by the BH-RAN-node over Xn interface.

There may be other neighbour cells of WAB-gNB, which are supposed in scope of the neighbour cells of the BH-RAN-node. The WAB-gNB can also discover neighbour cells’ CGI from the neighbor cells of the BH-RAN-node. The information of BH-RAN-node’s neighbour cells is sent from BH-RAN-node via Neighbour Information NR IE or Neighbour Information E-UTRA IE in some Xn messages, including Xn SETUP REQUEST/RESPONSE message, NG-RAN NODE CONFIGURATION UPDATE message, and NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

After discovering the neighbour cells’ CGI, the WAB-gNB can add the neighbour cells to the NCRT and look up the TNL address of the neighbour nodes to set up Xn interface. The WAB-gNB can get the IP addresses of the neighbour RAN-nodes using the legacy Xn-C TNL address discovery mechanism. This procedure impacts only stage-2.

Proposal 1-3: If WAB-gNB establishes Xn connection with BH-RAN-node, it discovers neighbour cells through Xn messages sent from the BH-RAN-node.

Proposal 1-4: WAB-gNB looks up the IP address(es) of the neighbour nodes using legacy Xn-C TNL address discovery mechanism and set up Xn connections with surrounding RAN nodes.

2.1.2 Avoiding Xn setup between WAB-gNBs

According to the WID approved during RAN#105 [3], setting up Xn between WAB-gNBs should be avoided. Due to the mobility of WAB-node is unpredictable, the WAB-gNBs is recommended not to set up Xn connection with another WAB-gNB in most cases, in order to avoid the frequent Xn setup and removal. For avoidance of setting up Xn between WAB-gNBs, WAB cell specific info (e.g., including a WAB-cell indication) can be introduced to Xn Setup Request message, so that a WAB-gNB can refuse the Xn setup request from another WAB-gNB.

On the other hand, when each WAB-gNB sets up Xn connections with its BH-RAN-node or other neighbour RAN nodes, it can indicate the WAB cell specific info using the Xn Setup Request/Response messages. If the BH-RAN node and neighbour RAN nodes are upgraded to support WAB enhancements, they can recognize that information. Then the BH-RAN-node or neighbour nodes can further spread the information to their neighbour by the neighbour cell information IE in Xn messages, that the WAB-gNB can discover the WAB cells in the neighbour cell information of Xn messages. It is observed that the WAB-gNB can discover the neighboring WAB cells and prevent to set up Xn with the neighbor WAB-gNB(s) if WAB specific info is added to the serving cell information IE and the neighbour cell information IE of Xn interface.

Take following figure as an example: WAB-node 1 indicates the WAB cell info (including a WAB-Cell indicator) to BH-RAN-node 1 by Xn connection and WAB-node 2 indicates the WAB cell info to both BH-RAN-node 2 and BH-RAN-node 1 over Xn. BH-RAN-node 1 can know the WAB cell of WAB-node 2 directly via Xn message from WAB-node 2 or via Xn message from BH-RAN-node 2. BH-RAN-node 2 knows the WAB cell of WAB-node 1 via Xn messages from BH-RAN-node 1. As a result, WAB-node 1 can know the WAB cell of WAB-node 2 through the neighbour cell information sent by BH-RAN-node 1, WAB-node 2 can know the WAB cell of WAB-node 1 from the neighbour cell information sent by BH-RAN-node 2 or BH-RAN-node 1.

Proposal 1-5: WAB cell specific info can be added to the serving cell information and the neighbour cell information in Xn messages.

2.1.3 Dynamic Xn removal

Due to the mobility of WAB-node, the NCRT in the WAB-gNB may change more frequently than legacy gNB. The WAB-node would need to set up Xn with new neighbour nodes and release Xn with the original neighbour nodes during its movement. The WAB-node can discover PCI of the new neighbour cell through the UE’s measurement result.

If the WAB-MT connects to a new BH-RAN-node and WAB-gNB establishes Xn with the new BH-RAN-node, the WAB-gNB can derive CGI of the current neighbour cells through serving cell information or neighbour cell information sent by the new BH-RAN-node over Xn. The WAB-gNB may add the new neighbour cells to the neighbour relationship, and set up Xn connection with the new neighbour nodes. Accordingly, the WAB-gNB would have to remove cells from the NCRT dynamically.

The WAB-gNB will consider the cell(s), which is not included in the serving/neighbour cell information of current BH-RAN-node, no longer acting as the neighbouring cells and remove the cell(s) from the NCRT. If there is any NG-RAN node which has already set up Xn connection with the WAB-gNB no longer serves the neighbour cell of the WAB-gNB, the WAB-gNB can initiate Xn removal to the NG-RAN node.

Proposal 1-6: WAB-gNB considers the cell not included in the serving/neighbour cell information of current BH-RAN-node as no longer acting as the neighbour cell.

Proposal 1-7: WAB-gNB removes Xn with the with the NG-RAN nodes which do not serve any neighbour cell of the WAB-gNB.

2.2 Resource multiplexing

Based on the WID approved during RAN#105 [3], radio-resource coordination between access and backhaul links should be specified. According to the TR [4], the resource coordination mechanism introduced for IAB can be considered as the starting point to resource multiplexing for WAB.

Resource multiplexing for IAB is specified in Rel-16/17. The transmission of each IAB-DU’s cell depends on resource configuration or implicit determination to make sure the access link will have no impact to the BH link. The same principle should be applied to WAB, that DL/UL transmission of each WAB-gNB’s cell should be constrained for potential impacts to transmission over the BH link.

Each slot is assigned to be hard, soft, or unavailable for D/U/F symbols for IAB-DU’s cell. When a symbol is configured as hard, the IAB-DU can use it for schedule. When a symbol is configured to be unavailable, the IAB-DU cannot use it except in some special cases. When a symbol is configured as soft, the IAB-DU can conditionally use it in cases where the parent node explicitly indicates the availability to the co-located IAB-MT via DCI format 2_5, or up to IAB-node’s decision by implementation.

Similar to IAB, there can be three types for D/U/F symbols in a slot for WAB-gNB’s cell. The hard symbols can be used by WAB cell, the unavailable symbols cannot be used by WAB cell and the soft symbols can be conditionally used by WAB cell depending on whether it has interference to the backhaul link. WAB-gNB can determine whether the soft symbols can be used relying on implementation. Different with IAB, a DCI for indicating the availability of soft symbols is not recommended to be considered for WAB, as the impact to RAN1 spec.

Proposal 2-1: Three types of symbol (i.e., hard, soft and unavailable) can be used for resource multiplex of WAB. WAB-gNB can determine whether the soft symbols is used relying on implementation.

- Resource coordination info

To perform resource multiplexing between access link and backhaul link, resource information for each cell of BH-RAN-node and each cell of WAB-gNB is exchanged between the BH-RAN-node and the WAB-node. WAB-node can send resource configurations of the WAB-node to the BH-RAN-node. The resource configuration of the WAB-gNB’s cell contains the configuration for slot format and symbol type (H/S/NA) of each slot. The slot format indicates the number and position of D/U/F symbols. On the other hand, the BH-RAN-node can also send the proposed slot format and/or symbol type of each slot for each WAB-gNB’s cell to the WAB node. As the consequence, the WAB node can modify the resource configuration (slot format and/or symbol type of each slot) base on the proposal from the BH-RAN-node.

The BH-RAN-node can send resource configurations of its cell(s) to the WAB-node. Since BH-RAN-node is legacy NG-RAN node, the resource configuration of BH-RAN-node’s cell only needs to include the slot format to indicate the number and position of D/U/F symbols. Besides that, the WAB node can send the proposed slot format of the BH-RAN-node’s cell to the BH-RAN-node, then BH-RAN-node can modify the slot format based on the proposal from the WAB-node.

Observation 1: The resource configuration of WAB-gNB’s cell(s) including slot format and symbol type of each slot can be exchanged between WAB-gNB and BH-RAN node. The resource configuration of BH-RAN-node’s cell(s) including slot format can be exchanged between WAB-gNB and BH-RAN node.

The multiplexing capability of WAB-node should also be sent to the BH-RAN-node. The multiplexing capability includes simultaneous operation of WAB-MT Rx / WAB-gNB Rx, WAB-MT Tx / WAB-gNB Tx, WAB-MT Rx / WAB-gNB Tx, WAB-MT Tx / WAB-gNB Rx, should also be supported by WAB. If the simultaneous operation of WAB-MT and WAB-gNB is not supported, time multiplexing between access and BH links should be applied, otherwise, frequency multiplexing can be utilized between access and BH links.

In IAB, the cell specific resources configured for the IAB-DU cell are considered as hard resources of IAB-DU cell, thus the relevant configurations are sent to the IAB-donor CU, including STC configuration, RACH Config Common, CSI-RS Configuration, SR Configuration and PDCCH Configuration SIB1, and used for resource coordination. Similarly, the cell specific resources configured for the WAB-gNB cell should be sent to the BH-RAN node for resource coordination.

Observation 2: The multiplexing capability of WAB-node and cell specific resources configuration for WAB-gNB cell(s) should be sent to the BH-RAN node.

Proposal 2-2: The resource coordination information sent from WAB-node to BH-RAN node includes:

- Resource configuration including slot format and symbol type for each WAB-gNB cell,

- (Proposed) resource configuration including slot format for BH-RAN-node’s cell(s),

- WAB-node duplex capability,

- Cell specific resources configured for each WAB-gNB cell.

Proposal 2-3:  The resource coordination information sent from BH-RAN-node to WAB-node includes:

- Resource configuration including slot format for each cell,

- (Proposed) resource configuration including slot format and/or symbol type for WAB-gNB’s cell(s).

- Use Xn or RRC to exchang resource coordination information

The resource coordination information would be exchanged between WAB-node and its BH-RAN-node via RRC or XnAP (If Xn interface between BH-RAN-node and WAB-node is established).

As the solution of using RRC signaling can be done without the Xn connection between WAB-node and BH-RAN-node, this solution is less constraint. However, it has large impact to the RRC specification.

Using XnAP signaling to achieve resource coordination is similar with the mechanism of IAB, but the disadvantage is that it relies on the Xn connection between the BH-RAN-node and WAB-gNB. Further, it requires co-location discovery of WAB-MT and WAB-gNB.

Observation 3: If the resource coordination is done by XnAP, the BH-RAN-node needs to discover co-location of WAB-MT and WAB-gNB.

However, considering resource multiplexing is optional and Xn is also optional, using Xn is feasible. Although co-location discovery is required, limited impact is expected for XnAP. In our view, the resource coordination information can be sent between access and backhaul links via XnAP.

Proposal 2-4: Exchanging resource coordination information between BH-RAN-node and WAB-node over Xn.

- Discover co-location of WAB-MT and WAB-gNB

The identity of WAB-MT is used over RRC between the BH-RAN-node and the WAB-node, while the gNB ID of WAB-gNB is used over Xn between the BH-RAN-node and the WAB-node. BH-RAN-node can discover co-location of the WAB-MT and the WAB-gNB by either the gNB ID of WAB-gNB or the identity of WAB-MT. According to the WAB-node integration procedure, the WAB-MT of a WAB-node connects to the network in the way as a UE by performing RRC connection setup procedure with the BH-RAN-node in phase 1. The WAB-gNB establishes Xn connection towards the BH-RAN-node in phase 2-3.

There are two options for co-location discovery:

Option 1: WAB-MT reporting identity of WAB-gNB.

The WAB-MT reports the gNB ID of its co-located WAB-gNB to the BH-RAN-node via RRC message, e.g., during phase 1 for WAB-MT setup. The BH-RAN-node can discover the WAB-MT is co-located with the WAB-gNB through the gNB ID, e.g., when the WAB-gNB sets up Xn interface with it during phase 2-3.

Option 2: WAB-gNB reporting identity of WAB-MT.

The BH-RAN-node allocates C-RNTI in the servicing cell to WAB-MT in phase 1. WAB-gNB sends the C-RNTI and the cell identity of the serving cell of its co-located WAB-MT to the BH-RAN-node using XnAP signaling, e.g., during the Xn setup procedure in phase 2-3. BH-RAN-node can discover co-location of the WAB-MT and the WAB-gNB via the C-RNTI and cell identity.

Comparing the two options, option 2 has an advantage over option 1 on avoiding the impact to RRC. It is proposed to adopt option 2 for co-location discovery.

Proposal 2-5: BH-RAN-node discovers the co-location of WAB-gNB WAB-MT via the WAB-gNB reporting the identity of the WAB-MT over Xn.

3 Conclusion

In the previous sections we made the following proposals:

Xn connection management

Proposal 1-1: WAB-gNB can recognize BH-RAN-node via SIB1 of the cell serving the WAB-MT.

Proposal 1-2: WAB-gNB obtains IP address of the BH-RAN-node via legacy Xn-C TNL address discovery mechanism and sets up Xn connection with it.

Proposal 1-3: If WAB-gNB establishes Xn connection with BH-RAN-node, it discovers neighbour cells through Xn messages sent from the BH-RAN-node.

Proposal 1-4: WAB-gNB looks up the IP address(es) of the neighbour nodes using legacy Xn-C TNL address discovery mechanism and set up Xn connections with surrounding RAN nodes.

Proposal 1-5: WAB cell specific info can be added to the serving cell information and the neighbour cell information in Xn messages. Proposal 1-6: WAB-gNB considers the cell not included in the serving/neighbour cell information of current BH-RAN-node as no longer acting as the neighbour cell.

Proposal 1-6: WAB-gNB considers the cell not included in the serving/neighbour cell information of current BH-RAN-node as no longer acting as the neighbour cell.

Proposal 1-7: WAB-gNB removes Xn with the with the NG-RAN nodes which do not serve any neighbour cell of the WAB-gNB.

Resource multiplexing

Proposal 2-1: Three types of symbol (i.e., hard, soft and unavailable) can be used for resource multiplex of WAB. WAB-gNB can determine whether the soft symbols is used relying on implementation.

Observation 1: The resource configuration of WAB-gNB’s cell(s) including slot format and symbol type of each slot can be exchanged between WAB-gNB and BH-RAN node. The resource configuration of BH-RAN-node’s cell(s) including slot format can be exchanged between WAB-gNB and BH-RAN node.

Observation 2: The multiplexing capability of WAB-node and cell specific resources configuration for WAB-gNB cell(s) should be sent to the BH-RAN node.

Proposal 2-2: The resource coordination information sent from WAB-node to BH-RAN node includes:

- Resource configuration including slot format and symbol type for each WAB-gNB cell,

- (Proposed) resource configuration including slot format for BH-RAN-node’s cell(s),

- WAB-node duplex capability,

- Cell specific resources configured for each WAB-gNB cell.

Proposal 2-3:  The resource coordination information sent from BH-RAN-node to WAB-node includes:

- Resource configuration including slot format for each cell,

- (Proposed) resource configuration including slot format and/or symbol type for WAB-gNB’s cell(s).

Observation 3: If the resource coordination is done by XnAP, the BH-RAN-node needs to discover co-location of WAB-MT and WAB-gNB.

Proposal 2-4: Exchanging resource coordination information between BH-RAN-node and WAB-node over Xn.

Proposal 2-5: BH-RAN-node discovers the co-location of WAB-gNB WAB-MT via the WAB-gNB reporting the identity of the WAB-MT over Xn.

4 Reference

- R3-245820, Support for Wireless Access Backhaul, Qualcomm, Ericsson, CATT, ZTE, Nokia, Nokia Shanghai Bell, RAN3#125bis

- R3-245836, Support for Wireless Access Backhaul,  Ericsson, ZTE, Nokia, Nokia Shanghai Bell, RAN3#125bis

- RP-242359, New WID on additional topological enhancements for NR, NTT DOCOMO, INC., AT&T, RAN#105

- R3-244849, 3GPP TR 38.799, Study on additional topological enhancements for NR, v1.2.0 (2024-08)