3GPP TSG-RAN WG3 Meeting #126                                                                    R3-247270

Orlando, USA, November 18th– 22nd, 2024

Source:	CATT

Title:	Discussion on stage-2 NR Femto

Agenda Item:	12.3

Document for:	Discussion and Decision

1 Introduction

In the last meeting, additional topological enhancements for NR are studied and a TP [1] are produced. According [1], the architecture and protocol stacks of NR Femto are captured in stage-2 spec.

In this contribution, we will continue our discussion about the other impact on stage-2 for NR Femto.

2 Discussion

2.1 Architecture

According to the WID [2] in RAN#105 meeting, the NR Femto architecture with optional NR Femto GW for NG interface is to be supported. In [1], the architecture of NR Femto is captured into TS 38.300. Further, refinement or supplement would be needed for the architecture. Besides the current architecture in the TR, following figure should also be captured into TS 38.300 to provide an overall picture of NG-RAN when applying NR Femto and NR Femto GW. Note that the NR Femto GW can optionally terminate the NR-U to the NR Femto and to the UPF, and relay NR-U data between the NR Femto and the UPF, even if NR Femto GW is deployed for concentrating NG-C interface between NR Femto and AMF.

Figure 1: NR Femto Architecture

Proposal 1: Capture above figure into TS 38.300 for NR Femto architecture.

2.2 Protocol stack

As a TP to BLCR for TS 38.300 [1], the protocol stack for NG control plane with the NR Femto GW is captured into TS 38.300. However, the protocol stacks for NG user plane is FFS.

For NG user plane, some companies proposed that the NG-U interface for NR Femto is same as the existing protocol stack in TS 38.300 [3], so there is no need to capture a separate protocol stack for NG user plane for NR Femto.

That is true when NR Femto GW is not deployed or when the NR Femto GW does not terminate the user plane between the NR Femto and the UPF. But the transport of user plane may be optionally concentrated in the NR Femto GW, which is already captured in the TP [1]. Considering that, the GTP-U tunnel would be changed by the GW based on implementation during transport between NR Femto and UPF. As the Tunnel Endpoint Identifier (TEID) contained in GTP-U header uniquely identifies the endpoint of a GTP-U tunnel, the identity of the GTP-U tunnel may be modified by the NR Femto GW. That means the NR Femto GW may terminate the user plane between NR Femto and UPF. This procedure cannot be illustrated by the existing protocol stack. Therefore, the protocol stack for NG user plane for NR Femto which illustrates the NR Femto GW terminating user plane needs to be captured into TS 38.300.

The protocol stack for NG User plane with the NR Femto GW terminating the user plane is shown as below:

Figure 2: User plane for NG interface for NR Femto to UPF with NR Femto GW

Proposal 2: TS 38.300 captures the protocol stack of NG user plane for NR Femto with NR Femto GW terminating the user plane.

In the last meeting, the protocol stacks for NG control plane without the NR Femto GW is also FFS.

If the NR Femto GW is not deployed, the control plane can just refer to the existing protocol stack in 38.300 [3], a separate protocol stack for NR control plane is not needed in this case.

So, there is no need to capture the protocol stacks for both NG-C and NG-U in case NR Femto GW is not deployed.

Proposal 3: TS 38.300 does not capture the protocol stacks for NG-U and NG-C for NR Femto without the NR Femto GW.

Proposal 4: Approve the TP for protocol stacks for NG interface in Annex.

2.3 Function

A NR Femto node hosts the same functions as a gNB, with the following additional specifics in case of connection to the NR Femto GW:

- A NR Femto node shall only connect to a single NR Femto GW at one time. The NR Femto node should not simultaneously connect to another NR Femto GW, or other AMF/UPF.

- The TAC and PLMN ID used by the NR Femto node shall also be supported by NR Femto GW.

- Selection of an AMF at UE attachment is hosted by the NR Femto GW instead of the NR Femto node. Upon reception of the GUAMI from a UE, the NR Femto node shall include it in the INITIAL UE MESSAGE message.

- NR Femto nodes may be deployed without network planning. A NR Femto node may be moved from one geographical area to another and therefore it may need to connect to different NR Femto GWs depending on its location.

- Signalling the GUAMI of the Source AMF to the NR Femto GW in the PATH SWITCH REQUEST message.

If the NR Femto GW is deployed, we propose that the NR Femto GW hosts the following functions in TS 38.300:

- Relaying UE-associated NG application part messages between AMF serving the UE and the NR Femto node serving the UE, for example INITIAL CONTEXT SETUP REQUEST message, HANDOVER REQUEST message, HANDOVER REQUEST ACKNOWLEDGE message, PATH SWITCH REQUEST message, PATH SWITCH REQUEST ACKNOWLEDGE message.

- Terminating non-UE associated NG application part procedures towards the NR Femto node and towards the AMF, for example NG SETUP REQUEST message, PWS RESTART INDICATION message, PWS FAILURE INDICATION message, OVERLOAD START message, OVERLOAD STOP message.

- Optionally terminating NG-U interface with the NR Femto node and with the UPF.

- Supporting TAC and PLMN ID used by the NR Femto node.

- Xn interfaces shall not be established between the NR Femto GW and other nodes.

- Routing the PATH SWITCH REQUEST message towards the AMF based on the GUAMI of the source AMF received from the NR Femto node.

Proposal 5: Capture above functions of NR Femto node and functions of NR Femto GW into TS 38.300.

2.4 Access control

According to TR 38.799 [4], existing CAG mechanism for PNI-NPN is reused for access control for NR Femto.

Some companies proposed that the definitions for access modes about “open, hybrid, closed” as the TP [4] captured for NR Femto access control should be introduced in TS 38.300. However, access control for PNI-NPN have already been specified without definition of “open, hybrid and closed” access modes. Since those three kinds of access control have been supported and PNI-NPN is a superset of NR Femto, the definition for those access modes is not necessary. Thus, we proposed that the description for three kinds of access control is captured as a note, and corresponding access modes are not mentioned in TS 38.300 spec. Having said that, access modes can be added to the note as needed in future in case reference to those access modes is foreseen.

Proposal 6: Three kinds of access control is captured as a note without corresponding access modes being mentioned in spec.

Regarding to the text in the TR [4] for three types of access control, it would need to be simplified by removing some detailed description of stage-3 aspects, such as how to set the RRC IEs.

From our point of view, following text can be used to describe 3 kinds of access control for NR Femto:

- The NR Femto cell is accessible by UEs without access control of CAG.

- The NR Femto cell is shared by both PLMN and CAG. That means the cell is accessible as a NPN cell by UEs which have the allowed CAG list includes a CAG-ID broadcasted by the cell and is accessible as a normal PLMN cell by other UEs.

- The NR Femto cell is NPN-only, which means it is only accessible by UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

The TP for access control to TS 38.300 BLCR is provided in Annex.

Proposal 7: Approve the TP for access control in the Annex.

3 Conclusion

In the previous sections we made the following observations and proposals:

Proposal 1: Capture above figure into TS 38.300 for NR Femto architecture.

Proposal 2: TS 38.300 captures the protocol stack of NG user plane for NR Femto with NR Femto GW terminating the user plane.

Proposal 3: TS 38.300 does not capture the protocol stacks for NG-U and NG-C for NR Femto without the NR Femto GW.

Proposal 4: Approve the TP for protocol stacks for NG interface in Annex.

Proposal 5: Capture above functions of NR Femto node and functions of NR Femto GW into TS 38.300.

Proposal 6: Three kinds of access control is captured as a note without corresponding access modes being mentioned in spec.

Proposal 7: Approve the TP for access control in the Annex.

4 Reference

- R3-245754, TP for BL CR on Introduction of NR Femto Architecture and Protocol Aspects, Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, Huawei, LG Electronics, Samsung, Lenovo, Baicells, RAN3#125bis

- RP-242359, New WID on additional topological enhancements for NR, NTT DOCOMO, INC., AT&T, RAN#105

- 3GPP TS 38.300, NR and NG-RAN Overall Description, Stage-2, v18.3.0 (2024-09)

- R3-244849, 3GPP TR 38.799,  Study on additional topological enhancements for NR, v1.2.0 (2024-08)

5 Annex (TP for TS 38.300 BL CR)

<<<<<<<<<<<<<<<<<<<< Start of change >>>>>>>>>>>>>>>>>>>>

### 4.X.3	Interfaces

#### 4.X.3.1	Protocol Stack for NG User Plane

When the NR Femto GW is not present, the user plane is terminated at the NR Femto and the AMF.

When the NR Femto GW is deployed, it may optionally terminate the user plane towards the NR Femto and towards the UPF, and relay User Plane data between the NR Femto and the UPF. The figures below show the NG-U protocol stack with the NR Femto GW when the NR Femto GW terminates the user plane.

Figure 4.X.3.1: User plane for NG interface for NR Femto to UPF with NR Femto GW

#### 4.X.3.2	Protocol Stacks for NG Control Plane

When the NR Femto GW is not present, all the NGAP procedures are terminated at the NR Femto and the AMF.

The figure below shows the NG-C protocol stack with the NR Femto GW.

Any protocol function associated to a UE-associated procedure shall reside within the NR Femto and the AMF only.

Editor’s note: The sentence above will be captured in details in the functional split section.

Figure 4.X.3.2: Control plane for NG Interface for NR Femto to AMF with the NR Femto GW

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

### 4.X.4	Access Control

- The access control by an NR Femto node reuses CAG mechanism for PNI-NPN (see clause 4.8).

NOTE: The NR Femto node may support three kinds of access control to restrict access to UEs according to the respective subscription:

-	The NR Femto cell is accessible by UEs without access control of CAG.

-	The NR Femto cell is shared by both PLMN and CAG. That means the cell is accessible as a NPN cell by UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell and is accessible as a normal PLMN cell by other UEs.

-	The NR Femto cell is NPN-only, which means it is only accessible by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

<<<<<<<<<<<<<<<<<<<< End of changes >>>>>>>>>>>>>>>>>>>>