3GPP TSG-RAN WG3 Meeting #126	R3-247359

Orlando, USA, 18 – 22 November, 2024

Agenda item:	8.1

Source: 	Huawei

Title: 	Discussion on the usage of multi-modality information

Document for:	Discussion and Decision

# 1. Introduction

As requested by SA2 in S2-2409444 [1], RAN2 has clarified the usage of information for multi-modality awareness in R2-2409272 [2], and RAN3 was required to assess the potential usage. In this contribution, we will analyse RAN2’s feedback and provide our views on the reply to SA2 and RAN2 in [4].

# 2. Discussion

During the previous discussion in SA2 and RAN2, it has been focused on whether to forward the MMSID to NG-RAN. To confirm the solution, RAN2 provided feedback on the usage of multi-modality information in R2-2409272 as shown following.

# 1. Overall Description: RAN2 thanks SA2 for the feedback on multi-modality awareness. For the usage of multi-modality information, RAN2 has discussed and reached the following agreements during the previous RAN2 meetings. First, RAN2 agreed that the multi-modality information (e.g., MMSID), can be used for joint admission control and QoS flow to DRB mapping. - RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation). Furthermore, RAN2 also agreed that traffic synchronization may be achieved by packet level synchronization information, if it can be provided from CN to RAN, which is up to SA2/4 to discuss. - For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN. - RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the mutli-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.. # 2. Actions: # To SA2, SA4: ACTION: RAN2 kindly asks SA2, and SA4 to take the above feedback on multi-modality awareness into consideration. # To RAN3, SA4: # ACTION: According to the request from SA2, RAN2 would kindly asks RAN3 and SA4 to review the RAN2 feedback above.

From RAN3 perspective, the usage of MMSID for joint admission control is feasible. As RAN3 explained in R3-245682 [3], it is technically feasible to forward MMSID to NG-RAN via NGAP. As long as the NG-RAN node obtains the MMSID, it can take the information into consideration when performing admission control for multiple QoS flows with the same MMSID. For example, the NG-RAN node may determine to accept all the QoS flows with the same MMSID to provide a complete service for the user so that the user experience can be satisfying. However, such solution can be performed based on the network implementation. There is no need to specify any enhancement for joint admission control.

From RAN2’s LS, another usage of the MMSID is for QoS flow to DRB mapping, RAN2 has concluded it is up to the gNB implementation. So, there is no specification impacts from RAN3 point of view.

According to the original LS from SA2 in R3-245017, RAN3 is asked by SA2 to confirm the applicability and usage of MMSID, but RAN2’s reply LS in R3-247009 also mentions packet level synchronization information besides the MMSID. For the potential usages on traffic synchronization and PDU Set discard based on additional synchronization information, it is unclear what the synchronization information will be and how the information is obtained and  transferred to NG-RAN. SA2/SA4 can analyse the definition of the information first. Then RAN3 may further evaluate the applicability and feasibility if required by SA2.

Proposal 1: RAN3 reply to SA2 and RAN2 to confirm the usage of MMSID, i.e., for admission control, but how to use it should be up to RAN implementation.

Proposal 2: For the usage on traffic synchronization and PDU Set discard, RAN3 wait for SA2/SA4 on the design of synchronization information and may further evaluate the applicability if required by SA2.

# 3. Conclusion

In this contribution, we analyzed the usage of multi-modality awareness based on the feedback from RAN2. The following proposal was made:

Proposal 1: RAN3 reply to SA2 and RAN2 to confirm the usage of MMSID, i.e., for admission control, but how to use it should be up to RAN implementation.

Proposal 2: For the usage on traffic synchronization and PDU Set discard, RAN3 wait for SA2/SA4 on the design of synchronization information and may further evaluate the applicability if required by SA2.

# 4. Reference

[1]. S2-2409444, TSG SA WG2, LS reply on multi-modality awareness at RAN.

[2]. R2-2409272, TSG RAN WG2, Reply to SA2 LS on multi-modality awareness.

[3]. R3-245682, TSG RAN WG3, Reply LS on multi-modality awareness at RAN.

[4]. R3-247360, Huawei, [draft] Reply LS on usage of multi-modality information