3GPP TSG-RAN WG3 Meeting #126	R3-247416

Orlando, US, 16th – 22nd November, 2024

Title: 	(TPs for AI/ML BLCR to TS 38.423 and AI/ML BLCR to TS 38.473) Signalling details for AI/ML-based Coverage and Capacity Optimization

Source: 	Huawei

Agenda item:	11.3

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 started discussing the objectives of the new Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based CCO (Coverage and Capacity Optimization).

This contribution analyzes the signalling details to enable the AI/ML-based CCO use case to be specified over Xn and F1 interfaces, and provides proposals for discussion based on the outcome from the last RAN3 meeting.

Timing information aspects and feedback information are discussed in a separate discussion paper [6].

# 2. Discussion

In the last RAN3 meeting the following agreements and open issues were achieved [4]:

Stage3 signalling design: Xn interface: Transferring Future Coverage State and the associated Coverage Modification Cause in the NG-RAN NODE CONFIGURATION UPDATE message. The Coverage Modification Cause reflects the predicted CCO issue. F1 interface: WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1. Timing information: Introducing separate time information for future coverage state and predicted CCO issue. Time for future coverage state: The point in time when the future coverage state will be applied. Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen. Information to validate the prediction is needed.

We discuss here further the Stage 3 signalling details for Xn and F1 interfaces for the AI/ML-based CCO use case.

## 2.1 XnAP Stage 3 signalling details

In the last RAN3 meeting it was agreed to transfer the future CCO state (“Future Coverage State” in the agreement) and the associated coverage modification cause by enhancing the existing NG-RAN node Configuration Update procedure (i.e., within the NG-RAN NODE CONFIGURATION UPDATE message). From a signalling perspective, a new Future Coverage Modification List IE (similar to the existing Coverage Modification List IE within the NG-RAN NODE CONFIGURATION UPDATE message) needs to be introduced.

Proposal 1: RAN3 to agree to introduce a new Future Coverage Modification List IE within the NG-RAN NODE CONFIGURATION UPDATE message.

The new IE contains the Future Coverage Modification Item IE which, in turn, includes the NR CGI, the corresponding Future Cell Coverage State and the Future SSB Coverage Modification List IEs.

Proposal 2: RAN3 to agree to introduce, within the Future Coverage Modification Item included in the Future Coverage Modification List IE, the NR CGI indicating the Cell ID of the cell whose coverage will be modified in the future, the corresponding Future Cell Coverage State and the Future SSB Coverage Modification List IEs.

The new Future SSB Coverage Modification List IE contains the Future SSB Coverage Modification Item IE which, in turn, includes the SSB Index and the corresponding Future SSB Coverage State IEs.

Proposal 3: RAN3 to agree to introduce, within the Future SSB Coverage Modification Item included in the Future SSB Coverage Modification List IE, the SSB Index and the corresponding Future SSB Coverage State IEs.

For each Future Coverage Modification Item included in the Future Coverage Modification List IE the associated coverage modification cause needs to be indicated. Therefore, the Future Coverage Modification List IE also includes the Future Coverage Modification Cause IE which, as per the agreement from RAN3#125bis, reflects the predicted CCO issue.

Proposal 4: RAN3 to agree that each Future Coverage Modification Item included in the Future Coverage Modification List IE has a Future Coverage Modification Cause IE associated to it.

Proposal 5: RAN3 to agree that the Future Coverage Modification Cause IE follows the same design of the existing Coverage Modification Cause IE, i.e., it is an ENUMERATED type, but with codepoints “coverage” and “cell edge capacity” only.

A TP for AI/ML BLCR to TS 38.423 for AI/ML-based CCO covering the above proposals can be found in Annex 1.

## 2.2 F1AP Stage 3 signalling details

With respect to the Xn interface, the discussion from the last meeting concerning the procedures to be used to convey over F1 the predicted CCO issue (from gNB-CU to gNB-DU) and future coverage state (from gNB-DU to gNB-CU) information was not concluded and only the following Working Assumption (WA) was minuted in the meeting minutes [4]:

Stage3 signalling design: F1 interface: WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

RAN3 discussed whether the provisioning of the above information should be either:

- subscription-based mechanism, where the gNB-DU requests the gNB-CU to provide prediction information via e.g. a procedure like Data Collection Reporting Initiation and, in turn, the gNB-CU provides the predicted CCO issue (and affected cells and SSBs) to the gNB-DU via a procedure like the Data Collection Reporting. Or,

- legacy CCO mechanism, that is, the gNB-CU provides the gNB-DU with the CCO issue type and affected cells/SSBs via the gNB-CU Configuration Update procedure and, in turn, the gNB-DU determines the new CCO state for the affected gNB-DU’s cells/SSBs and provides it to the gNB-CU via the gNB-DU Configuration Update procedure.

The WA from the last meeting reflects the majority of companies in RAN3 which see benefits in re-using the existing Configuration Update procedures also for the AI/ML-based CCO use case. Companies in support of the subscription-based mechanism for the AI/ML-based CCO [5], instead, think that the subscription-based mechanism based on Data Collection Reporting Initiation and Data Collection Reporting procedures is typically suitable when a logical node (in this case the gNB-DU) is a consumer of some information that it would like another logical node (the gNB-CU) to generate – and this is the case for CCO issue prediction – while the Configuration Update procedure is more suitable for scenarios where a local node has applied configuration changes and need to inform another logical node about these changes so that the latter can take proper actions. They also claim that a subscription-based solution has some advantages compared to using Configuration Update procedures, e.g., to enable the gNB-DU to inform the gNB-CU about which cells support alternative CCO states, hence avoiding potentially costly and energy consuming processing in the gNB-CU for cells for which corrective CCO actions would anyway not be available in the gNB-DU and would therefore be useless.

Actually, also in Rel-18 we have examples for which a prediction is provided without a preceding request from the receiving node, e.g., the cell-based UE trajectory: in this case, in fact, the AI/ML model in the source node predicts the UE trajectory across cells of the next-hop target gNB and provides this prediction to the target node as part of the Handover Preparation procedure, without the target node having previously requested for such prediction. It will be up to the target node to decide whether to use the UE trajectory prediction for subsequent mobility events. Moreover, it remains unclear to us whether there could be cases where cells managed by a gNB-DU for which alternative CCO states cannot be supported, and hence corrective CCO actions cannot be available. Finally, it should be also considered that AI/ML in general should be seen as a tool enhancing an existing mechanism and, for the specific case of CCO, we see no clear motivation to enable such tool by deviating from the legacy CCO mechanism.

Proposal 6: RAN3 to agree to turn the WA from RAN3#125bis into an agreement: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

If Proposal 6 is agreed, we need to discuss the enhancements to be introduced in gNB-CU Configuration Update and gNB-DU Configuration Update procedures to enable transferring of predicted CCO issue and future CCO state, respectively.

gNB-CU Configuration Update

In the existing GNB-CU CONFIGURATION UPDATE message (from gNB-CU to gNB-DU) we propose to introduce a new IE Predicted CCO Assistance Information (based on the legacy CCO Assistance Information IE) carrying information related to the predicted CCO issue and associated affected cells/SSBs.

Proposal 7: RAN3 to agree to introduce a new Predicted CCO Assistance Information IE within the gNB-CU CONFIGURATION UPDATE message.

The new Predicted CCO Assistance Information IE contains the Predicted CCO issue and Predicted affected Cells and SSBs IEs.

Proposal 8: RAN3 to agree to introduce Predicted CCO issue and Predicted affected Cells and SSBs IEs into the new Predicted CCO Assistance Information IE.

gNB-DU Configuration Update

In the existing GNB-DU CONFIGURATION UPDATE message (from gNB-DU to gNB-CU) we propose to introduce a new IE Future Coverage Modification Notification (based on the legacy Coverage Modification Notification IE) carrying information related to the list of cells/SSBs with the corresponding coverage configuration selected by the gNB-DU as part of the future coverage state and the coverage modification cause.

Proposal 9: RAN3 to agree to introduce a new Future Coverage Modification Notification IE within the gNB-CU CONFIGURATION UPDATE message.

The new Future Coverage Modification Notification IE is basically a Future Coverage Modification List IE and each Future Coverage Modification Item of this list includes the NR CGI, the corresponding Future Cell Coverage State and the Future SSB Coverage Modification List IEs.

Proposal 10: RAN3 to agree to introduce, within the Future Coverage Modification Item included in the Future Coverage Modification List IE, the NR CGI indicating the Cell ID of the cell whose coverage will be modified in the future, the corresponding Future Cell Coverage State and the Future SSB Coverage Modification List IEs.

The new Future SSB Coverage Modification List IE contains the Future SSB Coverage Modification Item IE which, in turn, includes the SSB Index and the corresponding Future SSB Coverage State IEs.

Proposal 11: RAN3 to agree to introduce, within the Future SSB Coverage Modification Item included in the Future SSB Coverage Modification List IE, the SSB Index and the corresponding Future SSB Coverage State IEs.

For each Future Coverage Modification Item included in the Future Coverage Modification List IE the associated coverage modification cause needs to be indicated. Therefore, the Future Coverage Modification List IE also includes the Future Coverage Modification Cause IE which, as per the agreement from RAN3#125bis, reflects the predicted CCO issue.

Proposal 12: RAN3 to agree that each Future Coverage Modification Item included in the Future Coverage Modification List IE has a Future Coverage Modification Cause IE associated to it.

Proposal 13: RAN3 to agree that the Future Coverage Modification Cause IE follows the same design of the existing Coverage Modification Cause IE, i.e., it is an ENUMERATED type, but with codepoints “coverage” and “cell edge capacity” only.

A TP for AI/ML BLCR to TS 38.473 for AI/ML-based CCO reflecting the above proposals on the F1AP Stage 3 signalling can be found in the Annex 2, where also timing information over F1 are included based on our proposals in [6].

# 3. Conclusion

Based on the above discussion, we have the following proposals:

XnAP Stage 3 signalling details

Proposal 1: RAN3 to agree to introduce a new Future Coverage Modification List IE within the NG-RAN NODE CONFIGURATION UPDATE message.

Proposal 2: RAN3 to agree to introduce, within the Future Coverage Modification Item included in the Future Coverage Modification List IE, the NR CGI indicating the Cell ID of the cell whose coverage will be modified in the future, the corresponding Future Cell Coverage State and the Future SSB Coverage Modification List IEs.

Proposal 3: RAN3 to agree to introduce, within the Future SSB Coverage Modification Item included in the Future SSB Coverage Modification List IE, the SSB Index and the corresponding Future SSB Coverage State IEs.

Proposal 4: RAN3 to agree that each Future Coverage Modification Item included in the Future Coverage Modification List IE has a Future Coverage Modification Cause IE associated to it.

Proposal 5: RAN3 to agree that the Future Coverage Modification Cause IE follows the same design of the existing Coverage Modification Cause IE, i.e., it is an ENUMERATED type, but with codepoints “coverage” and “cell edge capacity” only.

A TP for AI/ML BLCR to TS 38.423 for AI/ML-based CCO covering the above proposals can be found in Annex 1.

F1AP Stage 3 signalling details

Proposal 6: RAN3 to agree to turn the WA from RAN3#125bis into an agreement: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

Proposal 7: RAN3 to agree to introduce a new Predicted CCO Assistance Information IE within the gNB-CU CONFIGURATION UPDATE message.

Proposal 8: RAN3 to agree to introduce Predicted CCO issue and Predicted affected Cells and SSBs IEs into the new Predicted CCO Assistance Information IE.

Proposal 9: RAN3 to agree to introduce a new Future Coverage Modification Notification IE within the gNB-CU CONFIGURATION UPDATE message.

Proposal 10: RAN3 to agree to introduce, within the Future Coverage Modification Item included in the Future Coverage Modification List IE, the NR CGI indicating the Cell ID of the cell whose coverage will be modified in the future, the corresponding Future Cell Coverage State and the Future SSB Coverage Modification List IEs.

Proposal 11: RAN3 to agree to introduce, within the Future SSB Coverage Modification Item included in the Future SSB Coverage Modification List IE, the SSB Index and the corresponding Future SSB Coverage State IEs.

Proposal 12: RAN3 to agree that each Future Coverage Modification Item included in the Future Coverage Modification List IE has a Future Coverage Modification Cause IE associated to it.

Proposal 13: RAN3 to agree that the Future Coverage Modification Cause IE follows the same design of the existing Coverage Modification Cause IE, i.e., it is an ENUMERATED type, but with codepoints “coverage” and “cell edge capacity” only.

A TP for AI/ML BLCR to TS 38.473 for AI/ML-based CCO reflecting the above proposals on the F1AP Stage 3 signalling can be found in the Annex 2, where also timing information over F1 are included based on our proposals in [6].

# 4. Reference

- RP-242385, New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#105

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- R3-245558, Initial discussion on stage 3 impact for AI/ML-based Coverage and Capacity Optimization (CCO), Nokia, Jio, Deutsche Telekom

- R3-247415, Timing information and feedback for AI/ML-based Coverage and Capacity Optimization, Huawei

# Annex 1 - TP for AI/ML BLCR to TS 38.423

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.4.2	NG-RAN node Configuration Update

#### 8.4.2.1	General

The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE:	Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The NG-RAN node1 initiates the procedure by sending the NG-RAN NODE CONFIGURATION UPDATE message to a peer NG-RAN node2.

**** skip unchanged parts ****

Update of Cell Coverage:

If the Coverage Modification List IE is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information in the Cell Coverage State IE to identify the cell deployment configuration enabled by the NG-RAN node1 and for configuring the mobility towards the cell(s) indicated by the Global NG-RAN Cell Identity IE, as described in TS 38.300 [9].

-	If the Cell Deployment Status Indicator IE is present in the Coverage Modification List IE, the NG-RAN node2 shall consider the cell deployment configuration of the cell to be modified as the next planned configuration and shall remove any planned configuration stored for this cell.

-	If the Cell Deployment Status Indicator IE is present and the Cell Replacing Info IE contains non-empty cell list, the NG-RAN node2 may use this list to avoid connection or re-establishment failures during the reconfiguration, e.g. consider the cells in the list as possible alternative handover targets.

-	If the Cell Deployment Status Indicator IE is not present, the NG-RAN node2 shall consider the cell deployment configuration of cell to be modified as activated and replace any previous configuration for the cells indicated in the Coverage Modification List IE.

If the SSB Coverage Modification List IE is present in the Coverage Modification List IE, the NG-RAN node2 may use the information in the SSB Coverage State IE to identify the SSB beam deployment configuration enabled by the NG-RAN node1 and for configuring the mobility towards the beam(s) indicated by the SSB Index IE, as described in TS 38.300 [9].

If the Coverage Modification Cause IE set to "coverage" or "cell edge capacity" is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information for deducing the CCO issue detected at NG-RAN node1 and for configuring coverage state of its served cell(s).

If the Coverage Modification Cause IE set to "network energy saving" is present and the SSB Coverage State IE is zero for a set of SSB beams in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information to decide the SSB beam activation for the concerned beams when necessary.

Interactions with other procedures:

**** skip unchanged parts ****

If the NG-RAN node1 receives a NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message containing a Local NG-RAN Node Identifier within the Neighbour NG-RAN Node List IE identical to the Local NG-RAN Node Identifier included in the corresponding NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node1 may initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE message a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

Future update of Cell Coverage:

If the Future Coverage Modification List IE is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information in the Future Cell Coverage State IE to identify the cell deployment configuration that will be enabled by the NG-RAN node1 and for configuring the future mobility actions towards the cell(s) indicated by the NR CGI IE, as described in TS 38.300 [9].

If the Future SSB Coverage Modification List IE is present in the Future Coverage Modification List IE, the NG-RAN node2 may use the information in the Future SSB Coverage State IE to identify the SSB beam deployment configuration that will be enabled by the NG-RAN node1 and for configuring the future mobility actions towards the beam(s) indicated by the SSB Index IE, as described in TS 38.300 [9].

If the Future Coverage Modification Cause IE set to "coverage" or "cell edge capacity" is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information for deducing the CCO issue predicted at NG-RAN node1 and for configuring future coverage state of its served cell(s).

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| **** skip unchanged parts **** |  |  |  |  |  |  |
| Coverage Modification List |  | 0 .. 1 |  | List of cells with modified coverage. | GLOBAL | reject |
| >Coverage Modification Item |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  | Global Cell Identity 9.2.2.73 | Global Cell Identity of the cell to be modified. In this version of the specification, only a NG-RAN cell identifier can be included. | – |  |
| >>Cell Coverage State | M |  | INTEGER (0..63, …) | Value ‘0’ indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | – |  |
| >>Cell Deployment Status Indicator | O |  | ENUMERATED(pre-change-notification, ...) | Indicates the Cell Coverage State is planned to be used at the next reconfiguration. | – |  |
| >>Cell Replacing Info | C-ifCellDeploymentStatusIndicatorPresent |  |  |  | – |  |
| >>>Replacing Cells |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>>>Global NG-RAN Cell Identity |  |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of a cell that may replace all or part of the coverage of the cell to be modified. | – |  |
| >>SSB Coverage Modification List |  | 0..1 |  | List of SSB beams with modified coverage. | – |  |
| >>>SSB Coverage Modification Item |  | 0..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam to be modified. | – |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value ‘0’ indicates that the SSB beam is inactive. Other values Indicates that the SSB beam is active and also indicates the coverage configuration of the concerned SSB beam. | – |  |
| >>Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the reason for the coverage modification in NG-RAN node1. | YES | ignore |
| **** skip unchanged parts **** |  |  |  |  |  |  |
| Local NG-RAN Node Identifier Removal | O |  | Local NG-RAN Node Identifier 9.2.2.101 |  | YES | Ignore |
| Future Coverage Modification List |  | 0 .. 1 |  | List of cells with coverage to be modified. | GLOBAL | reject |
| >Future Coverage Modification Item |  | 1 .. < maxnoofCellsinNG-RAN node> |  |  | - |  |
| >>NR CGI | M |  | 9.2.2.7 |  | - |  |
| >>Future Cell Coverage State |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the coverage configuration of the concerned cell. | - |  |
| >>Future SSB Coverage Modification List |  | 0..1 |  | List of SSB beams with coverage to be modified. | - |  |
| >>>Future SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SS/PBCH block will be inactive. Other values indicate that the SS/PBCH block will be active and also indicate the coverage configuration of the concerned SS/PBCH block. | - |  |
| >>Future Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, …) |  | YES | Ignore |

<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex 2 - TP for AI/ML BLCR to TS 38.473

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

**** skip unchanged parts ****

If the BAP Address IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, store the received BAP address and use it as specified in TS 38.340 [30].

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a new cell and/or beam configuration.

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust coverage of its cells.If the CCO issue detection IE set to "network energy saving" is included in the CCO Assistance Information IE, the gNB-DU may consider the indicated SSB beams by the Affected Cells and Beam IE are deactivated due to network energy saving.

**** skip unchanged parts ****

If the Mobile IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows mobile IAB-node access.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Predicted Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a new future cell and/or beam configuration.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Predicted Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust coverage of its cells in the future.

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

The gNB-DU initiates the procedure by sending a GNB-DU CONFIGURATION UPDATE message to the gNB-CU including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU responds with GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

**** skip unchanged parts ****

If the Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization and network energy saving. If the Coverage Modification Cause IE is set to the "network energy saving", gNB-CU may consider those deactivated SSB beams are due to network energy saving.

**** skip unchanged parts ****

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Future Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for proactive Coverage and Capacity Optimization.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| **** skip unchanged parts **** |  |  |  |  |  |  |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| CCO Assistance Information | O |  | 9.3.1.211 | Indicates CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | ignore |
| **** skip unchanged parts **** |  |  |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Predicted CCO Assistance Information | O |  | 9.3.1.a | Indicates Predicted CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | Ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.a	Predicted CCO Assistance Information

This IE provides assistance information for the Capacity and Coverage (CCO) actions for specific predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO issue | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the type of predicted CCO issue. |
| Predicted Affected Cells and Beams | O |  | 9.3.1.212 |  |
| Time for Predicted CCO issue | O |  | FFS | It indicates the time point in the future at which the Predicted CCO issue is expected to occur. |
| Time For Future Coverage Modification | O |  | FFS | It indicates the time point in the future at which the Future Cell Coverage State of cells and beams not served by the gNB-DU is activated. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| **** skip unchanged parts **** |  |  |  |  |  |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Coverage Modification Notification | O |  | 9.3.1.213 |  | YES | ignore |
| **** skip unchanged parts **** |  |  |  |  |  |  |
| Mobile IAB-MT User Location Information | O |  | 9.3.1.307 |  | YES | ignore |
| Future Coverage Modification Notification | O |  | 9.3.1.b |  | YES | Ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.b	Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding coverage configuration selected by the gNB-DU as part of the future coverage state.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Future Coverage Modification List |  | 1 |  |  | - |  |
| >Future Coverage Modification Item |  | 1 .. <maxCellingNBDU> |  |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Future Cell Coverage State |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the coverage configuration of the concerned cell. | - |  |
| >>Future SSB Coverage Modification List |  | 0..1 |  |  | - |  |
| >>>Future SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SS/PBCH block will be inactive. Other values indicate that the SS/PBCH block will be active and also indicate the coverage configuration of the concerned SS/PBCH block. | - |  |
| >>Future Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, …) |  | YES | Ignore |
| >>Time For Future Coverage Modification | O |  | FFS | It indicates the time point in the future at which the Future Cell Coverage State is activated. | YES | Ignore |

<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>