3GPP TSG-RAN WG3 Meeting #126	R3-247420

Orlando, US, 16th – 22nd November, 2024

Title:	Discussion on the LS reply from SA2 on AI/ML Data Collection

Agenda Item:	20.1

Source:	Huawei

Document for:	Discussion

# 1	Introduction

In SA2#165 meeting (October 2024) SA2 approved the LSout in [1] to RAN2 and RAN3, which is a reply to the LS sent by RAN in [2] to SA, SA2, SA3, and SA5 (Cc RAN1, RAN2 and RAN3) on the topic of UE side data collection for UE-side model training currently under discussion in RAN2 as part of the Rel-19 AI/ML for air interface WI – note that the LS from RAN in [2] has been Noted in the last RAN3 meeting.

In the LS [2], RAN summarized the solutions identified and analysed in RAN2 for UE side data collection for UE-side model training and, in order for RAN to make a decision on how to progress on this topic in RAN#106 meeting (December 2024), input from SA and SA WGs is needed. The standardized solutions for data collection (if standardized) need to meet the requirements identified by RAN and listed in [2] as well.

In [1] SA2 provided an interim response and feedback on the solutions identified by RAN2, due to the fact that some aspects of such solutions and terminology being used to describe them require further clarifications from RAN2.

Among these aspects, SA2 mentions the UE data collection controllability, for which SA2 wonders whether there is the need to involve NG-RAN, and hence whether there could be RAN3 impacts.

This contribution discusses the aspect raised by SA2 on the UE data collection controllability from a RAN3 perspective, also in light of the e-mail discussion [POST127bis][020][AI PHY] Reply LS to SA2/SA5 (InterDigital/Nokia) in RAN2 collecting companies’ views on the questions in reply LS from SA2 [1] (and from SA5 in [3] on the same topic).

# 2	Discussion

The LS from RAN in [2] is based on a TP to TR 38.843 agreed in RAN2 (and included in the LS as an Annex) showing the options allowing for UE side data collection for UE-side model training:

The following options were discussed in RAN2: 1a. UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected. 1b. UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). 2. UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. 3. UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server. The options listed above were analysed to understand potential specification impact. The analysis included aspects such as termination entities, data transfer path, whether control plane (CP) or user plane (UP) should be used to transfer the data, protocol layers involved, MNO controllability and visibility over the collected data, etc. The result of this analysis can be found below in Table 7.2.1.3.2-1.

In the ACTION of the LS from RAN in [2], RAN asks SA2 to provide inputs on solutions 1b, 2 and 3 by RAN#106 (December 2024) so that RAN can make a decision on how to progress on the topic of UE side data collection.

The Table in the agreed TP to TR 38.843 capturing the RAN2 analysis of the above options (reported below for convenience, and included in [2] as well) lists the Controllability of MNO on data transfer as one of the aspects that RAN2 considered in its analysis of the different options for UE data collection and for which SA2 expects some NG-RAN involvement (and hence RAN3 impacts) as expressed in question Q1 in [1]:

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |

Note 1 in the above Table provides an explanation of what full controllability is: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. While Note 5, instead, indicates that RAN2 cannot reach consensus on the level of MNO controllability possible via solution 1b without input from SA groups.

The same concept as in Note 1 above of MNO full controllability of the data collection transfer process has been included by RAN in the list of requirements that a standardized solution for UE data collection shall fulfil [2]:

RAN has agreed to the following requirements for data collection for UE sided model training for standardized solution (if standardized) (i.e. Option 1b, 2, 3). Option 1a is not precluded. - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is futureproof and extendable. FFS/study if and how to handle non-standardized data (i.e. partial visibility). FFS controllability on data collection Standardized Solutions should follow the principle of aiming to minimize air interface overhead and impact to NW operation

In the reply LS in [1] SA2 lists a set of questions for clarification in the area of controllability of MNO of data transfer (among other areas) and, in the Actions of [1], SA2 requires feedback from RAN3 on one of these questions, i.e., question Q1:

**** Skip non-relevant text **** SA2 requires further clarification on some essential terminology to enable SA2 to assess the feasibility of any of the options. SA2 would like to ask about the following terms: - Controllability of MNO on data transfer: In RP-242389 / S2-2409600, it is stated that full controllability is required when “the MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer” SA2 kindly request RAN2 to provide further clarification on the following areas: Q1: Are there any aspects of the UE-data collection controllability, that required NG-RAN involvement? If so, what is the involvement of NG-RAN in UE-data collection controllability, e.g., what aspects of MNO controllability would require NG-RAN involvement and what would such involvement be? As an example of the kind of feedback that is requested, some companies in SA2 understand that initiating (e.g., triggering), terminating collection of UE-side data and controlling data transfer may require NG-RAN involvement, and it is currently not clear what this involvement may be. **** Skip non-relevant text **** 2. Actions: **** Skip non-relevant text **** To RAN3 ACTION: SA2 kindly ask RAN3 to consider question #1 above and provide as early as possible any feedback.

At the time of drafting this contribution paper, RAN2 has an ongoing post-meeting e-mail discussion [4] to collect companies’ views in RAN2 on the questions raised by SA2 and SA5 in their respective reply LSs.

Concerning Q1 in SA2’s reply LS [1], we think that the following aspects should be considered:

- Initiation (e.g. triggering) of UE side data collection;

- Termination of UE side data collection;

- Provisioning the UE with measurement configuration associated to the data collection;

- Control of the data transfer process.

For all options, a. and b. may or may not need NG-RAN involvement: if OTT server/CN/OAM can initiate/terminate the data collection process, then there is no NG-RAN involvement, otherwise it may involve NG-RAN.

For all options, c. may need NG-RAN involvement, and this should be further discussed in RAN2.

The above statements on a. and c. reflect the agreement on data collection for model training from the last RAN2 meeting (RAN2#127bis) reported below:

Data collection initiation and configuration for data collection is under network control. FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

From the above RAN2 agreement, in fact, it is not clear whether “network control” means NG-RAN, as it could also be e.g. CN.

For d., it depends on the data transfer option (and on the protocol layer for data transfer) being considered:

- Option 1a: NG-RAN involvement is not needed;

- Option 1b: NG-RAN involvement is not needed;

- Option 2-CP: NG-RAN involvement may not be needed as NAS signalling should be transparent to NG-RAN. However, whether NG-RAN involvement is needed or not has not been discussed in RAN2;

- Option 2-UP: NG-RAN involvement may not be needed (similar analysis as for Option 2-CP);

- Option 3-CP: NG-RAN may be involved;

- Option 3-UP: whether NG-RAN involvement is needed depends on feasibility analysis by SA5.

Observation 1: NG-RAN involvement in the MNO controllability of UE side data collection for UE-side model training is still under discussion in RAN2 and no clear conclusion has been reached yet.

Moreover, still concerning aspect d., we also see some relation with question Q2 in the SA2 reply LS in [1], which asks where (which entities) and under what conditions should controllability be performed:

**** Skip non-relevant text **** - Controllability of MNO on data transfer: In RP-242389 / S2-2409600, it is stated that full controllability is required when “the MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer” SA2 kindly request RAN2 to provide further clarification on the following areas: **** Skip non-relevant text **** Q2: Furthermore, with regards to “initiating, terminating and fully managing data transfer” some companies in SA2 believe that further clarification is required, on a per use case basis, on where (which entities) and under what conditions, should controllability be performed, e.g., in NG-RAN, a NF, OAM, an MNO controlled AF, a 3rd party AF, a UE)?

For the under what conditions part of Q2, our understanding is that RAN2 has not discussed it yet and, as a consequence, RAN2 has not identified conditions. While concerning where (which entities) should perform controllability, it is still our understanding that Table 7.2.1.3.2-1 in the LS from RAN [2] should be used as a starting point, and more RAN2 discussions are needed considering, e.g., use cases, data transfer options and other aspects (e.g. whether/how to minimize Uu impacts).

Observation 2: NG-RAN involvement in the MNO controllability of UE side data collection for UE-side model training also depends on the outcome of the discussion in RAN2 concerning where (which entities) and under what conditions should controllability be performed.

Based on the above, we think that in general the NG-RAN involvement should be discussed further in RAN2 before starting the discussion in RAN3 and provide feedback to SA2. As a consequence, in our view it should be RAN2 first to identify the need of NG-RAN involvement, and hence officially ask RAN3 to work on such involvement based on clear requirements.

Proposal: RAN3 to wait for further RAN2 progress on the need to involve NG-RAN in the MNO controllability of UE side data collection for UE-side model training before replying to SA2. The discussion in RAN3 should be triggered only after RAN2 has identified clear requirements on the NG-RAN involvement.

# 3	Conclusion

This document proposes the following observation and proposals:

Observation 1: NG-RAN involvement in the MNO controllability of UE side data collection for UE-side model training is still under discussion in RAN2 and no clear conclusion has been reached yet.

Observation 2: NG-RAN involvement in the MNO controllability of UE side data collection for UE-side model training also depends on the outcome of the discussion in RAN2 concerning where (which entities) and under what conditions should controllability be performed.

Proposal: RAN3 to wait for further RAN2 progress on the need to involve NG-RAN in the MNO controllability of UE side data collection for UE-side model training before replying to SA2. The discussion in RAN3 should be triggered only after RAN2 has identified clear requirements on the NG-RAN involvement.

# 4	References

- R3-247017 (S2-2411191), Reply LS on AIML data collection, SA2

- R3-245014 (RP-242389), LS on AIML data collection, RAN

- R3-247116 (S5-246299), Reply LS on AIML Data Collection, SA5

- [POST127bis][020][AI PHY] Reply LS to SA2/SA5 (InterDigital/Nokia), RAN2